3GPP TSG RAN Meeting #99									RP-230497

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item:			9.3.4.4

Title		:	Status report for WI NR_FR2_multiRx_DL; rapporteur: Qualcomm, vivo

| WI / SI Name | Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR2_multiRX_DL |  |  |  |  |
| Unique ID | 950077 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223527 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: - | Core part: 09/2023 | Performance part: 03/2024 | Testing part: - |  |
| Overall Completion level | Study Item: - | Core part: 35% | Performance Part: 10% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Sumant Iyer |
|  | Company | Qualcomm Inc. |
|  | Email | sumanti@qti.qualcomm.com |
| Rapporteur | Name | Qian Yang |
|  | Company | vivo |
|  | Email | qian9.yang@vivo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### Void

## 2.2	RAN2

#### Void

## 2.3	RAN3

#### Void

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.1.1	UE RF Core

##### 2.4.1.1.1	From RAN4#104-e: WF R4-2214457

- To support 4L DL MIMO reception at the UE when configured with 2 active TCI states, polarization multiplex (2 layers/direction) + spatial multiplex (2 directions) is assumed at the UE.

  - Note: This proposal is for general deployment assumption, not aimed at UE RF assumption

- UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions.

- For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline.

- Work plan:

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#104 | Aug. 2022 | Discussion on system assumptions, UE assumptions | Agree on system assumptions |
| RAN4#104-Bis | Oct. 2022 | UE implementation assumptions and requirement concept |  |
| RAN4#105 | Nov. 2022 | UE implementation assumptions and requirement concept | Agree on UE assumptions |
| RAN4#106 | Feb. 2023 | Requirement concept and simulation discussion | Agree on details pertaining to simulation effort |
| RAN4#106-Bis | Apr. 2023 | Simulation results discussion | Draft CR? |
| RAN4#107 | May 2023 | Requirement parameter details | Draft CR |
| RAN4#108 | Aug. 2023 | Requirement parameter details | Feature CR |

##### 2.4.1.1.2	From RAN4#104-Bis-e:

###### 2.4.1.1.2.1	WF R4-2217731

- ‘Panel’ is not referenced in the final UE RF requirements

- Take the following panel assumption for DL reception, noting panel is a logical construct:

- ‘Panel’ is defined as one or multiple as combination of below depending on different UE implementation:

  - Unit of antenna group to control beam independently

  - Within a panel, one beam can be selected and used for DL reception.

  - Across different panels, multiple beams [(each selected per panel)] may be used for DL reception.

  - ‘Beam’ is assumed to mean spatial filter associated with reception.

- [The scenario where a single antenna module is used to receive two AoAs simultaneously should not be excluded. Whether a UE with single antenna module can satisfy the requirement or not will be an implementation issue].

- [If an antenna module can be used to receive two AoAs simultaneously, it is considered to consist of at least two panels, where the understanding of “panel” is based on 1.2 ]

- Further study the power imbalance together with how to specify 2AoA sensitivity

- For single DCI, take the RMC in Annex 1 as starting point for further discussion. (see WF)

###### 2.4.1.1.2.2	WF R4-2217732

- On UE capabilities for the feature ”simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO

  - For UEs supporting single DCI, assume the following set of UE capabilities as pre-requisites:

    - Support of simultaneousReceptionDiffTypeD-r16

    - Support of singleDCI-SDM-scheme-r16

    - Support of 4L DL MIMO

  - For UEs supporting multi-DCIs, assume the following set of UE capabilities as pre-requisites:

    - Support of simultaneousReceptionDiffTypeD-r16

    - Support of multiDCI-MultiTRP-r16

    - Support of 4L DL MIMO

  - Further consider the following options:

    - Option 1: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline, if the UE supports single DCI scheme. If the UE only support multi-DCI scheme, multi-DCI is used.

    - Option 2: UE RF requirements shall not preclude UEs that do not support the single DCI scheme.

- For core requirement and testing, it is assumed that both SSB and CSI-RS are configured to UE for beam management

- UE panel assumption should follow implementation agonistic manner.

  - The term “panel” is not used in test configuration.

- Requirement discussions need to consider testability issue so that the defined requirement can be properly verified.

##### 2.4.1.1.3	From RAN4#105: WF R4-2220553

- ‘The UE RF requirement is based on a test system that can support multiple fixed relative AoA locations during test.’ is the starting point:

  - Multiple fixed orientation of the AoAs or single fixed orientation of AoA can be considered for test

  - Multiple fixed AoA offset values or single fixed AoA offset value can be considered for core requirement

- Candidate proposals for UE RF requirement concept (others not precluded):

  - Option 1: Requirement is based on 2AoA directional sensitivity statistics

  - Option 2: Requirement is based on the spherical coverage EIS of AoA1 (which is swept over the full sphere), and a fixed/pre-defined power level for AoA2

  - Option 3: Only verify the UE functionality (e.g., go or no-go) under two AoAs with a fixed DL power level

  - Option 4: RAN4 considers specifying the demod requirements of multi-Rx in FR2 with pre-defined side condition, instead of defining the two-AoA spherical coverage requirement

- No more than 2AoA cases are considered in this release as specified in the WID.

- DL polarizations:

  - Option 1: Multiple aspects may be chosen:

    - Aspect 1: The RF requirement can be defined for any AoA pair with assumption that TRP1 uses  polarization when TRP2 uses  polarization and vice-versa without any limitation of angular separation between two AOAs. (R4-2219125)

    - Aspect 2: Limit the polarization combinations (from 4 possible:   and )) for the 2-DL spherical coverage test case (R4-2219852)

  - Option 2: Take the mTRP system level assumptions in TR38.802 as the baseline and further discuss whether any additional constraints on DL polarization are necessary for the RF requirement definition.

  - Other options are not precluded.

- Strive to define single set of requirements for both sDCI and mDCI.

- The terms ‘antenna module’ and “panel” are not referenced in the final UE RF requirement and test configuration

- The scenario where a single antenna module is used to receive two AoAs simultaneously should not be excluded. If an antenna module can be used to receive two AoAs simultaneously, it is considered to consist of at least two panels, where the understanding of “panel” is based on Proposal 1 of 1.2.11

- On “panel”,

  - ‘Panel’ is defined as a group of antenna element that controls beam independently and has the following attributes

    - Within a panel, one beam can be selected and used for DL reception.

    - Across different panels, multiple beams (each selected per panel) may be used for DL reception.

    - ‘Beam’ is assumed to mean spatial filter associated with reception.

  - Confirm that a physical panel with dual polarization is assumed as two “panels”.

  - [Assuming four antenna panels per UE as the reference architecture for deriving the spherical coverage requirement of the spherical coverage requirement for the multi-Rx chain DL reception. The four panels are separated into two pairs, and each pair is composed of two antenna panels with orthogonal polarizations that are co-located.

    - Antenna panels used to receive two AoAs simultaneously is up to UE implementation ]

- For the UE supporting multi-DCI, the RMC of the single carrier can be reused for each layer

- test time optimization can be further discussed and decided in OTA SI

- Test procedures for sDCI and mDCI can be further discussed and decided in OTA SI once the requirement concept and test methodologies are confirmed. Test procedure discussion should be also aligned with requirement discussion and decision including requirement concept

- Consider AoA setup and deployment scenario as a package.

  - the mTRP assumptions achieved by RAN1 in the physical layer design (including TR38.802) can be taken as one of the references in deployment scenario discussion. Interested companies are encouraged to provide their analysis on deployment scenario and associated AoA setup in next meeting.

- The same numerology (SCS and CP) is assumed for the two AoAs in Rel-18.

##### 2.4.1.1.4	From RAN4#106: WF R4-2303708

- 2TRP grid for UE performance evaluation:

  - The 2TRP data set is only complete when each TRP traverses the entire surface of the test sphere.

  - For optimized AoA1 and AoA2 test point/perceived DL direction coverage, apply a full rotation in  and a half rotation in . For optimized AoA1 and AoA2 test point/perceived DL direction coverage, utilize constant-step size grids only.

  - In the coordination system of z-axis pointing to AoA1 (P0), the two AoAs (probes) shall be located in xz plane.

  - Testing both +AoA offset and –AoA offset for each test point shall be the starting point for simulation. The intention is to accommodate the impact from directionality of the AoA1-AoA2 DL orientation vectors. In other words we want to ensure no testing bias is introduced.

- Requirement Concept for UE RF (baseline): Only verify the UE functionality (e.g., go or no-go) under two AoAs with a fixed DL power level. In other words, the UE can achieve EIS performance not worse than YdBm on the test point pair (corresponding to 2 AoAs) and the ratio of qualified test points over the whole sphere is M%.

- UE orientation during verification or simulation: UE orientation w.r.t P0 position (z-axis) is part of core requirement consideration.

- DL pol. assumption for derivation of the UE RF requirement: The UE RF requirement is derived assuming the worst case polarization match between the 2 TRPs. The requirement applies for any combination of DL polarizations from each TRP.

- AoA separation for UE RF requirement:

  - AoA offset value should be an integer multiple of the step size of the constant step size measurement test grid.

  - In the simulation, all AoA separation values in the list {30°, 60°, 90°, 120°, 150°, 180°} shall be simulated.

- Further details for baseline RF requirement: Simulation assumption starting point: Re-use the legacy spherical coverage receiver sensitivity level (single probe) as the DL power for each probe to set the core requirement.

- sDCI vs mDCI:

  - Define the RF requirement based on mDCI with understanding that UE supporting sDCI can also meet the requirement.

  - To have the unified requirement concept for UEs supporting multi-DCIs as for UEs supporting single DCI, RAN4 can focus on fully overlapping in time and in frequency, supported by UE capabilities “multiDCI-MultiTRP-r16” and “overlapPDSCHsFullyFreqTime-r16.”

  - No “joint detect/decode” is considered in simulation for mDCI. In other words, Process only 2 TRP-RX pairs– In this case we only consider two TRP to RX pairs - TRP1-RX1 and TRP2-RX2. The signal from TRP2 to RX1 and TRP1 to RX2 is treated as interference.

- UE simulation assumptions:

  - Simulation output: Statistics per TRP (further combining not precluded from final requirement)

    - Y% @legacy EIS spherical coverage value in dBm as baseline

    - X dBm for legacy coverage fraction (example PC1 = 15%, PC3=50%) in 2TRP operation is encouraged based on contribution driven for the sake of comparison

    - For mDCI, if any one of the two test directions in a pair fails, the result is a fail for the AoA pair.

  - antenna module assumptions.:

    - (2 dual-pol antenna modules, 4x1 elements, combination of front/back, left/right, top/bottom)

    - company choice on bias, antenna module location, module count, antenna gain, but legacy spherical coverage requirement must be met.

- UE RF simulation calibration step: calibrate the simulation baseline with legacy peak EIS spec and legacy spherical EIS spec.

#### 2.4.1.2	UE RRM Core

##### 2.4.1.2.1	From RAN4#104-e: WF R4-2214344

Number of searchers for cell detection and measurements

  - 2 searchers are assumed.

Feasibility/necessity of enhancing requirements for L1 measurements

  - Following L1 measurements related requirements are considered in the multi-Rx chain WI

    - L1-RSRP measurement

    - BFD/CBD

    - RLM

    - L1-SINR

    - TCI state switching with dual TCI

    - Scheduling/measurement restrictions

- RRM  core part work plan

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#104 | Aug. 2022 | Study and initial discussion on the feasibility/necessity of specifying RRM core requirements for the following procedures/measurements, and corresponding core requirements - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) | Conclusion on the study/feasibility/necessity of at least some of the requirements Approve the RRM work plan |
| RAN4#104-Bis | Oct. 2022 | Continue discussion on the feasibility/necessity of specifying RRM core requirements for the following procedures/measurements, and corresponding core requirements - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) | Conclusion on the study/feasibility/necessity of all the requirements Agreements on assumptions of receive timing difference between different directions Initial agreements on the confirmed RRM requirements |
| RAN4#105 | Nov. 2022 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements |
| RAN4#106 | Feb. 2023 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements |
| RAN4#106-Bis | Apr. 2023 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements Work split on the Draft CRs |
| RAN4#107 | May 2023 | Continue discussion on the remaining issues for the RRM core requirements | Initial discussion on the Draft CRs Further agreements on the confirmed RRM requirements |
| RAN4#108 | Aug. 2023 | Continue discussion on the remaining issues for the RRM core requirements | Conclude all the RRM requirements Endorse all the Draft CRs for core part of the WI |

##### 2.4.1.2.2	From RAN4#104-Bis-e:

###### 2.4.1.2.2.1	WF R4-2217246

- Scope and scenarios:

  - Joint combination of multi-Rx chain with NR-U, RedCap and NTN are not in the scope of the WI.

  - RRM requirement discussion shall be focused on the case with different QCL TypeD RSs on a single component carrier.

  - Clarify the SSB and CSI-RS based simultaneous L1 measurement scenario before making conclusion on scenarios for Rel-18 multi-Rx DL reception, i.e., for simultaneous reception of different QCL type D RSs, what RSs combination(s) can be considered and which scenario(s) it is aimed at.

- General aspects:

  - UE behaviour and capability of handling Rx signal level difference between two channels may be discussed in RF/demodulation session.

  - Enhanced RRM requirements for multi-Rx chain UE should maintain the existing accuracy requirements.

  - Whether to define RRM requirements for activation delay from a single antenna panel to multi-antenna panels should be decided after RF conclusion.

- Receive timing difference:

  - Not to discuss UL transmission in R18 FR2 multi-Rx WI.

- Applicability and conditions:

  - RRM requirement of simultaneous DL reception from different directions shall be defined based on the applicable condition to be specified in UE RF session.

- To clarify the requirement in new section that R18 multi-Rx is applicable for FR2-1 only, i.e., no FR2-2.

###### 2.4.1.2.2.2	WF R4-2217587

- L1 RSRP/SINR Measurements: The number of samples assumed to define the accuracy requirements should not be changed

- BFD/BFR: CSI-RS based BFD/BFR is in the scope of the current WI

- Multi-Rx and other features: Do not discuss join requirements for multi-Rx and NR-U, NTN and RedCap at least until all the requirements for multi-Rx are finalized

###### 2.4.1.2.2.3	WF R4-2217248

- Requirements and Scenarios: Define DL TCI state switch requirements. UL TCI state switching, and UL spatial relation info switch are not in the scope of the WI.

- Scenarios to be considered w.r.t Intra-cell/Inter-cell multi-TRP: Follow conclusion from thread 211

- Scenarios to be considered w.r.t single DCI/multi-DCI: Follow conclusion from thread 211.

- TCI Switching:

  - Dual TCI state switching requirements shall be based on Rel-15/Rel-16 TCI framework

  - Can the TCI switch is assumed to be independent on each RX chain?: For each of the two TCI states, the TCI state switch is assumed to be independent

  - Switch command for dual TCI state switch

  - When two TCI states are switched simultaneously, assumption on the switch commands: Wait for conclusion on scenarios in other thread w.r.t sDCI vs mDCI.

  - TCI state switch scenarios to be considered: Wait for conclusion on scenarios in other thread w.r.t sDCI vs mDCI.

  - Does the cross-panel switch time needs to be defined: RRM not to define additional TCI state switching delay for cross panel TCI state switching. If RF session achieves a new conclusion on panels ON/OFF switch time, RRM session may revisit the issue if required

##### 2.4.1.2.3	From RAN4#105:

###### 2.4.1.2.3.2	WF R4-2220742

- Assumption on supporting 4-layer MIMO with simultaneous DL reception with two different QCL TypeD RSs

  - group based reporting is one of the means to enable simultaneous reception

  - companies proposing to enable simultaneous reception should bring more analysis on how this can be done

- Consider both sDCI and mDCI

- L3 measurements in RRC Connected mode requirements enhancements should be studied and specified if necessary

  - Note: other enhancements are deprioritized

###### WF R4-2220743

- Overall handling of L1 measurements: Discussion on Rx beam sweeping scaling factor, simultaneous reception of DL data and RS, and other multi-Rx related topics can be done jointly for L1-RSRP, RLM, L1-SINR and CBD/BFD.

  - Note: issues specific to each of the above measurements can be discussed separately.

###### 2.4.1.2.3.3	WF R4-2220402

- Regarding group-based reporting and simultaneous reception with different QCL type-D: group based reporting is one of the means to enable simultaneous reception

- As per RAN1 agreements, group-based reporting is supported for single DCI only. If group-based reporting is a prerequisite, this AI can focus on single DCI case only.

- Definition of Dual TCI state switch

  - Dual TCI state switching refers to the case when two active TCI states with QCL type D for simultaneous reception of PDCCH/PDSCH are involved before and/or after TCI switching command.

  - In other words, following are considered as dual TCI state switching to discuss the requirements

    - Single DCI to Dual TCI

    - Dual TCI to Dual TCI

    - FFS: Dual TCI to Single TCI

- TCI state switch indication method for PDCCH and PDSCH

  - PDCCH non-SFN

    - Only single PDCCH is supported. So Single MAC CE should be used

  - FFS: PDCCH SFN

    - One MAC CE is used to indicate both TCI states

  - PDSCH

    - For both SFN and non-SFN, only one DCI code point is indicated using single DCI and the code point may indicate one or two TCI states

- Known and unknown states combination: Dual TCI state switching requirements are defined for simultaneous reception. It is FFS whether we need to define any new requirements for non-simultaneous reception. The dual TCI states known, or unknown are FFS.

##### 2.4.1.2.4	From RAN4#106:

###### 2.4.1.2.3.1	WF R4-2303302

- Scope of the WI

  - No further discussion is needed for Option 1.

  - Discuss the requirements for the objectives in the WID directly.

- m-TRP scenarios for Rel-18 multi-Rx DL reception

  - Focus on intra-cell multi-TRP operation scenario

  - No requirements will be introduced for inter-cell multi-TRP operation in R18.

- Necessity of group-based beam reporting for simultaneous reception

  - Rel-17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception

  - Note: Simultaneous reception term above includes Data/Data, RS/RS and Data/RS simultaneous reception cases.

- Simultaneous L3 measurements and L1 measurements

  - Simultaneous L3 and L1 measurements is not supported in this release for multi-Rx.

  - Note: The agreement has no impact on scheduling restriction discussion for L3 measurements. Feasibility and benefit of scheduling restriction relaxation for L3 measurements is FFS.

- Baseline L1 measurement requirements for potential multi-Rx enhancement

  - For cell specific RLM: section 8.1

  - For cell specific link recovery: section 8.5

  - For TRP specific link recovery: section 8.18

  - For L1-RSRP measurement: section 9.5.

###### 2.4.1.2.3.2	WF R4-2220743

- TCI State Switching

  - Group Based Reporting and s/mDCI

    - Group based reporting applies to both sDCI and mDCI

  - Investigate and if needed specify requirements for the following TCI state switching scenarios

    - single TCI to dual TCI

    - dual TCI to dual TCI

    - dual TCI to single TCI

  - TCI Switch triggering method:

    - MAC-CE based dual TCI state switch for PDCCH reception

    - DCI based dual TCI state switch for PDCSH reception

- MRTD Handling:

  - Define RRM requirements for MRTD < CP

  - Note: the MRTD above is the maximum receive time difference between the signals coming from two directions on the same carrier

#### 2.4.1.3	UE Performance

##### 2.4.1.3.1	From RAN4#104-e: WF R4-2214344

- UE RRM performance part work plan

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#106 | Feb. 2023 | Initial discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Initial discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused |  |
| RAN4#106-Bis | Apr. 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused Initial discussion on list of test cases |  |
| RAN4#107 | May 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused | Initial agreement on list of test cases |
| RAN4#108 | Aug. 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused | Work split on the Conclude the list of test cases Work split on the Draft CRs for performance requirements and test cases |
| RAN4#108-Bis | Oct. 2023 | Continue discussion on the remaining issues for performance requirements and test cases Initial discussion on the Draft CRs for performance requirements and test cases |  |
| RAN4#109 | Nov. 2023 | Continue discussion on the remaining issues for performance requirements and test cases Continue discussion on the Draft CRs for performance requirements and test cases |  |
| RAN4#110 | Feb. 2024 | Continue discussion on the remaining issues for performance requirements and test cases | Conclude all the performance requirements and test cases Endorse all the Draft CRs for the performance part of the WI |

##### 2.4.1.3.2	From RAN4#106: WF R4-2302941

- General Aspects:

  - Use the existing agreement from RF in [R4-2220533] for definition of the terms of antenna module and antenna panel.

  - Rel-18 FR2 Multi-RX chain UE WI, Focus on multi-TRP transmission schemes with intra-cell mTRP scenarios only.

  - Whether to introduce a new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements: For initial simulation purpose, MIMO correlation matrix approach considered as starting point.

  - Whether to introduce requirements for both single-DCI and multi-DCI scenarios: For initial evaluation purpose, below transmission schemes with layer combinations considered

    - Single DCI with SDM scheme: 1+1 and 2+2 as layer combination

    - Multi-DCI with fully overlapping scheme: 1+1 and 2+2(high priority) as layer combination

    - Multi-DCI with non-overlapping scheme: 2+2 as layer combination

    - Receiver assumption: MMSE-IRC

      - Option 1: UE joint processing with 4x4

      - Option 2: UE processing with 2x2 per TRP

  - No PDCCH requirements will be introduced for Rel-18 FR2 Multi-Rx chain UE WI.

  - No PBCH performance requirements will be introduced.

  - No SDR performance requirements will be introduced.

  - Whether to introduce performance requirements for CSI reporting: Introduce PMI reporting requirements for single-DCI transmission scheme.

  - Whether to introduce performance requirements for CA: Prioritize single carrier performance requirements.

  - PN model and PN compensation (Tentative Agreement): For evaluation, consider PN impact for the decision of MCS, based on expected PN degradation of [X] dB; Interested companies can bring performance simulations with and without PN modelling in the next meeting.

    - Consider Example 2 with priority;

    - Consider PN Models for FC = [30, 47] GHz;

- Test scope for PDSCH demodulation requirements

  - Channel model (Tentative agreement): Include TDLA30-75. Other channel models are not precluded.

  - SCS/Bandwidth (Tentative agreement):

    - Encourage interested companies to provide simulation results with 120kHz/100 MHz with priority;

    - Interested companies are welcome to provide simulations with 120kHz/200MHz, to investigate SNR limitations for this BW based on evaluation results;

#### 2.4.2	Remaining Open issues

#### 2.4.2.1	Objective of Core part WI

Introduce necessary requirement(s) for enhanced FR2 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO

  - Enhanced RF requirements:

    - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs

    - The legacy spherical coverage requirement for reception from a single direction will be kept

    - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized

  - Enhanced RRM requirements:

    - The following requirements should be studied and specified if necessary:

      - L1-RSRP measurement delay

      - L3 measurement delay (both cell detection delay and measurement period can be considered)

        - The starting point is the enhancements related to L1-RSRP measurement enhancements

      - RLM and BFD/CBD requirements

      - Scheduling/measurement restrictions

      - TCI state switching delay with dual TCI

      - Receive timing difference between different directions (different QCL Type D RSs)

NOTE:

- The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited.

#### 2.4.2.2	Objective of Performance part WI

- Introduce necessary requirement(s) for enhanced FR2 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier

  - RRM Performance requirements and test cases corresponding to the new core requirements

  - RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused

  - UE demodulation requirements:

    - Specify performance requirements for enhanced FR2 UEs supporting up to 4 DL MIMO layers with dual TCI with different QCL typeD on a single component carrier.

      - Demod requirements on up to 4 DL MIMO layers

      - CSI requirements to support up to 4-layer with 2 TCIs and Rel-17 mTRP Type I codebook

    - Extension of requirements to cover intra-band CA can be considered after the single carrier requirements are defined

NOTE:

− For the dual-TCI demodulation requirement case, focus on UEs supporting “simultaneousReceptionDiffTypeD-r16” UE capability, and for the 2- or 4-layer downlink MIMO reception, focus on UEs supporting the basic mTRP CSI reporting capability (FG 23-7-1/1b of NR FeMIMO).

## 2.5	RAN5

#### Void

## 2.6	RAN6

#### Void

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A.

## 4.	References

## 4.1	Discussion Tdocs from RAN4#104-e:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2211642 | Discussion on Rel-18 multi Rx pannel RRM | CATT |
| R4-2211768 | Views on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2211814 | Proposal on spherical coverage requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2211815 | Enhanced spherical coverage requirements for multi-rx chains UEs | Nokia, Nokia Shanghai Bell |
| R4-2211882 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2211883 | RRM requirements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2211940 | Discussion on RRM requirements for simultaneous DL reception from different directions | CMCC |
| R4-2211978 | On multi-RX chain | Xiaomi |
| R4-2211993 | Requirements for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2212062 | Initial discussion on RRM requirements for simultaneous DL reception from different directions | OPPO |
| R4-2212180 | Impacts on RRM to support FR2 multi-Rx chain DL reception from multi-TRP | Qualcomm Incorporated |
| R4-2212185 | Discussion on UE RF requirements for simultaneous DL reception with up to 4 layer MIMO | LG Electronics |
| R4-2212219 | Discussion on RRM for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2212332 | System assumptions for multi-Rx chain DL reception | Qualcomm Incorporated |
| R4-2212333 | Work plan for FR2 UE multi-Rx chain DL reception | Qualcomm Incorporated, vivo |
| R4-2212466 | Discussion on RRM requirements for simultaneous DL reception from different directions | Samsung |
| R4-2212512 | Discussion on simultaneous DL reception from different directions | MediaTek Inc. |
| R4-2212593 | Discussion on simultaneous multi-Rx chain DL reception | Xiaomi |
| R4-2212688 | Discussion on RRM requirements for FR2 multi-Rx chain reception | Nokia, Nokia Shanghai Bell |
| R4-2212806 | General analysis on Multi-Rx requirements | vivo |
| R4-2213053 | Work plan for RRM requirement for NR FR2 multi-Rx chain DL reception | vivo, Qualcomm |
| R4-2213054 | Discussion on RRM requirements for multi-chain DL reception | vivo |
| R4-2213289 | Discussion on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2213495 | Discussion on RRM impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2213568 | Views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2213737 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2213872 | Discussion on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2213957 | Discussion on general RRM issues for simultaneous DL reception from different directions | Ericsson |
| R4-2214245 | Email Discussion Summary for [104-e][134] FR2_multiRx_UERF | Moderator (Qualcomm) |
| R4-2214282 | Email Discussion Summary for [104-e][233] FR2_multiRx_RRM | Moderator (Vivo) |

## 4.2	Discussion Tdocs from RAN4#104-Bis-e:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2215581 | Proposal on spherical coverage requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2215621 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215781 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2216127 | Discussion on RF requirement for multi-Rx DL reception | vivo |
| R4-2216353 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2216786 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2216253 | Views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2216589 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216875 | On Defining Coverage Requirements for Multi-Rx Chain Downlink Reception | Lenovo |
| R4-2215579 | Proposals on consideration of UE architecture for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2215620 | UE implementation assumptions for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215701 | System assumption and UE assumption for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2215778 | Discussion for FR2 multi-Rx chain DL reception | Murata Manufacturing Co Ltd. |
| R4-2216125 | Discussion on UE implementation assumption of multi-Rx DL reception | vivo |
| R4-2216352 | Discussion on UE architecture to support simultaneous DL reception | Xiaomi |
| R4-2216445 | Considerations on the UE assumptions for FR2 multi-Rx chain DL reception | OPPO |
| R4-2217784 | Email discussion summary for [104-bis-e][132] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2215360 | Discussion on FR2 multi Rx chain RRM impacts and general aspects | Intel Corporation |
| R4-2215361 | Discussion on RRM impacts for L1 measurement based on FR2 multi Rx chain | Intel Corporation |
| R4-2215362 | Discussion on RRM impacts for TCI state switching based on FR2 multi Rx chain | Intel Corporation |
| R4-2215462 | on the multi-RX chain general aspects | Xiaomi |
| R4-2215463 | on the multi-RX chain L1 measurement | Xiaomi |
| R4-2215464 | on the multi-RX chain L3 measurement | Xiaomi |
| R4-2215465 | on the multi-RX chain TCI state switching | Xiaomi |
| R4-2215622 | General aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215623 | On L3 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215624 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215710 | Discussions on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2215720 | Discussion on general aspects for FR2 multi-Rx chain DL reception | CMCC |
| R4-2215721 | Discussion on L1 measurement for FR2 multi-Rx chain DL reception | CMCC |
| R4-2215722 | Discussion on L3 measurement for FR2 multi-Rx chain DL reception | CMCC |
| R4-2215759 | Discussion on simultaneous DL reception from different directions for general issues | MediaTek Inc. |
| R4-2215760 | Discussion on simultaneous DL reception from different directions for L3 measurement | MediaTek Inc. |
| R4-2215761 | Discussion on simultaneous DL reception from different directions for L1 measurement | MediaTek Inc. |
| R4-2215762 | Discussion on simultaneous DL reception from different directions for TCI state switching | MediaTek Inc. |
| R4-2215803 | Discussion on general aspects of RRM for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2215804 | Discussion on L3 measurement related RRM for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2215805 | Discussion on L1 measurement related to RRM for simultaneous DL reception from different | LG Electronics Inc. |
| R4-2215806 | Discussion on TCI state switching for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2215812 | Discussion on general requirements for FR2_multiRX_DL | OPPO |
| R4-2215813 | Discussion on L3 requirements for FR2_multiRX_DL | OPPO |
| R4-2215814 | Discussion on L1 requirements for FR2_multiRX_DL | OPPO |
| R4-2215815 | Discussion on TCI state switching for FR2_multiRX_DL | OPPO |
| R4-2215867 | Further analysis on RRM impacts and general aspects | vivo |
| R4-2215868 | On L3 measurement for multi-Rx chain | vivo |
| R4-2215869 | On L1 measurement for multi-Rx chain | vivo |
| R4-2215870 | On TCI state switching for multi-Rx chain | vivo |
| R4-2216277 | Discussion RRM requirements of TCI state switching for multi-Rx | Huawei, HiSilicon |
| R4-2216285 | Discussion on RRM general impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216286 | Discussion on L3 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216287 | Discussion on L1 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216474 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216475 | Discussion on L1 part RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216476 | Discussion on L3 part RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216477 | Discussion on TCI state related RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216578 | General considerations on RRM requirements for multi-RX RRM | Nokia, Nokia Shanghai Bell |
| R4-2216579 | Discussion on RRM L3 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2216580 | Discussion on RRM L1 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2216581 | Discussion on RRM TCI State Switching for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2216713 | Further Analysis of RRM requirement impacts for simultaneous DL reception from different directions | Samsung |
| R4-2216824 | Discussion on scenarios for simultaneous DL reception from different directions | Ericsson |
| R4-2216825 | Discussion on L3 measurements and procedures | Ericsson |
| R4-2216826 | Discussion on L1 measurements and procedures | Ericsson |
| R4-2216827 | Discussion on active TCI state requirements | Ericsson |
| R4-2216866 | Impacts on RRM to support FR2 multi-Rx chain based 4 layer DL reception from multi-TRP | Qualcomm Incorporated |
| R4-2217246 | WF on RRM impacts and general aspects for multi-Rx | Vivo |
| R4-2217248 | WF on TCI state switching for multi-RX chain DL reception | Ericsson |
| R4-2217587 | WF on L1 measurements, beam sweeping factors and simultaneous reception | Qualcomm |

## 4.3	Discussion Tdocs from RAN4#105:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2218042 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2218043 | On specifying requirements for mDCI UEs | Qualcomm Incorporated |
| R4-2218165 | System parameter and UE assumption for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218166 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218329 | Proposal on UE RF System parameter assumption, UE architecture and conditions of UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2218330 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2218528 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2218556 | System and UE assumptions for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2218557 | RF requirements for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2218755 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2218873 | Further consideration on system parameter of multi-Rx DL reception | vivo |
| R4-2218874 | Discussion on RF requirement for multi-Rx DL reception | vivo |
| R4-2219124 | Discussion on UE RF system assumption supporting simultaneous DL reception | Xiaomi |
| R4-2219125 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2219192 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2219193 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2219497 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219600 | Consideration on small AoA angular separation | OPPO |
| R4-2219852 | System Parameter Assumptions for Multi AoA Rx Testing | Keysight Technologies UK Ltd |
| R4-2219868 | Discussion on RF test methods for UEs with multi-Rx | ROHDE & SCHWARZ |
| R4-2220017 | Complexity Reduction for Multi-Rx Multi-AOA Coverage Evaluation | Lenovo |
| R4-2220529 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2220530 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2220111 | Topic summary for [105][132] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2220112 | Topic summary for [105][132] FR2_multiRx_UERF_part2 | Moderator (Apple) |
| R4-2218167 | General aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218168 | On L3 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218169 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218170 | TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218335 | Discussion on FR2 multi Rx chain scope and scenarios | Intel Corporation |
| R4-2218336 | Discussion on FR2 multi Rx chain general aspects | Intel Corporation |
| R4-2218337 | Discussion on RRM impacts for L1 measurement based on FR2 multi Rx chain | Intel Corporation |
| R4-2218393 | Discussion on scope and scenarios for FR2 multi-Rx chain DL reception | CMCC |
| R4-2218394 | Discussion on L3 measurement for FR2 multi-Rx chain DL reception | CMCC |
| R4-2218408 | Analysis of scope and scenarios | Qualcomm Incorporated |
| R4-2218409 | Analysis of general aspects | Qualcomm Incorporated |
| R4-2218410 | L3 measurement specifically related | Qualcomm Incorporated |
| R4-2218586 | on the multi-RX chain scenarios | Xiaomi |
| R4-2218587 | on the multi-RX chain general aspects | Xiaomi |
| R4-2218588 | on the multi-RX chain L3 measurement | Xiaomi |
| R4-2218589 | on the multi-RX chain L1 measurement | Xiaomi |
| R4-2218590 | on the multi-RX chain TCI state switching | Xiaomi |
| R4-2218596 | Discussion on scope and scenario of RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218597 | Discussion on general aspects of RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218598 | Discussion on L3 measurement related for RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218599 | Discussion on L1 measurement related for RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218738 | Discussion on simultaneous DL reception from different directions for scope and scenarios | MediaTek Inc. |
| R4-2218739 | Discussion on simultaneous DL reception from different directions for general aspects | MediaTek Inc. |
| R4-2218740 | Discussion on simultaneous DL reception from different directions for L3 measurement | MediaTek Inc. |
| R4-2218741 | Discussion on simultaneous DL reception from different directions for L1 measurement | MediaTek Inc. |
| R4-2218742 | Discussion on simultaneous DL reception from different directions for TCI state switching | MediaTek Inc. |
| R4-2218775 | On scope and scenarios for FR2 multi-Rx | vivo |
| R4-2218776 | On general aspects for FR2 multi-Rx | vivo |
| R4-2218777 | On L3 measurement for multi-Rx | vivo |
| R4-2218778 | On L1 measurement for multi-Rx | vivo |
| R4-2218779 | On TCI state switching for multi-Rx | vivo |
| R4-2218977 | On scope and scenarios for FR2_multiRX_DL | OPPO |
| R4-2218978 | On general requirements for FR2_multiRX_DL | OPPO |
| R4-2218979 | Discussion on L3 requirements for FR2_multiRX_DL | OPPO |
| R4-2218980 | Discussion on L1 requirements for FR2_multiRX_DL | OPPO |
| R4-2218981 | Discussion on TCI state switching for FR2_multiRX_DL | OPPO |
| R4-2219008 | Scope and scenarios for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219009 | General aspects discussions for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219010 | Discussions for L3 measurements on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219011 | Discussions for L1 measurements on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219242 | Discussion RRM requirements of TCI state switching for multi-Rx | Huawei, HiSilicon |
| R4-2219250 | Discussion on scope and scenarios for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219251 | Discussion on RRM general aspects for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219252 | Discussion on L3 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219253 | Discussion on L1 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219290 | Discussion on scope and general scenarios for FR2 multi-RX | Samsung |
| R4-2219291 | Analysis on general aspects for FR2 multi-RX | Samsung |
| R4-2219434 | Discussion on scope and scenarios for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2219435 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2219436 | Discussion on L3 measurement related aspects for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2219739 | Discussion on scope and scenarios for the requirements for multi-RX RRM | Nokia, Nokia Shanghai Bell |
| R4-2219740 | Discussion on general aspects for multi Rx DL | Nokia, Nokia Shanghai Bell |
| R4-2219741 | Discussion on RRM L3 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2219742 | Discussion on RRM L1 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2219743 | Discussion on TCI state switching requirements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2219946 | Discussion on scope and UE capabilities | Ericsson |
| R4-2219947 | Discussion on general aspects | Ericsson |
| R4-2219948 | Discussion on L3 measurements and procedures | Ericsson |
| R4-2219949 | Discussion on L1 measurements and procedures | Ericsson |
| R4-2219950 | Discussion on active TCI state requirements | Ericsson |
| R4-2220370 | Ad-hoc meeting minutes: Rel-18 NR FR2 multi-Rx chain DL reception WI | Qualcomm |
| R4-2220402 | WF on NR FR2 multi-Rx chain DL reception – Part 3 | Ericsson |
| R4-2220742 | WF on NR FR2 multi-Rx chain DL reception – Part 1 | Vivo |
| R4-2220743 | WF on NR FR2 multi-Rx chain DL reception – Part 2 | Qualcomm |

## 4.4	Discussion Tdocs from RAN4#106:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2300101 | Overall procedure to enable dual TCI based PDSCH reception | Qualcomm Incorporated |
| R4-2300102 | L1 measurement requirements for support of simultaneous receptions from two TRPs | Qualcomm Incorporated |
| R4-2300103 | RLM and BM requirements for sDCI | Qualcomm Incorporated |
| R4-2300104 | SSB based scheduling and measurement restrictions | Qualcomm Incorporated |
| R4-2300105 | TCI and UE beam switching delay requirements | Qualcomm Incorporated |
| R4-2300106 | Maximum reception timing difference for support of simultaneous receptions from two TRPs | Qualcomm Incorporated |
| R4-2300146 | Discussion for FR2 multi-Rx FOM | Murata Manufacturing Co Ltd. |
| R4-2300195 | Discussion on System parameter assumption, UE architecture and conditions of UE RF requirements | Nokia, Nokia Shanghai Bell |
| R4-2300196 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2300255 | Discussion on Demodulation Requirements for FR2 with multi-RX chain reception | Apple |
| R4-2300267 | System parameter and UE assumption for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300268 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300269 | On general aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300270 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300271 | On RLM and BFD/CBD for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300272 | On TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300273 | On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300285 | On scheduling/measurement restrictions for multiple Rx chains | Apple |
| R4-2300453 | Discussion on general aspects for FR2 multi-RX DL reception | Samsung |
| R4-2300454 | Discussion on L1-RSRP measurement delay for FR2 multi-RX | Samsung |
| R4-2300455 | Discussion on receive timing difference between different directions for FR2 multi-RX | Samsung |
| R4-2300461 | Discussion on FR2 multi Rx chain general aspects | Intel Corporation |
| R4-2300462 | Discussion on RRM impacts for L1 measurement based on FR2 multi Rx chain | Intel Corporation |
| R4-2300463 | Discussion on RRM impacts for scheduling/measurement restriction on FR2 multi Rx chain | Intel Corporation |
| R4-2300464 | Discussion on TCI state switching requirement based on FR2 multi Rx chain | Intel Corporation |
| R4-2300465 | Discussion on FR2 multi Rx chain timing offset | Intel Corporation |
| R4-2300522 | General Aspects on RRM requirements for multi Rx | Nokia, Nokia Shanghai Bell |
| R4-2300523 | L1-RSRP Measurement Delays for multi Rx | Nokia, Nokia Shanghai Bell |
| R4-2300524 | Discussion RLM and BFD/CBD requirements for FR2 Multi Rx chain reception | Nokia, Nokia Shanghai Bell |
| R4-2300525 | Discussion on Scheduling Restrictions for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2300526 | Discussion on TCI state switching requirements for FR2 Multi Rx chain reception | Nokia, Nokia Shanghai Bell |
| R4-2300527 | Discussion on Multi-Rx Receive Time Difference (MRTD) | Nokia, Nokia Shanghai Bell |
| R4-2300701 | On MultiRx Demodulation performance and CSI requirements | Nokia, Nokia Shanghai Bell |
| R4-2300709 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2300832 | discussion on demodulation performance and CSI requirements for NR FR2 multi-Rx chain DL reception | Samsung |
| R4-2300910 | on the L1-RSRP measurement delay | Xiaomi |
| R4-2300911 | on the RLM and BFD CBD requirement. | Xiaomi |
| R4-2300912 | on the scheduling measurement restrictions | Xiaomi |
| R4-2300913 | on the TCI state switching delay with dual TCI | Xiaomi |
| R4-2300914 | on the RTD between different directions | Xiaomi |
| R4-2300940 | Discussion on general aspects for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2300941 | Discussion on L1-RSRP measurement for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2300942 | Discussion on scheduling restriction for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2300943 | Discussion on RTD for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2300949 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2300970 | General aspects discussions for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2300975 | Views on Performance Requirements for FR2 Multi-Rx DL Tests | Qualcomm Incorporated |
| R4-2300987 | Discussion on FR2 Multi-RX DL UE RF requirements | Samsung |
| R4-2301040 | Discussion on general aspects for multi-Rx UE | MediaTek inc. |
| R4-2301041 | Discussion on L1 measurement for multi-Rx UE | MediaTek inc. |
| R4-2301042 | Discussion on RLM/BFD/CBD requirements for multi-Rx UE | MediaTek inc. |
| R4-2301043 | Discussion on scheduling and measurement restriction for multi-Rx UE | MediaTek inc. |
| R4-2301044 | Discussion on dual TCI state switch for multi-Rx UE | MediaTek inc. |
| R4-2301045 | Discussion on receive timing difference for multi-Rx UE | MediaTek inc. |
| R4-2301233 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2301234 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2301326 | On general aspects for FR2 multi-Rx | vivo |
| R4-2301327 | On L1-RSRP measurement delay for multi-Rx | vivo |
| R4-2301328 | On RLM and BFD/CBD requiremets for FR2 multi-Rx | vivo |
| R4-2301329 | On scheduling and measurement restrictions for multi-Rx | vivo |
| R4-2301330 | On TCI state switching delay for multi-Rx | vivo |
| R4-2301331 | On RTD between different directions for multi-Rx | vivo |
| R4-2301363 | Views on NR FR2 multi-Rx chain DL reception requirements | NTT DOCOMO, INC. |
| R4-2301409 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2301410 | Discussion on L1-RSRP measurement requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2301411 | Discussion on Receive timing difference between different directions | ZTE Corporation |
| R4-2301412 | Discussion on scheduling restriction and measurement restriction for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2301461 | On AoA separation range and value for RF requirement | OPPO |
| R4-2301572 | Evaluation on UE requirement of multiRx DL reception | vivo |
| R4-2301573 | Discussion on AoA separation for multiRx DL reception | vivo |
| R4-2301622 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2301650 | On general aspects for FR2_multiRX_DL | OPPO |
| R4-2301651 | On L1 measurement for FR2_multiRX_DL | OPPO |
| R4-2301652 | On measurement restriction for FR2_multiRX_DL | OPPO |
| R4-2301653 | On TCI state switching for FR2 multi-Rx DL | OPPO |
| R4-2301759 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2301812 | Discussion on TCI state switching delay with dual TCI for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2301813 | Discussion on receiving timing difference between different directions for NR FR2 multi-Rx | Huawei, HiSilicon |
| R4-2301895 | UE demodulation and CSI reporting requirements for FR2 DL multi-Rx chain | Ericsson |
| R4-2301899 | On timing requirements regarding MRTD | Ericsson |
| R4-2302005 | Discussion on RRM general impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302006 | Discussion on L1-RSRP measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302007 | Discussion on RLM/BFD/CBD measurements for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302008 | Discussion on scheduling/measurement restrictions for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302155 | Discussion on TCI state switching for FR2 multi-RX DL reception | Samsung |
| R4-2302201 | Discussion on UE demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302250 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2302522 | System Parameter Assumptions for Multi-AoA Rx Testing | Keysight Technologies UK Ltd |
| R4-2302650 | Discussion on general aspects | Ericsson |
| R4-2302651 | Discussion on L1 measurements | Ericsson |
| R4-2302652 | Discussion on RLM and beam management | Ericsson |
| R4-2302653 | Discussion on Scheduling and measurement restrictions | Ericsson |
| R4-2302654 | Discussion on dual active TCI state switching requirements | Ericsson |
| R4-2302767 | Topic summary for [106][209] FR2_multiRx_part1 | Moderator (Vivo) |
| R4-2302768 | Topic summary for [106][210] FR2_multiRx_part2 | Moderator (Qualcomm) |
| R4-2302824 | Topic summary for [106][131] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2302825 | Topic summary for [106][132] FR2_multiRx_UERF_part2 | Moderator (Apple) |
| R4-2303773 | Topic summary for [106][328] NR_FR2_multiRX_DL_Demod | Moderator - Qualcomm |
| R4-2300146 | Discussion for FR2 multi-Rx FOM | Murata Manufacturing Co Ltd. |
| R4-2300195 | Discussion on System parameter assumption, UE architecture and conditions of UE RF requirements | Nokia, Nokia Shanghai Bell |
| R4-2300196 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2300267 | System parameter and UE assumption for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300268 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2300709 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2300949 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2300987 | Discussion on FR2 Multi-RX DL UE RF requirements | Samsung |
| R4-2301233 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2301234 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2301461 | On AoA separation range and value for RF requirement | OPPO |
| R4-2301572 | Evaluation on UE requirement of multiRx DL reception | vivo |
| R4-2301573 | Discussion on AoA separation for multiRx DL reception | vivo |
| R4-2301622 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2301759 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2302250 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2302522 | System Parameter Assumptions for Multi-AoA Rx Testing | Keysight Technologies UK Ltd |
| R4-2302824 | Topic summary for [106][131] FR2_multiRx_part1 | Moderator (Qualcomm) |
| R4-2302825 | Topic summary for [106][132] FR2_multiRx_part2 | Moderator (Apple) |
| R4-2303708 | WF on NR FR2 multi-Rx chain DL reception | Samsung, Apple |
| R4-2300255 | Discussion on Demodulation Requirements for FR2 with multi-RX chain reception | Apple |
| R4-2300701 | On MultiRx Demodulation performance and CSI requirements | Nokia, Nokia Shanghai Bell |
| R4-2300832 | discussion on demodulation performance and CSI requirements for NR FR2 multi-Rx chain DL reception | Samsung |
| R4-2300975 | Views on Performance Requirements for FR2 Multi-Rx DL Tests | Qualcomm Incorporated |
| R4-2301363 | Views on NR FR2 multi-Rx chain DL reception requirements | NTT DOCOMO, INC. |
| R4-2301895 | UE demodulation and CSI reporting requirements for FR2 DL multi-Rx chain | Ericsson |
| R4-2302201 | Discussion on UE demodulation requirements for NR FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2303773 | Topic summary for [106][328] NR_FR2_multiRX_DL_Demod | Moderator - Qualcomm |
| R4-2302941 | WF for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception | Moderator - Qualcomm |