3GPP TSG-RAN4 Meeting #106	R4-2303527

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0860 | rev | 1 | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Time mask for Tx switching for NSA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Qualcomm, China Telecom. |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1 |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is a common understanding that the configuration of “switchedUL” means that there would be no simultaneous transmission in the two bands that involves Tx Swtiching. Current RAN4 requirements for Tx Switching are based on switchedUL and not fully consider dualUL support, especially for the two points: - Requirement pictures have no case for simultaneous transmission in two bands - Emphasize for 2 antenna connectors/2-layer support cannot be used in case of simultaneous transmission in two bands Though this is a legacy issue, it is being discussed in Rel-18 enhancement and a discussion paper can be referenced to R4-2218855. The WF in RAN4#105 R4-2220546 was also include this issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The current time maske pictures have bedn updated to to include the case of simulantaneous transmission for dualUL case. Since it is mandated in the requirements that carrier 1 has one antenna and carrier 2 has two, the switching case for simultaneous transmission would also be limited. The following clarification note is proposed to be added for each switching case: “The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.” Issues have been addressed in the clarification: - The requirements in case of simultaneous transmission in two bands need to be specified/clarified and inline with the behavior defined in 38.214 - Unified wording between different test cases is preferred, e.g. band is more preferred than carrier - The additional clarification should be easy to be extended to Rel-18 It is noted that the concept of switchedUL or dualUL is not further clarifed, since they are basically defined and mostly used in RAN1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be no clear requirements for “dualUL” configuration, while actually needed. The alignment between different WGs would remain problematic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3B.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolation impact analysis This revision does not impact the UE implementaion, and the revision is in line with current RAN1 and RAN2 specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3B.4	Output power dynamics for switching between two uplink carriers

6.3B.4.1	E-UTRA and NR switching time mask between two uplink carriers

In addition to the requirements in 6.3B.0 and the maximum output power requirement specified in Table 6.2B.1.3-1 with inter-band EN-DC (two bands), the switching time mask specified in this sub-clause is applicable for an uplink band pair of a inter-band EN-DC configuration without SUL band when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in sub-clause 6.1.6 of TS 38.214 [14], where E-UTRA UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3B.4.1-1 are only located in NR carrier, and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3B.4.1-1: Time mask for switching between E-UTRA UL carrier and NR UL carrier, where the switching period is located in NR carrier

The requirements apply for the case of co-located and synchronized network deployment with the max receiving timing difference of 3us between the two carriers.

The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.