3GPP TSG-RAN4 Meeting #106	R4-2301992

# Athens, Greece, Feburary 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2978 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CBW change TCs_R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Perf, LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To update UE specific CBW change TCs in following aspects: - BWP configuration in 5.5.10.1 doesn’t align with other CBW change TCs. Both initial BWP and dedicated BWP are configured in 5.5.10.1 while only initial BWP is configured in other TCs. Suggest remove dedicated BWP. - Active BWP ID should be 0 since active BWP is initial BWP. - Correlation configuration “low” is missing. - To remove brackets on test parameters. - To correct Io values. - To fix typos. - To fix incorrect format - 38.133 R17 version doesn’t align with R16 version. FR1 EN-DC CBW change TC (A.4.5.9.1 in 38.133 R16 version) introduced under R16 RRM enhancement WI are not implemented in 38.133 R17 version. To make things worse, this TC number A.4.5.9.1 is used by TCs introduced in Rel-17 WIs. To avoid bringing confusion and impact to RAN5 specs. We suggest - Add FR1 EN-DC CBW change TC in R17 38.133 back as A.4.5.9.1. - Change PSCell activation and deactivation delay TC and Conditional PSCell addition and release delay TC to A.4.5.10 and A.4.5.11 respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE specific CBW change TCs are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.9.1, A.4.5.10, A.4.5.11, A.5.5.10.1, A.6.5.8.1, A.7.5.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Test description of A.4.5.9.1/A.6.5.8.1 are updated. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC

A.4.5.9.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE specific CBW change delay requirement defined in clause 8.13.1. Supported test configurations are shown in Table A.4.5.9.1.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1) and one NR PSCell (Cell 2) as given in Table A.4.5.9.1.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell are specified in Table A.4.5.9.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and to Cell 2 (PSCell) on radio channel 2 (PSCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1 (PSCell).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PSCell.

-	UE is indicated in SCS-SpecificCarrier that the active CBW is CBW-1 of initial condition in PSCell.

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated CBW configuration, sent from the test equipment to the UE, is completely received at the UE side in PSCell’s slot # denoted i. The UE shall reconfigure its CBW with the updated CBW of final condition (CBW-2).

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (i+ TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length) as defined in clause 8.13.1 and be ready for the reception of uplink grant for the PSCell no later than at the beginning of the DL slot right after slot (i+ TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length). The UE shall be continuously scheduled on PSCell’s BWP-1 of CBW-1 starting from the beginning of the DL slot right after slot (i+ TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length).

TRRCprocessingDelay and TCBWchangeDelayRRC are defined in clause 8.13.1.

The test equipment verifies the UE specific CBW change time in PSCell by counting the time from the time when the RRC Reconfiguration message including updated CBW configurationis sent till the time when RRC Reconfiguration Complete message is received.

Table A.4.5.9.1.1-1: UE specific CBW change supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.4.5.9.1.1-2: General test parameters for UE specific CBW change in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| T1 | s | 0.2 |  |

A.4.5.9.1.1-3: NR Cell specific test parameters for UE specific CBW change in synchronous EN-DC

| Parameter |  |  | Unit | Cell 2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1,4 |  | FDD |
|  |  | Config 2,3,5,6 |  | TDD |
| TDD configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2,5 |  | TDDConf.1.1 |
|  |  | Config 3,6 |  | TDDConf.1.2 |
| BWchannel |  | Config 1,4 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2,5 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3,6 |  | 40 MHz: NRB,c = 106 |
| Active DL BWP ID |  |  |  | 0 |
| Initial DL BWP |  | Config 1,4 |  | DLBWP.0.2 |
| Configuration (BWP-1) before and after UE specific CBW change |  | Config 2,5 |  |  |
|  |  | Config 3,6 |  |  |
| Initial UL BWP (BWP-1) |  | Config 1,4 |  | ULBWP.0.2 |
| Configuration before and after UE specific CBW change |  | Config 2,5 |  |  |
|  |  | Config 3,6 |  |  |
| Initial | Active DL | Config 1,4 |  | DLCBW.1.1 |
| Condition | CBW | Config 2,5 |  |  |
|  | Configuration (CBW-1) | Config 3,6 |  |  |
|  | Active UL | Config 1,4 |  | ULCBW.1.1 |
|  | CBW | Config 2,5 |  |  |
|  | Configuration (CBW-1) | Config 3,6 |  |  |
| Final | Active DL | Config 1,4 |  | DLCBW.1.2 |
| Condition | CBW | Config 2,5 |  |  |
|  | Configuration (CBW-2) | Config 3,6 |  |  |
|  | Active UL | Config 1,4 |  | ULCBW.1.2 |
|  | CBW | Config 2,5 |  |  |
|  | Configuration (CBW-2) | Config 3,6 |  |  |
| PDSCH Reference |  | Config 1,4 |  | SR.1.1 FDD |
| measurement channel |  | Config 2,5 |  | SR.1.1 TDD |
|  |  | Config 3,6 |  | SR2.1 TDD |
| RMSI CORESET |  | Config 1,4 |  | CR.1.1 FDD |
| parameters |  | Config 2,5 |  | CR.1.1 TDD |
|  |  | Config 3,6 |  | CR2.1 TDD |
| Dedicated CORESET |  | Config 1,4 |  | CCR.1.1 FDD |
| parameters |  | Config 2,5 |  | CCR.1.1 TDD |
|  |  | Config 3,6 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |
|  |  | Config 3,6 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2,5 |  | TRS.1.1 TDD |
|  |  | Config 3,6 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  | 1x2 low |
| Propagation Condition |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  |  | dBm/15 kHz | -104 |
| SS-RSRP Note 3 |  |  | dBm/15 kHz | -87 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2,4,5 | dBm/9.36MHz | -58.96 |
|  |  | Config 3,6 | dBm/38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

A.4.5.9.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for PSCell in the beginning of the DL slot right after  slot (i+ TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length).

All of the above test requirements shall be fulfilled in order for the observed PSCell UE specific CBW change delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

A.4.5.10	PSCell activation and deactivation delay

A.4.5.10.1	PSCell activation and deactivation delay

A.4.5.10.1.1	Test purpose and environment

The purpose of this test is to verify that the NR PSCell activation and deactivation delay under EN-DC are within the requirements stated in clause 7.38 in TS 36.133 [15] for the case when UE configured with one deactivated SCG and when PScell in one SCG is being activated where the PSCell is known by the UE at the time of activation.

Supported test configurations are shown in A.4.5.10.1.1-1. The test parameters for the E-UTRA cell are given in Table A.3.7.2.2-1. The E-UTRA cell once set up is not changed across time.

The test parameters for NR cell are given in Tables A.4.5.10.1.1-2, cell-specific parameters in A.4.5.10.1.1-3 below. The test consists of four successive time periods with duration of T1, T2, T3 and T4. There are two carriers each with one cell. The UE is connected to Cell 1 (E-UTRA PCell) on radio channel 1 (PCC) and PSCell (Cell2) is in deactivated state. During T1, both Cell1 and Cell2 are known to UE and UE performs measurement on deactivated PCell. Before the test starts the UE is configured RLM and BFD on deactivated PSCell. During T1, UE performs RLM and BFD on the deactivated PSCell and TCI state is known.

The test system shall send a RRC message to the UE to activate PSCell (Cell 2) on radio channel 2, where no any PSCell parameter is modified in the RRC message. The RRC message (to activate PSCell) also includes a request for the UE to transmit scheduling request on PUCCH for the PSCell after the PSCell has been successfully activated. The RRC message to activate PSCell shall be sent to the UE during period T1. The point in time at which the RRC message to activate PSCell (Cell2) is received at the UE antenna connector defines the start of period T2.

The test system shall observe the periodic reporting of CSI for PSCell during T3. The point in time at which the UE has sent scheduling request on PUCCH for PSCell (Cell 2) defines the start of period T3.

The test system shall send a RRC message to the UE to deactivate PSCell (Cell 2) on radio channel 2. The RRC message to deactivate PSCell (Cell2) shall be sent to the UE during period T3, after the UE has sent at least one CQI report with non-zero CQI index for PSCell (Cell 2). The point in time at which the RRC message to deativate PSCell (Cell2) is received at the UE antenna connector defines the start of period T4.

Table A.4.5.10.1.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, FDD |
| 2 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, TDD |
| 3 | LTE FDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| 4 | LTE TDD, NR SCS 15 kHz, BW 10 MHz, FDD |
| 5 | LTE TDD, NR SCS 15 kHz, BW 10 MHz, TDD |
| 6 | LTE TDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.4.5.10.1.1-2: General Test Parameters for PSCell activation and deactivation

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. One for E-UTRA cell and second for NR Cell |
| Initial | Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Condition | Deactivated cell |  | Cell2 | To be activated PSCell on RF channel number 2. |
| Final | Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Condition | Deactivated cell |  | Cell2 | PSCell deactivated on RF channel number 2. |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Scheduling request resource priodicity |  |  | 20ms | At the starting of period T3, UE sends a SR on PUCCH for PSCell |
| T1 |  | s | 1 | During this time the PCell shall be known and cell2 shall be unknown. |
| T2 |  | s | 1 | During this time the UE adds the PSCell. |
| T3 |  | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T4 |  | s | 1 | During this time the UE releases the PSCell. |

Table A.4.5.10.1.1-3: Cell Specific Parameters for PSCell activation and deactivation

| Parameter | Unit | Config | Test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1,2,3,4,5,6 | 1 |  |  |  |  |
| NR RF Channel Number |  | 1,2,3,4,5,6 | 2 |  |  |  |  |
| TDD |  | 1,4 | Not Applicable |  |  |  |  |
| configuration |  | 2,5 | TDDConf.1.1 |  |  |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  |
| BWchannel | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |  |
|  |  | 2,5 | 10: NRB,c = 52 |  |  |  |  |
|  |  | 3,6 | 40: NRB,c = 106 |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  |
| measurement |  | 2,5 | SR.1.1 TDD |  |  |  |  |
| channel |  | 3,6 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference |  | 1,4 | CCR.1.1 FDD |  |  |  |  |
| Channel |  | 2,5 | CCR.1.1 TDD |  |  |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  |
| SSB configuration |  | 1,2,4,5 | SSB.1 FR1 |  |  |  |  |
|  |  | 3,6 | SSB.2 FR1 |  |  |  |  |
| SMTC configuration |  | 1,2,4,5 | SMTC.1 |  |  |  |  |
|  |  | 3,6 | SMTC.1 |  |  |  |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  |  |  |  |
|  |  | 2,5 | TRS.1.1 TDD |  |  |  |  |
|  |  | 3,6 | TRS.1.2 TDD |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1,4 | CSI-RS.1.1 FDD |  |  |  |  |
|  |  | 2,5 | CSI-RS.1.1 TDD |  |  |  |  |
|  |  | 3,6 | CSI-RS.2.1 TDD |  |  |  |  |
| reportConfigType |  | 1,2,3,4,5,6 | periodic |  |  |  |  |
| reportQuantity |  | 1,2,3,4,5,6 | cri-RI-PMI-CQI |  |  |  |  |
| CSI reporting periodicity | slot | 1,2,4,5 | 5 |  |  |  |  |
|  |  | 3,6 | 10 |  |  |  |  |
| CSI reporting offset | slot | 1,2,4,5 | 2 |  |  |  |  |
|  |  | 3,6 | 4 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | N/A | -85 |  |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | N/A | -85 |  |  |  |
|  |  | 3,6 | N/A | -82 |  |  |  |
|  |  | 1,2,3,4,5,6 | -infinity | 0 |  |  |  |
|  |  | 1,2,3,4,5,6 | -infinity | 0 |  |  |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -infinity | -85 |  |  |  |
|  |  | 3,6 | -infinity | -82 |  |  |  |
| IoNote3 | dBm/9.36MHz | 1,2,4,5 | N/A | -57 |  |  |  |
|  | dBm/38.1MHz | 3,6 | N/A | -51 |  |  |  |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

A.4.5.10.1.2	Test Requirements

The UE performs RACH-less based PSCell activation. UE shall transmit the SR on PUCCH for PSCell at latest 65 msNote1 into T2.

The UE shall send at least one PUSCH on PSCell during T3.

The UE shall stop transmit PUSCH for PSCell in at latest 20 ms into T4.

All the above test requirements shall be fulfilled for the observed PSCell activation delay and PSCell deactivation delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

Note1:	The PSCell addition delay can be expressed as follows as specified in clause 7.38 in TS 36.133 [15]:

Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms

Where:

TRRC_delay = 20ms

Tprocessing = 5 ms

Tsearch = 0 ms

T∆ = 20ms

TIU= max 20 ms

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)

A.4.5.11.1	Addition and Release Delay of PSCell

A.4.5.11.1.1	Test purpose and environment

The purpose of this test is to verify that the NR conditional PSCell addition and release delays under EN-DC are within the requirements stated in clause 8.9A.2.

A.4.5.11.1.2	Test Parameters

Supported test configurations are shown in A.4.5.11.1.2-1. The test parameters for the E-UTRA cell are given in Table A.4.5.11.1.2-2. The E-UTRA cell once set up is not changed across time.

The test parameters for NR cell are given in Tables A.4.5.11.1.2-2 and cell-specific parameters in A.4.5.11.1.2-3 below. The test consists of four successive time periods with duration of T1, T2, T3 and T4 respectively. There are two carriers each with one cell. Before the test starts the UE is connected to Cell 1 (E-UTRA PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (NR PSCell) on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

At the start of time duration T1, the UE does not have any timing information of Cell 2. NR shall configure a condition implying addition to cell 2 during T1, at a time earlier than TRRC_delay before the beginning of T2.

At the start of T2, cell 2 becomes detectable and meets the addition condition. Reception by the test system of the PRACH preamble defines the start of T3.

During T3, the UE shall send periodic CSI reports in PSCell. After having received at least one such report, the test system shall send an RRC message instructing the UE to release the PSCell. Reception by the UE of the RRC message defines the start of T4.

During T4, the UE shall release the PSCell.

Table A.4.5.11.1.2-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, FDD |
| 2 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, TDD |
| 3 | LTE FDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| 4 | LTE TDD, NR SCS 15 kHz, BW 10 MHz, FDD |
| 5 | LTE TDD, NR SCS 15 kHz, BW 10 MHz, TDD |
| 6 | LTE TDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.4.5.11.1.2-2: General Test Parameters for Conditional PSCell Addition and Release

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. One for E-UTRA cell and second for NR Cell |
| Initial | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell on RF channel number 2. |
| Final | Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Condition | Neighbour Cell |  | Cell2 | PSCell released on RF channel number 2. |
| B1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event B1. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event B1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time to Trigger | S | 0 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured before T2 and released before T3. |
| PRACH configuration on cell2 |  |  | FR1 PRACH configuration 1 | Captured in A.3.8.2.1 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T1 |  | s | 5 | During this time the PCell is known and Cell 2 is unknown. |
| T2 |  | s | 7 | During this time Cell 2 meets the addition condition and UE adds this PSCell. |
| T3 |  | s | 1 | During this time the UE sends CSI reports for Cell 2. And the test system shall send an RRC message instructing the UE to release the PSCell. |
| T4 |  | s | 1 | During this time the UE releases the Cell 2. |

Table A.4.5.11.1.2-3: Cell Specific Parameters for Conditional PSCell Addition and Release

| Parameter | Unit | Config | Test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1,2,3,4,5,6 | 1 |  |  |  |  |
| NR RF Channel Number |  | 1,2,3,4,5,6 | 2 |  |  |  |  |
| TDD |  | 1,4 | Not Applicable |  |  |  |  |
| configuration |  | 2,5 | TDDConf.1.1 |  |  |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  |
| BWchannel | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |  |
|  |  | 2,5 | 10: NRB,c = 52 |  |  |  |  |
|  |  | 3,6 | 40: NRB,c = 106 |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  |
| measurement |  | 2,5 | SR.1.1 TDD |  |  |  |  |
| channel |  | 3,6 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference |  | 1,4 | CCR.1.1 FDD |  |  |  |  |
| Channel |  | 2,5 | CCR.1.1 TDD |  |  |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  |
| SSB configuration |  | 1,2,4,5 | SSB.1 FR1 |  |  |  |  |
|  |  | 3,6 | SSB.2 FR1 |  |  |  |  |
| SMTC configuration |  | 1,2,4,5 | SMTC.1 |  |  |  |  |
|  |  | 3,6 | SMTC.1 |  |  |  |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  |  |  |  |
|  |  | 2,5 | TRS.1.1 TDD |  |  |  |  |
|  |  | 3,6 | TRS.1.2 TDD |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1,4 | CSI-RS.1.1 FDD |  |  |  |  |
|  |  | 2,5 | CSI-RS.1.1 TDD |  |  |  |  |
|  |  | 3,6 | CSI-RS.2.1 TDD |  |  |  |  |
| reportConfigType |  | 1,2,3,4,5,6 | periodic |  |  |  |  |
| reportQuantity |  | 1,2,3,4,5,6 | cri-RI-PMI-CQI |  |  |  |  |
| CSI reporting periodicity | slot | 1,2,4,5 | 5 |  |  |  |  |
|  |  | 3,6 | 10 |  |  |  |  |
| CSI reporting offset | slot | 1,2,4,5 | 2 |  |  |  |  |
|  |  | 3,6 | 4 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | N/A | -85 |  |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | N/A | -85 |  |  |  |
|  |  | 3,6 | N/A | -82 |  |  |  |
|  |  | 1,2,3,4,5,6 | -infinity | 0 |  |  |  |
|  |  | 1,2,3,4,5,6 | -infinity | 0 |  |  |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -infinity | -85 |  |  |  |
|  |  | 3,6 | -infinity | -82 |  |  |  |
| IoNote3 | dBm/9.36MHz | 1,2,4,5 | N/A | -57 |  |  |  |
|  | dBm/38.1MHz | 3,6 | N/A | -51 |  |  |  |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

A.4.5.11.1.3	Test Requirements

TRRC_delay + TEvent_DU occurs during T1 as the addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 1040+10+62ms=1112 ms from the start of T2.

The UE shall transmit at least one periodic CSI report for PSCell during T3.

The UE shall stop transmitting CSI reports for PSCell at latest 20 ms into T4.

All of the above test requirements shall be fulfilled in order for the observed conditional PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90%.

<End of modified section 1>

<Start of modified section 2>

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell

A.5.5.10.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE specific CBW change delay requirement defined in clause 8.13. Supported test configurations are shown in Table A.5.5.10.1.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1) and one NR PSCell (Cell 2) as given in Table A.5.5.10.1.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell are specified in Table A.5.5.10.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and to Cell 2 (PSCell) on radio channel 2 (PSCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 2 (PSCell).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PSCell.

-	UE is indicated in SCS-SpecificCarrier that the active CBW is CBW-1 of initial condition in PSCell.

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration which reconfigure the UE specific CBW parameter, sent from the test equipment to the UE, is received at the UE side in PSCell’s slot # denoted i. The UE shall reconfigure its UE specific CBW with the updated UE specific CBW of final condition (CBW-2).

The UE shall be able to completely receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length) as defined in clause 8.13 and be ready for the reception of uplink grant for the PSCell no later than at the beginning of the DL slot right after slot (i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length). The UE shall be continuously scheduled on PSCell’s BWP-1 starting from the beginning of the DL slot right after slot (i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length).

TRRCprocessingDelay and TCBWchangeDelayRRC are defined in clause 8.13.

The test equipment verifies the UE specific CBW change switch time in PSCell by counting the time from the time when the RRC Reconfiguration message including updated UE specific CBW configuration is sent till the time when RRC Reconfiguration Complete message is received.

Table A.5.5.10.1.1-1: UE specific CBW change supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.5.5.10.1.1-2: General test parameters for UE specific CBW change in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| T1 | s | 0.2 |  |

Table A.5.5.10.1.1-3: NR Cell specific test parameters for UE specific CBW change in synchronous EN-DC

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR2 |
| Duplex mode |  |  | TDD |
| TDD configuration |  |  | TDDConf.3.1 |
| BWchannel |  |  | 100 MHz: NRB,c = 66 |
| Active BWP ID |  |  | 0 |
| Initial DL BWP Configuration (BWP-1) |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration (BWP-1) |  |  | ULBWP.0.2 |
| Initial Condition | Active DL CBW Configuration (CBW-1) |  | DLCBW.1.1 |
|  | Active UL |  | ULCBW.1.1 |
|  | CBW |  |  |
|  | Configuration (CBW-1) |  |  |
| Final Condition | Active DL CBW Configuration (CBW-2) |  | DLCBW.1.2 |
|  | Active UL |  | ULCBW.1.2 |
|  | CBW |  |  |
|  | Configuration (CBW-2) |  |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |
| RMSI CORESET parameters |  |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  |  | CCR.3.1 TDD |
| OCNG Patterns |  |  | OP.1 |
| SSB Configuration |  |  | SSB.1 FR2 |
| SMTC Configuration |  |  | SMTC.1 |
| TCI State |  |  | TCI.State.0 |
| TRS Configuration |  |  | TRS.2.1 TDD |
| Antenna Configuration |  |  | 1x2 |
| Propagation Condition |  |  | AWGN |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |

Table A.5.5.10.1.1-4: OTA related test parameters for UE specific CBW change test case

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  |  | Setup 1 according to table A.3.15 |
| Assumption for UE beamsNote 5 |  |  | Fine |
| Note1 | NR_TDD_FR2_A | dBm/15kHz | -112 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| Note1 | NR_TDD_FR2_A | dBm/SCS | -103 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note3 | -85 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
|  |  | dB | 18 |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz | -56 |
|  | NR_TDD_FR2_B | Note4 |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

A.5.5.10.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for PSCell in the beginning of the DL slot right after slot (i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length).

All of the above test requirements shall be fulfilled in order for the observed PSCell UE specific CBW change switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of modified section 2>

<Start of modified section 3>

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX

A.6.5.8.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE specific CBW change delay requirement defined in clause 8.13.

The supported test configurations are shown in Table A.6.5.8.1.1-1. The test scenario comprises of one Cell (Cell 1), which is PCell as given in Table A.6.5.8.1.1-2. Cell-specific parameters are specified in Table A.6.5.8.1.1-3.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE sends ACK/NACK during the test.

Before the test starts:

- UE is connected to Cell 1 (PCell) on radio channel 1.

- UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1 (PCell).

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PCell.

- UE has been configured with UE specific CBW (CBW-1).

- UE is indicated in SCS-SpecificCarrier [2] that the UE specific CBW is CBW-1 as the initial condition in Cell 1 (PCell).

Cell1 (PCell) has constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration containing SCS-SpecificCarrier with updated UE specific CBW, sent from the test equipment to the UE, is completely received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its UE specific CBW with the updated CBW-2 for the final condition (CBW-2).

The UE shall be able to receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length as defined in clause 8.13 and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1 on the PCell’s BWP-1 on CBW-2 for the final condition. The UE shall be continuously scheduled on the PCell’s BWP-1 on CBW-2  for the final condition starting from the first DL slot right after slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length.

TRRCprocessingDelay and TCBWchangeDelayRRC are defined in clause 8.13.

The test equipment verifies the UE specific CBW switching delay in PCell by estimating the time from the moment the RRC Reconfiguration message including updated UE specific CBW configuration is sent until the moment a vaild ACK/NACK is received.

Table A.6.5.8.1.1-1: Supported test configurations for UE specific CBW change in SA scenario

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.8.1.1-2: General test parameters for UE specific CBW change in SA scenario

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

Table A.6.5.8.1.1-3: NR Cell specific test parameters for UE specific CBW change in SA scenario

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  |  | FDD |
|  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  | Config 1 |  |  | Not Applicable |
|  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  | Config 1 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  |  | 40 MHz: NRB,c = 106 |
| Active DL BWP ID |  | Config 1,2, 3 |  |  | 0 |
| Initial DL BWP Configuration (BWP-1) |  | Config 1,2, 3 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration (BWP-1) |  | Config 1,2, 3 |  |  | ULBWP.0.2 |
| Initial Condition | Active DLCBW Configureation (CBW-1) | Config 1, 2, 3 |  |  | DLCBW.1.1 |
|  | Active UL CBW Configuration (CBW-1) | Config 1, 2, 3 |  |  | ULCBW.1.1 |
| Final Condition | Active DLCBW Configureation (CBW-2) | Config 1, 2, 3 |  |  | DLCBW.1.2 |
|  | Active UL CBW Configuration (CBW-2) | Config 1, 2, 3 |  |  | ULCBW.1.2 |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |
|  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD |
|  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD |
|  |  | Config 2 |  |  | CCR.1.1 TDD |
|  |  | Config 3 |  |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2 |  |  | SSB.1 FR1 |
|  |  | Config 3 |  |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  | Config 1 |  | TRS.1.1 FDD |
|  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  | Config 1,2 |  | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.1 is linked with ULBWP.0.1; DLBWP.1.1 is linked with ULBWP.1.1; as defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.6.5.8.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for the PCell from the first DL slot that occurs right after the begining of slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

All of the above test requirements shall be fulfilled in order for the observed UE specific CBW change delay on the PCell to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of modified section 3>

<Start of modified section 4>

A.7.5.10	UE specific CBW change

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA

A.7.5.10.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE specific CBW change delay requirement defined in clause 8.13. Supported test configurations are shown in Table A.7.5.10.1.1-1.

The test scenario comprises of one PCell (Cell 1) as given in Table A.7.5.10.1.1-2. Cell-specific parameters of PCell are specified in Table A.7.5.10.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK transmission.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1 (PCell).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PCell.

-	UE has been configured with UE-specific CBW (CBW-1)

-	UE is indicated in SCS-SpecificCarrier [2] that the UE-specific CBW is CBW-1 as the initial condition in Cell 1 (PCell).

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated CBW configuration, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its CBW with the updated CBW of final condition (CBW-2).

The UE shall be able to completely receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length as defined in clause 8.13.2 and starts to report valid ACK/NACK for the PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1. The UE shall be continuously scheduled on PCell’s new CBW starting from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length.

TRRCprocessingDelay and TCBWchangeDelayRRC are defined in clause 8.13.

The test equipment verifies the UE specific CBW switch time in PCell by counting the time from the time when the RRC Reconfiguration message including updated CBW configurations sent till the time when RRC Reconfiguration Complete message is received.

Table A.7.5.10.1.1-1: UE specific CBW change supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.10.1.1-2: General test parameters for UE specific CBW change in NR SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| T1 | s | 0.2 |  |

Table A.7.5.10.1.1-3: NR Cell specific test parameters for UE specific CBW change in NR SA

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR2 |
| Duplex mode |  |  | TDD |
| TDD configuration |  |  | TDDConf.3.1 |
| BWchannel |  |  | 100 MHz: NRB,c = 66 |
| Active DL BWP ID |  |  | 0 |
| Initial DL BWP Configuration (BWP-1) |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration (BWP-1) |  |  | ULBWP.0.2 |
| Initial Condition | Active DL CBW Configuration (CBW-1) |  | DLCBW.1.1 |
|  | Active UL CBW Configuration (CBW-1) |  | ULCBW.1.1 |
| Final Condition | Active DL CBW Configuration (CBW-2) |  | DLCBW.1.2 |
|  | Active UL CBW Configuration (CBW-2) |  | ULCBW.1.2 |
| 'PDSCH Reference measurement channel |  |  | SR.3.1 TDD |
| RMSI CORESET parameters |  |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  |  | CCR.3.1 TDD |
| OCNG Patterns |  |  | OP.1 |
| SSB Configuration |  |  | SSB.1 FR2 |
| SMTC Configuration |  |  | SMTC.1 |
| TCI State |  |  | TCI.State.0 |
| TRS Configuration |  |  | TRS.2.1 TDD |
| Antenna Configuration |  |  | 1x2 |
| Propagation Condition |  |  | AWGN |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.7.5.10.1.1-4: OTA related test parameters for UE specific CBW change test case

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  |  | Setup 1 according to table A.3.15 |
| Assumption for UE beams Note 5 |  |  | Fine |
| Note1 | NR_TDD_FR2_A | dBm/15kHz | -112 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| Note1 | NR_TDD_FR2_A | dBm/SCS | -103 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note3 | -85 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
|  |  | dB | 18 |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -56 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |

A.7.5.10.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for PCell from the first DL slot that occurs after the beginning of slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length and starts to report valid ACK/NACK for the PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

All of the above test requirements shall be fulfilled in order for the observed PCell UE specific CBW change delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of modified section 4>