3GPP TSG RAN Meeting #99			RP-230573

Rotterdam, Netherlands, March 20-23, 2023

Agenda item:		9.2.5

Source:	Nokia, Nokia Shanghai Bell

Title:	View on open issues for Rel-18 RedCap

Document for:		Discussion and Decision

# Introduction

In this RAN#98-e there were discussions on the potential support of peak rate reduction (PR1) as a stand-alone feature within Rel-18 RedCap, without conclusion. Many companies have showed concerns on device ecosystem fragmentation, while others showed strong support for the feature. In this contribution we present our views on how the preferred way forward regarding this issue.

# Discussion

In RAN#98-e no conclusion has been reached on Rel-18 RedCap scope regarding support of PR1 as a standalone feature [1]:

From draft RAN#98-e report [2]:

conclusion:

- Proposal for Issue – 1 UE peak rate reduction as a standalone feature: revisit at RAN #99 (no additional discussion in RAN1 in Q1/23)

It is clear from the discussion regarding issue 1 in [1] that there are multiple views on what characterizes a (new) device type, which creates confusion on the discussion. In Rel-17, RAN2 already agreed to introduce RedCap as a single device type in the specifications, with all other variants indicated by means of UE capabilities, as per normal procedure. However, when discussing on implications of supported features to availability of devices in the market, one often refers to the variety of devices to be developed and manufactured, which is independent of how they are signalled. Yet another dimension to the discussion is the variety of devices that the network needs to be prepared to handle, in particular those that are not backward compatible to existing deployments.

The general understanding is that in NR it is assumed that devices can evolve by adding support to more optional capabilities over time, without normally implying new device types. However, some new features imply the device is not fully backward compatible to existing deployments, and those might require updates to the network to detect those devices or allow continuity of basic services once such devices are present. This has been done with Rel-17 RedCap due to their reduced bandwidth and other characteristics, and in RAN#98-e it has been agreed to introduce an early indication specific to Rel-18 RedCap due to the fact devices supporting the bandwidth reduction (BW3) and peak rate reduction are not backward compatible to networks supporting Rel-17 RedCap.

From signalling point of view, one could say that the early indication gives the network a warning about the device type, and further capability exchange will refine the mutual understanding on the exact features supported by the device. This is essentially aligned to the formulation moderator’s proposals shown in the beginning of this section. However, one can argue that regardless of the signalling framework to indicate their support by the device, there is a fragmentation in the market in case both PR1 and BW3 are supported as independent features, which is particularly concerning for cost-constrained devices such as in case of RedCap.

Another issue is the network support, as each of those features is reducing functionality instead of adding more advanced support. Hence, from the network point of view, each of those features requires handling non-backward compatible devices, essentially making support of all possible variants and combinations mandatory in the network.

Based on the considerations above and on the status of work in the working groups so far, we see the following possible ways forward:

- Combine peak data rate reduction with UE baseband bandwidth reduction in a single UE capability

- Support peak data rate reduction as optional capability on top of UE baseband bandwidth reduction

- Do not support UE peak data rate reduction

We understand option a) above provides the best compromise between network complexity and minimization of device ecosystem fragmentation, and option b) should be avoided due to the extra fragmentation and inherent complexity.

Proposal: Support combining peak data rate reduction and UE baseband bandwidth reduction in a single capability.

# Conclusions

Based on the discussion above, we make the following proposals, observations, and conclusions:

Proposal: Support combining peak data rate reduction and UE baseband bandwidth reduction in a single capability.

# References

- RP-223551, “Moderator's summary for discussion [98e-19-R18-eRedCap],” Moderator (RAN1 VC, CMCC), Dec. 2022

- Draft meeting report RAN#98-e, Dec. 2022