3GPP TSG RAN Meeting #99			RP-230575

Rotterdam, Netherlands, March 20th – 23rd, 2023

Agenda item:	9.2.3

Source:	KT Corp.

Title:	Considerations for Ambient IoT latency requirements

Document for:	Discussion

# Introduction

Study on Ambient IoT was approved in RAN#97e [1]. This contribution proposes to add Latency in RAN design targets for Ambient IoT study item.

# Latency Requirements from SA1

SA1 had been working on Study on Ambient power-enabled Internet of Things [2] and SI had identified 28 use cases which is being captured in TR 22.840 [3]. As endorsed in [4], KT has been re-arranging SA1 use cases with Grouping A and Grouping B with their latency requirement identified as below:

| Use Case | Grouping A | Grouping B | Latency Requirement | Note |
| --- | --- | --- | --- | --- |
| Automated warehousing | Indoor | Inventory | 1s | End to end latency refers to the time taken for an Ambient IoT device to transmit the message |
| Medical instrument inventory management and positioning | Indoor | Inventory Positioning | Hundreds ms level | It is one trip delay between an Amibient-IoT device and the interface of application platform |
| Remote monitoring of transmission and distribution networks in smart grids | Outdoor | Sensors | 1s | This is calculated based on assumption that the sensor data are collected once per several seconds. |
| NPN for logistics | Outdoor | Inventory Positioning | NA | - |
| Automatic intralogistics in automobile manufacturing | Indoor | Inventory Positioning | >100ms | This value corresponds to peak reading rate of 100 tags per second. The average tag reading rate is lower |
| Ambient IoT sensors in smart homes | Indoor | Sensors | 20s | - |
| Airport Terminal/Shipping Port | Outdoor Indoor | Positioning | >1sec | 128 bits for the Electronic Product Code (EPC) of the tracked object and additional 128 bits assumed for control/other data (e.g. location-related) |
| Remote lost item finding | Indoor Outdoor | Positioning | >5s |  |
| Absolute Positioning | Indoor Outdoor | Positioning | NA |  |
| Ranging for Ambient IoT | Indoor | Positioning | NA |  |
| Medical instrument inventory management and positioning | Indoor Outdoor | Positioning | Hundreds ms level | It is one trip delay between an Ambient-IoT device and the interface of application platform |
| Personal belongings finding | Indoor Outdoor | Positioning | 1s | - |
| BSMR environmental supervision | Indoor | Sensor | 1s | End to end latency refers to the time taken for an Ambient IoT device to transmit the data |
| Indoor positioning in shopping centre using Ambient IoT | Indoor | Positioning | 0.5s | - |
| Ambient IoT devices for Smart Laundry | Indoor | Sensor | >10s | - |
| Ambient IoT devices for automated supply distribution | Indoor | Inventory | >10s | - |
| Device Activation and Deactivation | Indoor Outdoor | Sensor | NA | - |
| Fresh food supply chain | Indoor Outdoor | Inventory Sensor | >1minute | - |
| Forest fire monitoring | Outdoor | Sensor | >10sec | - |
| Smart agriculture | Indoor | Sensor | >1s | - |
| Museum guide | Indoor | Sensor | [2]s | - |
| Smart dairy farm | Outdoor | Sensor | >1s | Latency is not critical |
| Smart livestock farming (pig barns) | Indoor | Sensor | >10s | Latency is not critical for this use case |
| Smart manhole cover safety monitoring | Outdoor | Sensor | 10 - 30s | Latency is not critical. Per GB/T 41401[5], the max latency is smaller than 30 seconds |
| Smart bridge health monitoring | Outdoor | Sensor | 10s | Latency of smart bridge health monitoring is not critical, as the sensor information is needed by the monitoring application on a per-15 minutes basis |
| Elderly health care | Indoor Outdoor | Sensor | NA | - |
| End-to-end logistics | Indoor Outdoor | Positioning | NA | - |
| Pressure powered switch | Indoor | Command | NA | - |

Table 1. Ambient IoT use cases with Groups and latency requirement

It is observed that for the Grouping A, use cases incorporates indoor and outdoor with some use cases applied to both indoor and outdoor. Also, from 28 use cases from SA1, Grouping B can be categorized as 6 Inventory, 11 Positioning, 14 Sensors, 1 Command use cases with some use cases can apply multiple category. It can be also observed that the end-to-end latency requirement is not as stringent as what we target for 5G NR eMBB and URLLC use cases. With the most stringent latency requirement needs to meet below 100ms level, many use cases can be supported with below tens of seconds level of latency.

Observation 1: For Grouping B, 6 inventory, 11 positioning, 14 sensors, 1 command use cases with some use cases with multiple category can be identified

Proposal 1: Depending on the potential new use cases identified apart from TR 22.840, ‘command’ can be removed from Grouping B if no further use case is added/identified

Observation 2: From use cases in TR 22.840, end-to-end latency requirements varies from below 100ms to 1 minute

Observation 3: From use cases in TR 22.840, latency requirements can be considered not too stringent as what we can achieve with 5G NR

Based on Grouping A, the end-to-end latency requirement for the Indoor use cases ranged from below 100ms to 20 seconds. For outdoor use cases, the range varied from 1 to 30 seconds. For the use cases which can be applied both indoor and outdoor varied from hundreds of msecs. to 1 minute.

Observation 4: For grouping A, each category has different level of latency requirements

From above observations, KT proposes to add sub-clause on Latency under 5 RAN design targets and capture target latency requirements in TR 38.848 so that different latency requirements compared to typical 5G NR can be applied to Ambient IoT use cases.

Proposal 2: TR 38.848 to add sub-clause “5.z Latency” in RAN design target

# Conclusion

Based on the observations found in the above section, the following proposals are made as below:

Observation 1: For Grouping B, 6 inventory, 11 positioning, 14 sensors, 1 command use cases with some use cases with multiple category can be identified

Observation 2: From use cases in TR 22.840, end-to-end latency requirements varies from below 100ms to 1 minute

Observation 3: From use cases in TR 22.840, latency requirements can be considered not too stringent as what we can achieve with 5G NR

Observation 4: For grouping A, each category has different level of latency requirements

Proposal 1: Depending on the potential new use cases identified apart from TR 22.840, ‘command’ can be removed from Grouping B if no further use case is added/identified

Proposal 2: TR 38.848 to add sub-clause “5.z Latency” in RAN design target

# Reference

- RP-222685, New SID: Study on Ambient IoT, RAN#97e

- SP-220085, New SID: Study on Ambient power-enabled Internet of Things, SA#95e

- TR 22.840, Study on Ambient power-enabled Internet of Things (Release-19)

- RP-223451, [98e-11-Ambient-IoT] Extended round summary and GTW proposal, RAN#98e

- Intelligent manhole cover national standard, GB/T 41401, 2022, the Ministry of Housing and Urban-Rural Development of the People’s Republic of China