3GPP TSG RAN								RP230798-

Rotterdam, March 20-23, 2023

Agenda Item:	9.2.2

Source:	AT&T

Title:	TP for TR38.893: conclusions and recommendations

Document for:	Discussion/Approval

# Introduction

The SID for this study item [1] treats the potential for misalignment between 3GPP frequency bands and actual regional band allocations. Band n77 is cited as the case in point. The SID solicits solutions to the problem, including solutions not using new bands or new band numbers.

The study item has progressed through several meetings. Several perspectives have emerged regarding potential approaches for resolving similar problems should such problems arise in future frequency allocations [2,3].  The details of these approaches have been discussed in sufficient detail for the needs of the study item.

This document provides observations for the potential approaches and draws recommendations and conclusions from those proposals.

# Discussion

## New Bands vs. New Band Numbers

A distinction is made in [2] between New Band and New Band Number approaches. In either case, the specification of a regionally-defined subband which is a subrange of a 3GPP frequency band would result in a new 3GPP band identifier for the regulatory allocation. Introduction of a new band involves at least drafting a significant amount of standards text for the new band and may also complicate the specification of band combinations for carrier aggregation requirements.

It is clearly a duplication of work to repeat the same requirements for regionally-defined bands derived from existing 3GPP bands, and the only utility is to signal differences in certification status. This same utility can be achieved by the new band numbers approach. [2] suggests this approach would designate new band numbers, with the associated text simply referring to the “parent” band, i.e., the 3GPP band from which the smaller allocation was derived. In terms of protocol support, the approaches are roughly equivalent.

Observation 1:	The New Band Numbers provides the benefits of the New Bands approach with potentially less standards effort.

However, although implementation of the new band number approach may seem obvious, RAN does not have a model for this process.

Observation 2: RAN4, and potentially RAN2, would need to agree on the process for introducing new band numbers to incorporate regional subbands in the standards.

## Reuse NR band number with new signalling

[2] describes alternatives for augmenting existing signaling to indicate a) network identity to the UE to prevent the UE from camping on disallowed bands and b) UE band status to prevent the network from initiating a handover which is not allowed for the UE. Further work would be required to provide details for these alternatives before they could be used successfully to address new regionally-defined subband issues.

Observation 3:	Resolution of new regionally-defined subband issues using augmented signaling would require some study, preferably in RAN2, to select augmented signaling procedures for addressing new regionally-defined subbands.

## UE implementation

[3] suggests that regionally-defined subbands can be accommodated by reliance on UE implementation, possibly supported by signaling UE capabilities or by adding standards requirements. This places the onus on the UE to ensure that all regulatory requirements are met. This approach does require the UE to provide information regarding subband capabilities to the network. However, as UE predictability is important in regulatory matters, it is not sufficient to assume that UE implementations will always exhibit compliant behaviours.

Observation 4:	Some degree of standardization is necessary to ensure UEs comply with regional regulations.

## General observations

Each of the proposals for addressing future mismatches between regionally-defined subbands and 3GPP bands involves some amount of work and uncertainty. Each proposal has a mix of benefits and drawbacks. Selecting one option as the best would depend on the situation being addressed. If band mismatch becomes a frequent issue, then a generalized signaling-based solution could provide an efficient mechanism for managing a large, varied set of subbands. Alternately, if band mismatch turns out to be a rare problem, then a more constrained solution like new band numbers might be a more efficient solution.

Observation 5:	The best solution to the problem is dependent on the context of the problem.

In fact, the study item was predicated on the n77 situation, where different, non-overlapping subbands became available at different times. Future cases of band mismatches may present technical problems which are better addressed by one approach, while other cases might be best addressed by a different approach.

Observation 6:	There may not be a single best approach.

Based on the above observations, the study item can conclude with the following recommendations.

For band standardization issues which arise from cases where regionally-defined subbands do not occupy an entire 3GPP band and multiple definitions are likely:

Proposal 1:	The assignment of band numbers referring to pre-existing 3GPP “parent” bands is preferred over the specification of a new band for the regionally-defined band.

Proposal 2:	RAN2 and RAN4 should fully evaluate the specification, implementation, and test impacts of the new band number approach during the first work item associated with a regionally-defined subband. If the evaluation indicates that significant complexity is incurred due to the adoption of the new band number approach, RAN2 and RAN4 should also evaluate specification of new signalling as an alternative approach.

Proposal 3:	Care should be taken to ensure that the UE behaviour when operating within the regionally-defined subband is predictable.

# Conclusion

Observation 1:	The New Band Numbers provides the benefits of the New Bands approach with potentially less standards effort.

Observation 2:	RAN4, and potentially RAN2, would need to agree on the process for introducing new band numbers to incorporate regional subbands in the standards.

Observation 3:	Resolution of new regionally-defined subband issues using augmented signaling would require some study, preferably in RAN2, to select augmented signaling procedures for addressing new regionally-defined subbands.

Observation 4:	Some degree of standardization is necessary to ensure UEs comply with regional regulations.

Observation 5:	The best solution to the problem is dependent on the context of the problem.

Observation 6:	There may not be a single best approach.

Based on these observations, the study item can conclude with the following recommendations.

For band standardization issues which arise from cases where regionally-defined subbands do not occupy an entire 3GPP band and multiple definitions are likely:

Proposal 1:	The assignment of band numbers referring to pre-existing 3GPP “parent” bands is preferred over the specification of a new band for the regionally-defined band.

Proposal 2:	RAN2 and RAN4 should fully evaluate the specification, implementation, and test impacts of the new band number approach during the first work item associated with a regionally-defined subband. If the evaluation indicates that significant complexity is incurred due to the adoption of the new band number approach, RAN2 and RAN4 should also evaluate specification of new signalling as an alternative approach.

Proposal 3:	Care should be taken to ensure that the UE behaviour when operating within the regionally-defined subband is predictable.

# Reference

[1]	RP-223556, Revised SID: Study on UE support of regionally-defined subsets of an NR band, RAN#98-e, September 2022

[2]	RP-223547, Text proposal for Study on UE support of regionally-defined subsets of an NR band, RAN#98-e, December 2022

[3]	RP-223548, TP for TR 38.893 to introduce the potential solutions, RAN#98-e, December 2022

Text proposal:

7 Conclusions and recommendations

The study item has reviewed possible responses to situations in which regional regulatory bodies may allocate subbands of the existing 3GPP bands in a phased fashion. In these cases, UEs may be certified for different subsets of the 3GPP band(s) and must therefore operate only within the subbands for which certification has been received. Further, networks should not attempt to redirect these UEs to frequencies not in the subbands for which certification has been received. Different proposals for ensuring these behaviours are documented herein.

For band standardization issues which arise from cases where regionally-defined subbands do not occupy an entire 3GPP band, we offer the following conclusions.

Solutions A, B or C are valid approaches which could be used to solve the problem with the regional sub-bands. Solution BWhich method is chosen for a particular band can be decided on a case by case basis under a dedicated work item.  is expected to have a lower standardization workload and hence, should be adopted over Solution A if no issues are identified. However due to related workload, it is recommended that solution B with applying the same requirements (i.e., “parent” band requirements), is preferred over solution C if this approach is confirmed by the working groups. Which method  (e.g., new band number, new NS flag, new band, etc.) is chosen for a particular band can be decided on a case by case basis under a dedicated work item.

RAN2 and RAN4 should fully evaluate the specification, implementation, and test impacts of the new band number approach during the first work item associated with a regionally-defined subband and adopt an appropriate solution based on their evaluation.

Finally, it is recommended that care should be taken to ensure that the UE behaviour is predictable when operating within the regionally-defined subband and when the UE is operating in the parent band when it in a location where the regionally-defined subband is not applicable.