3GPP TSG RAN WG1 #100bis	       	R1-2001672

e-Meeting, April 20th – 30th, 2020

Source:	vivo

Title:	Enhancement for Scheduling/HARQ

Agenda Item:	7.2.5.4

Document for:	Discussion and Decision

- Introduction

In RAN1#99 meeting, enhancement for Scheduling/HARQ are discussed. The following agreements are achieved [1]:

Conclusion:

  - For Rel. 16 URLLC, no support of out-of-order/overlap PDSCH/HARQ and out-of-order/overlap PUSCH operation.

Conclusion

In Rel. 16 URLLC:

  - The UE is not expected to be scheduled with two DG-PUSCH overlap in the time domain on the same carrier.

In this contribution, we share our view on enhancement for Scheduling/HARQ.

- Two overlapping PUCCHs

In RAN1#99 meeting, a left issue is whether UE can be scheduled with two PUCCHs carrying HARQ-ACK with different priorities associated with two DG-PDSCHs scheduled on the same carrier overlapping in the time domain.

In Rel-15, for HARQ-ACK feedback with one priority, out-of-order HARQ may occur, i.e. gNB can indicate out of order HARQ-ACK timing within a slot. As shown in Figure 1, the starting symbol of PUCCH resource corresponding to the second PDSCH transmission is earlier than the start symbol of PUCCH resource corresponding to the first PDSCH. However, in Rel-15 UE will multiplex the two HARQ-ACK bits into one PUCCH resource within the slot such that there is no out-of-order issue for the actual PUCCH transmission. UE processes both PDSCH and feeds back HARQ-ACK using one PUCCH.

Figure 1 Rel-15 scheduling

For PUCCHs carrying HARQ-ACK with different priorities, it is not necessary to restrict that gNB can only schedule two non-overlapping PUCCH in time domain. In UCI enhancement section, it has been agreed if two PUCCHs carrying HARQ-ACK corresponding to the different priorities are overlapping in time, HARQ-ACK with lower priority would be dropped and HARQ-ACK with the higher priority would be transmitted. It can be supported for both slot and sub-slot granularity. Taken figure 2 as an example, for a UE with eMBB and URLLC services, URLLC DL service may arrive after an eMBB transmission. In this case, a UE can be scheduled with a URLLC PDSCH after an eMBB PDSCH is scheduled and HARQ-ACK for the URLLC PDSCH is fed back in the same sub-slot where the HARQ-ACK for the eMBB PDSCH is transmitted.

Figure 2 Rel-16 scheduling

Proposal 1: UE can be scheduled with two PUCCHs carrying HARQ-ACK with different priorities associated with two DG-PDSCHs scheduled on the same carrier overlapping in the time domain.

- Conclusion

In the contribution, we have some investigations on enhancement to scheduling/HARQ, and propose that,

Proposal 1: UE can be scheduled with two PUCCHs carrying HARQ-ACK with different priorities associated with two DG-PDSCHs scheduled on the same carrier overlapping in the time domain.

References

[1] Chairman notes RAN1 #99 meeting