3GPP TSG RAN WG1 #100bis		    R1-2001674

e-Meeting, April 20th – 30th, 2020

Source:	vivo

Title:	Enhanced UL configured grant transmissions for URLLC

Agenda Item:	7.2.5.6

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the remaining issues on enhanced UL configured grant for URLLC, including DMRS configuration for Type 2 CG activated by DCI format 0_2 in case antenna ports field is not present in DCI format 0_2, agreements that has not been captured in the specification and some editorial corrections.

- Discussion

  - DMRS configuration for Type 2 CG activated by DCI format 0_2

Based on [1] in RAN1#100-e meeting, the issue on how the UE determines the DMRS configuration for Type 2 CG activated by DCI format 0_2 when the Antenna ports field is not present in DCI format 0_2 with CRC scrambled by C-RNTI is postponed to be discussed in this meeting because it is better to wait for the decision made for dynamic grant scheduled by DCI format 0_2 first and then, it is more straightforward to apply the unified behaviour for both DG and CG.

For dynamic PUSCH scheduled by DCI format 0_2, it was agreed to introduce independent RRC parameters AntennaPorts-FieldPresence-ForDCIFormat0_2 for DCI format 0_2 and AntennaPorts-FieldPresence-ForDCIFormat1_2 for DCI format 1_2 [2]. Accordingly, TS 38.212 [3] captures above agreement for dynamic PUSCH as following:

7.3.1.1.3 Format 0_2 <Unchanged parts are omitted> - Antenna ports – number of bits determined by the following: - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured; - 2, 3, 4, or 5 bits otherwise, - 2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1; - 4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2; - 3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook. where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 and is configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. <Unchanged parts are omitted>

Based on above, it is straightforward to first agree following proposals:

Proposal 1: If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2,

- the UE determines that the bit width of the Antenna ports field in the DCI format 0_2 with CRC scrambled by CS-RNTI with NDI=0 is zero.

- the UE shall not use the parameter cg-DMRS-Configuration in configuredGrantConfig to derive the bit width of the Antenna ports field in DCI format 0_2 with CRC scrambled by CS-RNTI with NDI= 0.

- for the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of TS 38.212.

Drafted TP for above proposals are presented below:

7.3.1.1.3 Format 0_2 <Unchanged parts are omitted> - Antenna ports – number of bits determined by the following: - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured; - 2, 3, 4, or 5 bits otherwise, - 2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1; - 4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2; - 3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook. where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 and is configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with cg-DMRS-Configuration in configuredGrantConfig, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 for PUSCH transmission scheduled by DCI format 0_2 with CRC scrambled by CS-RNTI with NDI=0. <Unchanged parts are omitted> A UE does not expect that the bit width of a field in DCI format 0_2 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_2 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_2 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2, the UE determines that the bit width of the Antenna ports field in the DCI format 0_2 with CRC scrambled by CS-RNTI is zero. <Unchanged parts are omitted>

Proposal 2: adopt above TP in 38.212.

  - Missed agreement in specification

In RAN1 #AH 1901 meeting, following agreement was made for UL configured grant.

Agreement: - In Rel-16, for both Type 1 and Type 2 configured grant and when multiple active configurations are configured in a BWP, transmission of a TB based on the configured grant is associated with a single active configuration, even if the transmission is repeated

In current 38.214 [4], it seems the above agreement is not clearly captured in section 6.1.2.3. Therefore, we suggest to capture the following text in 38.214 to reflect the above agreement.

Proposal 3:  adopt the following TP in 38.214.

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant <Unchanged parts are omitted> For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberofrepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. If a UE is provided with more than one configuration for Type 1 and/or Type 2 configured grant, for a PUSCH transmissions with a configured grant configuration, when K ≥ 1, the UE shall transmit the TB across the transmission occasion candidates within the same configured grant configuration. <Unchanged parts are omitted>

  - Editorial correction

For Type 2CG and SPS activation and release PDCCH validation, the title for Table 10.2-1 ~ Table 10.2-3 may cause confusion on which table should be used when a UE is provided a single configuration and when a UE is provided more than one configuration. For example, the title for Table 10.2-2 is “Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation”, it seems the Table 10.2-2 can also be used as separate release in case a UE is provided more than one configuration. However, the intention is the Table 10.2-2 is only used for the case of a UE provided a single configuration. To make it clear, following was proposed:

Proposal 4: adopt following TP in 38.213.

#### 10.2 PDCCH validation for DL SPS and UL grant Type 2 A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or configured UL grant Type 2 PDCCH if - the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and - the new data indicator field in the DCI format for the enabled transport block is set to '0'. If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2. If a UE is provided more than one configurations for UL grant Type 2 PUSCH or for SPS PDSCH, a value of the HARQ process number field in a DCI format indicates an activation for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by Configuredgrantconfig-index or by SPSconfig-index, respectively. Validation of the DCI format is achieved if the RV field for the DCI format is set as in Table 10.2-3. If a UE is provided more than one configurations for UL grant Type 2 PUSCH or for SPS PDSCH - if the UE is provided Type2Configuredgrantconfig-ReleaseStateList or SPS-ReleaseStateList, a value of the HARQ process number field in a DCI format indicates a corresponding entry for scheduling release of one or more UL grant Type 2 PUSCH or SPS PDSCH configurations - if the UE is not provided Type2Configuredgrantconfig-ReleaseStateList or SPS-ReleaseStateList, a value of the HARQ process number field in a DCI format indicates a release for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by Configuredgrantconfig-index or by SPSconfig-index, respectively Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-4. If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format. Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration DCI format 0_0/0_1/0_2; DCI format 1_0/1_2; DCI format 1_1. HARQ process number; set to all '0's/0_2; set to all '0's; set to all '0's. Redundancy version; set to all '0's; set to all '0's; For the enabled transport block: set to all '0's. Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. HARQ process number; set to all '0's/0_1/0_2; set to all '0's. Redundancy version; set to all '0's; set to all '0's. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's; set to all '0’s for FDRA Type 0 set to all '1's for FDRA Type 1. Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations DCI format 0_0/0_1/0_2; DCI format 1_0/1_2; DCI format 1_1. Redundancy version; set to all '0's; set to all '0's; For the enabled transport block: set to all '0's. Table 10.2-4: Special fields for single or multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. Redundancy version; set to all ‘0’s; set to all ‘0’s. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's; set to all '0’s for FDRA Type 0 set to all '1's for FDRA Type 1. A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after symbols from the last symbol of a PDCCH providing the SPS PDSCH release. For UE processing capability 1 [6, TS 38.214] and for the SCS of the PDCCH reception, for 15 kHz, for 30 kHz, for 60 kHz, and for 120 kHz. For a UE with capability 2 [6, TS 38.214] in FR1 and for the SCS of the PDCCH reception, for 15 kHz, for 30 kHz, and for 60 kHz.

- Conclusion

In this contribution, we provide our views on remaining issues on UL configured grant. The proposals and corresponding TPs are summarized below.

Proposal 1: If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2,

- the UE determines that the bit width of the Antenna ports field in the DCI format 0_2 with CRC scrambled by CS-RNTI with NDI=0 is zero.

- the UE shall not use the parameter cg-DMRS-Configuration in configuredGrantConfig to derive the bit width of the Antenna ports field in DCI format 0_2 with CRC scrambled by CS-RNTI with NDI= 0.

- for the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of TS 38.212.

Proposal 2: adopt the following TP in 38.212.

7.3.1.1.3 Format 0_2 <Unchanged parts are omitted> - Antenna ports – number of bits determined by the following: - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured; - 2, 3, 4, or 5 bits otherwise, - 2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1; - 4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2; - 3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook. where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 and is configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with cg-DMRS-Configuration in configuredGrantConfig, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 for PUSCH transmission scheduled by DCI format 0_2 with CRC scrambled by CS-RNTI with NDI=0. <Unchanged parts are omitted> A UE does not expect that the bit width of a field in DCI format 0_2 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_2 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_2 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2, the UE determines that the bit width of the Antenna ports field in the DCI format 0_2 with CRC scrambled by CS-RNTI is zero. <Unchanged parts are omitted>

Proposal 3:  adopt the following TP in 38.214.

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant <Unchanged parts are omitted> For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberofrepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. If a UE is provided with more than one configuration for Type 1 and/or Type 2 configured grant, for a PUSCH transmissions with a configured grant configuration, when K ≥ 1, the UE shall transmit the TB across the transmission occasion candidates within the same configured grant configuration. <Unchanged parts are omitted>

Proposal 4: adopt following TP in 38.213.

#### 10.2 PDCCH validation for DL SPS and UL grant Type 2 A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or configured UL grant Type 2 PDCCH if - the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and - the new data indicator field in the DCI format for the enabled transport block is set to '0'. If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2. If a UE is provided more than one configurations for UL grant Type 2 PUSCH or for SPS PDSCH, a value of the HARQ process number field in a DCI format indicates an activation for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by Configuredgrantconfig-index or by SPSconfig-index, respectively. Validation of the DCI format is achieved if the RV field for the DCI format is set as in Table 10.2-3. If a UE is provided more than one configurations for UL grant Type 2 PUSCH or for SPS PDSCH - if the UE is provided Type2Configuredgrantconfig-ReleaseStateList or SPS-ReleaseStateList, a value of the HARQ process number field in a DCI format indicates a corresponding entry for scheduling release of one or more UL grant Type 2 PUSCH or SPS PDSCH configurations - if the UE is not provided Type2Configuredgrantconfig-ReleaseStateList or SPS-ReleaseStateList, a value of the HARQ process number field in a DCI format indicates a release for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by Configuredgrantconfig-index or by SPSconfig-index, respectively Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-4. If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format. Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration DCI format 0_0/0_1/0_2; DCI format 1_0/1_2; DCI format 1_1. HARQ process number; set to all '0's/0_2; set to all '0's; set to all '0's. Redundancy version; set to all '0's; set to all '0's; For the enabled transport block: set to all '0's. Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. HARQ process number; set to all '0's/0_1/0_2; set to all '0's. Redundancy version; set to all '0's; set to all '0's. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's; set to all '0’s for FDRA Type 0 set to all '1's for FDRA Type 1. Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations DCI format 0_0/0_1/0_2; DCI format 1_0/1_2; DCI format 1_1. Redundancy version; set to all '0's; set to all '0's; For the enabled transport block: set to all '0's. Table 10.2-4: Special fields for single or multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. Redundancy version; set to all ‘0’s; set to all ‘0’s. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's; set to all '0’s for FDRA Type 0 set to all '1's for FDRA Type 1. A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after symbols from the last symbol of a PDCCH providing the SPS PDSCH release. For UE processing capability 1 [6, TS 38.214] and for the SCS of the PDCCH reception, for 15 kHz, for 30 kHz, for 60 kHz, and for 120 kHz. For a UE with capability 2 [6, TS 38.214] in FR1 and for the SCS of the PDCCH reception, for 15 kHz, for 30 kHz, and for 60 kHz.

- References

- R1-2001424, ‘[URLLC-eCG-03] Email discussion/approval on DMRS configuration for Type 2 CG activated by DCI format 0_2,’ NTT DOCOMO Inc.

- R1-2001404, ‘Summary of email discussion [100e-NR-L1enh_URLLC_PDCCH-01] on remaining issues on DCI format,’ Huawei

- R1-2001463, ‘TS 38.212, v16.0.0, Corrections for URLLC after RAN1#100-e,’ RAN1

- R1-2001444, ‘TS 38.214, v16.0.0, Corrections on NR URLLC support,’ RAN1