3GPP TSG RAN WG1 #100bis	R1-2001878

e-Meeting, April 20th–April 30th, 2020

Agenda Item:	7.2.4.3

Source: 	Fujitsu

Title:	Remaining issues on sidelink synchronization procedure

Document for:	Discussion/Decision

Introduction

In RAN1#100-e online meeting, 3 issues identified as top priority went into email discussions, and the following agreements have been achieved [1]:

Agreement:

- Slot-level indication is supported in TDD configuration indication.

Agreements:

The TDD configuration indication is done as follows:

- X bits to indicate patterns + Y bits to indicate periodicity + Z bits to indicate UL slots.

  - FFS the values of X, Y and Z.

  - Total Z bits to indicate UL slots in pattern 1 and pattern 2 respectively if two patterns are configured.

If any TP, to be discussed next meeting.

Agreement:

- SL SSID is used for DM-RS sequence initialization in PSBCH.

Agreements:

The DM-RS sequence initialization for PSBCH is to be down-selected one from the following Alts:

- Alt 1: cinit=211iS-SSB+1NIDSL+1+26iS-SSB+1, where iS-SSB is 3 LSBs of S-SSB index.

- Alt 2: cinit=NIDSL

Agreement:

- Scrambling for PSBCH is processed after rate matching.

Agreement:

- NR PSBCH scrambling sequence is initialized with Cinit=NIDSL at the start of each PSBCH block.

Agreements:

- Downlink pathloss based S-SSB open-loop power control is supported.

- The transmission power is same for S-PSS, S-SSS, and PSBCH symbol.

Besides, there were some remaining issues identified but not treated due to no time in the last meeting [2]. In this document, we would like to share our views on how to determine the timing of an S-SS/PSBCH block in the sidelink synchronization procedures of a UE.

Discussion

In TS 38.213 subclause 16.1 [3], there are the following descriptions on how a UE determines the indexes of slots that include S-SS/PSBCH:

The UE determines indexes of slots that include S-SS/PSBCH block as NoffsetS-SSB+NintervalS-SSB⋅iS-SSB, where

-	index 0 corresponds to a first slot in a frame with SFN satisfying (SFN mod 16)=0

-	iS-SSB is a S-SS/PSBCH block index within the number of S-SS/PSBCH blocks in the period, with 0≤iS-SSB≤NperiodS-SSB-1

-	NoffsetS-SSB is a slot offset from a start of the period to the first slot including S-SS/PSBCH block, provided by timeOffsetSSB-SL

-	NintervalS-SSB is a slot interval between S-SS/PSBCH blocks, provided by timeIntervalSSB-SL

It captures the following agreement reached in the RAN1#99 meeting directly [4].

Agreements: - The S-SSBs within the period of 160ms are distributed with the same interval with the following (pre-)configured parameters: - The offset from the start of the S-SSB period to the first S-SSB - The interval between neighboring S-SSBs

According to the description in subclause 16.1, the slot index for S-SSB is a running index in a 160ms period. But referring to agreed PSBCH contents, the slot index in its payload is the index within a frame. As the definition of the indexes of slots in 38.213 is inconsistent with the definition of the slot index in PSBCH, the UE behavior for deriving the timing of transmitting S-SSBs and receiving S-SSBs needs to be further clarified in the text of the specification.

Figure 1. Slot indexes and SFNs of multiple S-SSBs in one S-SSB period

As shown in Figure 1, a configured pattern with 8 S-SSBs is illustrated for 60kHz subcarrier spacing of FR2. According to the current text in 38.213, a UE determines the slots with indexes of 33, 35, 37, 39, 41, 43, 45, 47 for transmitting 8 S-SSBs based on NoffsetS-SSB+NintervalS-SSB⋅iS-SSB. These slot indexes indicate the timing information of an S-SSB within a 160ms period, but not within a 10ms frame. A UE still need to derive the absolute timing information for each S-SSB either for transmitting or for receiving, which are slot index 33, 35, 37, 39 in the frame with (SFN mode 16)=1, and slot index 1, 3, 5, 7 in the frame with (SFN mode 16)=2 as illustrated in Figure 1.

More exactly, for transmitting an S-SSB, a UE should determine the SFN/DFN and the slot index in this frame that includes the S-SSB according to the configured high layer parameters including subcarrier spacing, number of S-SSB in a period, S-SSB offset and S-SSB interval. Then, the UE will use 10-bit to indicate the DFN and use 7-bit to indicate the slot index in this DFN in the contents of PSBCH. In the receiving procedure, a UE can determine the timing information of a receiving S-SSB by decoding the DFN and the slot index in this DFN carried by PSBCH directly. With the consideration of the half duplex constraint, a UE should select the synchronization resources which are different from the received synchronization resources to transmit S-SSB(s). In this sense, it is also necessary to align the definition of the slot index in PSBCH and the slot index in 38.213.

Therefore, the procedure of UE determining the indexes of slots where S-SS/PSBCH blocks are transmitted in TS 38.213 subclause 16.1 should be corrected, and we have the following proposal:

Proposal: The procedure of UE determining the indexes of slots that include S-SS/PSBCH block in TS 38.213 subclause 16.1 should include the determination of both the SFN that includes S-SS/PSBCH block(s) and the slot index in this SFN that includes S-SS/PSBCH block (see text proposals in Annex).

Conclusions

Based on the above discussion we offer the following proposal:

Proposal: The procedure of UE determining the indexes of slots that include S-SS/PSBCH block in TS 38.213 subclause 16.1 should include the determination of both the SFN that includes S-SS/PSBCH block(s) and the slot index in this SFN that includes S-SS/PSBCH block (see text proposals in Annex).

- Annex

Text proposal for TS 38.213

---------------------------------start of text proposal------------------------------------

### 16.1	Synchronization procedures

----------------------------------------unchanged parts omitted------------------------------------------

A UE is provided, by numSSBwithinPeriod-SL, a number NperiodS-SSB of S-SS/PSBCH blocks in a period of 16 frames. The UE assumes that a transmission of the S-SS/PSBCH blocks in the period is with a periodicity of 16 frames. The UE determines SFNs indexes of slots that include S-SS/PSBCH block by using as NoffsetS-SSB+NintervalS-SSB⋅iS-SSB, with the SFN satisfying,

-	SFN mod 16=(NoffsetS-SSB+NintervalS-SSB⋅iS-SSB)/Nslotframe,μ

and the UE determines the slot indexes in the frame with the SFN including S-SS/PSBCH block according to

-	(NoffsetS-SSB+NintervalS-SSB⋅iS-SSB )mod Nslotframe,μ

where,

-	index 0 corresponds to a first slot in a frame with SFN satisfying (SFN mod 16)=0

-	iS-SSB is an a S-SS/PSBCH block index within the number of S-SS/PSBCH blocks in the period, with 0≤iS-SSB≤NperiodS-SSB-1

-	NoffsetS-SSB is a slot offset from a start of the period to the first slot including S-SS/PSBCH block, provided by timeOffsetSSB-SL

-	NintervalS-SSB is a slot interval between S-SS/PSBCH blocks, provided by timeIntervalSSB-SL

-	Nslotframe,μ is the slot number within a frame for subcarrier spacing configuration μ

----------------------------------------unchanged parts omitted------------------------------------------

--------------------------------------end of text proposal-------------------------------------

Reference

[1] 3GPP TSG RAN WG1 Meeting #100-e, RAN1 Chairman’s Notes, e-Meeting, Feb.24th-Ma.6th, 2020

[2] R1-2000833, Feature lead summary on AI 7.2.4.3 #2 Sidelink synchronization, RAN1#100-e, Feb. 24th – Mar. 6th, 2020.

[3] 3GPP TS 38.213 V16.1.0 (2020-03).

[4] 3GPP TSG RAN WG1 Meeting #99, RAN1 Chairman’s Notes, Reno, USA, Nov. 18th – 22nd, 2019