3GPP TSG RAN WG1 Meeting #100bis-e                                                         	   R1-2001991

E-meeting, April 20th – 30th, 2020

Agenda Item:	7.2.2.2.5

Source: 	Intel Corporation

Title:	Wideband operation for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

This contribution provides clarification on intra-cell guard bands to be aligned with RAN2 agreement.

# Discussion

## Intra-cell guard bands

Current RAN1 spec 38.214 [1] describes the intra-carrier guard band using the starting CRB and ending CRB of the guard band but RAN2 agreed to use starting CRB and number of CRBs as given below. Therefore, the description in RAN1 spec needs to be updated accordingly and the corresponding TP is proposed in this contribution

RAN2 agreement RAN2#109e has discussed the signaling of guard bands and agreed on the following: - The guard bands for a cell are signalled by using a starting index and length for each guard band, only when the network wants to configure it. (FFS – move to offline) RAN2 should further discuss the signalling for the cases when there is no guard band, when RAN4 specs should be used, and when/if the UE does not support guard bands. This is captured in the CR as follows for the case RRC signals an actual guard band: IntraCellGuardBand-r16 ::= SEQUENCE (SIZE (1..ffsValue)) OF GuardBand-r16 -- FFS upper size 4, assuming 100Mhz cell GuardBand-r16 ::= SEQUENCE { startCRB-r16 INTEGER (0..ffsValue), --FFS upper range 275 nrofCRBs-r16 INTEGER (1..ffsValue) }

Proposal: The following TP is agreed for 38.214.

Text proposal for TS 38.214 # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by start CRB, GB sstart,μ and the number of CRBs GB ssizeend,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16 respectively. The intra-cell guard bands separate NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ , and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB sendstart,μ+GB ssize,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets of cardinality NRB-setBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.]

# Conclusions

It is proposed to agree on the TP provided in section 2.

# References

[1] 	3GPP TS 38.214 v16.1.0