3GPP TSG RAN WG1 #100bis	R1-2002126

# e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.4.2.2

Source: 	Samsung

Title: 	On Mode 2 for NR Sidelink

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about key remaining issues for Mode 2 resource allocation and provide our views with text proposals.

- Remaining issues for Mode 2 resource allocation

Remaining details of sensing and resource (re-)selection window

In RAN1#98bis [1], the following agreements were made for resource (re-)selection procedure as

- Resource (re-)selection procedure supports re-evaluation of Step 1 and Step 2 before transmission of SCI with reservation

  - The re-evaluation of the (re-)selection procedure for a resource reservation signalled in a moment ‘m’ is not required to be triggered at moment > ‘m – T3’ (i.e. resource reselection processing time needs to be ensured)

  - FFS relationship of T1 and T3, if any

Then, the followings were agreed for sensing and resource selection window in the email discussion of 98b-NR-16 [2]:

- For a given time instance n when resource (re-)selection and re-evaluation procedure is triggered

  - The resource selection window starts at time instance (n + T1), T1 ≥ 0 and ends at time instance (n + T2)

    - The start of selection window T1 is up to UE implementation subject to T1 ≤ Tproc,1

    - T2 is up to UE implementation with the following details as a working assumption:

      - T2 ≥ T2min

      - If T2min > Remaining PDB, then T2min is modified to be equal to Remaining PDB

      - FFS other details of T2min including whether the minimum window duration T2min - T1 is a function of priority

    - UE selection of T2 shall fulfil the latency requirement, i.e. T2 ≤ Remaining PDB

  - A sensing window is defined by time interval [n – T0, n – Tproc,0)

    - T0 is (pre-)configured, T0 > Tproc,0 FFS further details

  - FFS, if Tproc,0 and Tproc,1 are defined separately or as a sum

  - FFS relation of T3, Tproc,0, Tproc,1

  - Time instances n, T0, T1, T2, T2min are measured in slots, FFS Tproc,0 and Tproc,1

- A UE is expected to select resources for all intended (re-)transmissions within the PDB, i.e. the number of intended (re-)transmissions is an input to the resource (re-)selection procedure

In RAN1#99 [3], it was agreed that

- T0 is (pre)-configured between: 1000+[100]ms and [100]ms.

- T2min is (pre-)configured per priority indicated in SCI from the following set of values:

  - {1, 5, 10, 20}*2µ, where µ = 0,1,2,3 for SCS 15,30,60,120 respectively

Figure 1. Sensing window and resource selection window

Based on agreements above, Figure 1 describes the sensing windows and resource selection windows when resource (re-)selection is triggered in slot n and re-evaluation procedure is triggered in slot nresel>n.

First of all, the remaining issue on sensing window is to decide T0 and Tproc,0 values. For T0, there are brackets for long sensing window (1000+[100]ms) and short sensing window ([100]ms). We propose that T0 is (pre-)configured between 1000ms and 100ms since there is no motivation to add another 100ms for 1000ms long sensing window. In the current version of specification [4], the sensing window is defined by the range of physical slots [n –T0,n–Tproc,0). Since the reservation interval is defined as the number of logical slots and there may be SLSS slots and reserved slots configured, the accuracy of RSRP and SL RSSI measurement of some related resources in the sensing window may be degraded if sensing window is defined in physical slots. Therefore, the sensing window need to be defined as logical slots before slot ‘n’ as shown in Figure 1. Next, Tproc,0 is the time for processing sensing results. In LTE V2X, Tproc,0 is defined as one subframe. In NR V2X, multiple SCSs are supported and one slot for Tproc,0 may not be enough for larger SCS. Therefore, two slots for Tproc,0 can be considered for larger SCS such as 60 kHz and 120 kHz. However, if the short sensing window such as 100ms is configured, then one slot for Tproc,0 can be enough even for lager SCS. In this regard, Tproc,0 can be configurable between {1, 2} slots for larger SCS. Therefore, we propose:

Proposal 1: The followings are proposed for T0 and Tproc,0:

- T0 is (pre-)configured between 1000ms and 100ms.

- T0 is converted from units of ms to units of logical slots before slot n.

- Tproc,0 is defined as 1 slot for SCS {15, 30}kHz and (pre-)configured between {1, 2} slots for SCS {60, 120} kHz.

The following table provides the corresponding text proposal for Proposal 1:

Text proposal for Proposal 1: In TS 38.214 [4], Section 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 The following higher layer parameters affect this procedure: … - t0_processing: internal parameter Tproc,0 is set to the corresponding value from higher layer parameter t0_processing for the subcarrier spacing of the sidelink. … 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0) The UE shall monitor slots tn'-T0SL, tn'-T0+1SL, …, tn'-Tproc,0SLexcept for those in which its transmission occur, where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL where T0 is defined above and Tproc,10 is TBDfrom higher layer parameter t0_processing. The UE shall monitor slots which can belong to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. …

Secondly, the remaining issue on resource selection window is to decide for Tproc,1 value. Tproc,1 value should be decided considering required time for implementation related to resource selection procedure. In LTE V2X, Tproc,1 was defined as 4 subframes. In NR V2X, we can use this value. Since selection of T1 is up to UE implementation subject to T1 ≤ Tproc,1, the fixed 4ms for Tproc,1 is reasonable. Therefore, we propose:

Proposal 2: Tproc,1 defined as 4∙2μ physical slots where μ is obtained from the higher-layer parameter subcarrierSpacing-SL.

The following table provides the corresponding text proposal for Proposal 2:

Text proposal for Proposal 2: In TS 38.214 [4], Section 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 … The following steps are used: 1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot tySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1 , where Tproc,1 is TBD is 4∙2μ slots and μ is obtained from the higher-layer parameter subcarrierSpacing-SL; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). The total number of candidate single-slot resources is denoted by Mtotal. …

Remaining details on re-evaluation

For resource re-evaluation procedure, the followings were agreed in RAN1#100-e [5] as:

- For re-evaluation of a pre-selected resource contained in a slot ‘k’ to be first time signaled in a slot ‘m’, where k ≥ m,

  - Step 1 of the resource (re-)selection procedure is performed at least at the moment ‘m-T3’, and if the pre-selected resource is not in the identified candidate resource set, Step 2 is triggered for reselection of the resource

    - Re-evaluations before the moment ‘m-T3’ or after ‘m-T3’ but before ‘m’ are not precluded and are up to UE implementation

      - FFS whether to mandate a UE to perform Step 1 checking every slot before ‘m-T3’

    - FFS whether evaluation of Step 2 has to ensure any introduced timing restrictions between pre-selected and re-selected resources when re-evaluation is triggered, and whether it is allowed to change the pre-selected but not reserved resources which are still in the candidate resource set in order to ensure the timing restrictions

- FFS whether for the case of enabled periodic reservation, already reserved resources in upcoming periods can be re-evaluated

We first discuss about FFS points in the agreements above. At first, it was discussed whether a UE to mandate to perform every slot Step 1 checking before ‘m-T3’. This is to enhance the performance of resource re-selection. However, this would require increased UE processing burden. Current agreement allows this aggressive re-evaluation operation by UE implementation. Therefore, we do not support to mandate this feature. Secondly, it was agreed in the last meeting [5] that Step 2 has to ensure a minimum time gap ‘Z = a + b’ between any two selected resources of a TB for HARQ RTT. Therefore, this should be applied also for resource re-selection by re-evaluation procedure. Lastly, there is a FFS point whether to support re-evaluation for periodic reserved resource or not. However, we have agreed on resource pre-emption procedure to re-select reserved resource(s). Thus, there is no motivation additional application of re-evaluation for reserved resource(s). In these regards, we propose:

Proposal 3: For the FFS points on resource re-evaluation procedure in RAN1#100-e meeting,

- Every slot Step 1 checking before ‘m-T3’ should not be mandated for re-evaluation procedure since this requires increased UE processing burden.

- Re-evaluation of Step 2 has to ensure a minimum time gap Z = a + b between any two selected resources of a TB for HARQ RTT as agreed in RAN1#100-e meeting.

- Re-evaluation for periodically reserved resources is not supported because pre-emption procedure can be used for this purpose.

We think that text proposal is not necessary on Proposal 3.

Next for resource re-evaluation procedure, T3 value should be decided considering required time for implementation related to re-evaluation procedure. Specifically, T3 is the processing time related to resource re-selection after re-evaluation. Besides to resource selection time, T3 needs to consider additional processing time for dropping previously reserved resource(s) as well as overlapping between previously reserved resource(s) and newly selected resource(s). According to these aspects, we propose to define T3 such that T3 =T1+1 slots. Here, T1 is the selected processing time for resource selection by UE within upper bound Tproc,1 and 1 slot becomes an additional processing time required in re-evaluation procedure as explained above. Therefore, we propose:

Proposal 4: In resource re-evaluation procedure, T3 is T1 +1 slots where T1 is the selected processing time for resource selection by UE within upper bound Tproc,1.

The following table provides the corresponding text proposal for Proposal 4 and agreements for re-evaluation procedure made in RAN1#100-e [5]:

Text proposal for Proposal 4 and agreements on re-evaluation procedure in RAN1#100-e: In TS 38.214 [4], Section 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 … The UE shall report set SA to higher layers. After resource (re-)selection is triggered in slot n by higher layer and a pre-selected resource contained in a slot ‘k’ to be first time signaled in a slot ‘m’ where k≥m, the UE shall trigger resource re-evaluation in slot nresel>n. The re-evaluation is triggered at least a slot nresel=m-T3 where T3=T1 +1 slots. It is up to UE implementation that re-evaluation is performed before slot m-T3 or after slot m-T3 but before slot m. If the re-evaluation is triggered in slot nresel, step 1-7 are used with the following modifications: - n is replaced with nresel - SA is replaced with SB where SB is the set of the candidate single-slot resources for re-evaluation If the pre-selected resource is not in the set SB, the UE shall report set SB to higher layers.

Remaining details of resource (re-)selection procedure

In RAN1#98 [6], the following agreements were made for resource (re-)selection procedure as

- The resource (re-)selection procedure includes the following steps

  - Step 1: Identification of candidate resources within the resource selection window

    - FFS details

  - Step 2: Resource selection for (re-)transmission(s) from the identified candidate resources

    - FFS details

For Step 1 resource (re-)selection procedure, it was agreed in RAN1#99 [3] that

- On a per resource pool basis, when reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is enabled:

  - A period is additionally signalled in SCI and the same reservation is applied with respect to resources indicated within NMAX within window W at subsequent periods

  - A set of possible period values is the following: 0, [1:99], 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000 ms

    - <= 4 bits are used in SCI to indicate a period

    - An actual set of values is (pre-)configured

  - Regarding the number of periods

    - The number of remaining periodic reservations is not explicitly indicated in SCI

  - (working assumption) Procedure of mapping of periodic semi-persistent resources into the resource selection window is reused from LTE

    - By reusing TS 36.213, section 14.1.1.6, steps 5 and 6 of non-partial sensing, as applicable

In addition, the following agreement and conclusion were made in RAN1#100-e [5] as:

- On a per resource pool basis, when reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is enabled:

  - A set of possible period values additionally includes all integer values from 1 to 99 ms

- Evaluate till the next meeting whether given the agreed set of configurable reservation periodicities, the change to the exclusion procedure is necessary, wherein currently all configured period values are used for exclusion as inherited from LTE

Since all integer values from 1 to 99 ms are supported in NR sidelink, we need to discuss how to handle short reservation periodicities in resource (re-)selection procedure. According to the working assumption for reusing step 5 from LTE as described in the above, current version of specification [4] includes step 5 where a UE shall exclude any candidate single-slot resource for all possible reservation periodicity values if the UE has not monitored a certain slot in the sensing window due to its own transmission. This resource exclusion procedure was designed assuming the worst case where the UE does not know the reservation periodicity of the other possible colliding UE(s). However, if we apply this principle in NR considering short reservation periodicities, this may lead to over excluding resources. Thus, we need to change step 5 resource exclusion procedure. A simple solution is not to perform step 5 in NR resource (re-)selection procedure. In addition, we also have agreed to reuse step 6 from LTE as above. In step 6, the current specification [4] defines Tscal value in order to handle the overlapped slots in resource exclusion procedure but the value was not decided yet. Since upper bound of the resource selection window would be decide by remaining packet delay budget based on the agreement, we propose that Tscal becomes remaining packet delay budget in slots. Therefore, we propose:

Proposal 5: The followings are proposed for Step 1 resource (re-)selection procedure:

- Working assumption for reusing LTE step 5 from TS 36.213 is not applied in NR. Remove Step 5 in section 8.1.4 of TS 38.214.

- In order to handle the overlapped slots in the Step 1 resource identification procedure, Tscal is defined as remaining packet delay budget.

The following table provides the corresponding text proposal for Proposal 5:

Text proposal for Proposal 5: In TS 38.214 [4], Section 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 … The following higher layer parameters affect this procedure: … - reservationPeriodAllowed: this higher parameter provide an actual set of values from {0, 1:99, 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000}ms … The resource reservation interval, Prsvp_TX, if provided by higher layer from reservationPeriodAllowed, is converted from units of ms to units of logical slots, resulting in Prsvp_TX'. … 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot tmSL in Step 2. - for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed and a hypothetical SCI format 0-1 received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 56) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 0-1 in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 0-1 indicate the values Prsvp_RX and prioRX, respectively according to Clause [TBD] in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 0-1, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 0-1, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause [TBD] in [6, TS 38.213] the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0,1,…,Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is FFSremaining packet delay budget (in slots). …

Next, for Step 2 resource (re-)selection procedure, one of controversial issues in RAN1#100-e meeting was about UE behaviour on resource selection for a retransmission of a TB. In Step 2, it was agreed that randomized resource selection is used from the identified candidate resources from Step 1 resource (re-)selection procedure. In addition, resource reservation for feedback-based PSSCH retransmissions is supported by a prior SCI. Therefore, the following options were discussed to define a UE behaviour on resource selection for retransmission with a prior transmission of the same TB:

- Option 1: A sidelink resource for a retransmission of a TB shall be reserved by a prior SCI

- Option 2: A sidelink resource for a retransmission of a TB should be reserved by a prior SCI if the prior SCI is within distance of 32 logical slots from the resource for the retransmission.

Option 1 is about resource selection to secure chain reservation by SCI. Therefore, Option 1 mandates the UE to select retransmission resources always within distance of 32 logical slots from the prior transmission. However, in Option 1, the chain can be broken by re-evaluation and/or pre-emption procedure. Furthermore, we do not want to give a restriction on UE implementation of resource selection procedure. In this aspect, our preference is Option 2 where chain reservation is allowed when the prior transmission is within distance of 32 logical slots from the selected resource for the retransmission.

Proposal 6: A sidelink resource for a retransmission of a TB should be reserved by a prior SCI if the prior SCI is within distance of 32 logical slots from the resource for the retransmission.

In addition, text proposal on the following agreement was not made in the last meeting as:

- In Step 2, a UE ensures a minimum time gap Z = a + b between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected

  - ‘a’ is a time gap between the end of the last symbol of the PSSCH transmission of the first resource and the start of the first symbol of the corresponding PSFCH reception determined by resource pool configuration and higher layer parameters of MinTimeGapPSFCH and periodPSFCHresource

  - ‘b’ is a time required for PSFCH reception and processing plus sidelink retransmission preparation including multiplexing of necessary physical channels and any TX-RX/RX-TX switching time and is determined by UE implementation

The following table provides the corresponding text proposal on Step 2 resource (re-)selection procedure agreed in RAN1#100-e [5]:

Text proposal for agreements on Step 2 resource (re-)selection procedure in RAN1#100-e: In TS 38.214 [4], add Section 8.1.X Condition for selecting resources when the number of HARQ transmissions is more than two in sidelink resource allocation mode 2 If PSFCH transmission in the resource pool is enabled, the UE shall ensure a minimum time gap k=a+b between any two transmission opportunities of PSSCH where - a is a time gap between the end of the last symbol of the PSSCH transmission of the first resource and the start of the first symbol of the corresponding PSFCH reception in slots determined by resource pool configuration and high layer parameter of MinTimeGapPSFCH and periodPSFCHresource. - b is a time required for PSFCH reception and processing plus sidelink retransmission preparation including multiplexing of necessary physical channels and any TX-RX/RX-TX switching time and is determined by UE implementation. …

Remaining details of pre-emption

For resource pre-emption procedure, the followings were agreed in RAN1#100-e [5] as:

- For pre-emption, both full and partial frequency domain overlap in the same slot are considered as the overlapping condition to trigger resource reselection, wherein the whole resource is reselected even if the partial overlap happened

- (Re-)selection procedure for an already reserved but pre-empted resource to be used for transmission in a slot ‘m’ is not required to be triggered at moment > ‘m – T3’

  - T3 here is identical to T3 introduced for the re-evaluation

- FFS whether re-selection of the already-reserved, but pre-empted resource applies only to the resource transmitted in slot ‘m’ or to other already-reserved and pre-empted resource(s) signaled in the SCI in slot ’m’ as well

In order to support the resource pre-emption mechanism, we need to decide detailed conditions for TX resource (re-)selection. Simply, we can reuse Mode 2 resource (re-)selection and re-evaluation procedure for pre-emption. However, RSRP threshold for pre-emption can be configured separately. This means that RSRP threshold for pre-emption can be different from that of resource (re-)selection and re-evaluation procedure. In RAN1#100-e meeting, there was a proposal that an actual pre-emption depends on priority level. For example, even though pre-emption is enabled by higher layer, pre-emption cannot be activated for UE with high priority. However, it was already agreed that re-selection trigger for pre-emption is occurred only when other UE’ priority for overlapped resource is higher and SL-RSRP measurement for overlapped resource is larger than an associated SL-RSRP threshold. Therefore, additional condition such as priority dependent pre-emption activation is not necessary. In addition, there is one FFS point in the agreement above. Between two options, our preference is that re-selection of the already-reserved, but pre-empted resource applies to other already-reserved and pre-empted resource(s) signaled in the SCI in slot ’m’ as well. In summary, we propose:

Proposal 7: The followings are proposed for resource pre-emption procedure:

- If a resource pre-emption is enabled in resource pool and the pre-emption is triggered, UE behaviour follows Mode-2 resource (re-)selection and re-evaluation procedure with separate RSRP threshold for pre-emption.

- Priority dependent pre-emption activation is not supported.

- Re-selection of the already-reserved, but pre-empted resource applies to other already-reserved and pre-empted resource(s) signaled in the SCI in slot ’m’ as well.

- Conclusions

This contribution discusses on issues for maintenance of Mode 2 resource allocation in NR sidelink and proposes the followings depending on the discussion:

Proposal 1: The followings are proposed for T0 and Tproc,0:

- T0 is (pre-)configured between 1000ms and 100ms.

- T0 is converted from units of ms to units of logical slots before slot n.

- Tproc,0 is defined as 1 slot for SCS {15, 30}kHz and (pre-)configured between {1, 2} slots for SCS {60, 120} kHz.

Proposal 2: Tproc,1 defined as 4∙2μ physical slots where μ is obtained from the higher-layer parameter subcarrierSpacing-SL.

Proposal 3: For the FFS points on resource re-evaluation procedure in RAN1#100-e meeting,

- Every slot Step 1 checking before ‘m-T3’ should not be mandated for re-evaluation procedure since this requires increased UE processing burden.

- Re-evaluation of Step 2 has to ensure a minimum time gap Z = a + b between any two selected resources of a TB for HARQ RTT as agreed in RAN1#100-e meeting.

- Re-evaluation for periodically reserved resources is not supported because pre-emption procedure can be used for this purpose.

Proposal 4: In resource re-evaluation procedure, T3 is T1 +1 slots where T1 is the selected processing time for resource selection by UE within upper bound Tproc,1.

Proposal 5: The followings are proposed for Step 1 resource (re-)selection procedure:

- Working assumption for reusing LTE step 5 from TS 36.213 is not applied in NR. Remove Step 5 in section 8.1.4 of TS 38.214.

- In order to handle the overlapped slots in the Step 1 resource identification procedure, Tscal is defined as remaining packet delay budget.

Proposal 6: A sidelink resource for a retransmission of a TB should be reserved by a prior SCI if the prior SCI is within distance of 32 logical slots from the resource for the retransmission.

Proposal 7: The followings are proposed for resource pre-emption procedure:

- If a resource pre-emption is enabled in resource pool and the pre-emption is triggered, UE behaviour follows Mode-2 resource (re-)selection and re-evaluation procedure with separate RSRP threshold for pre-emption.

- Priority dependent pre-emption activation is not supported.

- Re-selection of the already-reserved, but pre-empted resource applies to other already-reserved and pre-empted resource(s) signaled in the SCI in slot ’m’ as well.

References

3GPP TSG RAN1#98bis, Final Report of 3GPP TSG RAN.

98b-NR-16, Email discussion on resource selection and sensing windows bounds.

3GPP TSG RAN1#99, Final Report of 3GPP TSG RAN.

TS 38.214, “NR; Physical layer procedures for data,” v16.0.0.

3GPP TSG RAN1#100-e, Final Report of 3GPP TSG RAN.

3GPP TSG RAN1#98, Final Report of 3GPP TSG RAN.