3GPP TSG RAN WG1 #100bis-e	R1-2002150

# e-Meeting, April 20th-30th, 2020

Agenda Item:		       7.2.11.1

Source:			Samsung

Title:		     UE features for two-step RACH

Document for:		Discussion and Decision

# Introduction

This contribution discusses the remaining issues on UE features for 2step RACH.

# Feature Group 9-1

During the previous email discussion on UE features, we have submitted our comments for the initial version on the 2step RACH UE feature. A general comment shared by many companies is that the initial version is somehow too detail especially for feature group (FG) 9-1. With that understanding, the updated UE feature list provides two types of summary for the 2step RACH: 1) Alt.1 - Detailed feature group, 2) Alt. 2 - Simplified basic feature group

Alt.1 is still the “detailed” summary with reduction from the initial version, and Alt.2 is super simple summary which barely gives any details on what is needed for 2step RACH. That drives us to think about what is qualified to be listed in the UE feature. One of the most important symptoms is that it requires a UE to do something new rather than a UE is capable of doing. E.g., for 2step RACH, we think it might be good to capture these items requiring UE to have new behaviour than a Rel-15 UE. With above said, we think the Alt.1 might be more appropriate and a further reduction on the lists are needed.

Proposal 1: For FG 9-1, Alt.1 should be supported and further reduction on the list is needed.

Following this logic, we have the comments for the each item in FG 9-1 for 2step RACH as shown in the following table.

Table 1 – discussion on UE feature group 9-1 for 2step RACH

| Index | Listed UE feature in Alt.1 | New features compared to Rel-15 UE? | To be kept? |
| --- | --- | --- | --- |
| 1 | RACH type selection based on a SSB-based RSRP threshold | Yes. Rel-15 did not do RACH type selection. | Yes |
| 2 | Separately configured ROs not applicable to 4-step RO configuration | No. this separate configuration is actually the same manner of that for 4step RACH, just use different parameters. | No |
| 3 | Same ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration | Yes. Rel-15 UE did not find the start of preamble set by the end of another preamble set. | Yes |
| 4 | Maximum two MsgA PUSCH configurations in an UL BWP from UE perspective | Yes. The msgA PUSCH configuration is new to Rel-15 UE. | Yes |
| 5 | Validation of MsgA PRACH and PUSCH | Yes, the PUSCH validation is new to Rel-15 UE. | Yes |
| 6 | Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH | Yes, this mapping between preamble and PUSCH is different from SSB-RO mapping for Rel-15 UE. | Yes |
| 7 | MsgA PUSCH transmission including scrambling, DMRS sequences and ports, RV, etc | No. only the parameters are different. | No |
| 8 | MsgA open loop power control | No. only the parameters are different. | No |
| 9 | Monitoring of MsgB with PDCCH addressed to msgB-RNTI or C-RNTI, and receiving MsgB including SuccessRAR, fallbackRAR, and backoff indication | Yes, Rel-15 UE only need to monitor the msg.2 with RA-RNTI or using C-RNTI. But Rel-16 2step RACH requires UE to monitor the msgB by C-RNTI and msgB-RNTI. | Yes |
| 10 | Support PUCCH transmission for HARQ-ACK feedback to a MsgB | Yes, the PUCCH resource determination method is different from Rel-15. | Yes |
| 11 | MsgA PRACH configuration and preamble formats | Yes, with removing the preamble formats, which are the same with Rel-15. | Yes, but MsgA PRACH configuration |
| 12 | PUCCH power control for HARQ-ACK feedback to a MsgB | No, the power control principle is the same as Rel-15, the TPC command is indicated by the same manner even though the TPC command is coming from new types of RAR for 2step RACH. | No |
| 13 | Minimum TX gap between PRACH and PUSCH (for both TDD and FDD, FR1 and FR2, single CC and intra-band CA) as specified in Rel-15 | NO. | No |
| 14 | Minimum TX gap between last DL SSB reception symbol and PRACH (TDD. FR1 and FR2, single CC and intra-band CA) as specified in Rel-15 | NO. | No |
| 15 | MsgA PRACH and PUSCH transmissions in different PRACH and PUSCH slots | NO. | No |
| 16 | Minimum TX gap between the last symbol of MsgB PDSCH and the first symbol of PUCCH carrying HARQ-ACK to MsgB PDSCH | NO. | No |
| 17 | 2-step RACH operation in RRC_IDLE/INACTIVE/CONNECTED state | Yes, some BWP could have 2step RACH only. | Yes |
| 18 | Intra-slot frequency hopping for MsgA PUSCH | No, Intra-slot frequency hopping is also the mandatory feature for Rel-15 UE. | No |

Proposal 2: Adopt the revised UE feature group in the appendix by removing item 2),7),8),12),13),14),15),16),18) in FG9-1 for 2step RACH Rel-16 and modify the 11) to be “MsgA PRACH configuration ”.

# Conclusion

In this contribution, considerations on 2step RACH UE feature are presented. In particular, the following are proposed:

Proposal 1: For FG 9-1, Alt.1 should be supported and further reduction on the list is needed.

Proposal 2: Adopt the revised UE feature group in the appendix by removing item 2),7),8),12),13),14),15),16),18) in FG9-1 for 2step RACH Rel-16 and modify the 11) to be “MsgA PRACH configuration ”.

# Reference

- TS38.213, “Physical layer procedures for control”, V16.1.0.

# Appendix- 2step RACH UE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - [RACH type selection based on a SSB-based RSRP threshold] - [Same ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration] - [Maximum two MsgA PUSCH configurations in an UL BWP from UE perspective] - [Validation of MsgA PRACH and PUSCH] - [Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH] - [Monitoring of MsgB with PDCCH addressed to msgB-RNTI or C-RNTI, and receiving MsgB including SuccessRAR, fallbackRAR, and backoff indication] - [Support PUCCH transmission for HARQ-ACK feedback to a MsgB] - [MsgA PRACH configuration] - [2-step RACH operation in RRC_IDLE/INACTIVE/CONNECTED state] |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-2 | Supported 2 symbols DMRS for msgA PUSCH | Supported 2 symbols DMRS for msgA PUSCH (‘len2’) | 9-1, 2-18 | Yes | N/A | If UE does not support ‘len2’, and if msgA-maxLength is configured as ‘len2’, the UE cannot use 2-step RACH resources | per UE | No | Yes |  | Shall be aligned with 2-18 | Conditionallymandatory for UE supporting both 9-1 and 2-18 |
|  | 9-3 | Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel MsgA and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell | 9-1 | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | per band combination | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-4 | MsgA operation in a band combination including SUL | MsgA operations in a band combination including SUL | 9-1, 6-16 | Yes | N/A | UE does not support msgA operations in a band combination including SUL | per band combination | N/A | N/A |  |  | Optional with capability signalling |