3GPP TSG RAN WG1 #100bis		R1-2002183

e-Meeting, April 20th – 30th, 2020

Agenda item:	6.2.5.3

Source: 	Qualcomm Incorporated

Title: 	UE features for MIMO

Document for:	Discussion / Decision

This document contains our comments for the UE feature list for LTE MIMO, based on the version shared in RAN1 reflector on April 8th. The comments are introduced using track changes with respect to the baseline version.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.LTE DL MIMO efficiency enhancements | 3-1 | Additional SRS symbols within normal UL subframes | 1. Support of additional 1~13 SRS symbols within normal UL subframes with repetitions, | SRS | Yes | N/A | Network cannot utilize additional SRS symbols within normal UL subframes | Per BoBC | N/A | N/A | UE reports the maximum number of SRS symbols the UE can transmit in one subframe, | Optional with capability signalling |
|  | 3-1A | Additional SRS symbols within normal UL subframes with frequency hopping | with frequency hopping | 3-1 | Yes | N/A | Network cannot utilize frequency hopping for additional SRS symbols within normal UL subframes | Per BoBC | N/A | N/A | UE reports up to 2 SRS bandwidth thresholds, and for the 3 sets of bandwidth, whether it supports FH with gaps, FH without gaps, or none. Each of these is reported separately for the case of repetition and no repetition. | Optional with capability signalling |
|  | 3-1B | Additional SRS symbols within normal UL subframes with antenna switching | With antenna switching | 3-1 | Yes | N/A | Network cannot utilize antenna switching for additional SRS symbols within normal UL subframes | Per BoBC | N/A | N/A | UE reports, per BoBC, whether it supports 1T2R, 1T4R, 2T4R (2/3 pairs), or no switching. UE reports whether it needs gaps or not, for the case of repetition and no repetition. | Optional with capability signalling |
|  | 3-2 | Virtual cell Id | 1. Support of virtual cell ID for legacy (Rel-15 and earlier releases) SRS. 2. Support of virtual cell ID for additional SRS symbol(s) within normal UL subframes. | SRS | Yes | N/A | Network cannot utilize the virtual cell ID for SRS | Per BoBC | N/A | N/A | FFS whether component 2 is to be a separate FG. | Optional with capability signalling |