3GPP TSG RAN WG1 #100bis-e		                                                 R1-2002480

e-Meeting, April 20th – 30th, 2020

Agenda item: 	7.2.11.2

Source: 	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR Unlicensed

Document for:	Discussion and Decision

# Introduction

The Rel-16 UE feature e-mail discussions have gone through two phases in RAN1 e-mail reflector, with the current proposals available in [1]. In this contribution we present our further views regarding the UE features for NR Unlicensed.

# Discussion

The detailed comments are provided below following the numbering of features in [1], and which can be found in the Annex for reference:

- 10-1/10-1a/10-2/10-2a: Missing clear relationship with 10-30 (COT duration). It should be a pre-requisite at least for 10-1/10-1a.

- 10-1, components 8, 9, 10: remove “with Q” or clarify the text so that it becomes self-contained.

- 10-1a: remove “with Q” or clarify the text so that it becomes self-contained.

- 10-2:

  - Components 4, 5, 6: remove “with Q” or clarify the text so that it becomes self-contained.

  - Component 8: add 2ms support

- 10-2a:

  - Component 1: remove “with Q” or clarify the text so that it becomes self-contained.

  - Component 2: add 2ms support

- 10-2b: it is unclear why this component would be needed.

- 10-3a/3b/3c: to be merged as a single feature group as originally proposed. If interlace is required it is required for all formats.

- 10-8: Preference to have length 9/10 included in this feature for clarity. However, if length 9/10 is assumed to be covered by 14-3 then it needs to be added as a pre-requisite.

- 10-9/10-9a/9b 10-9b should be baseline and required for UEs implementing 10-9 or 10-9a.

- 10-12: It should be combined with 10-3a/b/c (PUCCH component).

- 10-14: fix typo “imapplicable time -> inapplicable time”. It is OK to support it for licensed use as well.

- 10-15: Remove “FFS if need to further split under other group DAI/NFI configured or not”

- 10-16 and 10-16a: to be merged into a single feature

- 10-17: It is OK to support it for licensed use as well.

- 10-19: Numbers need to be more clear, otherwise we need to assume maximum is supported always. Is this applicable for all LBT types with ED measurement equally?

- 10-19b: Feature is not needed as it is not fundamentally different single-RB set operation.

- 10-19c: Feature is under discussion in R16 NR-U maintenance.

- 10-20a: It is OK to support it for licensed use as well.

- 10-31: Feature is under discussion in R16 NR-U maintenance.

# Summary

In this contribution we presented our proposals on UE features for NR Unlicensed.

# References

[1] R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, AT&T, NTT DOCOMO, April 2020

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-1 | UE stand-alone (DL and UL) operation in shared spectrum under dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access (FFS if move this to separate feature) 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. Contention window adjustment 7. CP extension up to 1 symbol for PUSCH/PUCCH transmission 8. SSB/MIB/RMSI reception with Q 9. SSB RRM with Q in DMTC 10. SSB-RLM with Q in DMTC window 11. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band with both DL and UL transmission support under dynamic channel access | Optional with capability signalling |
|  | 10-1a | UE DL only operation in shared spectrum under dynamic channel access mode | 1. SSB RRM with Q in DMTC | 6-5 |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band with DL only operation | Optional with capability signalling |
|  | 10-2 | UE stand-alone (DL and UL) operation in shared spectrum under semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth 4. SSB/MIB/RMSI reception with Q 5. SSB RRM with Q in DMTC 6. SSB-RLM with Q in DMTC window 7. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 8. Support fixed frame period of 5ms and 10ms |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for FBE operation, including fixed frame period, Cat 2 LBT, Cat 1 LBT | Optional with capability signalling |
|  | 10-2a | UE DL only operation in shared spectrum under semi-static channel access mode | 1. SSB RRM with Q in DMTC 2. Support fixed frame period of 5ms and 10ms | 6-5 |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band with DL only operation | Optional with capability signalling |
|  | 10-2b | UE stand-alone (DL and UL) operation in shared spectrum under semi-static channel access mode | 1. Support fixed frame periods shorter than 5ms | 10-2 or 10-2a |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-3 | PRB interlace mapping for PUSCH | 1. PRB interlace frequency domain resource allocation for PUSCH | 10-1 or 10-2 Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  | Support of PRB interlace PUSCH | Optional with capability signalling |
|  | 10-3a | PRB interlace mapping for PUCCH format 0 and format 1 | 1. PRB interlace frequency domain resource allocation for PUCCH format 0 and format 1 | 10-1 or 10-2 Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  | Support of PRB interlace PUCCH format 0/1 | Optional with capability signalling |
|  | 10-3b | PRB interlace mapping for PUCCH format 2 | 1. PRB interlace frequency domain resource allocation for PUCCH format 2 | 10-1 or 10-2 Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  | Support of PRB interlace PUCCH format 2 | Optional with capability signalling |
|  | 10-3b | PRB interlace mapping for PUCCH format 3 | 1. PRB interlace frequency domain resource allocation for PUCCH format 3 | 10-1 or 10-2 Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  | Support of PRB interlace PUCCH format 3 | Optional with capability signalling |
|  | 10-7 | LBT bandwidth size of 10MHz | FFS the components | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-8 | Type B PDSCH length | Length 3, 5, 6, 8, 11, 12, 13 FFS the capability are separate for each length or some groups are formed to signal the capability together |  | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Support of additional length (other than 2/4/7) for type B PDSCH. Note length 9/10 are already covered by 14-3 | Optional with capability signalling |
|  | 10-9 | Search space set group switching with explicit DCI 2_0 bit field trigger | 1. Two groups of search space sets 2. Monitor DCI 2_0 with a search space set switching field 3. Support a timer to switch back to original search space set group 4. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | FFS: Per UE or per band or per BC | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
|  | 10-9a | Search space set group switching with implicit PDCCH decoding with DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group 4. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
|  | 10-9b | Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
|  | 10-9c | Joint search space group switching across multiple cells | 1. Configured with a group of cells and switch search space set group jointly over these cells | 10-9, 10-9a, or 10-9b | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Without this capability, the UE will switch search space set groups for different cells independently | Optional with capability signalling |
|  | 10-10 | RSSI and channel occupancy measurement and reporting | 1. RSSI measurement 2. Channel occupancy reporting | 10-1, 10-1a, 10-2, or 10-2a | Yes | N/A |  | per band | N/A | N/A |  | RSSI measurement for channel occupancy. | Optional with capability signalling |
|  | 10-11 | SRS starting position at any OFDM symbol in a slot |  |  | Yes | N/A |  | Per band or per UE | N/A | N/A |  | Support transmitting SRS starting in all symbols (0,…,13) of a slot | Optional with capability signalling |
|  | 10-12 | OCC for PRB interlace mapping for PF2 and PF3 FFS if need this capability, or if we split this for PF2 and PF3 separately | 1. OCC2 2. OCC4 | 10-3b or 10-3c | Yes | N/A |  | Per band | N/A | N/A |  | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |
|  | 10-13a | Extended CP range of more than one symbol for CG-PUSCH FFS if need this capability |  | 10-1, Do we need this for 10-2? 5-19 or 5-20 | Yes | N/A |  | Per band | N/A | N/A |  | How long a UE can generate the CP extension beyond 1 symbol for CG-PUSCH | Optional with capability signalling |
|  | 10-14 | Non-numerical PDSCH to HARQ-ACK timing | 1. Support configuration of a value for dl-DataToUL-ACK indicating an imapplicable time to report HARQ ACK | 10-1 or 10-2 Need discussion for licensed use | Yes | N/A |  | Per band or per UE | N/A | N/A |  | If non-numerical K1 value is supported | Optional with capability signalling |
|  | 10-15 | Enhanced dynamic HARQ codebook | 1. Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 2. Support of bit field in DCI 0_1 for other group total DAI if configured. 3. Support the retransmission of HARQ ACK FFS if need to further split under other group DAI/NFI configured or not | 10-1 or 10-2 Need discussion for licensed use | Yes | N/A |  | Per band or per UE | N/A | N/A |  | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |
|  | 10-16 | One-shot HARQ ACK feedback | 1. Support feedback of HARQ-ACK codebook containing all configured HARQ processes for all configured CCs, triggered by a DCI 1_1 scheduling a PDSCH | 10-1 or 10-2 Need discussion for licensed use | Yes | N/A |  | Per band or per UE | N/A | N/A |  | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
|  | 10-16a | One-shot HARQ ACK feedback trigger with empty DCI 1_1 | 1. Support feedback of HARQ-ACK codebook containing all configured HARQ processes for all configured CCs with a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value | 10-16 Need discussion for licensed use | Yes | N/a |  | Per band or per UE | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-17 | Multi-PUSCH UL grant | 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1 | 10-1 or 10-2 Need discussion for licensed use | Yes | N/A |  | Per band or per UE | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-18 | Configured grant with retransmission in CG resources | 1. Support retransmission in CG resources 2. Support configured grant retransmission timer 3. Support DFI monitoring 4. Support CG-UCI in CG-PUSCH | 10-1 or 10-2, 5-19 or 5-20 Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
|  | 10-19 | Number of LBT bandwidth FFS if this is needed | Number of ED measurements the UE is able to perform simultaneously [Whether the UE in WB operating mode can support nx20MHz measurement | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | This is the number of LBT bandwidth a UE can perform separate ED check on simultaneously | Optional with capability signalling |
|  | 10-19a | Support DL reception with subset of RB sets | 1. When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will assume all RB sets in the DL BWP are all transmitted or none of them are transmitted | Optional with capability signalling |
|  | 10-19b | Support UL transmission with subset of RB sets passing LBT | 1. When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only the RB sets the UL signals or channels are located | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will transmit UL when all RB sets in the UL BWP pass LBT | Optional with capability signalling |
|  | [10-19c] | Suppport intra-cell guard band(s) for DL carrier BW > 20MHz | Support DL frequency domain resource allocation type 0 with PRG partially overlapped with Type-2 intra-cell gard band between two adjacent LBT RB sets where CCA is clear on only one of RB sets as defined by RAN4, and rate matching around the guard bands either at RB level (in case of wideband precoding) or at PRG level (in case of precoding granularity of 2 or 4 RBs) according to the information provided by DCI Format 2_0 available RB set bitmap and RRC intra-cell guard band configurations | 10-1a | Yes | N/A |  | [Per band] | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-20 | Support search space set configuration with freqMonitorLocation-r16 | 1. Support search space set configuration with freqMonitorLocations-r16 | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-20a | Support coreset configuration with rb-Offset | 1. Support coreset configuration with rb-Offset | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-21 | Support using ED threshold for UL to DL COT sharing | 1. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL to share COT with gNB for DL 2. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for CG-PUSCH to share COT with gNB for DL 3. Indicate in CG-UCI the COT sharing information as configured in cg-COT-Sharing-r16 | 10-1 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-22 | No gap 2-step RACH msgA transmission | 1. Support transmitting PRACH and PUSCH of msgA without gap in between FFS if RAN1 can disallow this in NR-U. | 10-1 or 10-2 and 9-1 | Yes | N/A |  | Per band | N/A | N/A |  | For licensed case, a minimum gap between PRACH and PUSCH of msgA is introduced, but is not applicable to NR-U | Optional with capability signalling |
|  | 10-23 | CGI reading based on off-sync raster SSB for ANR functionality | 1. Support acquisition of relevant information from a neighbouring NR unlicensed cell in an unlicensed carrier by reading the RMSI of the neighbouring unlicensed cell and reporting the acquired information to the network | 10-1, 10-1a, 10-2, or 10-2a | Yes | N/A |  | Per band | N/A | N/A |  | Support reading RMSI from SCell from an off-sync raster SSB for ANR | Optional with capability signalling |
|  | 10-24 | CG-UCI multiplexing with HARQ ACK | 1. Support multiplexing CG-UCI with HARQ ACK | 10-18, 5-19 or 5-20 Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-25 | Enable configured UL transmission out of COT | 1. Support configuration of enableConfiguredUL-r16 and enable Cat 4 LBT based transmission of RRC configured UL *SRS, PUCCH, CG-PUSCH etc) out of COT when DCI 2_0is configured but not detected | 10-1 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-26 | CSI-RS based RLM outside of discovery burst transmission window | Support RLM measurements using CSI-RS resources that are outside of discovery burst transmission window | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-27 | Wideband PRACH | Enhanced PRACH design for NR-U by adopting a single long ZC sequence, with ZC sequence = 1151 for 15kHz and ZC sequence = 571 for 30kHz | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-28 | Configured grant with Rel-16 enhanced resource configuration | 1. Support configuration of resources with cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, | 10-18 5-19 or 5-20 | Yes | N/A |  | FFS Per band or Per UE | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-29 | Support available RB set indicator field in DCI 2_0 | Support monitoring DCI 2_0 to read availableRB-Sets-r16 | 10-1, 10-1a, 10-2, or 10-2a | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | Support monitoring DCI 2_0 to read COT duration | 10-1, 10-1a, 10-2, or 10-2a | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | [10-31] | Support of CSI-RS measurements for CSI reporting and tracking without COT duration from DCI 2_0 | · Perform CSI measurements for reporting and tracking using CSI-RS resources that are not within a COT duration indicated by DCI 2_0 · Note: This includes the cases when DCI 2_0 is not configured and when DCI 2_0 is configured but COT duration is not provided by either CO duration field or SFI. | 10-1a | Yes | N/A |  | [Per band] | N/A | N/A |  |  | Optional with capability signaling |