3GPP TSG RAN WG1 #100b-e 	R1-2002519

# April 20th – April 30th, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS38.214 | CR | draft | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.214 clarification on CSI report on deactivated or deconfigured resources |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2020-04-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A CSI report setting can be linked to at least one CSI-RS resource for channel measurement. When a resource or a resource set deactivated or deconfigured, the CSI report setting may still be active in the following scenarios. - Scenario 1: CSI-RS resource is deconfigured or released, but the associated CSI report is not reconfigured. - Scenario 2: SP CSI-RS is deactivated, but the associated AP-CSI or SP-CSI are still alive: - An AP/SP CSI trigger state comprises {Rep0 w/ SP CSI-RS 0, Rep1 w/ SP CSI-RS 1}. If the gNB becomes uninterested in Rep0, it may deactivate the SP CSI-RS 0 to save reference signal overhead. However, the gNB may still be interested in Rep1. - Scenario 3: SP-CSI report based on SP CSI-RS, but the SP CSI-RS is deactivated before deactivating the SP-CSI report There would be two problems if UE is forced to report a CSI if the associated resource is deactivated or deconfigured: - First, the reported CSI would be out-dated which is as useless as a junk report because it is based on a CSI measurement long time ago - Second, there is misalignment in resource occupation. Current spec says the resource occupation for a SP resource ends at its deactivation commant. However, if UE is forced to report its associated CSI, UE has to buffer the last CSI measurement till the time the report is sent. In our view, Scenario 1 and 3 are error cases, if the gNB is not interested in a CSI report, it should deactivate/reconfigure the report. In Scenario 2, UE shall drop the CSI and report the remaining CSI reports. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If an aperiodic or semi-persistent CSI report on PUSCH is linked to semi-persistence CSI-RS set, UE is not expected to report the CSI if the associated semi-persistent CSI-RS set is deactivated when receiving the DCI carrying the CSI request. For SP CSI report linked to SP CSI-RS resource set, UE does not expect to deactivate the SP CSI-RS resource set if the SP-CSI report is activated. UE does not expect to be configured with a CSI report setting if the associated CSI-RS resource is not configured or released. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC failure, or Wasteful CSI report and ambiguous resource/port counting if the associated resource is deactivated/deconfigured but the CSI report is still active. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4, 5.2.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | - No isolated impact, it is an alignment of UE and BS behavior based on common understanding among companies. |  |  |  |  |  |  |  |  |

### 5.2.1.4 Reporting configurations

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

The Reporting configuration for CSI can be aperiodic (using PUSCH), periodic (using PUCCH) or semi-persistent (using PUCCH, and DCI activated PUSCH). The CSI-RS Resources can be periodic, semi-persistent, or aperiodic. Table 5.2.1.4-1 shows the supported combinations of CSI Reporting configurations and CSI-RS Resource configurations and how the CSI Reporting is triggered for each CSI-RS Resource configuration. Periodic CSI-RS is configured by higher layers. Semi-persistent CSI-RS is activated and deactivated as described in Clause 5.2.1.5.2. Aperiodic CSI-RS is configured and triggered/activated as described in Clause 5.2.1.5.1.

UE is not expected to be configured with a CSI report setting if the associated CSI-RS resource is not configured or released.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

### 5.2.1.5	.2 CSI processing criteria

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

If the UE has an active semi-persistent CSI-RS/CSI-IM resource configuration, or an active semi-persistent ZP CSI-RS resource set configuration, and has not received a deactivation command, the activated semi-persistent CSI-RS/CSI-IM resource set or the activated semi-persistent ZP CSI-RS resource set configurations are considered to be active when the corresponding DL BWP is active, otherwise they are considered suspended.

If the UE is configured with carrier deactivation, the following configurations in the carrier in activated state would also be deactivated and need re-activation configuration(s): semi-persistent CSI-RS/CSI- IM resource, semi-persistent CSI reporting on PUCCH, semi-persistent SRS, semi-persistent ZP CSI-RS resource set.

If an aperiodic or semi-persistent CSI report on PUSCH is linked to semi-persistence CSI-RS resource set, UE is not expected to report the CSI if the associated semi-persistent CSI-RS set is deactivated when receiving the DCI carrying the CSI request.

If a semi-persistent CSI report linked to semi-persistent CSI-RS resource set, UE does not expect to deactivate the semi-persistent CSI-RS resource set if the semi-persistent CSI report is activated.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<