3GPP TSG RAN WG1 Meeting #100bis-e	R1- 2002537

April 20th – April 30th, 2020

Agenda item:	7.2.4.1

Source: 	Qualcomm Incorporated

Title: 	Considerations on Physical Layer aspects of NR V2X

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide text proposals for NR V2X specifications regarding physical layer structure. Specifically, the included text proposals address PSCCH, second-stage control details, PSSCH DMRS and TB size determination, and PSBCH synchronization and DMRS sequence.

# Second-Stage Control

## Bit-Scrambling Procedure

The bit scrambling pseudocode has a bug leading to a discontinuous sequence being used for 2nd-stage control when SL-SCH is mapped to two layers. In such a case, the resulting scrambled sequence is

b(q)=bq0+cq0,  bq1+cq1,  bq2+cq4,  bq3+cq5,…

instead of

b(q)=bq0+cq0,  bq1+cq1,  bq2+cq2,  bq3+cq3,…

This can be addressed by accounting for placeholder bits when indexing into the scrambling sequence. Since the index into the scrambling sequence is already adjusted by M(q), this variable can also be used to account for the placeholder bits and resolve the issue.

Observation 1: The 2nd-stage control scrambling procedure in 38.211 does not correctly apply the scrambling sequence when mapping to two layers.

Proposal 1: Adopt the following text proposal to fix the discontinuity in the 2nd-stage control scrambling sequence in TS 38.211.

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.3.1.1	Scrambling

For the single codeword q=0, the block of bits bq0,…,bqMbit(q)-1, where Mbit(q)=Mbit,SCI2(q)+Mbit,data(q) is the number of bits in codeword q transmitted on the physical channel as defined in [4, TS 38.212], shall be scrambled prior to modulation.

Scrambling shall be done according to the following pseudo code

set i=0

set k=0

while i<Mbitq

if b(q)i=x	// SCI placeholder bits

b(q)i=b(q)i-2

k=k+1

else

b(q)i=b(q)i+cq(i-Mq(i,k)) mod 2

end if

i = i + 1

end while

where the scrambling sequence cq(i) is given by clause 5.2.1 and

-	for 0≤i<Mbit,SCI2(q)

-	M(q)=0 Mq(i,k)=k

-	The scrambling sequence generator shall be initialized with

cinit

-	for Mbit,SCI2(q)≤i< Mbit(q)

-	Mq(i,k)=Mbit,SCI2q

-	The scrambling sequence generator shall be initialized with

cinit

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

## Number of Modulation Symbols

The equation for determining the number of modulation symbols for 2nd-stage SCI is mostly reused from the UCI resource determination equation. However, there are some differences between 2nd-stage SCI and UCI:

- 2nd-stage SCI will always use QPSK regardless of the SL-SCH modulation, whereas UCI always uses the same modulation order as SL-SCH.

- 2nd-stage modulation symbols are duplicated across both layers when SL-SCH is mapped to two layers, whereas UCI is rate-matched and mapped to the same number of layers as SL-SCH.

Those differences between 2nd-stage control and UCI will cause the equation in TS 38.212 to provide very high coding rates for 2nd-stage SCI. For example, when the using 64 QAM, R = 616/1024, 20 PRB allocation, and single layer, the effective coding rate for SCI2 will exceed 1.

Observation 2: the equation to determine the number of modulation symbols of 2nd-stage control does not account for the difference in modulation order or number of layers between 2nd-stage control and SL-SCH, leading to very high coding rate for 2nd-stage control.

In this section, we address these issues individually and then group them into a single text proposal.

The potential differences in the modulation order and layer mapping between 2nd-stage control and SL-SCH needs to be addressed in order to maintain reasonable effective code rate for 2nd-stage SCI. Here, the number of SCI-2 modulation symbols is scaled by the number of SL-SCH layers and by ratio of the number of bits in SCL-SCH modulation order (Qm) to 2nd-stage control modulation order (QPSK, 2).

Updating the equation to account the difference in modulation order and layer mapping between 2nd-stage control and SL-SCH to maintain reasonable 2nd-stage control effective coding rate:

QSCI2'=minν⋅Qm⋅OSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)2⋅r=0CSL-SCH-1Kr, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

The intention of the ratio r=0CSL-SCH-1Kr/l=0NsymbolPSSCH-1MscSCI2(l) is to approximate the spectral efficiency of SCH. However, the sum r=0CSL-SCH-1Kr does not actually provide the TBS of SCH since Kr includes the filler null bits and is not the number of information bits in a codeblock. For Base-graph 2, used for many of the smaller TB sizes, the proportion of shortened bits can be greater than 40%. This is a known issue that was discussed in [10].

Rather than replace Kr with K', we propose to use the target SCH spectral efficiency (Qm⋅R⋅ν) from MCS as a simpler approach that also resolves a circular dependency with SL-SCH TB size calculation as will be discussed in Section 3.1.

The circular dependency between TBS determination and 2nd-stage control resource determination can be eliminated by using the target spectral efficiency of PSSCH as follows:

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)Qm⋅R⋅νr=0CSL-SCH-1Kr, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

Finally, the overall text proposal combing the changes related to the 2nd-stage control rate-matching equation is provided.

Proposal 2: Adopt the following text proposal on 2nd-stage control rate-matching in TS 38.212 to account for the difference in modulation order and number of layers between 2nd-stage control and SL-SCH and to remove the dependency on TBS.

----------------------------------------------------begin text proposal for 38.212----------------------------------------------------

8.4.4 Rate Matching

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols for 2nd-stage SCI transmission, denoted as QSCI2', is determined as follows:

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)2⋅Rr=0CSL-SCH-1Kr, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

where

-	OSCI2 is the number of the SCI format 0-2 bits

-	LSCI2 is the number of CRC bits for SCI format 0-2, which is 24 bits.

<<<unchanged text omitted>>>

Krr

-	R is the target code rate signalled by the MCS field in SCI format 0-1.

-	α is configured by higher layer parameter [SL-scaling].

-----------------------------------------------------end text proposal for 38.212-----------------------------------------------------

## Modulation

The PSSCH modulation procedure in TS 38.211 maps the same 2nd-stage control modulation symbol to both layers when SL-SCH is mapped to two layers. However, this operation is already performed by the scrambling procedure where placeholder bits are replaced by prior 2nd-stage control bits. We provide a text proposal to remove this duplicate functionality.

Proposal 3: Adopt the following text proposal to remove redundant duplication of 2nd-stage control modulated symbols in TS 38.211.

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.3.1.2	Modulation

<<<unchanged text omitted>>>

Modulation for 0≤i<Mbit,SCI2(q) shall be done as described in clause 5.1 b(q)2i, b(q)2i+1υ∈1,2d(q)iυ=1d(q)2i, d(q)2i+1υ=2Msymb,1(q)=υMbit,SCI2(q)2 using QPSK where Msymb,1q=Mbit,SCI2q/2.

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

## Scrambling Seed

PUSCH and PDSCH scrambling sequences are initialized using RNTIs, which are 16 bits long, therefore we propose to also use 16 bits to initialize the 2nd-stage control scrambling sequence. Like other sequences in NR sidelink, those 16 bits are the least significant bits of the 1st-stage control CRC.

Proposal 4: Adopt the following proposal for initializing the 2nd-stage control scrambling sequence in TS 38.211.

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.3.1.1 Scrambling

<<<unchanged text omitted>>>

where the scrambling sequence cq(i) is given by clause 5.2.1 and

-	for 0≤i<Mbit,SCI2(q)

-	M(q)=0

-	The scrambling sequence generator shall be initialized with

cinit=NC

where NC=NCXmod 216 where the quantity NCX equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the PSSCH according to NCX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in [TS 38.212].

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

## CRC Length

RAN1 agreed to use PDCCH polar coding for 2nd stage control [6]:

Conclusion:

- If two-stage SCI is supported, the following details are used.

  - Information related to channel sensing is carried on 1st-stage.

  - 2nd-stage is decoded by using PSSCH DMRS.

  - Polar coding used for PDCCH is applied to 2nd-stage

  - Payload size for 1st-stage in two-stage SCI case is the same for unicast, groupcast, and broadcast in a resource pool.

  - After decoding the 1st-stage, the receiver does not need to perform blind decoding of 2nd-stage.

  - FFS other details

- Companies are encouraged to perform analysis (e.g., flexibility, complexity, forward compatibility, overhead, spec impact, latency, robustness, reliability, etc.)/evaluations with details of the SCI contents comparing single-stage vs. two-stage SCI. Aim to conclude in RAN1#98

The CRC length and polynomial are an integral part of the PDCCH polar coding chain, which was designed around the CRC of length 24 bits using the gCRC24C(D) polynomial.

Proposal 5: Adopt the following text proposal capturing the CRC length for 2nd stage control in TS 38.212.

----------------------------------------------------begin text proposal for 38.212----------------------------------------------------

8.4.4 Rate Matching

<<<unchanged text omitted>>>

-	LSCI2 is the number of CRC bits for SCI format 0-2, which is 24 bits.

-----------------------------------------------------end text proposal for 38.212-----------------------------------------------------

## Procedure for Determining Beta Offset Value

Current specification indicates that the beta offset value is provided by high layer parameter sl-BetaOffsets2ndSCI [2]. however, the specification does not define which the procedure to obtain the actual beta offset from the indicator in SCI. For UCI, a mapping between the indicator in DCI and the beta values in tables is explicitly defined [3]. A similar mapping needs to be defined for SCI as well. Table 9.3-2 in 38.213 provides finer granularity values that more suitable for SCI-2 compared to the coarser granularity values in Table 9.3-1

Proposal 6: Adopt the following text proposals to complete the procedure for determining beta offset value

----------------------------------------------------begin text proposal for 38.212----------------------------------------------------

### 8.4.4	Rate Matching

<<<unchanged text omitted>>>

-	βoffsetSCI2 is determined according to beta_offset indicator in the corresponding SCI format 0-1 and Table 8.4.4-1.

<<<unchanged text omitted>>>

Table 8.4.4-1: Determination of βoffsetSCI2

| beta_offset indicator | βoffsetSCI2 |
| --- | --- |
| ‘00’ | 1st offset index provided by higher layers into Table 9.3-2 of [5, 38.213] |
| ‘01’ | 2nd offset index provided by higher layers into Table 9.3-2 of [5, 38.213] |
| ‘10’ | 3rd offset index provided by higher layers into Table 9.3-2 of [5, 38.213] |
| ‘11’ | 4th offset index provided by higher layers into Table 9.3-2 of [5, 38.213] |

The input bit sequence to rate matching is d0, d1,d2,d3, ⋯,dN-1, where N is the number of coded bits.

-----------------------------------------------------end text proposal for 38.212-----------------------------------------------------

# PSSCH

## TB Size Determination

The following agreement was made in RAN1-100e [9]:

Agreement:

For sidelink TBS determination, N_RE’ and/or N_RE are calculated based on the procedure step 1) in 5.1.3.2 Transport block size determination of TS38.214 with the following considerations.

- For the number of PSSCH symbols,

  - AGC symbol and GP symbol in the end of slot are excluded.

- PSCCH overhead

  - The exact number of REs for PSCCH (including PSCCH DMRS) is considered

- 2nd SCI overhead

  - FFS: How to consider the 2nd SCI

  - FFS: How to handle the relationship between PSSCH TBS determination and 2nd stage SCI modulated symbols determination.

- FFS: how to consider PSFCH, PSSCH DMRS, GP symbols before PSFCH, SL PT-RS, SL CSI-RS

  - FFS: N_oh^PRB is introduced or not

- It is RAN1’s understanding that a UE is not expected to receive a retransmission with a TB size that is different from the last valid TB size signalled for this TB.

- Note: The design will be such that the TBS is the same between a transmission and its re-transmission(s).

Transport-block size determination for PSSCH has to accommodate the following factors that are different from PDSCH/PUSCH:

- The number of PSSCH OFDM symbols in a slot is not under transmitter control. The total number of sidelink symbols in a slot is fixed, and some slots contain PSFCH resources, while others do not.

- PSCCH could occupy a large proportion of sidelink resources if the number of allocated PSSCH sub-channels is small, or a small proportion if the number of allocated sub-channels is large.

- 2nd-stage SCI could occupy a large proportion of sidelink resources, especially if the number of allocated PSSCH sub-channels is small.

- The number of RBs between initial and retransmission of a TB is fixed.

- PSSCH DMRS could be on 2, 3, or 4 symbols.

To account for those differences, we propose a TBS determination scheme based on that of PDSCH/PUSCH with the following changes:

- The transmitter indicates in SCI whether PSFCH overhead (NsymbPSFCH) is accounted for in TBS determination or not. This is achieved by a new SCI field “PSFCH overhead indicator.”

- Nsymbsh is the number of sidelink symbols in a slot, not only those used by PSSCH. The first symbol is excluded from this number since it is used for AGC training and is a repetition of the following symbol.

- The overhead of PSCCH is subtracted from the total number of available REs.

- The overhead of 2nd-stage control is subtracted from the total number of available REs. To avoid a circular dependency between TBS determination and calculating the number of modulation symbols for 2nd-stage control, the latter is adjusted to use the target spectral efficiency instead of values based on the TBS as described in Section 2.2.

- To account for the AGC symbol at the beginning of the slot and the gap symbol at the end of the slot, the maximum number of REs per PRB is limited to 144 (12 symbols) instead of 156 (13 symbols).

## Text Proposal for TBS Determination

Proposal 7: Adopt the following text proposal capturing a TBS determination procedure for PSSCH in TS 38.214 and TS 38.212.

----------------------------------------------------begin text proposal for 38.214----------------------------------------------------

8.1.3.2	Transport block size determination

The UE shall first determine the number of REs (NRE) within the slot.

-	A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NSCRB⋅(Nsymbsh-NsymbPSFCH)-NDMRSPRB-NohPRB, where

- NSCRB=12 is the number of subcarriers in a physical resource block,

- Nsymbsh is the number of sidelink symbols within the slot, excluding the first symbol in the slot,

- NsymbPSFCH (a value from 0 or 3) is the number of overhead symbols for PSFCH as indicated by the PSFCH overhead field in SCI format 0_2A, 0_2B, 0_2C, or 0_2D. NsymbPSFCH=3 when the indicator in SCI is 1; otherwise NsymbPSFCH=0.

- NDMRSPRBNDMRSPRBNDMRSPRB (a value from 12, 18, or 24) is the number of REs for DM-RS per PRB in the scheduled PSSCH, as indicated by SCI format 0_1, and

- NohPRB is the overhead configured by higher layer parameter xOverhead. If xOverhead is not configured (a value from 0, 6, 12, or 18), NohPRB is set to 0.

-	A UE determines the total number of REs allocated for PSSCH (NRE) by NRE=min144, NRE'⋅nPRB-NRESCI,1-NRESCI,2, where

- nPRB is the total number of allocated PRBs for the PSSCH,

- NRESCI,1 is the total number of REs allocated for PSCCH, and

- NRESCI,2 is the total number of REs allocated for second stage control as determined in clause 8.4.4 of [5, 38.212].

-----------------------------------------------------end text proposal for 38.214-----------------------------------------------------

----------------------------------------------------begin text proposal for 38.212----------------------------------------------------

8.4.1.1	SCI format 0-2

SCI format 0-2 is used for the decoding of PSSCH.

The following information is transmitted by means of the SCI format 0-2:

-	HARQ Process ID – [x] bits as defined in clause x.x.x of [6, TS 38.214].

-	New data indicator – 1 bit as defined in clause x.x.x of [6, TS 38.214].

-	Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214].

-	PSFCH overhead indicator – 1 bit as defined in clause 8.1.3.2 of [6, TS 38.214]

-----------------------------------------------------end text proposal for 38.212-----------------------------------------------------

## Scrambling Seed

PUSCH and PDSCH scrambling sequences are initialized using RNTIs, which are 16 bits long, therefore we propose to also use 16 bits to initialize the PSSCH scrambling sequence. Like other sequences in NR sidelink, those 16 bits are the least significant bits of the 1st-stage control CRC.

Proposal 8: Adopt the following proposal for initializing the PSSCH scrambling sequence in TS 38.211

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.3.1.1 Scrambling

<<<unchanged text omitted>>>

-	for Mbit,SCI2(q)≤i< Mbit(q)

-	M(q)=Mbit,SCI2q

-	The scrambling sequence generator shall be initialized with

cinit=NC

where NC=NCXmod 216 where the quantity NCX equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the PSSCH according to NCX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in [TS 38.212].

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

## Mapping of CSI-RS

Given the configurability of CSI-RS and DMRS, there are configurations that could lead to RE-level collisions between the two for some transmission. In Uu, it is stated that the UE does not expect to encounter such transmissions. Such a restriction on collision is necessary in sidelink as well.

Proposal 9: Adopt the following text proposal to disallow resource collision between CSI-RS and DMRS

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.4.1.5.3   Mapping to physical resources

Mapping to resource elements shall be done according to clause 7.4.1.5.3 with the following exceptions:

-	only 1 and 2 antenna ports are supported, X∈1,2;

-	only density ρ=1 is supported;

-	zero-power CSI-RS is not supported;

-	the quantity βCSIRS is an amplitude scaling factor to conform with the transmit power specified in clause 8.2.1 of [6, TS 38.214].

The UE is not expected to be configured such that CSI-RS and DM-RS can be mapped to the same resource element

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

Another issue that needs clarification is that CSI-RS cannot be present in a symbol that also contains second-stage control. This is due to the CSI report trigger being in SCI-2. Therefore, the UE needs to decode SCI-2 to determine whether CSI-RS is present or not. If CSI-RS can be mapped to a symbol that contains second-stage control, there will be ambiguity about the rate-matching of second-stage control. Therefore, mapping CSI-RS to a symbol with second-stage control should be explicitly disallowed.

Proposal 10: Adopt the following text proposal to prevent a cyclic dependency between CSI-RS and second-stage control rate-matching

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.4.1.5.3   Mapping to physical resources

Mapping to resource elements shall be done according to clause 7.4.1.5.3 with the following exceptions:

-	only 1 and 2 antenna ports are supported, X∈1,2;

-	only density ρ=1 is supported;

-	zero-power CSI-RS is not supported;

-	the quantity βCSIRS is an amplitude scaling factor to conform with the transmit power specified in clause 8.2.1 of [6, TS 38.214].

CSI-RS cannot be mapped to an OFDM symbol with second-stage control resources.

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

# Resource Pool Configuration

In RAN1-100e, the following agreements regarding resource-pool configuration were made:

Agreement:

- For resource pool configuration, slots for a resource pool is (pre-)configured with bitmap, which is applied with periodicity.

Agreement:

- For derivation of the set of slots to be included in the resource pool, the baseline is the derivation with bitmap and periodicity based on Subclause 14.1.5 of TS36.213 with the following modifications.

  - FFS:  Periodicity and L_bitmap value

  - The slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell if serving cell timing reference is in use, or DFN 0 otherwise

- The following procedure is used.

  - The set includes all the slots except the following slots:

    - Slots in which SLSS resource is configured,

    - (Working assumption) slots not having at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where

      - X is sl-LengthSymbols

      - Y is sl-StartSymbol

    - (Working assumption) reserved slots which are determined by the similar steps in Subclause 14.1.5 of TS36.213

Working Assumption:

- For the number of PRBs for resource pool, allow configuration of all  number of PRBs in a SL BWP.

- FFS until RAN1#100bis-e whether/how to deal with remaining PRBs if the configured PRBs for resource pool is not a multiple of subchannel size.

In LTE V2X, the periodicity of the resource pool configuration is 1024 frames (10240 ms). While, compared to LTE Uu, NR Uu has more flexible TDD pattern indication, the procedure to be reused from LTE already excludes sub-frames and ensures that the bitmap can be applied to the 10240 ms period. The same mechanism would be used in NR.

Proposal 11: Use a periodicity of 10240 ms for the resource pool bitmap configuration.

Multiple proposals were brought up in the email discussion [100e-NR-5G_V2X_NRSL-PHYstructure-02] leading to the working assumption on allowing the configuration of all number of PRBs in a resource pool. Assigning the additional PRBs to other sub-channels or grouping them into a sub-channel would lead to sub-channels of different sizes, which is incompatible with existing design, has not been investigated in RAN1, and would lead variable performance between the sub-channels. Given the current Release-16 design and to provide a forward compatible design where future UEs can communicate with Release-16 in the same resource pool, we propose to keep those PRBs reserved in Release-16.

Proposal 12: The remaining PRBs if the configured PRBs for resource pool is not a multiple of subchannel size are reserved in Release-16.

# 4-Symbol PSSCH DMRS Pattern

During the [99-NR-05] email discussion, it was shown that DMRS pattern with DMRS on symbols (1, 4, 7, 10) exhibited degraded performance in high-speed, high-spectral efficiency scenarios compared to other DMRS patterns due to the additional extrapolation at the end of the slot. Figure 1 compares the performance of two patterns: the current 4-symbol pattern (1, 4, 7, 10) and (1, 4, 8, 12).

Figure 1 Performance comparison between the current 4-symbol PSSCH DMRS pattern and (1, 4, 8, 12)

Given the performance results, an additional 4-symbol DMRS pattern should be introduced to enable NR V2X to provide good performance at all speeds and spectral efficiency.

Proposal 13: Adopt the following text proposal to add an additional 4-symbol PSSCH DMRS pattern for high-speed scenarios

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.4.1.1.2 Mapping to physical resources

<<<unchanged text omitted>>>

Table 8.4.1.1.2-1: PSSCH DM-RS time-domain location.

| ld in symbols | DM-RS position l |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSCCH duration 2 symbols |  |  | PSCCH duration 3 symbols |  |  |
|  | Number of PSSCH DM-RS |  |  | Number of PSSCH DM-RS |  |  |
|  | 2 | 3 | 4 | 2 | 3 | 4 |
| 6 | 1, 5 |  |  | 1, 5 |  |  |
| 7 | 1, 5 |  |  | 1, 5 |  |  |
| 8 | 1, 5 |  |  | 1, 5 |  |  |
| 9 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 10 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 11 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 12 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 13 | 3, 10 | 1, 6, 11 | 1, 4, 7, 10 | 4, 10 | 1, 6, 11 | 1, 4, 7, 10 |
| 13 |  |  | 1, 4, 8, 12 |  |  | 1, 4, 8, 12 |

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

# Conclusion

Observation 1: The 2nd-stage control scrambling procedure in 38.211 does not correctly apply the scrambling sequence when mapping to two layers.

Observation 2: the equation to determine the number of modulation symbols of 2nd-stage control does not account for the difference in modulation order or number of layers between 2nd-stage control and SL-SCH, leading to very high coding rate for 2nd-stage control.

Proposal 1: Adopt the following text proposal to fix the discontinuity in the 2nd-stage control scrambling sequence in TS 38.211.

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.3.1.1	Scrambling

For the single codeword q=0, the block of bits bq0,…,bqMbit(q)-1, where Mbit(q)=Mbit,SCI2(q)+Mbit,data(q) is the number of bits in codeword q transmitted on the physical channel as defined in [4, TS 38.212], shall be scrambled prior to modulation.

Scrambling shall be done according to the following pseudo code

set i=0

set k=0

while i<Mbitq

if b(q)i=x	// SCI placeholder bits

b(q)i=b(q)i-2

k=k+1

else

b(q)i=b(q)i+cq(i-Mq(i,k)) mod 2

end if

i = i + 1

end while

where the scrambling sequence cq(i) is given by clause 5.2.1 and

-	for 0≤i<Mbit,SCI2(q)

-	M(q)=0 Mq(i,k)=k

-	The scrambling sequence generator shall be initialized with

cinit

-	for Mbit,SCI2(q)≤i< Mbit(q)

-	Mq(i,k)=Mbit,SCI2q

-	The scrambling sequence generator shall be initialized with

cinit

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

Proposal 2: Adopt the following text proposal on 2nd-stage control rate-matching in TS 38.212 to account for the difference in modulation order and number of layers between 2nd-stage control and SL-SCH and to remove the dependency on TBS.

----------------------------------------------------begin text proposal for 38.212----------------------------------------------------

8.4.4 Rate Matching

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols for 2nd-stage SCI transmission, denoted as QSCI2', is determined as follows:

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)2⋅Rr=0CSL-SCH-1Kr, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

where

-	OSCI2 is the number of the SCI format 0-2 bits

-	LSCI2 is the number of CRC bits for SCI format 0-2, which is 24 bits.

<<<unchanged text omitted>>>

Krr

-	R is the target code rate signalled by the MCS field in SCI format 0-1.

-	α is configured by higher layer parameter [SL-scaling].

-----------------------------------------------------end text proposal for 38.212-----------------------------------------------------

Proposal 3: Adopt the following text proposal to remove redundant duplication of 2nd-stage control modulated symbols in TS 38.211.

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.3.1.2	Modulation

<<<unchanged text omitted>>>

Modulation for 0≤i<Mbit,SCI2(q) shall be done as described in clause 5.1 b(q)2i, b(q)2i+1υ∈1,2d(q)iυ=1d(q)2i, d(q)2i+1υ=2Msymb,1(q)=υMbit,SCI2(q)2 using QPSK where Msymb,1q=Mbit,SCI2q/2.

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

Proposal 4: Adopt the following proposal for initializing the 2nd-stage control scrambling sequence in TS 38.211.

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.3.1.1 Scrambling

<<<unchanged text omitted>>>

where the scrambling sequence cq(i) is given by clause 5.2.1 and

-	for 0≤i<Mbit,SCI2(q)

-	M(q)=0

-	The scrambling sequence generator shall be initialized with

cinit=NC

where NC=NCXmod 216 where the quantity NCX equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the PSSCH according to NCX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in [TS 38.212].

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

Proposal 5: Adopt the following text proposal capturing the CRC length for 2nd stage control in TS 38.212.

----------------------------------------------------begin text proposal for 38.212----------------------------------------------------

8.4.4 Rate Matching

<<<unchanged text omitted>>>

-	LSCI2 is the number of CRC bits for SCI format 0-2, which is 24 bits.

-----------------------------------------------------end text proposal for 38.212-----------------------------------------------------

Proposal 6: Adopt the following text proposals to complete the procedure for determining beta offset value

----------------------------------------------------begin text proposal for 38.212----------------------------------------------------

### 8.4.4	Rate Matching

<<<unchanged text omitted>>>

-	βoffsetSCI2 is determined according to beta_offset indicator in the corresponding SCI format 0-1 and Table 8.4.4-1.

<<<unchanged text omitted>>>

Table 8.4.4-1: Determination of βoffsetSCI2

| beta_offset indicator | βoffsetSCI2 |
| --- | --- |
| ‘00’ | 1st offset index provided by higher layers into Table 9.3-2 of [5, 38.213] |
| ‘01’ | 2nd offset index provided by higher layers into Table 9.3-2 of [5, 38.213] |
| ‘10’ | 3rd offset index provided by higher layers into Table 9.3-2 of [5, 38.213] |
| ‘11’ | 4th offset index provided by higher layers into Table 9.3-2 of [5, 38.213] |

The input bit sequence to rate matching is d0, d1,d2,d3, ⋯,dN-1, where N is the number of coded bits.

-----------------------------------------------------end text proposal for 38.212-----------------------------------------------------

Proposal 7: Adopt the following text proposal capturing a TBS determination procedure for PSSCH in TS 38.214 and TS 38.212.

----------------------------------------------------begin text proposal for 38.214----------------------------------------------------

8.1.3.2	Transport block size determination

The UE shall first determine the number of REs (NRE) within the slot.

-	A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NSCRB⋅(Nsymbsh-NsymbPSFCH)-NDMRSPRB-NohPRB, where

- NSCRB=12 is the number of subcarriers in a physical resource block,

- Nsymbsh is the number of sidelink symbols within the slot, excluding the first symbol in the slot,

- NsymbPSFCH (a value from 0 or 3) is the number of overhead symbols for PSFCH as indicated by the PSFCH overhead field in SCI format 0_2A, 0_2B, 0_2C, or 0_2D. NsymbPSFCH=3 when the indicator in SCI is 1; otherwise NsymbPSFCH=0.

- NDMRSPRBNDMRSPRBNDMRSPRB (a value from 12, 18, or 24) is the number of REs for DM-RS per PRB in the scheduled PSSCH, as indicated by SCI format 0_1, and

- NohPRB is the overhead configured by higher layer parameter xOverhead. If xOverhead is not configured (a value from 0, 6, 12, or 18), NohPRB is set to 0.

-	A UE determines the total number of REs allocated for PSSCH (NRE) by NRE=min144, NRE'⋅nPRB-NRESCI,1-NRESCI,2, where

- nPRB is the total number of allocated PRBs for the PSSCH,

- NRESCI,1 is the total number of REs allocated for PSCCH, and

- NRESCI,2 is the total number of REs allocated for second stage control as determined in clause 8.4.4 of [5, 38.212].

-----------------------------------------------------end text proposal for 38.214-----------------------------------------------------

----------------------------------------------------begin text proposal for 38.212----------------------------------------------------

8.4.1.1	SCI format 0-2

SCI format 0-2 is used for the decoding of PSSCH.

The following information is transmitted by means of the SCI format 0-2:

-	HARQ Process ID – [x] bits as defined in clause x.x.x of [6, TS 38.214].

-	New data indicator – 1 bit as defined in clause x.x.x of [6, TS 38.214].

-	Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214].

-	PSFCH overhead indicator – 1 bit as defined in clause 8.1.3.2 of [6, TS 38.214]

-----------------------------------------------------end text proposal for 38.212-----------------------------------------------------

Proposal 8: Adopt the following proposal for initializing the PSSCH scrambling sequence in TS 38.211

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.3.1.1 Scrambling

<<<unchanged text omitted>>>

-	for Mbit,SCI2(q)≤i< Mbit(q)

-	M(q)=Mbit,SCI2q

-	The scrambling sequence generator shall be initialized with

cinit=NC

where NC=NCXmod 216 where the quantity NCX equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the PSSCH according to NCX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in [TS 38.212].

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

Proposal 9: Adopt the following text proposal to disallow resource collision between CSI-RS and DMRS

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.4.1.5.3   Mapping to physical resources

Mapping to resource elements shall be done according to clause 7.4.1.5.3 with the following exceptions:

-	only 1 and 2 antenna ports are supported, X∈1,2;

-	only density ρ=1 is supported;

-	zero-power CSI-RS is not supported;

-	the quantity βCSIRS is an amplitude scaling factor to conform with the transmit power specified in clause 8.2.1 of [6, TS 38.214].

The UE is not expected to be configured such that CSI-RS and DM-RS can be mapped to the same resource element

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

Proposal 10: Adopt the following text proposal to prevent a cyclic dependency between CSI-RS and second-stage control rate-matching

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.4.1.5.3   Mapping to physical resources

Mapping to resource elements shall be done according to clause 7.4.1.5.3 with the following exceptions:

-	only 1 and 2 antenna ports are supported, X∈1,2;

-	only density ρ=1 is supported;

-	zero-power CSI-RS is not supported;

-	the quantity βCSIRS is an amplitude scaling factor to conform with the transmit power specified in clause 8.2.1 of [6, TS 38.214].

CSI-RS cannot be mapped to an OFDM symbol with second-stage control resources.

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

Proposal 11: Use a periodicity of 10240 ms for the resource pool bitmap configuration.

Proposal 12: The remaining PRBs if the configured PRBs for resource pool is not a multiple of subchannel size are reserved in Release-16.

Proposal 13: Adopt the following text proposal to add an additional 4-symbol PSSCH DMRS pattern for high-speed scenarios

----------------------------------------------------begin text proposal for 38.211----------------------------------------------------

8.4.1.1.2 Mapping to physical resources

<<<unchanged text omitted>>>

Table 8.4.1.1.2-1: PSSCH DM-RS time-domain location.

| ld in symbols | DM-RS position l |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSCCH duration 2 symbols |  |  | PSCCH duration 3 symbols |  |  |
|  | Number of PSSCH DM-RS |  |  | Number of PSSCH DM-RS |  |  |
|  | 2 | 3 | 4 | 2 | 3 | 4 |
| 6 | 1, 5 |  |  | 1, 5 |  |  |
| 7 | 1, 5 |  |  | 1, 5 |  |  |
| 8 | 1, 5 |  |  | 1, 5 |  |  |
| 9 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 10 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 11 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 12 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 13 | 3, 10 | 1, 6, 11 | 1, 4, 7, 10 | 4, 10 | 1, 6, 11 | 1, 4, 7, 10 |
| 13 |  |  | 1, 4, 8, 12 |  |  | 1, 4, 8, 12 |

-----------------------------------------------------end text proposal for 38.211-----------------------------------------------------

# References

- 3GPP TS 38.211 V16.1.0, NR, Physical Channels and Modulation (Release 16).

- 3GPP TS 38.212 V16.1.0, NR, Multiplexing and Channel Coding (Release 16).

- 3GPP TS 38.213 V16.1.0, NR, Physical Layer Procedures for Control (Release 16).

- 3GPP TS 38.214 V16.1.0, NR, Physical Layer Procedures for Data (Release 16).

- Chairman’s Notes, RAN1 #96, February 2019, Athens, Greece.

- Chairman’s Notes, RAN1 #97, May 2019, Reno, USA.

- Chairman’s Notes, RAN1 #98bis, October 2019, Chongqing, China.

- Chairman’s Notes, RAN1 #99, November 2019, Reno, USA.

- Chairman’s Notes, RAN1 #100e, March 2020.

- R1-1811489, “Maintenance issues of physical uplink control channel,” Ericsson, RAN1 #94bis, Chengdu, China.