3GPP TSG RAN WG1 #100bis	R1-2002609

e-Meeting, April 20th – 30th, 2020

Agenda item:		7.2.10.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintanence of unaligned frame boundary with slot alignment and partial SFN alignment for inter-band CA

Document for:		Discussion and Decision

# Introduction

For the unaligned frame boundary with slot and partial SFN alignment for inter-band CA, RAN1#100e endorsed TPs for 38.211, 38.213 and 38.214. The three specs are not exactly aligned, and this contribution proposes additional modifications to make the specification more ready for implementation.

# Discussion

## TS38.211 subclause 4.5

TS38.211 4.5 defines the slot offset Nslot, offsetCA as follows (revisions according to the updates made in the RAN1#100e):

## 4.5 Carrier aggregation Transmissions in multiple cells can be aggregated. Unless otherwise noted, the description in this specification applies to each of the serving cells. For carrier aggregation of cells with non-aligned frames, the slot offset Nslot, offsetCA between a PCell/PScell and an SCell is determined by higher-layer parameter CA-slot-offset for the SCell. The quantity μoffset is defined as the maximum of the lowest subcarrier spacing configuration among the subcarrier spacings given by the higher-layer parameters SCS-SpecificCarrierList configured for PCell/PSCell and the SCell, respectively.. The slot offset Nslot, offsetCA fulfills - when the lowest subcarrier spacing configuration among the subcarrier spacings configured for the cell is μ=2 for both cells or μ=3 for both cells, the start of slot 0 for the cell whose point A has a lower frequency coincides with the start of slot qNslot, offsetCA mod Nslotframe,μ for the other cell where q=-1 if point A of the PCell/PSCell has a frequency lower than the frequency of point A for the SCell, otherwise q=1; - otherwise, - If the lowest subcarrier spacing configured by the higher-layer parameter SCS-SpecificCarrierList for the two cells is the same, the start slot 0 of the PCell/PSCell coincides with the start of slot qNslot, offsetCA mod Nslotframe,μ for the SCell. - Otherwise, the start of slot 0 for the cell which has the lowest subcarrier spacing in the higher-layer parameter SCS-SpecificCarrierList for the two cells coincides with the start of slot qNslot, offsetCA mod Nslotframe,μ for the other cell. qNslot, offsetCA mod Nslotframe,μ- where q=-1 if the lowest subcarreier spacing configuration given by SCS-SpecificCarrierList of the PCell/PSCell is smaller than or equal to the lowest subcarrier spacing given by SCS-SpecificCarrierList for the SCell, otherwise q=1.

## TS38.214 subclauses 5.1.2.1, 5.2.1.5.1a and 6.1.2.1

Update proposal to TS38.214 subclause 5.1.2.1:

#### 5.1.2.1 Resource allocation in time domain When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception. Given the parameter values of the indexed row: - The slot allocated for the PDSCH is Ks, where , if UE is configured with CA-slot-offset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and - Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and theof the PDCCH carrier with the scheduling DCI as defined in clause 4.5 of [4, TS 38.211], - Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the of the scheduled PDSCH carrier as defined in clause 4.5 of [4, TS 38.211]. Nslot, offsetCA

Proposal: Similar update should be made to the 5.2.1.5.1a and 6.1.2.1 of TS28.214.

## TS38.213 subclause 9.1.2.1

[Parts not relevant not copied here for brevity] e) if CA-slot-offset is provided, the Nslot, offsetCA and the defined in [4, TS 38.211] correspond to the Nslot,offset,cDL and μoffset,DL,c for serving cell c, and Nslot,offsetUL and μoffsetUL for the cell of PUCCH transmission. [Parts not relevant not copied here for brevity] end while else while if modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0 Set – index of a DL slot within an UL slot while Set to the set of rows Set to the cardinality of Set – index of row in set if slot starts at a same time as or after a slot for an active DL BWP change on serving cell or an active UL BWP change on the PCell and slot nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD is before the slot for the active DL BWP change on serving cell or the active UL BWP change on the PCell nD=nD+1; else while if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD-NPDSCHrepeat+1 to slot nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD, at least one symbol of the PDSCH time resource derived by row is configured as UL where is the k-th slot timing value in set , ; else ; end if end while if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and , ; ; The UE does not expect to receive SPS PDSCH release and unicast PDSCH in a same slot; else Set to the cardinality of Set to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of while Set while if for start OFDM symbol index for row ; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row ; ; else ; end if end while ; Set to the smallest last OFDM symbol index among all rows of ; end while end if nD=nD+1; end if end while end if ; end while end if

# Conclusion

Proposal: Update the TS38.211 subclause 4.5 as suggested above

Proposal: Update the TS38.214 subclauses 5.1.2.1, 5.2.1.5.1a and 6.1.2.1 as suggested above

Proposal: Update the TS38.213 subclause 9.1.2.1 as suggested above

# Annex A: Relevant portion of TS38.211

TS38.211 4.5 defines the slot offset Nslot, offsetCA as follows (revisions according to the updates made in the RAN1#100e):

## 4.5 Carrier aggregation Transmissions in multiple cells can be aggregated. Unless otherwise noted, the description in this specification applies to each of the serving cells. For carrier aggregation of cells with non-aligned frames, the slot offset Nslot, offsetCA between a PCell/PScell and an SCell is determined by higher-layer parameter CA-slot-offset for the SCell. The quantity μoffset is defined as the maximum of the lowest subcarrier spacing configuration among the subcarrier spacings given by the higher-layer parameters SCS-SpecificCarrierList configured for PCell/PSCell and the SCell, respectively.. The slot offset Nslot, offsetCA fulfills - when the lowest subcarrier spacing configuration among the subcarrier spacings configured for the cell is μ=2 for both cells or μ=3 for both cells, the start of slot 0 for the cell whose point A has a lower frequency coincides with the start of slot qNslot, offsetCA mod Nslotframe,μ for the other cell where q=-1 if point A of the PCell/PSCell has a frequency lower than the frequency of point A for the SCell, otherwise q=1; - otherwise, the start of slot 0 for the cell with the lower subcarrier spacing of the lowest subcarrier spacing given by the higher-layer parameters SCS-SpecificCarrierList configured for the two cells, or the Pcell/PSCell if both cells have the same lowest subcarrier spacing given by the higher-layer parameters SCS-SpecificCarrierList configured for the two cells, coincides with the start of slot qNslot, offsetCA mod Nslotframe,μ for the other cell where q=-1 if the lowest subcarreier spacing configuration given by SCS-SpecificCarrierList of the PCell/PSCell is smaller than or equal to the lowest subcarrier spacing given by SCS-SpecificCarrierList for the SCell, otherwise q=1.

# Annex B: Relevant portion of TS38.214

TS38.214 5.1.2.1 (revisions according to the updates made in the RAN1#100e):

#### 5.1.2.1 Resource allocation in time domain When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception. Given the parameter values of the indexed row: - The slot allocated for the PDSCH is Ks, where , if UE is configured with CA-slot-offset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and - Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured CA-slot-offset, for the cell receiving the PDCCH respectively, Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the, respectively,which are determined by higher-layer configured CA-slot-offset for the cell receiving the PDSCH, as defined in clause 4.5 of [4, TS 38.211]. Nslot, offsetCA

TS38.214 5.2.1.5.1a

Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset. The CSI-RS triggering offset has the values of {0, 1,…31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with CA-slot-offset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where - n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset, - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively, - Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured CA-slot-offset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured CA-slot-offset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] subclause 4.5

TS38.214 6.1.2.1

in CSI-ReportConfig for the triggered CSI Reporting Settings and is the (m+1)th entry of . - The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with CA-slot-offset for at least one of the scheduled and scheduling cell, Ks =, otherwise, and where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and and are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, - Nslot, offset, PDCCHCAand μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured CA-slot-offset for the cell receiving the PDCCH, Nslot, offset, PUSCHCA and μoffset,PUSCH are the Nslot, offsetCA and the,respectively, which are determined by higher-layer configured CA-slot-offset for the cell transmitting the PUSCH, as defined in clause 4.5 of [4, TS 38.211], and Nslot, offsetCA