3GPP TSG-RAN WG1 Meeting #100bis-e	R1-2002623

e-Meeting, April 20th – April 30th, 2020

Agenda Item:	7.2.8.4

Source:	Ericsson

Title:	Maintenance of rel16 Physical-layer procedures to support UE - gNB measurements

Document for:	Discussion, Decision

# Introduction

This contribution address maintenance issues pertaining to Physical-layer procedures to support UE/gNB measurements for NR positioning.

# Discussion

### Clarification on RSTD measurement procedure

In clause 5.1.6.5 of TS38.214, the following text is included:

“The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference time for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met.”

The above sentence is a bit ambiguous as it needs to be clarified that the UE can use different DL PRS resources within a DL PRS resource set and the DL PRS resource set can be any DL PRS resource set associated with the [ID] provided by DL-PRS-RstdReferenceInfo.  A clarification is proposed in text proposal 3.

Endorse text proposal 1 in Section 5 for inclusion in 38.214.

### Regarding DL-PRS-QCL-Info

In RAN1 Meeting#100-e, the issue of whether a DL PRS can also be ‘QCL-Type-C’ (in addition to ‘QCL-Type-D’) with a DL PRS was discussed (see Section 2.6 of [2]).  The current specifications allow a target DL PRS to be ‘QCL-Type-D’ with another DL PRS which allows the target DL PRS to be received with the same spatial receive filters as the source DL PRS.  However, the current specification does not allow the Doppler shift and average delay estimated on the source DL PRS to be used for the target DL PRS.  We think it is beneficial to allow the Doppler shift and average delay estimated on the source DL PRS to be used for the target DL PRS.  Hence, we support allowing a DL-PRS to be ‘QCL-Type-C’ with another DL-PRS, and support text proposal 2.

Endorse text proposal 2 in Section 5 for inclusion in 38.214.

## Clarification on UE Rx-Tx Time-MeasRequestInfo

In RAN1 Meeting#100-e, the following clarification on UE Rx-Tx Time-MeasRequestInfo was discussed (see Section 2.6 of [2]).

“The UE can be configured in higher layer parameter UE Rx-Tx Time-MeasRequestInfo to report, subject to UE capability, up to 4 multiple UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in difference positioning frequency layers.”

We think this clarification is reasonable and support the proposal to add this clarification to TS 38.214.

Endorse text proposal 3 in Section 5 for inclusion in 38.214.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Endorse text proposal 1 in Section 5 for inclusion in 38.214.

Proposal 2	Endorse text proposal 2 in Section 5 for inclusion in 38.214.

Proposal 3	Endorse text proposal 3 in Section 5 for inclusion in 38.214.

- References

RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, Shenzhen, China, March, 2019

R1-2001364, “Summary of Email Discussion [100e-NR-Pos-PHY procedures-01]: PRS reception procedure”,  Qualcomm Incorporated, 3GPP TSG-RAN WG1 Meeting #100-e, February 24 – March 06, 2020.

# Text Proposals for Specification CRs

Text proposal 1 (38.214):

### 5.1.6.5 PRS reception procedure ----unchange text ommitted -------- The UE may be indicated by the network that a DL PRS resources can be used as the reference for the RSTD measurement in a higher layer parameter DL-PRS-RstdReferenceInfo. The reference time indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters DL-PRS-expectedRSTD and DL-PRS-expectedRSTD-uncertainty. The UE expects the reference time to be indicated whenever it is expected to receive the DL PRS. This reference time provided by DL-PRS-RstdReferenceInfo may include an [ID], a PRS resource set ID, and optionally a single PRS resource ID or a list of PRS resource IDs. The UE may use different DL PRS resources within a single DL PRS resource set or a different DL PRS resource set to determine the reference time for the RSTD measurement where the DL PRS resource set can be any DL PRS resource set associated with the [ID] provided by DL-PRS-RstdReferenceInfo as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference time than indicated by the network, then it is expected to report the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. ----unchange text ommitted --------

Text proposal 2 (38.214):

##### 5.1.6.5 PRS reception procedure -----unchanged text omitted ------ - DL-PRS-QCL-Info defines any quasi-colocation information of the DL PRS resource with other reference signals. The DL PRS may be configured to be ‘QCL-Type-C’ and 'QCL-Type-D' (when applicable) with a source DL PRS or SS/PBCH Block from a serving cell or a non-serving cell. The DL PRS may be configured to be 'QCL-Type-C' with a SS/PBCH Block from a serving or non-serving cell. If the DL PRS is configured as both 'QCL-Type-C' and 'QCL-Type-D' with a SS/PBCH Block then the SSB index indicated should be the same. -----unchanged text omitted ------ If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are from the same cell. If DL-PRS-QCL-Info is configured to the UE with 'QCL-Type-D' with a source DL-PRS-Resource then the DL-PRS-ResourceSetId and the DL-PRS-ResrouceId of the source DL-PRS-Resource are expected to be indicated to the UE. -----unchanged text omitted ------

Text proposal 3 (38.214):

##### 5.1.6.5 PRS reception procedure -----unchanged text omitted ------ The UE can be configured in higher layer parameter UE Rx-Tx Time-MeasRequestInfo to report, subject to UE capability, up to 4 multiple UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in difference positioning frequency layers. -----unchanged text omitted ------