3GPP TSG RAN WG1 Meeting #100bis-e                                                            R1-2002681

E-meeting, April 20 - April 30, 2020

Title:	[Draft] Reply LS on CSI-RS capabilities (FG 2-33/36/40/41/43)

Response to:	R1-2001519 (R2-2002415)

Release:	Release 16

Work Item:	TEI16, NR_newRAT-Core

Source:	Huawei, HiSilicon [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Min Zhang

E-mail Address:	min.zhang1{at} huawei.com

Attachments: -

1. Overall Description:

RAN1 would like to thank RAN2 for reply LS on CSI-RS capabilities (FG 2-33/36/40/41/43). Based on the discussions during R16, RAN1 would like to provide following answers.

Question1: Definition of CSI-RS ports/resources configured for the TDM case.

RAN2 understand that the legacy triplet included in SupportedCSI-RS-Resource is relevant to the following definition in sub-clause 5.2.1.6 of TS 38.214.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources than reported as capability.

RAN2 is wondering if the current running CR to 38.306 describing “active Tx ports/resources across multiple slots” by referring to sub-clause 5.2.1.6 of TS 38.214 is in line with RAN1’s understanding.

Answer: Before answering Question1, RAN1 is wondering what are definitions of the starting slot and the ending slot of “multiple slots” in the running CR to 38.306 so that the UE can report associated values. According to TS38.214, the definition of active CSI-RS depends on different type of CSI-RS configured for CSI reporting. For example, periodic CSI-RS is active starting when the periodic CSI-RS is configured by higher layer signalling and ending at the end of when the configuration is released, whilst aperiodic CSI-RS is active starting from the end of the PDCCH containing the request and ending at the end of PUSCH containing the report associated with this aperiodic CSI-RS.

.

Question2: The maximum value of simultaneous CSI-RS resources and CSI-RS ports.

In the existing SupportedCSI-RS-Resource, the maximum value of simultaneous resources is 64 and the one of total Tx ports is 256. RAN2 is wondering if the existing value is enough to address the total capability across all CCs or the larger value is desirable.

Answer: Yes

Question3: indication of maxNumberTxPortsPerResource in a per BC manner

In the RAN1 LS it is stated that “To address above issue, RAN1 has agreed to recommend to introduce new per band capability signaling and per BC capability signaling for component 1 of FG2-36/2-40/2-41/2-43”. The component 1 of FG2-36/2-40/2-41/2-43 contains maxNumberTxPortsPerResource. Currently RAN2 had no consensus to whether to introduce maxNumberTxPortsPerResource per BC. Without this additional field, the number of ports for each resource would be determined based on the values indicated for the band on which the resource is configured, like in Rel-15 signaling (given in the existing per-band signaling). See Annex A for an example. RAN2 would appreciate if RAN1 could provide feedback if this structure does not serve the intended purpose.

Answer: A list of supported combinations for each codebook, whereas each combination is a triplet of {maxNumberTxPortsPerResource, maxNumberResources, totalNumberTxPorts}, shall be signaled to gNB with a granularity of per BC.

2. Actions:

To RAN2:

ACTION:  RAN1 would like to respectfully ask RAN2 to take the above information into account.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN WG1 Meeting #101-e	25 – 29 May 2020,

TSG-RAN WG1 Meeting #102	24 – 28 August 2020, 		Toulouse, France

Reference

[1] R1-2002673 “Discussion on Reply LS on CSI-RS capabilities (FG2-33/36/40/41/43)”, 20th- 30th April, 2020.