3GPP TSG-RAN WG1 Meeting #100bis-e	R1-2002831

# E-meeting, April 20th – 30th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | 0022 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SS-RSRPB measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was recommended by LS R1-1909969/R4-1910610 from RAN4 that CSI reference signals could be removed from the definition of SS-RSRPB. Such recommendation was endorsed in R1-1909885 for Rel-16 specification. It should be also captured in Rel-15 specification. In RAN#87e, it was agreed in RP-200174 as follows, Following the observations under 2.1 it is recommended to not remove the SS-RSRPB definition from TS 38.215 REL-15 but to correct it by a resubmission of R1-1911445 to remove the CSI reference signals. This CR will require a corresponding cat. A REL-16 CR. Furthermore, a reference to RAN5 specifications will be added to the SS-RSRPB definition to point to corresponding measurement accuracy/range and signalling to make clear that this measurement is used for testing purposes only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove CSI reference signals from the definition of SS-RSRPB measurement Following the agreement at RAN #87e, add notes regarding the use of this UE measurement for conformance test purposes only and signaling to be defined in RAN5 specification TS 38.509 rather than RAN2/4 specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification on the definition of SS-RSRPB measurement for NR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impacts on SS-RSRPB measurement for NR Antenna test Function (ATF). Since it is for conformance test purposes only, there is no impact on gNB or UE signalling to the network. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.201: "NR; Physical Layer – General Description"

[3]	3GPP TS 38.211: "NR; Physical channels and modulation"

[4]	3GPP TS 38.212: "NR; Multiplexing and channel coding"

[5]	3GPP TS 38.213: "NR; Physical layer procedures for control channels"

[6]	3GPP TS 38.214: "NR; Physical layer procedures for data channels"

[7]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification"

[9]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification"

[11]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std."

[12]	3GPP TS 38.133: "NR; Requirements for support of radio resource management"

[13]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation"

[14]	3GPP TS 38.509: "5GS; Special conformance testing functions for User Equipment (UE)"

<Unchanged parts are omitted>

### 5.1.18	SS reference signal received power per branch (SS-RSRPB)

| Definition | SS reference signal received power per branch (SS-RSRPB) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals (SS). The measurement time resource(s) for SS-RSRPB are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. For SS-RSRPB determination demodulation reference signals for physical broadcast channel (PBCH) in addition to secondary synchronization signals may be used. SS-RSRPB using demodulation reference signal for PBCH shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in 3GPP TS 38.213 [5]. SS-RSRPB shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If higher-layers indicate certain SS/PBCH blocks for performing SS-RSRPB measurements, then SS-RSRPB is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, SS-RSRPB is not defined. For frequency range 2, SS-RSRPB shall be measured for each receiver branch based on the combined signal from antenna elements corresponding to the receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency |

NOTE 1:	The number of resource elements within the measurement period that are used by the UE to determine SS-RSRPB is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:	The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

NOTE 3:  This UE measurement is defined only for conformance test purposes. It is described along with test control entity signalling in [14].

<Unchanged parts are omitted>