3GPP TSG RAN WG1 Meeting #100bis-e     														R1-2002935

20th April– 30th April 2020

Agenda Item:		7.2.3.2

Source:				Qualcomm Incorporated

Title: 		Outcome of [100b-e-NR-IAB-01] regarding TP for section 6.3.3.2 of 38.211

Document for:		Discussion and decision

### Introduction

This contribution summarizes the outcome of the [100b-e-NR-IAB-01] discussion regarding a TP for section 6.3.3.2 of 38.211 and provides a summary of the preparation phase as part of agenda item 7.2.3.2 on the subject of extensions of RACH occasions and periodicities for backhaul RACH resources for planning the email discussion in RAN1 #100bis-e.

### Issues for discussion

There is one contribution [1] on this agenda item which addresses some issues with the current specifications in TS 38.211.

The issues relate to inaccuracies in the following paragraph of TS 38.211:

For the IAB-MT part of an IAB node, the following applies: - if the higher-layer parameter prach-ConfigurationPeriocScaling is configured, the variable x used in nSFN mod x=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by xIAB , where xIAB=δx and δ is given by the higher-layer parameter prach-ConfigurationPeriodScaling and the IAB node does not expect xIAB to be larger than 64; - if the higher-layer parameter prach-ConfigurationFrameOffset is configured, the variable y used in nSFN mod x=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by yIAB=y+Δy mod x where Δy is given by the higher-layer parameter prach-ConfigurationFrameOffset, and x is the value used in nSFN mod x=y; - if the higher-layer parameter prach-ConfigurationSOffset is configured, the subframe number in Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number in Table 6.3.3.2-4 shall be replaced by sn+Δs mod L where sn is the slot or subframe number, Δs∈0,1,…,L-1 is given by the higher-layer parameter prach-ConfigurationSOffset, and L is the number of subframes in a frame in Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame in Table 6.3.3.2-4.

Specifically, the main issue is that the text “and L is the number of subframes in a frame in Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame in Table 6.3.3.2-4” seems to imply that L is provided in those tables. However, that interpretation would not be accurate and a reader not familiar with the IAB RAN1 context may get confused.

Moreover, a potential issue is that the text “the subframe number in Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number in Table 6.3.3.2-4 shall be replaced by (s_n +Δs) mod L” may imply a value replacement in the tables while the previous two paragraphs describe a variable replacement in the equations using values from the tables.

A TP is proposed in [1] to address these issues.

A slightly simpler TP that minimizes the text changes is proposed:

Proposal 1:

Adopt the following TP for section 6.3.3.2 of 38.211:

## <Unchanged text is omitted> For the IAB-MT part of an IAB node, the following applies: - if the higher-layer parameter prach-ConfigurationPeriocScaling is configured, the variable x used in nSFN mod x=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by xIAB , where xIAB=δx and δ is given by the higher-layer parameter prach-ConfigurationPeriodScaling and the IAB node does not expect xIAB to be larger than 64; - if the higher-layer parameter prach-ConfigurationFrameOffset is configured, the variable y used in nSFN mod x=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by yIAB=y+Δy mod x where Δy is given by the higher-layer parameter prach-ConfigurationFrameOffset, and x is the value used in nSFN mod x=y; - if the higher-layer parameter prach-ConfigurationSOffset is configured, the subframe number sn from in Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number sn from in Table 6.3.3.2-4 shall be replaced by sn+Δs mod L where sn is the slot or subframe number, Δs∈0,1,…,L-1 is given by the higher-layer parameter prach-ConfigurationSOffset, and L is the number of subframes in a frame when using in Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame for SCS of 60 KHz when using in Table 6.3.3.2-4. <Unchanged text is omitted>

Discussion on Proposal 1:

| Company | Support Proposal 1? [Yes/No] | Comments |
| --- | --- | --- |
| Qualcomm | Yes | None. |
| ZTE, Sanechips | Yes | None. |
| LG | Yes | None. |
| Ericsson | Yes | We are fine with the TP. Please also note there is a spelling error in the first prach-ConfigurationPeriodScaling. |
| Nokia | Yes | None |
| Huawei | Yes | Regarding the proposed change, we would like to support Luca’s version with a slight update marked in blue this is already from the current spec. For the purpose of slot numbering in the tables, the following subcarrier spacing shall be assumed: - 15 kHz for FR1 - 60 kHz for FR2. if the higher-layer parameter prach-ConfigurationSOffset is configured, the subframe number sn from in Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number sn from in Table 6.3.3.2-4 shall be replaced by sn+Δs mod L where sn is the slot or subframe number, Δs∈0,1,…,L-1 is given by the higher-layer parameter prach-ConfigurationSOffset, and L is the number of subframes in a frame when using in Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame for SCS of 60 KHz when using in Table 6.3.3.2-4. Thanks for the further discussion. To me, the SCS for slot numbering is a general assumptions and should also be used to determine the number of slots within a frame. I don’t see why this assumption is not followed… Anyway, I will not object to repeating it again if everyone else feels differently. |
| Samsung |  | Generally we are fine with the TP, just want to confirm one following understanding: the prach-ConfigurationSOffset also give the offset in terms of subframe (15khz) for FR1 and slot (for 60khz) for FR2, right? which this should be aligned with the one for Sn dd, since I cannot tell the reference SCS from the 331. if above understanding is common, i think the last sentence for the scs of 60khz, could be replaced with "the number of 60khz slots in a frame when using in Table 6.3.3.2-4." Since in table, both “Slot number“ and "Number of PRACH slots within a 60 kHz slot" are used. we think it is more accurate to reflect the 60khz here for "L", in order to align with Sn, Delta_S. |
| Intel | Yes | We agree with Huawei, as TS38.211 6.3.3.2 already has a clear statement regarding the SCSs for the Tables (including Table 6.3.3.2-4) as below. For the purpose of slot numbering in the tables, the following subcarrier spacing shall be assumed: - 15 kHz for FR1 - 60 kHz for FR2. |

### Conclusion

Based on the support from companies during the preparation phase and during the [100b-e-NR-IAB-01] email discussion the following was agreed:

Agreement 1:

Adopt the following TP for section 6.3.3.2 of 38.211:

## <Unchanged text is omitted> For the IAB-MT part of an IAB node, the following applies: - if the higher-layer parameter prach-ConfigurationPeriocdScaling is configured, the variable x used in nSFN mod x=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by xIAB , where xIAB=δx and δ is given by the higher-layer parameter prach-ConfigurationPeriodScaling and the IAB node does not expect xIAB to be larger than 64; - if the higher-layer parameter prach-ConfigurationFrameOffset is configured, the variable y used in nSFN mod x=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by yIAB=y+Δy mod x where Δy is given by the higher-layer parameter prach-ConfigurationFrameOffset, and x is the value used in nSFN mod x=y; - if the higher-layer parameter prach-ConfigurationSOffset is configured, the subframe number sn from in Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number sn from in Table 6.3.3.2-4 shall be replaced by sn+Δs mod L where sn is the slot or subframe number, Δs∈0,1,…,L-1 is given by the higher-layer parameter prach-ConfigurationSOffset, and L is the number of subframes in a frame when using in Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame for SCS of 60 KHz when using in Table 6.3.3.2-4. <Unchanged text is omitted>

### References

[1]		R1-2001874 – “Remaining issues in IAB PRACH” – ZTE, Sanechips

### Appendix

##### Proposals from contributions:

| R1-2001874 | According to the discussion above, we provide the following proposal and text proposals on PRACH occasions of IAB node MT: #### 6.3.3.2 Mapping to physical resources ## <Unchanged text is omitted> For the IAB-MT part of an IAB node, the following applies: - if the higher-layer parameter prach-ConfigurationPeriocScaling is configured, the variable x used in nSFN mod x=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by xIAB , where xIAB=δx and δ is given by the higher-layer parameter prach-ConfigurationPeriodScaling and the IAB node does not expect xIAB to be larger than 64; - if the higher-layer parameter prach-ConfigurationFrameOffset is configured, the variable y used in nSFN mod x=y of Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by yIAB=y+Δy mod x where Δy is given by the higher-layer parameter prach-ConfigurationFrameOffset, and x is the value used in nSFN mod x=y; - if the higher-layer parameter prach-ConfigurationSOffset is configured, the subframe number in Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number in Table 6.3.3.2-4 for PRACH time resources shall be replaced by sn+Δs mod L where sn is the slot or subframe number, Δs∈0,1,…,L-1 is given by the higher-layer parameter prach-ConfigurationSOffset, and L is the number of subframes in a frame in Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame in Table 6.3.3.2-4. - For subframe number determination, sn is the subframe number in Tables 6.3.3.2-2 to 6.3.3.2-3 and L equals to 10. - For slot number determination, sn is the slot number in Tables 6.3.3.2-4 and L equals to 40. |
| --- | --- |