3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002945

E-meeting, April 20 – April 30, 2020

Agenda Item:	6.2.2.2

Source:	Huawei

Title:	Text Proposal on PUR power control for TS 36.213

Document for:	Discussion and Decision

Introduction

This contribution provides the text proposal produced as a result of the following email discussion:

[100b-e-LTE-NB_IoTenh3-PUR-02] Clarification on PUR power control by 4/24 and corresponding TP (if any) by 4/30 – Xiang (Huawei)

- Issue#7 in R1-2002714

- Reasons for change: the relationship between PO_NPUSCH,c3 and the HL parameter might be unclear; in  and , the character “c” overlaps the character “H”.

- Summary of changes: link PO_NPUSCH,c3 to the corresponding HL parameter; replacing “” by “PO_NPUSCH,cj”, and replacing “” by “MNPUSCH,c(i)”

- Specs/Sections impacted: Section 16.2.1.1.1 of TS 36.213

- Consequences if not approved: the relationship between  PO_NPUSCH,c3 and the HL parameter might be unclear; and  and  are not friendly for reading.

Text Proposal

------------------------------------- Start of Text Proposal for TS 36.213-------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- ##### 16.2.1.1.1 UE behavior … where, - is the configured UE transmit power defined in [6] in NB-IoT UL slot i for serving cell . - MNPUSCH,c(i) is {1/4} for 3.75 kHz subcarrier spacing and {1, 3, 6, 12} for 15kHz subcarrier spacing - PO_NPUSCH,cjis a parameter composed of the sum of a component provided from higher layers and a component provided by higher layers for j=1, 3 and for serving cell where . For NPUSCH (re)transmissions corresponding to a dynamic scheduled grant or a semi-persistent grant then j=1, for NPUSCH (re)transmissions corresponding to the random access response grant then j=2 and for NPUSCH transmission using preconfigured uplink resource then j=3. PO_UE_NPUSCH,c2=0. If enhanced random access power control is not applied, PO_NOMINAL_NPUSCH,c(2)=PO_PRE+∆PREAMBLE_Msg3, where the parameter preambleInitialReceivedTargetPower [8] () and ∆PREAMBLE_Msg3 are signalled from higher layers for serving cell . If enhanced random access power control is applied, PO_NOMINAL_NPUSCH,c2=MSG3_RECEIVED_TARGET_POWER +ΔPREAMBLE_Msg3 … -------------------------------------------- Unchanged parts omitted -------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

- Reference

- R1-2001570	Corrections on transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-2001851	Remaining issues for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-2002176	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-2002507	Corrections for Preconfigured UL resources for NB-IoT	Ericsson

- R1-2002645	Remaining issues for preconfigured UL resources	Nokia, Nokia Shanghai Bell

- R1-2001517	LS on open PUR issues for NB-IoT/eMTC	RAN2, Ericsson

- R1-2001849	Discussion on RAN2 LS on open PUR issues	ZTE

- R1-2002501	On the LS on open PUR issues for NB-IoT/eMTC	Ericsson

- R1-2002603	Draft reply LS on open PUR issues for NB-IoT/eMTC	Huawei, HiSilicon