3GPP TSG RAN WG1 #100bis		R1-2002948

e-Meeting, April 20th – 30th, 2020

Source:	Moderator (OPPO)

Title:	Text Proposal 2 in [100b-e-NR-eMIMO-multiTRP-03]

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

- Introduction

Rel-16 enhancement on MIMO WID includes objectives of enhancing multi-TRP/Panel transmission with ideal and non-ideal backhaul. During the work of rel-16, designs for multiple-PDCCH based and single-PDCCH based multi-TRP/Panel transmission were discussed and specified. This document provides the TP for issue #b-8 listed in [21] for TS 38.214

- #b-8: Specify mapping type for PDSCH in scheme 3

- Text Proposal

Reason for changes:

For scheme 3 (‘TDMSchemeA’), it has been agreed that the indicated SLIV is applied to the first transmission occasion, and the second transmission occasion has the same number of symbols as the first transmission occasion. The PDSCH mapping type for the first transmission occasion can be indicated by the DCI, but the mapping type for the second transmission occasion is not clearly specified yet.

Summary of changes:

In TS 38.214, calrify that the mapping type for PDSCH of scheme 3 is always mapping type B and the mapping type B is applied to both PDSCH transmission occasions.

Specs/Sections impacted:

TS 38.214 V16.1.0 /5.1.2.1

Consequences if not approved:

The mapping type for PDSCH transmission occasion in PDSCH of scheme 3 (‘TDMSchemeA’) is not clear and the UE behavior for receiving PDSCH of scheme 3 (‘TDMSchemeA’) is ambiguous.

The text proposal for TS 38.214 is:

#### 5.1.2.1 Resource allocation in time domain ---- Unchanged text are omitted ---- When a UE is configured by the higher layer parameter RepSchemeEnabler set to 'TDMSchemeA' and indicated DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. - If two TCI states are indicated by the DCI field 'Transmission Configuration Indication', the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. UE expects the PDSCH mapping type indicated by DCI field “Time domain resource assignment’ to be mapping type B, and the indicated PDSCH mapping type is applied to both PDSCH transmission occasions. - Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1. ---- Unchanged text are omitted ----

- Reference

- R1-2001563	Remaining issues on multi-TRP in R16	Huawei, HiSilicon

- R1-2001596	Maintenance of multi-TRP enhancements	ZTE

- R1-2001678	Discussion on remaining issues on multi TRP operation	vivo

- R1-2001726	Text proposals for enhancements on multi-TRP and panel Transmission	 OPPO

- R1-2001822	Enhancements on Multi-TRP/Panel Transmission	MediaTek Inc.

- R1-2001913	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

- R1-2001971	Remaining issues on multi-TRP/panel transmission	Lenovo, Motorola Mobility

- R1-2002004	Corrections to multi-TRP	Intel Corporation

- R1-2002052	TP on Multi-TRP/Panel Transmission	Futurewei

- R1-2002090	Remaining issues on multi-TRP/panel transmission	CATT

- R1-2002139	On Rel.16 multi-TRP/panel transmission	Samsung

- R1-2002212	Remaining issues on multi-TRP/panel transmission	CMCC

- R1-2002270	Discussion on remaining issues for multi-TRP operation	Spreadtrum Communications

- R1-2002294	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

- R1-2002337	Remaining issues for Multi-TRP enhancement	Apple

- R1-2002364	Remaining issues on multi-TRP transmission	NEC

- R1-2002378	Clarification on Multi-TRP URLLC Scheme 4	Convida Wireless

- R1-2002448	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC

- R1-2002472	Remaining issues on Multi-TRP/Panel Transmission	Ericsson

- R1-2002551	Multi-TRP Enhancements	Qualcomm Incorporated

- R1-2002730         FL summary #2 for Multi-TRP/Panel Transmission