3GPP TSG-RAN WG1 Meeting #100bis-e	                 R1-2002991

e-Meeting, April 20th – April 30th, 2020

Agenda Item:	7.2.3.3

Source:	AT&T

Title:	TP for 38.213 regarding usage of tdd-UL-DL-ConfigurationDedicated-IAB-MT

Document for:	Discussion/Approval

# Introduction

This contribution provides a TP on usage of tdd-UL-DL-ConfigurationDedicated-IAB-MT based on the following agreement reached during RAN1#100bis-e:

# TP for 38.213 regarding usage of tdd-UL-DL-ConfigurationDedicated-IAB-MT

| Reason for change: | During RAN1#100b-e the following agreement was reached regarding the usage of tdd-UL-DL-ConfiguraitonDedicated-IAB-MT in 38.213: Agreement TS 38.213 Section 14 should be updated with a TP capturing the following behaviors when an IAB-MT is provided with tdd-UL-DL-ConfigurationDedicated-IAB-MT: - Clarification that the behaviors described in Section 11.1 of 38.213 for a UE provided with tdd-UL-DL-ConfigurationDedicated are also applicable for an IAB-node MT when provided with tdd-UL-DL-ConfigurationDedicated-IAB-MT - The IAB-node MT does not expect to be configured with both tdd-UL-DL-ConfigurationDedicated and tdd-UL-DL-ConfigurationDedicated-IAB-MT. - Note: It is up to the 38.213 editor to decide whether or not to include this behavior in case of misconfiguration in the final TP to 38.213. However, this agreement has not yet been captured yet in current version of 38.213. |
| --- | --- |
| Summary of change: | Adds specification text describing that the behaviors described in Section 11.1 of 38.213 for a UE provided with tdd-UL-DL-ConfigurationDedicated are also applicable for an IAB-node MT when provided with tdd-UL-DL-ConfigurationDedicated-IAB-MT |
| Consequences if not approved: | The description of IAB-MT behavior when configured with tdd-UL-DL-ConfigurationDedicated-IAB-MT is incomplete. |
| Clauses affected: | TS 38.213 Section 14 |

< Unchanged parts are omitted >

#### For each serving cell, an IAB-node MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigurationDedicated-IAB-MT.

If the IAB-node MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the statements in clauses 11.1 containing the term "tdd-UL-DL-ConfigurationDedicated" apply to the IAB-node MT of an IAB node, with the term "tdd-UL-DL-ConfigurationDedicated" replaced with "tdd-UL-DL-ConfigurationDedicated-IAB-MT”, except that the tdd-UL-DL-ConfigurationDedicated-IAB-MT provides

#### If the IAB-node MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides only flexible symbols over the number of slots as provided by TDD-UL-DL-ConfigurationCommon.

The tdd-UL-DL-ConfigurationDedicated-IAB-MT provides

-	a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT

-	for each slot configuration from the set of slot configurations

-	a slot index for a slot provided by slotIndex

-	a set of symbols for a slot by symbols where

-	if symbols = allDownlink, all symbols in the slot are downlink

-	if symbols = allUplink, all symbols in the slot are uplink

-	if symbols = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible.

-	if symbols = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible.

For each slot having a corresponding index provided by slotIndex, the IAB-MT applies a format provided by a corresponding symbols.

< Unchanged parts are omitted >