3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002994

e-Meeting, April 20th - 30th, 2020

Agenda item:	6.2.1.8

Source:	Moderator (Nokia)

Title:	Text proposals for [100b-e-LTE-eMTC5-Use-of-LTE-control-channel-region-01]

Document for:	Discussion and Decision

# Introduction

This document includes the endorsed text proposals for the email discussion –

[100b-e-LTE-eMTC5-Use-of-LTE-control-channel-region-01] Email discussion on the following issues by 4/24 and corresponding TP (if any) by 4/30

# Clarification of “for other purposes” wording

Reasons for change:  In section 6.4.1 of 36.211, it is unclear what is meant by “resource elements k,l not reserved for other purposes” when the PDSCH is mapped onto the LTE control channel region.

Summary of changes:  Added clarification to have the mapping to the LTE Control Channel region be specified as an extension to the current specification.

Specs/Sections impacted: TS 36.211 Section 6.4.1

Consequences if not approved:  Unclear what is meant by “resource elements k,l not reserved for other purposes”.

Text proposal for TS 36.211

---------------------------------------------- Text start -------------------------------------------------------

6.4.1	Physical downlink shared channel for BL/CE UEs

<Unchanged parts are omitted>

-	If PDSCH transmission in the LTE control region is configured by higher layer parameter ce-dl-lte-control-region-config, after the initial mapping of the PDSCH to resource elements (k,l) klstarting from l=lDataStart  in the first slot to the last OFDM symbol available for downlink transmission in the subframe has been performed, the mapping shall continue with resource elements k,l not reserved for cell-specific reference signals in increasing order of first the index k over the assigned physical resource blocks and then the index l starting from l=0 in the first slot to l=lDataStart-1 in the first slot, where lDataStart  is given by clause 7.1.6.4 of 3GPP TS 36.213 [4].

<Unchanged parts are omitted>

-------------------------------------------------- Text end ----------------------------------------------------------

# MPDCCH transmission type for PUR

Reasons for change:  It is unclear what type of MPDCCH transmission (distributed or localized) is used for UE-specific search space configured by PUR C-RNTI.

Summary of changes:  Specify that distributed MPDCCH transmission is used for UE-specific search space configured by PUR C-RNTI.

Specs/Sections impacted: TS 36.213 Section 9.1.5.

Consequences if not approved:  Unclear what type of MPDCCH transmission is used for UE-specific search space configured by PUR C-RNTI.

Text proposal for TS 36.213

---------------------------------------------- Text start -------------------------------------------------------

9.1.5	MPDCCH assignment procedure

<Unchanged parts are omitted>

For Type1-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space and Type2A-MPDCCH common search space, distributed MPDCCH transmission is used.

For MPDCCH UE-specific search space configured by PUR C-RNTI, distributed MPDCCH transmission is used.

<Unchanged parts are omitted>

-------------------------------------------------- Text end ----------------------------------------------------------

# References

- R1-2002993, “FL summary on [100b-e-LTE-eMTC5-Use-of-LTE-control-channel-region-01],” Moderator (Nokia), RAN1#100bis-e, e-Meeting.

- R1-2001860, “Clarification on use of LTE control region for PUR related DL transmissions,” ZTE, RAN1#100bis-e, e-Meeting.

- R1-2002506, “Corrections for Use of LTE control channel region for DL transmission in LTE-MTC,” Ericsson, RAN1#100bis-e, e-Meeting.

- R1-2000229, “Corrections on use of LTE control channel region for DL transmission,” Huawei, HiSilicon, RAN1#100-e, e-Meeting.

- R1-2000374, “Correction on use of LTE control channel region,” ZTE, RAN1#100-e, e-Meeting.

- R1-202699, “Feature lead summary #1 on MPDCCH performance improvement,” Moderator (Huawei), RAN1#100bis-e, e-Meeting.

- R1-201574, “Corrections on MPDCCH performance improvement,” Huawei, HiSilicon, RAN1#100bis-e, e-Meeting.

- R1-201854, “Clarification on MPDCCH performance improvement,” ZTE, RAN1#100bis-e, e-Meeting.