3GPP TSG RAN WG1 Meeting #100bis	R1-2003114

# e-Meeting, 20th – 30th April 2020

Agenda Item:	6.2.1.3

Source:	Moderator (Ericsson)

Title:	TP for 36.213 on HARQ-ACK bundling size for LTE-MTC multi-TB scheduling

Document for:	Discussion, Decision

# Introduction

RAN1#100bis-e discussed HARQ-ACK bundling size for Multi-TB scheduling for Rel-16 LTE-MTC in the email discussion “[100b-e-LTE-eMTC5-Multi-TB-03] HARQ-ACK bundling”. For more background information, see “Issue #3” in the email discussion summary in R1-2002796. This contribution presents a 36.213 TP based on the outcome of the discussion.

| Reason for change: | RAN1#100bis-e made the following agreement regarding the HARQ-ACK bundling size, bundling patterns and DCI field mapping for Rel-16 LTE-MTC which needs to be captured in the specification: |
| --- | --- |
| Summary of change: | The agreement is implemented in the specification. |
| Consequences if not approved: | HARQ-ACK bundling may be incorrectly implemented. |
| Clauses affected: | 7.3 |

# Text proposal for 36.213

## 7.3	UE procedure for reporting HARQ-ACK

<Unchanged parts are omitted>

For a BL/CE UE, if the UE is configured with CEModeA, and if the UE is configured with higher layer parameter multi-TB-DL-HARQ-bundling and multiple TB are scheduled in the corresponding DCI format 6-1A with CRC scrambled by C-RNTI,

-	for HARQ-ACK transmission associated with the corresponding DCI, the UE shall generate M HARQ-ACK bits by performing a logical AND operation of HARQ-ACKs across all TBs in each TB bundle Ab where b = 1, …, M;

-	the set of TBs that belong to TB bundle Ab and the number of TB bundles M are given by Table XX.XX;

-	the value of is the number of scheduled TB determined in the corresponding DCI.

Table XX.XX: Value of Ab for different values of DCI field ‘Multi-TB HARQ-ACK bundling size’ and for different values of number of scheduled transport blocks NTB.

| DCI field ‘Multi-TB HARQ-ACK bundling size’ | NTB=1 | NTB=2 | NTB=4 | NTB=6 | NTB=8 |
| --- | --- | --- | --- | --- | --- |
| 00 | A1={TB0} | A1=TB0 A2=TB1 | A1=TB0 A2=TB1 A3=TB2 A4=TB3 | A1=TB0 A2=TB1 A3=TB2 A4=TB3 A5=TB4 A6=TB5 | A1=TB0 A2=TB1 A3=TB2 A4=TB3 A5=TB4 A6=TB5 A7=TB6 A8=TB7 |
| 01 | - | A1=TB0, TB1 | A1=TB0, TB1 A2=TB2, TB3 | A1=TB0, TB1 A2=TB2, TB3 A3=TB4, TB5 | A1=TB0, TB1 A2=TB2, TB3 A3=TB4, TB5 A4=TB6, TB7 |
| 10 | - | - | A1=TB0, TB1 A2=TB2 A3=TB3 | A1=TB0, TB1,TB2 A2=TB3, TB4,TB5 | A1=TB0, TB1,TB2 A2=TB3, TB4,TB5 A3=TB6, TB7 |
| 11 | - | - | A1=TB0, TB1,TB2, TB3 | A1=TB0, TB1,TB2,TB3 A2=TB4, TB5 | A1=TB0, TB1,TB2,TB3 A2=TB4, TB5,TB6,TB7 |