3GPP TSG RAN WG1 #100bis-e Meeting	    			             R1-2003135

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.4.1

Source: 	Moderator (Samsung)

Title: 	Text Proposal for indication of MCS tables

Document for:	Endorsement

Introduction

During WI and RAN1#100b-e meeting, the following were agreed.

Agreements: (in RAN1#98bis) - Three MCS tables supported in Rel-15 NR Uu CP-OFDM are also used for SL. - Support of the the low-spectral efficiency 64QAM MCS table is an optional UE feature in SL as in the Uu link - For each resource pool, at least one MCS table is (pre)-configured - FFS whether or not to introduce a case where the MCS table can be overwritten by PC5 RRC or indicated in SCI - Each resource pool is only configured with one 1st stage SCI PSCCH format Agreements: (in RAN1#100bis-e) - The MCS table is indicated by 1st SCI, the number of MCS tables is (pre-) configured per resource pool. - 64QAM table is (pre-)configured as default. - Zero, one or two additional can be additionally (pre-)configured. Tables - Using the 256QAM and/or low-SE MCS tables - The number of bits in the 1st SCI for the indication is determined based on the number of MCS tables (pre)-configured for the resource pool - 0, 1, or 2 bits - Over-writing the (pre-)configured MCS table(s) by PC5-RRC is NOT supported - A UE is not required to decode the 2nd SCI or the PSSCH associated with a 1st SCI if the 1st SCI indicates an MCS table that the UE does not support

Summary and justification

For NR sidelink, it was decided to support all three MCS tables defined for Rel-15 NR Uu. However, RAN1 has not determined how to use all the MCS tables in a resource pool.

In RAN1#10bis-e, RAN1 decided to define a bit field in a 1st SCI (SCI format 0-1) to indicate which MCS table is used for PSSCH among the candidate MCS table(s) (pre-)configured for the resource pool. The size of the bit field in the 1st SCI is determined by the number of the tables (pre-)configured, while 64QAM MCS table is a default table when there is no configuration.

For example, if there is no MCS table configuration for the resource pool, then there is no bit field for MCS table indication in the 1st SCI and UEs in the resource pool use 64QAM MCS table only. If there is an additional MCS table indication with 256QAM MCS table, then there is one bit in the 1st SCI to indicate whether the TX UE uses 64QAM MCS table or 256QAM MCS table.

The TPs in this document are to reflect those agreements/changes decided by RAN1 in RAN1#100bis-e.

Text Proposal for TS38.212

< Unchanged parts are omitted >

- 8.3.1.1	SCI format 0-1

SCI format 0-1 is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 0-1:

-	Priority – 3 bits as defined in clause x.x.x of [6, TS 38.214].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause x.x.x of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause x.x.x of [6, TS 38.214].

-	Resource reservation period – log2(NreservPeriod) bits as defined in clause x.x.x of [6, TS 38.214], if higher parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	DMRS pattern – [x] bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], if more than one DMRS patterns are configured by higher layer parameter sl-PSSCH-DMRS-TimePattern; 0 bit otherwise.

-	2nd-stage SCI format – [x] bits as defined in clause x.x.x of [6, TS 38.214].

-	Beta_offset indicator – [2] bits as provided by higher layer parameter sl-BetaOffsets2ndSCI.

-	Number of DMRS port – 1 bit as defined in Table 8.3.1.1-1.

-	Modulation and coding scheme – 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

- 	MCS table indicator – as defined in clause 8.1.3.1 of [6, TS 38.214]: 1 bit if one MCS table is configured by higher layer parameter sl-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl-MCS-Table; 0 bit otherwise.

-	Reserved – [2 - 4] bits as determined by higher layer parameter sl-NumReservedBits, with value set to zero.

Table 8.3.1.1-1: Number of DMRS port

| Value of the Number of DMRS port field | Antenna ports |
| --- | --- |
| 0 | 1000 |
| 1 | 1000 and 1001 |

< Unchanged parts are omitted >

Text Proposal for TS38.214

< Unchanged parts are omitted >

- 8.1.3	Modulation order, target code rate, redundancy version and transport block size determination

- 8.1.3.1	Modulation order and target code rate determination

IMCS is given by the "Modulation and coding scheme" field in SCI format 0-1.

The MCS table is determined as follows: Table 5.1.3.1-1 is used if no additional MCS table is confiigured by higher layer parameter sl-MCS-Table; otherwise a MCS table is determined according to Table 8.1.3.1-1 and Table 8.1.3.1-2 and MCS indicator field in SCI format 0-1.

Table 8.1.3.1-1: Mapping of one bit of MCS table indicator to MCS table

| MCS table indicator | MCS table |
| --- | --- |
| '0' | Table 5.1.3.1-1 |
| '1' | 1st table provided by higher layer parameter sl-MCS-Table |

Table 8.1.3.1-2: Mapping of two bits of MCS table indicator to MCS table

| MCS table indicator | MCS table |
| --- | --- |
| '00' | Table 5.1.3.1-1 |
| '01' | 1st table provided by higher layer parameter sl-MCS-Table |
| '10' | 2nd table provided by higher layer parameter sl-MCS-Table |
| '11' | reserved |

The UE shall use IMCS and the MCS table determined according to the previous step to determine the modulation order (Qm) and Target code rate (R) used in the physical sidelink shared channel.

< Unchanged parts are omitted >

- 8.3 UE procedure for receiving the physical sidelink shared channel

For sidelink resource allocation mode 1, a UE upon detection of SCI format 0-1 on PSCCH can decode PSSCH according to the detected SCI format 0-2, and associated PSSCH resource configuration configured by higher layers.

For sidelink resource allocation mode 2, a UE upon detection of SCI format 0-1 on PSCCH can decode PSSCH according to the detected SCI format 0-2, and associated PSSCH resource configuration configured by higher layers.

A UE is required to decode neither the corresponding SCI format 0-2 nor the PSSCH associated with an SCI format 0-1 if the SCI format 0-1 indicates an MCS table that the UE does not support.

< Unchanged parts are omitted >