3GPP TSG RAN WG1 Meeting #100bis                                                                 	R1-200xxxx

e-Meeting, April 20 - April 30, 2020

Title:	[Draft] LS reply on eMIMO RRC parameters

Response to: 	R1-2001505 (R2-2001683)

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	Ericsson [RAN1]

To:	RAN2

CC:

Contact Person:

#### Name:	Mattias Frenne

E-mail Address:	mattias.frenne@ericsson.com

# 1. Overall Description:

RAN1 would like to thank RAN2 for the LS R2-2001683 on eMIMO RRC and MAC layer parameters.

In R2-2001683, RAN2 asked five questions on the detailed parameters for UL full power, multi-beam and multi-TRP. RAN1 would like to provide the answers as follows.

# 2 Question 1 ULFPTX

RAN2 agreed (see LS R1-2001505):

Given the above analysis we propose to keep the three modes for ULFPTX.

RAN2 wrote (see LS R1-2001505):

The ULFPTX seems to have three distinct modes of operation which is why RAN2 agreed to support ULFPTX by parameter ul-FullPowerTransmission in IE PUSCH-Config as follows in Error: Reference source not found:

ul-FullPowerTransmission-r16            ENUMERATED {fullpower, fullpowerMode1, fullpoweMode2}     OPTIONAL    -- Need R

Question 1.

RAN2 would like to inform RAN1 about the above agreement and ask RAN1 to indicate if they find issues with the agreement.

Proposed reply:

RAN1 does not see any issue with the related RAN2 agreement.

Please provide comments below

| Company | Comment |
| --- | --- |
| Moderator-Ericsson | The majority view is OK with using a single RRC parameter to configure full power operation as RAN2 have agreed. |

# 3 Question 2 Parameter BDFactor (mTRP)

Question 2. RAN2 would like to ask RAN1 to inform RAN2 whether the parameter BDFactor should be per cell (e.g. within IE ServingCellConfig) or per cell group (e.g. within IE PhysicalCellGroupConfig)?

Proposed reply:

The parameter BDFactor shall be per cell group (within IE PhysicalCellGroupConfig).

Please provide comments below

| Company | Comment |
| --- | --- |
| Moderator-Ericsson | The majority propose this reply |

# 4 Question 3 Configuration of different repetition schemes

Question 3. RAN2 asks RAN1 to inform RAN2 if repetition schemes 2a/2b/3 (fdmSchemeA, fdmSchemeB and tdmScheme) and scheme 4 (slotBased) are mutually exclusive in all UE configuration options, and to provide a reference which could be used in TS 38.331 for referencing about the configuration limitations for the repetition schemes.

Proposed reply:

From RAN1 point of view, RAN2 specifications do not need to state any restrictions related to repetition schemes 2a/2b/3 and scheme 4 (RAN2 specs do not need to specify them as mutually exclusive).  Any applicable restrictions can instead be captured in TS 38.214 which can be the reference.

Please provide comments below

| Company | Comment |
| --- | --- |
| Moderator-Ericsson | The majority of companies indicate that RAN2 specs do not need to capture such restrictions. |

# 5 Question 4 SRS activation/deactivation MAC CE for multiple CCs/BWPs

Question 4. RAN2 would like to ask RAN1 whether the intention is to activate per SRS resource set or per SRS resource. Further, if per SRS resource, whether RAN1 sees any issues in indicating spatial relation, potentially different, for more than one resource in one MAC CE in order to save overhead.

Proposed reply:

RAN1 understands that the intention of the agreement is to activate per SRS resource. Furthermore, RAN1 has no issue in using one MAC CE (to save overhead) to activate/deactivate spatial relations for >1 SRS resources.

Please provide comments below

| Company | Comment |
| --- | --- |
| Moderator-Ericsson | The majority of companies seem to be ok with this reply |

# 6 Question 5 max number of PUCCH resources in a PUCCH group or max number of serving cells in the CC lists

Question 5. RAN2 asks RAN1 to provide values for maximum number of PUCCH resources in a PUCCH group and for maximum value of serving cells in per CC/BWP lists.

Proposed reply:

RAN1 understands that the maximum number of PUCCH resources is 128 and the maximum value of serving cells in a CC/BWP list is 32.

Please provide comments below

| Company | Comment |
| --- | --- |
| Moderator-Ericsson | The majority of companies seem to be ok with this reply |

2. Actions:

To RAN2

ACTION: RAN1 would like to respectfully ask RAN2 to take the above information into account.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #101-e	25 May - 6 June. 2020, 		Online

TSG-RAN WG1 Meeting #102	24 - 28 Aug. 2020, 		Toulouse, FR