3GPP TSG RAN WG1 Meeting #100bis                                                                 	R1-200xxxx

e-Meeting, April 20 - April 30, 2020

Title:	[Draft] reply LS on the starting point of MsgB window

Response to: 	R1-2001511 (R2-2002298)

Release:	Rel-16

Work Item:	NR_2step_RACH-Core

Source:	ZTE [RAN1]

To:	RAN2

CC:

Contact Person:

#### Name:	Li Tian

E-mail Address:	tian.li150@ zte.com.cn

1. Overall Description:

RAN1 would like to thank RAN2 for the LS on the starting point of MsgB window.

RAN1 confirms the starting point for MsgB window is not clear from RAN1 specs when the UE transmits only PRACH in a valid PRACH occasion if the PRACH occasion is not mapped to a valid PUSCH occasion.

RAN1 finds there could be more cases causing UE to transmit PRACH only MsgA, and RAN1 concludes that:

- For a transmission of only PRACH if the PRACH preamble is mapped to a valid PUSCH occasion associated with a DMRS resource but due to certain reason the PUSCH is not transmitted, UE determines the msgB window starting position by using end of corresponding PUSCH occasion as reference point;

- For a transmission of only PRACH if the PRACH preamble is not mapped to a valid PUSCH occasion associated with a DMRS resource, UE determines the msgB window starting position by using end of the PRACH occasion as reference point.

RAN1 would fix it in TS 38.213 as follows.

---------------------------------Text proposal starts for TS 38.213, Section 8.2A -------------------------------- =====unchanged text omitted ====== In response to a transmission of a PRACH and a PUSCH, or a transmission of a PRACH if the PRACH occasion corresponding to the PRACH transmission is mapped to a valid PUSCH occasion but the PUSCH is not transmitted, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MsgB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB-ResponseWindow. In response to a transmission of a PRACH if the PRACH occasion corresponding to the PRACH transmission is not mapped to a valid PUSCH occasion, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MsgB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB-ResponseWindow. ===== unchanged text omitted ====== ------------------------------- Text proposal ends for TS 38.213, Section 8.2A -----------------------------------

Any comments?

| Company | Comment |
| --- | --- |
| Ericsson | Please find some text proposals in the reply to RAN2 from our side. And do we need to put the RAN1 TP in the reply LS? |

2. Actions:

To RAN2

ACTION: RAN1 would like to respectfully ask RAN2 to take the above information into account.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #101-e	25 May - 6 June. 2020, 		Online

TSG-RAN WG1 Meeting #102	24 - 28 Aug. 2020, 		Toulouse, FR