3GPP TSG RAN WG1 #100b eMeeting	R1- 20xxxxx

April 20th – 30th, 2020

Agenda item:	7.2.5.4

Source: 	Qualcomm

Title: 	Summary of the Remaining Issues on HARQ and Scheduling Enhancements for URLLC

Document for:	Discussion and Decision

# 1	Introduction

In this document, proposals related to out-of-order operation for NR URLLC are summarized. The related conclusion from RAN1 #99 is as follows:

Conclusion:

- For Rel. 16 URLLC, no support of out-of-order/overlap PDSCH/HARQ and out-of-order/overlap PUSCH operation.

This issue was again discussed after the RAN1 #99 as part of email discussion [99-NR-13]; however, no agreement was made. The editor mentioned that the following note is placed for the related clause in Section 9 of TS 39.213:

a first PUCCH of larger priority index, a PUSCH or a second PUCCH of smaller priority index, and a transmission of the first PUCCH would overlap in time with a transmission of the PUSCH or the second PUCCH, the UE does not transmit the PUSCH or the second PUCCH.

Editor’s note: “TBD for two PUCCHs with HARQ-ACK, associated with two DG-PDSCHs on a scheduled cell with different priorities, overlap in time”

Hence, in the current Rel. 16 specification, the collision across two PUCCHs carrying HARQ-ACK with different priorities for the PDSCHs scheduled on the same serving cell is not supported.

# 2         Proposals

Issue #1: “Overlapping PUCCHs carrying HARQ-ACK with different priorities”

The related proposals are captured in the following table:

| Company | Proposal |
| --- | --- |
| Ericsson | - For two PUCCHs carrying two HARQ-ACK codebooks with different priorities, respectively, if the two PUCCHs overlap in time, the multiplexing/prioritization procedure is applied. This is already captured by existing specification. |
| vivo | - UE can be scheduled with two PUCCHs carrying HARQ-ACK with different priorities associated with two DG-PDSCHs scheduled on the same carrier overlapping in the time domain. |
| ZTE | - For overlapping of two PUCCHs carrying HARQ-ACK with different priorities in Rel-16, no additional restrictions should be introduced. - In case of two non-overlapping PUCCHs carrying HARQ-ACK with different priorities associated with two DG-PDSCHs scheduled on the same carrier in a slot/sub-slot, down-select between the following two interpretations and corresponding UE behaviors. - Interpretation 1: The restriction of one PUCCH transmission with HARQ-ACK in one slot/sub-slot is per HARQ-ACK codebook. - UE behavior 1-1: If the PDSCH-to-PUCCHs is in order, UE transmits the two PUCCHs in the same slot/sub-slot. - UE behavior 1-2: A UE is not expected that the PDSCH-to-PUCCHs is out of order. - Interpretation 2: The restriction of one PUCCH transmission with HARQ-ACK in one slot/sub-slot is for all configured HARQ-ACK codebook(s). It means a UE can transmit more than one PUCCH carrying HARQ-ACK only when it is configured with two sub-slot based HARQ-ACK codebooks. - UE behavior 2: The PDSCH-to-PUCCHs can be in order or out of order, and the UE does not transmit the PUCCH carrying low priority HARQ-ACK. Feature lead comment: The restrictions on the number of PUCCH transmissions per slot/sub-slot is related to the HARQ-ACK codebook design, and should be discussed under AI 7.2.5.2. |
| OPPO | - There is no reason to preclude the agreements of the collision handling of two PUCCHs carrying HARQ-ACK with different priorities made in scheduling/HARQ enhancement agenda. |
| CATT | In a given scheduled cell, the UE is not expected to be scheduled with a first PDSCH and a second PDSCH, which is starting later than the first PDSCH, if the PUCCH carrying HARQ-ACK for the first and second PDSCHs do not “overlap”, and if the PUCCH carrying HARQ-ACK for the second PDSCH “is transmitted” earlier than that of the first PDSCH. - If the two PUCCHs correspond to same HARQ-ACK CB, - Two PUCCHs are said to “overlap” when they are mapped to the same slot or sub-slot - If the two PUCCHs correspond to HARQ-ACK CBs with different priorities, - Two PUCCHs are said to “overlap” when they have time-domain overlap of at least one symbol In a given scheduled cell, the UE can be scheduled with two overlapping PUCCHs carrying HARQ-ACK with different priorities for two PDSCHs scheduled on the same carrier. |
| LGE | In rel-16, a UE can be scheduled with two overlapping PUCCHs carrying HARQ-ACK with different priorities for two PDSCHs scheduled on the same carrier. The UE transmits PUCCH carrying HARQ-ACK corresponding to high priority index and drops PUCCH carrying HARQ-ACK corresponding to low priority index. |
| Intel | Confirm the following: A UE, supporting intra-UE prioritization, can be scheduled with two overlapping PUCCHs carrying HARQ-ACK with different priorities for two PDSCHs scheduled on the same carrier. In a given scheduled cell, the UE is not expected to be scheduled with a first PDSCH and a second PDSCH, which is starting later than the first PDSCH, if the PUCCH carrying HARQ-ACK for the first and second PDSCHs do not overlap, and if the PUCCH carrying HARQ-ACK for the second PDSCH is transmitted earlier than that of the first PDSCH. - If the two PUCCHs correspond to same HARQ-ACK CB, - Two PUCCHs are said to overlap when they are mapped to the same slot or sub-slot - If the two PUCCHs correspond to HARQ-ACK CBs with different priorities, - Two PUCCHs are said to overlap when they have time-domain overlap of at least one symbol. It may be considered further as part of Rel-16 UE features discussion to introduce capability signaling for a UE to indicate support of intra-UE prioritization (at least between HARQ-ACK CBs), separately for when the corresponding PDSCHs are scheduled in the same DL carrier or not. Further separation of UE capability reporting on support of intra-UE prioritization (at least between HARQ-ACK CBs) based on relative timing of the starting symbols of the overlapping PUCCHs is not sufficiently justified. |
| CMCC | - The UE can be scheduled with two PUCCHs carrying HARQ-ACK with different priorities associated with two DG-PDSCHs scheduled on the same carrier overlapping in the time domain, and intra-UE UL collision handling behavior previously agreed should be followed. |
| Huawei/HiSi | - It is concluded that two PUCCHs with different priorities overlapping in time and a corresponding dropping rule are already supported by the current Rel-16 specification. |
| Nokia/NSB | - RAN1 to conclude that the UE can be scheduled with two PUCCHs overlapping in the time domain which carry HARQ-ACK with different priorities associated with two PDSCHs scheduled on the same carrier. |
| NTT DOCOMO | - Two PUCCHs carrying HARQ-ACK with different priorities associated with two DG-PDSCHs scheduled on the same carrier in one slot/sub-slot can be overlapping in time regardless of the order of the two PUCCH resources as long as the two PUCCH resources are in the same slot/sub-slot and dropping timeline is satisfied. |
| Qualcomm | - Two PUCCHs carrying HARQ-ACK with different priorities shall not collide in the time domain if their associated PDSCHs are dynamically scheduled on the same serving cell. - The editor note confirms that this feature is not supported in Rel. 16 URLLC. |

Issue #2: “Out-of-Order with Sub-slot Based HARQ-ACK Codebook”

Clarifying that out-of-order HARQ is not supported even in case of sub-slot based HARQ-ACK codebook by applying the following change [10]:

TP to TS 38.214, Sec. 5.1 UE procedure for receiving the physical downlink shared channel ## 5.1 UE procedure for receiving the physical downlink shared channel <Unchanged text is omitted> ## In a given scheduled cell, the UE is not expected to receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j or sub-slot j', and a second PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j or a sub-slot before sub-slot j'. ## <Unchanged text is omitted>

The contribution paper in [1], [5], and [8] also pointed out the same issue. In particular, the following changes are proposed in [5]:

A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1 or 1_2 decode the corresponding PDSCHs as indicated by that DCI. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to receive a PDSCH that overlaps in time with another PDSCH. The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6]. In a given scheduled cell, the UE is not expected to receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK associated with the same HARQ-ACK codebook assigned to be transmitted in a slot before slot j if subslotLength-ForPUCCH is not provided for the PUCCH with HARQ-ACK. In a given scheduled cell, the UE is not expected to receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to a sub-slot j, and a second PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK associated with the same HARQ-ACK codebook assigned to be transmitted in a sub-slot before sub-slot j if subslotLength-ForPUCCH is provided for the PUCCH with HARQ-ACK. In a given scheduled cell, the UE is not expected to receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot or sub-slot j, and a second PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK associated with a different HARQ-ACK codebook assigned to be transmitted ending earlier than the start of the HARQ-ACK for the first PDSCH. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i. In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration , with N=13 for =0, N=13 for =1, N=20 for =2, and N=24 for =3.

In [8], the following changes are proposed:

5.1	UE procedure for receiving the physical downlink shared channel

……

In a given scheduled cell, the UE is not expected to receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j or sub-slot j, and a second PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK, which is the same priority index with the first HARQ-ACK, assigned to be transmitted in a slot before slot j or a sub-slot before sub-slot j. The UE is not expected to be scheduled with a first PDSCH and the second PDSCH, which is starting later than the first PDSCH, if the PUCCH carrying HARQ-ACK for the first and second PDSCHs do not overlap in time domain, and if the PUCCH carrying HARQ-ACK for the second PDSCH is transmitted earlier than that of the first PDSCH.

Issue #3: “Out-of-Order with SPS Release”

In [4], it is proposed to disallow out-of-order HARQ between a PUCCH of a scheduled PDSCH and that for SPS release as shown in the figure below:

Proposal: The out-of-order HARQ with the same priority caused by the transmission of SPS PDSCH release should be avoid.

Issue #4: “Timeline for Cancellation”

In [9], it is proposed that for handling collision across two PUCCHs of different priorities, no extra timeline is needed for supporting dropping.

Comment from the feature lead: This proposal is about cancellation timeline and should be discussed under AI 7.2.5.2.

# 3         References

[1] R1-2001787, “Remaining issue of scheduling/HARQ for NR URLLC,” Ericsson

[2] R1-2001672, “Enhancements for scheduling/HARQ,” vivo

[3] R1-2001614, “Remaining issues on scheduling/HARQ enhancements for NR URLLC,” ZTE

[4] R1-2001776, “Enhancements to scheduling and HARQ,” OPPO

[5] R1-2002085, “Remaining issues on out-of-order scheduling/HARQ,” CATT

[6] R1-2001922, “Remaining issues of enhancements to scheduling/HARQ for NR URLLC,” LGE

[7] R1-2002001, “Remaining issues on scheduling and HARQ enhancements for URLLC,” Intel

[8] R1-2002210, “Remaining issues on scheduling and HARQ,” CMCC

[9] R1-2002583, “Corrections on operation of out-of-order,” Huawei, HiSilicon

[10] R1-2001700, “Maintenance of Rel. 16 URLLC scheduling/HARQ enhancements,” Nokia, NSB

[11] R1-2002445, “Remaining issue for scheduling and HARQ enhancements for URLLC,” NTT DOCOMO

[12] R1-2002547, “Remaining issues on scheduling/HARQ enhancements for URLLC,” Qualcomm