3GPP TSG-RAN WG1 Meeting 101e	R1-2005122

# Elbonia, May 25 – June 5, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0040 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR-based access to unlicensed spectrum |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-06-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Incorrect higher-layer parameter names. - The LSBs of k_ssb are ambiguous when operating in unlincesed spectrum - The CP extension for configured grant transmission is undefined. - The agreement on limit on the number of CRBs per interlace is not captured in the specifciations. - CP extension should be supported also for SRS |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction of higher-layer parameter name for cyclic shift hopping (R1-2002914) - Correction of kSSB when operating in shared spectrum (R1-2002915) - Correction of CP exension for configured grant transmisison (R1-2002982) - State that number of CRBs per interlace within a BWP can not be less than 10 (R1-2002912) - Clarify that CP extension applies for SRS (R1-2004995) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for NR in unlicensed spectrum. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.4.6, 5.3.1, 6.3.2.2.2, 7.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.4.4.6	Interlaced resource blocks

Multiple interlaces of resource blocks are defined where interlace m∈0,1,…,M-1 consists of common resource blocks m,M+m, 2M+m, 3M+m, …, with M being the number of interlaces given by Table 4.4.4.6-1. The relation between the interlaced resource block nIRB,mμ∈0,1,… in bandwidth part i and interlace m and the common resource block nCRBμ is given by

nCRBμ=MnIRB,mμ+NBWP,istart,μ+m-NBWP,istart,μ mod M

where NBWP,istart,μ is the common resource block where bandwidth part starts relative to common resource block 0. When there is no risk for confusion the index μ may be dropped.

The UE expects that the number of common resource blocks in an interlace contained within bandwidth part i is no less than 10.

Table 4.4.4.6-1: The number of resource block interlaces.

| μ | M |
| --- | --- |
| 0 | 10 |
| 1 | 5 |

### 5.3.1	OFDM baseband signal generation for all channels except PRACH and RIM-RS

The time-continuous signal  on antenna port p and subcarrier spacing configuration μ for OFDM symbol  in a subframe for any physical channel or signal except PRACH is defined by

sl(p,μ)t=sl(p,μ)ttstart,lμ≤t<tstart,lμ+Tsymb,lμ0otherwise
sl(p,μ)t=k=0Ngrid,xsize,μNscRB-1ak,l(p,μ)ej2πk+k0μ-Ngrid,xsize,μNscRB2Δft-NCP,lμTc-tstart,lμ
k0μ=Ngrid,xstart,μ+Ngrid,xsize,μ2NscRB-Ngrid,xstart,μ0+Ngrid,xsize,μ02NscRB2μ0-μ
Tsymb,lμ=Nuμ+NCP,lμTc

where t=0 at the start of the subframe,

and

-	 is given by clause 4.2;

-	 is the subcarrier spacing configuration;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList.

In case of cyclic prefix extension of the first OFDM symbol l allocated for PUSCH, SRS, or PUCCH transmission, the time-continuous signal sext(p,μ)t for the interval tstart,lμ-Text≤t<tstart,lμ preceding the first OFDM symbol for PUSCH, SRS, or PUCCH is given by

sext(p,μ)t=sl(p,μ)t

where t<0 refers to the signal in the previous subframe and

-	for dynamically scheduled PUSCH, SRS, and PUCCH transmissions

Text=minmaxText',0, Tsymb,(l-1)mod7∙2μμ

Text'=k=1CiTsymb,  l-kmod 7∙2μ μ-Δi

where Δi is given by Table 5.3.1-1 with C1=1 for μ∈0,1, C1=2 for μ=2, and C2 and C3 given by the higher-layer parameters cp-ExtensionC2-r16 and cp-ExtensionC3-r16, respectively, and TTA given by clause 4.3.1. For contention-based random access, or in absence of higher-layer configuration of C2 and C3, the value of Cishall be set to the largest integer fulfilling Text'<Tsymb,  (l-1)mod7∙2μ μ for each of the values of i∈2,3.

-	for a PUSCH transmission using configured grantText

Text=k=12μTsymb,  l-kmod 7∙2μ μ-Δi

where  Δi is given by Table 5.3.1-2 with the index i given by the procedure in [6, TS 38.214].

The starting position of OFDM symbol  for subcarrier spacing configuration in a subframe is given by

Table 5.3.1-1: The variables Ci and Δi for cyclic prefix extension

| Text index i | Ci | Δi |
| --- | --- | --- |
| 0 | - | - |
| 1 | C1 | 25∙10-6 |
| 2 | C2 | 16∙10-6+TTA |
| 3 | C3 | 25∙10-6+TTA |

Table 5.3.1-2: The variable Δi for cyclic prefix extension with configured grants.

| index i | Δi |
| --- | --- |
| 0 | 16∙10-6 |
| 1 | 25∙10-6 |
| 2 | 34∙10-6 |
| 3 | 43∙10-6 |
| 4 | 52∙10-6 |
| 5 | 61∙10-6 |
| 6 | k=12μTsymb, l-kmod 7∙2μ μ |

##### 6.3.2.2.2	Cyclic shift hopping

The cyclic shift α varies as a function of the symbol and slot number according to

αl=2πNscRBm0+mcs+mint+ncsns,fμ,l+l' mod NscRB

where

-	ns,fμ is the slot number in the radio frame

-	l is the OFDM symbol number in the PUCCH transmission where l=0 corresponds to the first OFDM symbol of the PUCCH transmission,

-	l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PUCCH transmission in the slot given by [5, TS 38.213]

-	 is given by [5, TS 38.213] for PUCCH format 0 and 1 while for PUCCH format 3 and 4 is defined in clause 6.4.1.3.3.1

-	 except for PUCCH format 0 when it depends on the information to be transmitted according to clause 9.2 of [5, TS 38.213].

-	mint is given by

-	mint=5nIRBμ for PUCCH formats 0 and 1 if PUCCH shall use interlaced mapping according to any of the higher-layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon or useInterlacePUCCH-PUSCH in BWP-UplinkDedicated, where nIRBμ is the resource block number within the interlace;

-	mint=0 otherwise

The function  is given by

ncsns,fμ,l=m=072mc8Nsymbslotns,fμ+8l+m

where the pseudo-random sequence  is defined by clause 5.2.1. The pseudo-random sequence generator shall be initialized with , where  is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell.

#### 7.4.3.1	Time-frequency structure of an SS/PBCH block

In the time domain, an SS/PBCH block consists of 4 OFDM symbols, numbered in increasing order from 0 to 3 within the SS/PBCH block, where PSS, SSS, and PBCH with associated DM-RS are mapped to symbols as given by Table 7.4.3.1-1.

In the frequency domain, an SS/PBCH block consists of 240 contiguous subcarriers with the subcarriers numbered in increasing order from 0 to 239 within the SS/PBCH block. The quantities  and  represent the frequency and time indices, respectively, within one SS/PBCH block. The UE may assume that the complex-valued symbols corresponding to resource elements denoted as 'Set to 0' in Table 7.4.3.1-1 are set to zero. The quantity  in Table 7.4.3.1-1 is given by v=NIDcell mod 4. The quantity  is the subcarrier offset from subcarrier 0 in common resource block NCRBSSB to subcarrier 0 of the SS/PBCH block, where NCRBSSB is obtained from the higher-layer parameter offsetToPointA and the 4 least significant bits of  are given by the higher-layer parameter ssb-SubcarrierOffset and for SS/PBCH block type A the most significant bit of  is given by aA+5 in the PBCH payload as defined in clause 7.1.1 of [4, TS 38.212]. For operation with shared spectrum channel access, the 4 least significant bits of kSSB are given by the higher-layer parameter ssb-SubcarrierOffset and the most significant bit of kSSB is given by aA+5 in the PBCH payload as defined in clause 7.1.1 of [4, TS 38.212]. If  kSSB≥24, kSSB=kSSB ; otherwise, kSSB=2kSSB2. If ssb-SubcarrierOffset is not provided, kSSB is derived from the frequency difference between the SS/PBCH block and Point A.

The UE may assume that the complex-valued symbols corresponding to resource elements that are part of a common resource block partially or fully overlapping with an SS/PBCH block and not used for SS/PBCH transmission are set to zero in the OFDM symbols partially or fully overlapping with OFDM symbols where SS/PBCH is transmitted.

For an SS/PBCH block, the UE shall assume

-	antenna port  is used for transmission of PSS, SSS, PBCH and DM-RS for PBCH,

-	the same cyclic prefix length and subcarrier spacing for the PSS, SSS, PBCH and DM-RS for PBCH,

-	for SS/PBCH block type A,  and  with the quantities , and NCRBSSB expressed in terms of 15 kHz subcarrier spacing, and

-	for SS/PBCH block type B,  and  with the quantity  expressed in terms of the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and NCRBSSB expressed in terms of 60 kHz subcarrier spacing;

-	the centre of subcarrier 0 of resource block NCRBSSB  coincides with the centre of subcarrier 0 of a common resource block with the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon. This common resource block overlaps with subcarrier 0 of the first resource block of the SS/PBCH block.

The UE may assume that SS/PBCH blocks transmitted with the same block index on the same center frequency location are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and, when applicable, spatial Rx parameters. The UE shall not assume quasi co-location for any other SS/PBCH block transmissions other than what is specified in [5, TS 38.213].

Table 7.4.3.1-1: Resources within an SS/PBCH block for PSS, SSS, PBCH, and DM-RS for PBCH.

| Channel or signal | OFDM symbol number relative to the start of an SS/PBCH block | Subcarrier number relative to the start of an SS/PBCH block |
| --- | --- | --- |
| PSS | 0 | 56, 57, …, 182 |
| SSS | 2 | 56, 57, …, 182 |
| Set to 0 | 0 | 0, 1, …, 55, 183, 184, …, 239 |
|  | 2 | 48, 49, …, 55, 183, 184, …, 191 |
| PBCH | 1, 3 | 0, 1, …, 239 |
|  | 2 | 0, 1, …, 47, 192, 193, …, 239 |
| DM-RS for PBCH | 1, 3 |  |
|  | 2 |  |