3GPP TSG-RAN WG1 Meeting #101-e	R1-2004224

e-Meeting, May 25th – June 5th, 2020

Agenda item:		7.2.5.3

Source:		Moderator (Apple Inc.)

Title:	Feature lead summary on PUSCH enhancements for NR eURLLC (AI 7.2.5.3)

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, Sections 2 to 6 summarize the issues raised in the contributions submitted under AI 7.2.5.3, PUSCH enhancements for NR Rel-16 URLLC.

Section 7 provides the suggestion for the scope of email discussions.

# 2	TP for RAN#100b-e agreements on SSB, CORESET#0 in MIB and gap handling

In RAN1#100bis-e, the following agreements have been made (see the FL summary in R1-2003005):

Agreements:
For operation in unpaired spectrum, symbols that are indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered invalid symbols for PUSCH repetition Type B, and segmentation occurs around these invalid symbols.

Agreements:

For operation in unpaired spectrum, symbols indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS are considered as invalid symbols for PUSCH repetition Type B, and segmentation occurs around these symbols.

Agreements:

For operation in unpaired spectrum, introduce a new RRC parameter numberInvallidSymbolsForDL-UL-Switching to indicate the number of symbols after the last semi-static DL symbol that are invalid symbols for PUSCH repetition Type B.

- The candidate values include {1, 2, 3, 4}.

- If not configured, it means no symbols are explicitly defined for DL-to-UL switching.

TPs are provided in Nokia/NSB[5], CATT[6], Apple[15], QC[18].

TP from Nokia/NSB[5]:

TP to TS 38.214, Sec. 6.1.2.1: < text omitted> For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, - symbols indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, and - symbols indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, and - a number of symbols after the last semi-static DL symbol, where the number of symbols is provided by numberInvalidSymbolsForDL-UL-Switching if configured and zero otherwise, are considered as invalid symbols for PUSCH repetition Type B transmission. < text omitted>

TP from CATT[6]:

#### 6.1.2.1 Resource allocation in time domain For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, a symbol that is indicated for reception of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, a symbol that is indicated for a CORESET for Type0-PDCCH CSS set by pdcch-ConfigSIB1 in MIB, is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, if RRC parameter numberInvallidSymbolsForDL-UL-Switching is configured, it indicates the number of symbols after the last semi-static DL symbol that are invalid symbols for PUSCH repetition Type B.

TP from Apple[15]:

#### 6.1.2.1 Resource allocation in time domain < Unchanged parts are omitted > For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, the following symbols are considered as invalid symbols for PUSCH repetition Type B transmission: - symbols indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks; - symbols indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set; - if numberInvalidSymbolsForDL-UL-Switching is configured, numberInvalidSymbolsForDL-UL-Switching symbol(s) after the last semi-static DL symbol in each consecutive set of all semi-static DL symbols as determined by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

TP from QC[18]:

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, symbols indicated to a UE: - by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, or - by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks. - Number of flexible or uplink symbols indicated to a UE by numberInvalidSymbolsForDL-UL-Switching, if it is configured, after a downlink symbol as determined by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated

It was pointed out in WILUS[19] that the subcarrier spacing of gap symbols configured by numberInvallidSymbolsForDL-UL-Switching should be clarified.

As tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are indicated based on referenceSubcarrierSpacing, it makes sense that numberInvallidSymbolsForDL-UL-Switching is also based on referenceSubcarrierSpacing.

Proposal:

Adopt the following TP for TS 38.214 Clause 6.1.2.1:

#### 6.1.2.1 Resource allocation in time domain < Unchanged parts are omitted > For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, the following symbols are considered as invalid symbols for PUSCH repetition Type B transmission: - symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks, and - symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, and - if numberInvalidSymbolsForDL-UL-Switching is configured, numberInvalidSymbolsForDL-UL-Switching symbol(s) after the last semi-static DL symbol in each consecutive set of all semi-static DL symbols as determined by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon. - The UE may be configured with the higher layer parameter InvalidSymbolPattern, which provides a symbol level bitmap spanning one or two slots (higher layer parameter symbols given by InvalidSymbolPattern). A bit value equal to 1 in the symbol level bitmap symbols indicates that the corresponding symbol is an invalid symbol for PUSCH repetition Type B transmission. The UE may be additionally configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by InvalidSymbolPattern), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap symbols, and a bit value equal to 1 indicates that the symbol level bitmap symbols is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40ms. The first symbol of periodicityAndPattern every 40ms/P periods is a first symbol in frame 𝑛𝑓 mod 4 = 0, where P is the duration of periodicityAndPattern in units of ms. When periodicityAndPattern is not configured, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. If InvalidSymbolPattern is configured, when the UE applies the invalid symbol pattern is determined as follows: < Unchanged parts are omitted >

# 3	A-CSI/SP-CSI on PUSCH with repetition Type B

It has been agreed in RAN1#100-e that:

Agreements: (RRC impact)

Introduce reportSlotOffsetList-r16-ForDCIFormat0_1 and reportSlotOffsetList-r16-ForDCIFormat0_2 and update TS 38.214 accordingly

- FFS whether or not to always assume the number of nominal repetitions is equal to 1 when PUSCH with repetition Type B carries A-CSI/SP-CSI only.

A-CSI/SP-CSI on PUSCH with repetition Type B was further discussed in RAN1#100bis-e (see the FL summary in R1-2003006), without any agreements reached.

## 3.1	A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH

Based on the contributions, it seems generally agreeable among the companies that for A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition, and other actual repetitions are discarded. The following options have been discussed:

- Option A: there is no scheduling restriction.

  - ZTE[1], Ericsson[3] (?), Nokia/NSB[5], Intel[7], Samsung[9], DOCOMO[17]

- Option B: UE is not expected to be indicated with numberofrepetitions >1.

  - vivo[2], Huawei/HiSilicon[4], Spreadtrum[11], Sharp[16]

- Option C: The first nominal repetition is not expected to be segmented into multiple actual repetitions.

  - CATT[6], LG[12] (For the number of nominal repetition, follow same behavior with pusch- aggregationfactor in Rel-15), OPPO[14], Apple[15], QC[18]

## 3.2	A-CSI/SP-CSI on PUSCH with repetition Type B with UL-SCH

For A-CSI on PUSCH with repetition Type B with UL-SCH, the following options have been discussed:

- CSI is transmitted on the first actual repetition.

  - vivo[2], Huawei/HiSilicon[4], Nokia/NSB[5] (first preference), Intel[7], Samsung[9], Spreadtrum[11], DOCOMO[17]

  - This may be more aligned with the agreements that UCI is multiplexed on the first overlapping actual repetition in case PUCCH would overlap with PUSCH repetition Type B.

- CSI is transmitted on the last actual repetition.

  - CATT[6], Apple[15] (excluding the single-symbol repetition), QC[18]

  - Beneficial for providing more updated CSI report without delaying PUSCH scheduling. But there may be unnecessary delay on CSI report in some cases.

- A-CSI is transmitted on the first actual repetition that fulfills the CSI multiplexing timeline.

  - ZTE[1], Ericsson[3], Nokia/NSB[5] (second preference)

  - There may be misalignment between gNB and UE due to TA impact.

OPPO[14]:

- UE is not expected that the gap between the first actual repetition and UL grant is smaller than CSI calculation timeline when A-CSI needs to be multiplexed in PUSCH. (FL: This seems too restrictive. Rel-15 does not have this requirement either. The only thing is that UE may not update CSI if CSI computation timeline is not satisfied.)

- A unified solution for A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH and with UL-SCH is preferred. (does this mean OPPO supports the first option above?)

## 3.3	A-CSI/SP-CSI on PUSCH with repetition Type A

Vivo[2]:

For PUSCH repetition type A,

- for A-CSI triggered without UL-SCH,

  - A-CSI only on PUSCH can be triggered with a PUSCH repetition with the number of repetitions >1, where the number of repetitions is configured by pusch-AggregationFactor. If UE is configured with pusch-AggregationFactor > 1, A-CSI/SP-CSI is transmitted on the first actual repetition, and other actual repetitions are discarded..

  - UE does not expect that A-CSI only on PUSCH is triggered with a PUSCH repetition with the number of repetitions >1, where the number of repetitions is indicated by numberofrepetitions in the time-domain resource allocation.

- for A-CSI triggered with UL-SCH,

  - When UE is configured with pusch-AggregationFactor>1, A-CSI is multiplexed on the first PUSCH repetitions.

  - When UE is indicated by numberofrepetition>1 in time-domain resource allocation, A-CSI is multiplexed on the first PUSCH repetitions.

Ericsson[3]: Use similar handling for A-CSI/SP-CSI on PUSCH repetition Type A as for Type B.

# 4	UCI multiplexing on PUSCH for PUSCH repetition Type B

In RAN1 #100bis-e, the following agreements were reached (see feature lead summary R1-2003004):

Agreements:

In case of PUCCH overlapping with PUSCH with repetition Type B,

- Option A: Multiplexing timeline conditions in Clause 9.2.5 of TS 38.213 shall be satisfied for all the overlapping actual repetitions. Otherwise it is considered as an error case.

Agreements:

In case PUCCH overlaps with multiple repetitions of PUSCH repetition Type B that satisfy the multiplexing timeline conditions, UCI is multiplexed on only one actual repetition (including the case where a PUCCH overlaps with a PUSCH with repetition Type B in multiple slots). To determine which actual repetition, down-select from the following 3 options:

- Option 1: the first overlapping actual repetition in the first overlapping slot that satisfies the multiplexing timeline

What still remains open is how to determine the number of REs for UCI. In addition, there are a few contributions discussing the overlapping between PUSCH repetition Type B and PUCCH repetitions.

Before getting into the details of UCI multiplexing on PUSCH, there was one discussion point from RAN1#100bis-e that an actual repetition of a single symbol should not be considered for UCI multiplexing.

- Samsung[9], Sharp[16]: An actual repetition of a single symbol in case of L>1 is not considered for UCI multiplexing.

- QC[18]: An actual repetition of a single symbol PUSCH repetition Type B is not considered for UCI multiplexing.

Currently, UCI is not expected to be multiplexed The following is proposed, which is generally applicable for any type of UCI (HARQ-ACK, P-CSI, A-CSI, SP-CSI).

Proposal:

For PUSCH repetition Type B, an actual repetition of a single symbol is not considered for UCI multiplexing.

ETRI[10] discussed the case of one PUSCH repetition overlapping with multiple PUCCHs with HARQ-ACK. But this is highly related to the open point under UCI enhancements AI. Therefore, the recommendation is to either discuss this together in UCI enhancements AI, or discuss after the decision is made there.

- ETRI[10]: Up to n HARQ-ACK codebooks are multiplexed for all PUSCH repetitions, where n is 1 for grant based PUSCH repetition type B.

## 4.1 UCI resource determination

Regarding how to determine the number of REs for UCI when multiplexed on PUSCH repetition Type B, the discussions in previous meetings and in the RAN1#101-e contributions have been mainly focused on the case for PUSCH repetition Type B with UL-SCH. But the issue also exists for PUSCH repetition Type B without UL-SCH. Note that it also applied to all different types of UCI, including HARQ-ACK, P-CSI, A-CSI, SP-CSI.

### 4.1.1 UCI resource determination on PUSCH repetition Type B without UL-SCH

In Rel-15, for PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK, CSI part 1 and CSI part 2 is calculated according to

,

and

,

respectively, where is the number of available REs in the PUSCH piggybacking the UCI.

The question is if the PUSCH is with repetition Type B, how the number of available REs should be calculated, i.e., whether it should be based on the nominal repetition or actual repetition.

Apple[15] proposed that in this case the calculation is based on actual repetition.

### 4.1.2 UCI resource determination on PUSCH repetition Type B with UL-SCH

In Rel-15, for PUSCH with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK, CSI part 1 and CSI part 2 is calculated according to

,

QCSI-1'=minOCSI-1+LCSI-1∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'

and

QCSI-2'=minOCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1',

respectively, where is the number of available REs in the PUSCH piggybacking the UCI, and  is the TBS of the PUSCH.

For PUSCH repetition Type B, TBS is determined based on the nominal repetition length. The question is then how the number of REs for UCI should be calculated? Based on the nominal repetition or actual repetition?

Using nominal repetition in the first part of the equation would provide more REs and better performance guarantee for UCI, while using actual repetition would allow more REs to be used for data transmission. This is also true for the second part of the equation.

Here is a summary of companies’ view based on the contributions:

- Option 1: The calculation is based on the nominal repetition

  - Huawei/HiSilicon

  - Option 1a: with the additional limit of no more than the resources available in the actual repetition

    - ZTE[1], Ericsson[3], Intel[7], Panasonic[8], OPPO[14], Docomo[17] (2nd preference)

  - Option 1b: UE does not expect that the number of REs required by UCI is more than the number of available REs in the actual repetition on which the UCI is multiplexed.

    - Vivo[2], Samsung[9], LG[12], OPPO[14], Apple[15], QC[18]

- Option 2: The calculation is based on the actual repetition.

  - LG[12], WILUS[19]

- Option 3: The first part of the equation is based on the nominal repetition, and the second part of the equation is based on the actual repetition.

  - Nokia/NSB[5] (TP provided), CATT[6], Intel[7], Panasonic[8], Spreadtrum[11], Docomo[17] (1st preference)

Samsung[9]: Scale the βoffsetPUSCH value provided by RRC by the PUSCH repetition number k as β'offsetPUSCH= k×βoffsetPUSCH for the RE number calculation of UCI multiplexing on PUSCH for both Type B and Rel-16 Type A repetition with dynamic indication of the repetition number in the DCI format.

ETRI[10]: For actual repetition, more alpha scaling value is provided. (not clear what this means exactly? One value for nominal repetition and one value for actual repetition?) If any given alpha scaling does not give a meaningful bound, drop the actual PUSCH instance and transmit PUCCH.

## 4.2 Overlapping between PUSCH repetition Type B and PUCCH repetitions

A few contributions discussed the overlapping between PUSCH repetition Type B and PUCCH repetitions.

- ETRI[10]: Regarding HARQ-ACK repetition using long PUCCH formats, PUCCH is transmitted and overlapped PUSCH repetition(s) are dropped, provided that their priority index are the same.

- Fujitsu[13]: For the UL channels of the same PHY layer priority, UE drops the actual repetition(s) for PUSCH repetition type B which is overlapped in time with a PUCCH over a first number NPUCCHrepeat>1 of slots.

- ASUSTek[20]

  - There are two possible UE behaviors based on the current specification text, which should be clarified:

    - (1)The UE would drop all actual repetition(s) in the overlapping slot(s) and transmit the PUCCH

    - (2)The UE would only drop the actual repetition(s) that overlap with the PUCCH and transmit the remaining actual repetition(s). – preferred (TP provided)

# 5	Remaining issues on the transmit power for PUSCH repetition Type B

It has been agreed in RAN1#100-e that:

Agreements:

For PUSCH repetition Type B, PUSCH transmit power is determined based on the nominal repetition duration.

The corresponding TP was agreed in RAN1#100bis-e (see FL summary R1-2003077).

It appears that CR is needed to cover the case where a PUSCH transmission occasion (i.e., a nominal repetition) overlaps with multiple slots on a second carrier with the same SCS. Ericsson[3] and CATT[6] provided TPs for TS 38.213.

Ericsson[3]

### 7.7.1 Type 1 PH report <Unchanged text omitted> If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration on active UL BWP of carrier of serving cell is smaller than a SCS configuration on active UL BWP of carrier of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP that overlaps with multiple slots on active UL BWP , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple slots on active UL BWP that fully overlaps with the slot on active UL BWP . If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration on active UL BWP of carrier of serving cell is smaller than or equal to a SCS configuration on active UL BWP of carrier of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission with PUSCH repetition Type B, whose nominal repetition spans more than one slot on active UL BWP and overlaps with multiple slots on active UL BWP , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple slots on active UL BWP that fully overlaps with the nominal repetition on active UL BWP . If a UE is configured with multiple cells for PUSCH transmissions, where a same SCS configuration on active UL BWP of carrier of serving cell and active UL BWP of carrier of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the slot on active UL BWP that overlaps with the slot on active UL BWP . <Unchanged text omitted>

CATT[6]

---------------------------------------------- Start of text proposal for 38.213 --------------------------------------- <unchanged part omitted> ### 7.7.1 Type 1 PH report If a UE determines that a Type 1 power headroom report for an activated serving cell is based on an actual PUSCH transmission then, for PUSCH transmission occasion on active UL BWP of carrier of serving cell , the UE computes the Type 1 power headroom report as [dB] where , , , , , and are defined in Clause 7.1.1. If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration on active UL BWP of carrier of serving cell is smaller than a SCS configuration on active UL BWP of carrier of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP that overlaps with multiple slots on active UL BWP , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple slots on active UL BWP that fully overlaps with the slot on active UL BWP . If a UE is configured with multiple cells for PUSCH transmissions, where a same SCS configuration on active UL BWP of carrier of serving cell and active UL BWP of carrier of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot on active UL BWP that overlaps with the PUSCH transmission slot on active UL BWP . <unchanged part omitted> ---------------------------------------------- End of text proposal for 38.214 ---------------------------------------

Huawei/HiSilicon[4], QC[18]: PHR calculation is based on nominal transmission duration.

# 6	Remaining Issues on Interaction with DL/UL Directions for PUSCH repetition Type B

## 6.1	Behavior for half-duplex CA

For half-duplex CA, according to the email discussions [95-NR-06] and [100b-e-NR-TEIs-02], the following agreements and conclusion have been endorsed. The 38.213 CR for half-duplex CA has been agreed in RAN1#100-e in R1-2001475.

Agreements: For a UE configured with multiple serving cells in a band or band combination, where the UE is not capable of simultaneous transmission and reception, support a behavior in the following table: - FFS: other combinations of Semi SFI, RRC and Dynamic - FFS: behaviour for SSB in RRM measurements - FFS: down-select further between Alt1 and Alt2. Down-selection may be performed independently for case 3 and 6. - For the agreed behaviour introduce a new feature as “mandatory with capability” - Note: Sep./Dec. UEs are still able to operate HD TDD CA if network ensures same transmission direction across all the serving cells. - Reference (Ref) cell is the cell with the lowest cell ID among cells: (i) within the band or band combination and (ii) with conflicting directions, and “Other cell” is any cell within the band or band combination other than the Ref cell. No; Ref cell; Other cell; UE behavior; Note. 1; Semi SFI D; Semi SFI U; Allowed to drop U for inter-band Error case in intra-band; Dropping U on other cell Error case in intra-band. 2; Semi SFI D; RRC U; Allowed to drop U; Dropping on other cell. 3; Semi SFI D; Dynamic U; Alt 1: Allowed to drop D for inter-band Error case in intra-band; Overriding semi SFI D to F on reference cell for the UE. Alt 2: Error; Error. 4; Semi SFI U; Semi SFI D; Allowed to drop D for inter-band Error case in intra-band; Dropping D on other cell Error case in intra-band. 5; Semi SFI U; RRC D; Allowed to drop D; Dropping on other cell. 6; Semi SFI U; Dynamic D; Alt 1: Allowed to drop U for inter-band Error case in intra-band; Overriding semi SFI U to F on reference cell for the UE. Alt 2: Error; Error. 7; RRC D; RRC U; Allowed to drop U; Dropping on other cell. 8; RRC U; RRC D; Allowed to drop D; Dropping on other cell. 9; Dynamic D; Dynamic U; Error; Error. 10; Dynamic U; Dynamic D; Error; Error. Note, in above tables, the terminology is as follows: - Semi SFI D and U: D and U symbols configured by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated - Semi SFI F: flexible symbols configured by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, when provided to a UE, or when TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated are not provided to the UE - RRC D: symbols corresponding to a higher-layer configured PDCCH, or a PDSCH, or a CSI-RS on semi SFI F of the same cell - RRC U: symbols corresponding to a higher-layer configured SRS, or PUCCH, or PUSCH, or PRACH on semi SFI F of the same cell - Dynamic D and U: symbols scheduled as D and U by DCI formats other than DCI format 2_0 on semi SFI F of the same cell - Note: there is no intention to change the existing behavior w.r.t. how to update the reference cell Agreement: - For case 3, down-select Alt 1 - For case 6, down-select Alt 2 - Note: above down-selection applies to NR CA only For the remaining cases, adopt the following behavior: No; Ref cell; Other cell; UE behavior; Note. 11; RRC U; Semi SFI D; Allowed to drop D; Dropping on other cell. 12; Dynamic U; Semi SFI D; Allowed to drop D; Dropping on other cell. 13; RRC D; Semi SFI U; Allowed to drop U; Dropping on other cell. 14; Dynamic D; Semi SFI U; Allowed to drop U; Dropping on other cell. 15; RRC U; Dynamic D; Alt 2: Error. 16; RRC D; Dynamic U; Alt 1: Allowed to drop D for inter-band Error case in intra-band. 17; Dynamic U; RRC D; Allowed to drop D; Dropping on other cell. 18; Dynamic D; RRC U; Allowed to drop U; Dropping on other cell. Agreement: Half-duplex CA UE determines reference cell per symbol as a cell with the lowest cell ID among multiple serving cells in a band or band combination having direction determined by RRC D/U or semi SFI D/U - Note: this overrides earlier agreements “ Reference (Ref) cell is the cell with the lowest cell ID among cells: (i) within the band or band combination and (ii) with conflicting directions, and “Other cell” is any cell within the band or band combination other than the Ref cell.” - Note: Agreed cases 12, 14, 17 and 18 are not needed anymore Note: Agreed cases 9 and 10 should apply to collisions between two cells irrespective of a cell being reference or other Conclusion: The reference cell is determined with excluding the effects of UL TA and DL and UL timing differences. The existence of directional conflict between cells is determined with including the effects of UL TA and DL and UL timing differences. The error cases resulting from the discrepancy between the methods of reference cell determination and duplex direction conflict determination will not be discussed further in Rel-16. These cases include at least: • duplex direction conflict between cells where both cells are reference • duplex direction conflict between cells where there is no reference • conflict between Dynamic D and Dynamic U

Based on all collided cases listed above, the cases with dropping uplink transmission are summarized as follows in ZTE[1].

| Case No. | Ref cell | Other cell | UE behavior | Note |
| --- | --- | --- | --- | --- |
| 1 | Semi SFI D | Semi SFI U | Allowed to drop U for inter-band Error case in intra-band | Dropping U on other cell Error case in intra-band |
| 2 | Semi SFI D | RRC U | Allowed to drop U | Dropping on other cell |
| 7 | RRC D | RRC U | Allowed to drop U | Dropping on other cell |
| 13 | RRC D | Semi SFI U | Allowed to drop U | Dropping on other cell |

Here is a short summary of the behavior from CATT[12].

- For PUSCH on reference cell, - For DG PUSCH and higher layer configured PUSCH on reference cell in a set of symbols, PUSCH transmission will not be affected by symbol direction indicated on other cells. - For PUSCH on other cell(s), - For DG PUSCH or the first activated type 2 CG PUSCH in a set of symbols, PUSCH transmission will not be affected by symbol direction indicated on other cells. - For higher layer configured PUSCH in a set of symbols, a symbol can’t be used for PUSCH transmission, when - the symbol is indicated semi-static downlink on reference cell, or - UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on reference cell on the symbol. For PUSCH on reference cell or other cell(s), when a symbol is indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, the symbol can’t be used for PUSCH transmission.

ZTE[1]:

- In case of half-duplex operation in CA, a symbol is considered as an invalid symbol for PUSCH repetition Type B transmission with Type 1 configured grant on another cell if the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on the reference cell.

- In case of half-duplex operation in CA, a symbol is considered as an invalid symbol for PUSCH repetition Type B transmission in any of multiple serving cells if the symbol is indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon.

TP

--------------------------------------------Text Proposal 1 for Section 6.1.2.1 in TS38.214 [3] -------------------------------

#### 6.1.2.1 Resource allocation in time domain <---------------------------Other parts are omitted ------------------------------- For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - A symbol is considered as an invalid symbol for PUSCH transmission with a Type 1 configured grant on another cell if the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on the reference cell, when a UE indicates support of capability for half-duplex operation in CA with unpaired spectrum and satisfies the conditions as described in [6, TS 38.213 Clause 11.1]. - A symbol is considered as an invalid symbol for PUSCH transmission in any of multiple serving cells if the symbol is indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when a UE indicates support of capability for half-duplex operation in CA with unpaired spectrum and satisfies the conditions as described in [6, TS 38.213 Clause 11.1]. <---------------------------Other parts are omitted ------------------------------->

CATT[6]

- For type 1 CG PUSCH and type 2 CG PUSCH other than the first activated PUSCH on the other cell in a set of symbols with PUSCH repetition type B, if one of the symbols is indicated as semi-static downlink on reference cell, or if the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on reference cell in the symbol, the symbol is invalid for PUSCH transmission and a PUSCH repetition is segmented around the symbol.

- For PUSCH on reference cell or other cell(s), if one of the symbols is indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, the symbol is invalid for PUSCH transmission and a PUSCH repetition is segmented around the symbol.

WILUS[19]: For a UE operating half-duplex CA, the symbols for SS/PBCH block reception in any of multiple serving cells are regarded as invalid symbols for PUSCH repetition type B. (TP provided)

Proposal:

In case of half-duplex CA operation, symbols that are indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks in any of multiple serving cells are considered invalid symbols for PUSCH repetition Type B, and segmentation occurs around these invalid symbols.

Proposal:

## 6.2	Other remaining issues

WILUS[19] proposed some further optimization regarding the handling of the gap symbols provided by numberInvallidSymbolsForDL-UL-Switching:

- Apply numberInvallidSymbolsForDL-UL-Switching to indicate the number of symbols after the last symbol for the reception of SS/PBCH block that are invalid symbols for PUSCH repetition Type B.

- Apply numberInvallidSymbolsForDL-UL-Switching to indicate the number of symbols after the last symbol of PDCCH monitoring occasion in Type-0 CSS that are invalid symbols for PUSCH repetition Type B.

- If the first nominal repetition scheduled by a PDCCH overlaps with gap symbols, do not consider the gap symbols as invalid symbols for PUSCH repetition Type-B (i.e., use the gap symbols for the first nominal repetition)

# 7	Other Issues

### Issue #1: Single transmission layer for PUSCH repetition Type B

Vivo[2]: If numberofrepetitions is configured, in case the number of nominal repetitions >1, PUSCH is limited to a single transmission layer. (TP provided for TS 38.214 s6.1.2.1.)

### Issue #2: Limit on the number of repetitions in a slot

We had the following agreements from RAN1#99:

Agreements: (RAN1#99 Reno)

For PUSCH repetition type A and type B, the number of bits to indicate numberofrepetitions is 3.

- {1, 2, [3], 4, [6], 7, [8], 12, 16} are supported.

- FFS whether to have a limit on the number of nominal repetitions in a slot

The value set for numberofrepetitions has been agreed in RAN1#100-e, but it remains open whether to have a limit on the number of nominal repetitions in a slot.

Here is a summary of companies’ views from RAN1#100bis-e contributions:

- Yes: Spreadtrum[11]

  - Limit on the number of actual repetitions per slot (SCS-dependent): Huawei/HiSi[1] (reuse Rel-15 UE features?),

- No:

  - CATT[6]

Feature lead recommendation: given that the issue is being addressed as part of UE feature discussion, it is recommended that we do not discuss it under this AI.

### Issue #3: Granularity for frequency hopping

Huawei/HiSi[4] pointed out using granularity of RB in frequency hopping starting position and offset may not be the best in aligning with RBG boundary, and proposed to use RBG as the granularity for PUSCH with RA type 1 scheduled by DCI format 0_2. Note that this is not an issue specifically for PUSCH repetition type B. Rather, it is introduced by the RBG scheduling granularity in DCI format 0_2 for RA type 1.

- In case of intra-slot frequency hopping, the starting RBG in each hop is given by: RBGstart =RBGstart i=0RBGstart +RBGoffsetmod NRBG i=1 - In case of inter-slot frequency hopping, the starting RBG during slot i is given by: RBGstart =RBGstart i mod 2=0RBGstart +RBGoffsetmod NRBG i mod 2=1 - In case of inter-repetition frequency hopping, the starting RBG in i-th repetition is given by: RBGstart =RBGstart i mod2=0RBGstart +RBGoffsetmod NRBG i mod2 =1

### Issue #4: TP for TDRA for configured grant

- Huawei/HiSi[4]:

  - A TP to add the definition of TDRA for a nominal/actual repetition for configured grant

##### 6.1.2.3.2 Transport Block repetition for uplink transmissions of PUSCH repetition Type B with a configured grant The procedures described in this Clause apply to PUSCH transmissions of PUSCH repetition type B with a Type 1 or Type 2 configured grant. For PUSCH transmissions with a Type 1 or Type 2 configured grant, the nominal repetitions and the actual repetitions are determined according to the procedures for PUSCH repetition Type B defined in Clause 6.1.2.1, and the starting slot as well as the starting symbol for each repetition bundle are determined according Clause 5.8.2 of [10, TS 38.321]. The higher layer configured parameters repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for each actual repetition with a configured grant shall be set to 0. Otherwise, for the nth transmission occasion among all the actual repetitions (including the actual repetitions that are omitted) of the K nominal repetitions, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If a configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the actual repetitions. Otherwise, the initial transmission of a transport block may start at < Unchanged parts are omitted >

### Issue #5: TP for invalid symbol pattern for configured grant

- Huawei/HiSi[4]:

  - A TP to clarify the invalid symbol pattern for configured grant

#### 6.1.2.1 Resource allocation in time domain < Unchanged parts are omitted > For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH transmission scheduled by DCI format 0_1, or scheduled by DCI format 0_2, or corresponding to a configured grant as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - The UE may be configured with the higher layer parameter InvalidSymbolPattern, which provides a symbol level bitmap spanning one or two slots (higher layer parameter symbols given by InvalidSymbolPattern). … < Unchanged parts are omitted >

The remaining issues do not seem to be critical at this point for this AI, therefore the recommendation is to de-prioritize the related discussions.

- Ericsson[3]: When only one bit is used to signal RV in DCI format 0_2, it indicates either RV 0 or RV 2 (instead of RV 0 or 3).

  - Reason: RV {0, 2} performs better than {0, 3}

  - FL recommendation: this is a generic issue introduced by one-bit RV field in DCI format 0_2. It is recommended to discuss this issue under AI 7.2.5.1.

------------------ Text Proposal for 38.212 Section 7.3.1.1.3 ------------------ - Redundancy version – 0, 1 or 2 bits determined by higher layer parameter NumberofbitsforRV-ForDCIFormat0_2 - If 0 bit is configured, rvid to be applied is 0; - 1 bit according to Table 7.3.1.2.3-1 7.3.1.1.2-34; - 2 bits according to Table 7.3.1.1.1-2. ----------------------------------------------End of proposed TP ----------------------------------------------------

- Fujitsu[13]: If a PUSCH of type B repetition is scheduled by a DCI and the DCI includes a DAI field, the value of the DAI field is applicable for the repetition(s) where HARQ-ACK information is multiplexed. (FL: any spec changes needed?)

- Apple[15]: Clarify UE behavior when NRE'≤0 (the overhead is configured too much)

- QC[18]

  - Sec. 5 on the conflict of low priority dynamic DL transmission and high priority configured UL transmission

    - FL comment: This seems to be a generic issue for intra-UE prioritization and multiplexing, not specific for PUSCH repetition Type B. Would suggest that this is discussed under AI 7.5.2.

# 6	Scope of the email discussions

As guided by the chairman, this AI can have up to 3 email discussions, while the TP in Section 2 will be discussed separately.

Based on the summary, the suggestion is to have the following email discussions:

- Email discussion #1: TP for RAN1#100b-e agreements (Section 2)

- Email discussion #2: A-CSI/SP-CSI on PUSCH with repetition Type B (Section 3)

- Email discussion #3: UCI multiplexing on PUSCH for PUSCH repetition Type B (Section 4)

- Email discussion #4: Remaining issues on transmit power and interaction with DL/UL directions for PUSCH repetition Type B (Section 5 and Section 6)

It is suggested to postpone the remaining issues in Section 7 to the next meeting.

# References

- R1-2003319	Remaining issues on PUSCH enhancements for NR URLLC	ZTE

- R1-2003389	PUSCH enhancements for URLLC	vivo

- R1-2003441	Remaining Issue of PUSCH Enhancements for NR URLLC	Ericsson

- R1-2003529	Corrections on PUSCH enhancement	Huawei, HiSilicon

- R1-2003579	Maintenance of PUSCH enhancements for Rel-16 NR URLLC	Nokia, Nokia Shanghai Bell

- R1-2003622	Remaining issues on PUSCH enhancements	CATT

- R1-2003739	Corrections on PUSCH enhancements for URLLC	Intel Corporation

- R1-2003815	Remaining issues on URLLC PUSCH enhancement	Panasonic Corporation

- R1-2003867	Remaining issues for PUSCH enhancements	Samsung

- R1-2003976	PUSCH enhancements	ETRI

- R1-2003986	Discussion on PUSCH enhancements for URLLC	Spreadtrum Communications

- R1-2004031	Remaining issues of PUSCH enhancements for NR URLLC	LG Electronics

- R1-2004046	Remaining issues on PUSCH enhancements for URLLC	Fujitsu

- R1-2004116	PUSCH enhancements for URLLC	OPPO

- R1-2004223	Remaining Issues on PUSCH enhancements for eURLLC	Apple

- R1-2004332	Remaining issues on PUSCH enhancements for NR URLLC	Sharp

- R1-2004391	Remaining issues for PUSCH enhancements for Rel.16 URLLC	NTT DOCOMO, INC

- R1-2004459	Remaining issues on PUSCH enhancements for URLLC	Qualcomm Incorporated

- R1-2004524	Remaining issues on PUSCH enhancement for NR URLLC	WILUS Inc.

- R1-2004572	Remaining issue of PUSCH enhancements for NR URLLC	ASUSTeK

# Appendix A: Previous agreements on potential enhancements for PUSCH

### RAN1#94bis (Oct. 2018)

Agreements:

- One PUSCH transmission instance is not allowed to cross the slot boundary at least for grant-based PUSCH.

### RAN1#95 (Nov. 2018)

Agreements:

Support at least one of the following for one TB:

- One UL grant scheduling two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots

- One UL grant scheduling two or more PUSCH repetitions in consecutive available slots, with one repetition in each slot with possibly different starting symbols and/or durations

- N (N>=2) UL grants scheduling N PUSCH repetitions on consecutive available slots, with one repetition in each slot, and the i-th UL grant can be received before the end of the PUSCH transmission scheduled by the (i-1)th UL grant.

- FFS the definition of available slots

### RAN1 AH#1901 (Jan. 2019)

Agreements:

At least for scheduled PUSCH, for the option “One UL grant scheduling two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots” (also called as “mini-slot based repetitions”), if supported, it further consists of:

- Time domain resource determination

  - The time domain resource assignment field in the DCI indicates the resource for the first repetition.

  - The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols.

    - FFS the detailed interaction with the procedure of UL/DL direction determination

  - Each repetition occupies contiguous symbols.

  - FFS whether/how to handle “orphan” symbols (the # of UL symbols is not sufficient to carry one full repetition)

- Frequency hopping (at least 2 hops)

  - Support at least inter-PUSCH-repetition hopping and inter-slot hopping

  - FFS other FH schemes

  - FFS number of hops larger than 2

- FFS dynamic indication of the number of repetitions

- FFS DMRS sharing

- FFS TBS determination (e.g. based on the whole duration, or based on the first repetition)

Agreements:

At least for scheduled PUSCH, for the option “One UL grant scheduling two or more PUSCH repetitions in consecutive available slots, with one repetition in each slot with possibly different starting symbols and/or durations” (also called as “twomulti-segment transmission”), if supported, it further consists of:

- Time domain resource determination

  - The time domain resource assignment field in the DCI indicates the starting symbol and the transmission duration of all the repetitions.

    - FFS multiple SLIVs indicating the starting symbol and the duration of each repetition

    - FFS details of SLIV, including the possibility of modifying SLIV to support the cases with S+L>14.

  - FFS the interaction with the procedure of UL/DL direction determination

- For the transmission within one slot,

  - If there are more than one UL period within a slot (where each UL period is the duration of a set of contiguous symbols within a slot for potential UL transmission as determined by the UE)

    - Alt1: One repetition spans across more than one UL periods.

      - This implies that DMRS is required for each UL period.

      - Note: it is agreed in previous meetings that one PUSCH instance is not across a slot boundary

      - Each repetition occupies contiguous symbols available for potential UL transmission across one or more UL periods

    - Alt2: One repetition is within one UL period.

      - FFS if more than one UL period is used for the transmission (If more than one UL period is used, this would override the previous definition of this option.)

      - Each repetition occupies contiguous symbols

  - Otherwise, a single PUSCH repetition is transmitted within a slot following Rel-15 behavior.

- FFS Transmission of the repetitions spanning across more than two slots is not supported.

- Frequency hopping

  - Support at least inter-slot FH

  - FFS other FH schemes

- FFS TBS determination (e.g. based on the whole duration, or based on the first repetition, overhead assumption)

Agreements:

- Down-select between “mini-slot based repetitions” and “two-segment transmission”, aiming in RAN1#96

- FFS the option of using separate grants to schedule PUSCH repetitions in consecutive available slots

Agreements:

Companies are encouraged to provide more details in RAN1#96 at least for the following for potential enhancements of PUSCH:

- Details of the time domain resource determination, including the interaction with the DL/UL direction of the symbols

- Details of TBS determination

- What is different for scheduled PUSCH and configured grant?

  - E.g. for configured grant, should the transmission be allowed to postpone when conflicting with DL symbols?

- Comparison between the two schemes, including the potential performance evaluation/analysis (including latency, reliability, etc), complexity, overhead, etc.

### RAN1#96 (Feb. 2019)

Agreements:

- Capture the descriptions of option 1 to 6 (see R1-1903797 and previous agreements) in the TR.

Here is the description of Option 4 from TR 38.824:

One or more actual PUSCH repetitions in one slot, or two or more actual PUSCH repetitions across slot boundary in consecutive available slots, is supported using one UL grant for dynamic PUSCH, and one configured grant configuration for configured grant PUSCH. It further consists of:

- The number of the repetitions signaled by gNB represents the “nominal” number of repetitions. The actual number of repetitions can be larger than the nominal number.

  - FFS dynamically or semi-statically signalled for dynamic PUSCH and type 2 configured grant PUSCH

- The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first “nominal” repetition.

- The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols.

  - FFS the detailed interaction with the procedure of UL/DL direction determination

- If a “nominal” repetition goes across the slot boundary or DL/UL switching point, this “nominal” repetition is splitted into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot.

  - Handling of the repetitions under some conditions, e.g., when the duration is too small due to splitting, is to be further investigated in the WI phase.

- No DMRS sharing across multiple PUSCH repetitions

- The maximum TBS size is not increased compared to Rel-15.

- FFS: L > 14

- S+L can be larger than 14

- FFS: The bitwidth for TDRA is up to 4 bits.

- Note: different repetitions may have the same or different RV.

Conclusion:

- Finalize the details regarding how to use “option 1” vs. “option 2” during the WI phase using option 4, 5, and 6 (as in R1-1903797) as a starting point.

Agreements:

- Capture the simulation results in Section 3 in the TR.

### RAN1#96bis (Apr. 2019)

Agreements:

- Option 5 is not considered further as part of PUSCH enhancements.

Agreements:

For option 4, dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH is supported for PUSCH enhancements. The dynamic indication can be enabled or disabled by the gNB.

- FFS the exact signaling method

- FFS the exact DCI format(s)

- FFS the exact mechanism to enable or disable

- FFS the DCI activating type 2 configured grant PUSCH

Agreements:

For option 6,

- For dynamic PUSCH

  - For semi-static DL symbol(s), to down-select

    - Option 1: it is not expected that the resource allocation has conflict with semi-static DL symbol(s).

    - Option 2: if the resource allocation has conflict with semi-static DL symbol(s), the repetition is not transmitted.

  - For dynamically indicated DL symbol(s) (via format 2_0), it is not expected at the UE that the resource allocation has conflict with dynamically indicated DL symbol(s).

    - Note: this is the same as Rel-15 behavior.

- For configured grant PUSCH,

  - For type 1 configured grant PUSCH, and PUSCH other than the first PUSCH (including all repetitions) associated with the type 2 configured grant activation,

    - If a repetition conflicts with semi-static DL symbol(s), the repetition is not transmitted.

    - FFS: If a repetition conflicts with dynamically indicated DL symbol(s) (via format 2_0), the repetition is not transmitted.

  - FFS For the first PUSCH (including all repetitions) associated with the type 2 configured grant activation, follow the same handling as dynamic PUSCH.

Agreements:

- For option 6, at least for dynamic grants, it is not expected that one repetition (i.e., one SLIV) spans across slot boundary.

Agreements:

For both option 4 and 6, frequency hopping is supported

- FFS details

### RAN1#97 (May 2019)

Agreements:

- Adopt option 4 with the following update:

- The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first “nominal” repetition.

  - FFS the detailed interaction with the procedure of UL/DL direction determination

### RAN1#98 (Aug. 2019)

Agreements:

In terms of how to interpret L and K for all PUSCH transmissions, down-select between the following two:

- Alt 1: The time window within which valid symbols are used for transmission is L*K.

  - FFS the definition of “valid symbols”

- Alt 2: The time window within which valid symbols are used for transmission can be longer than L*K symbols, and it is extended at least in case of semi-static DL symbols.

  - FFS extension of the time window in case of dynamic DL symbols and/or semi-static flexible symbols and/or reserved symbols (if defined) and/or SSB symbols and/or type-0 CSS in CORESET#0 (as indicated by MIB)

  - FFS the definition of “valid symbols”

  - FFS whether to define a maximum time window size and if so, details

Conclusion:

In terms of how to handle the interaction of enhanced PUSCH with DL/UL directions, consider the following options:

- For DG PUSCH

  - If dynamic SFI is not configured,

    - Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

  - If dynamic SFI is configured

    - Option 1: behavior not dependent on dynamic SFI

      - Option 1-1: Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

        - FFS whether the conflict between dynamic SFI and symbols used for PUSCH transmission is considered as an error case, e.g.

          - Option 1-1a: The UE does not expect any semi-static flexible symbol to be indicated as DL within the PUSCH transmission time window.

          - Option 1-1b: No error case is defined and in general all semi-static flexible symbols are used for PUSCH within the PUSCH transmission time window.

      - Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols.

      - Option 1-3: Dynamic indication in UL grant on which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the dynamically indicated invalid symbols.

      - Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the invalid symbols as defined in the rules.

    - Option 2: the UE uses SFI to determine the symbols to transmit

      - In case SFI is configured and received

        - Option 2-1: Segmentation occurs around semi-static DL symbols and dynamic DL/flexible symbols

        - Option 2-2: Dynamic flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL symbols and dynamic DL symbols

        - Option 2-3: Dynamic flexible symbols are used for PUSCH. A repetition is not transmitted if it conflicts with a dynamic DL symbol.

        - Option 2-4: A repetition is not transmitted if it conflicts with a dynamic DL/flexible symbol

      - In case SFI is configured and not received

        - A repetition is not transmitted if it conflicts with a semi-static flexible symbol.

- For CG PUSCH other than the first Type 2 CG PUSCH (including all the repetitions) activated by an UL grant

  - If dynamic SFI is not configured,

    - Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

  - If dynamic SFI is configured

    - Option 1: behavior not dependent on dynamic SFI

      - Option 1-1: Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

        - This does not seem to make much sense for CG. If semi-static flexible symbols are always used for CG PUSCH, the gNB can essentially configure these symbols as UL in semi-static configuration. – no need for this option?

      - Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols.

      - Option 1-3 from DG is not applicable for CG.

      - Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the invalid symbols as defined in the rules.

    - Option 2: the UE uses SFI to determine the symbols to transmit

      - In case SFI is configured and received

        - Option 2-1: Segmentation occurs around semi-static DL symbols and dynamic DL/flexible symbols

        - Option 2-2 does not make sense for CG. (Dynamic flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL symbols and dynamic DL symbols)

        - Option 2-3 does not make sense for CG. (Dynamic flexible symbols are used for PUSCH. A repetition is not transmitted if it conflicts with a dynamic DL symbol.)

        - Option 2-4: a repetition is not transmitted if it conflicts with a semi-static DL symbol and a dynamic DL/flexible symbol

      - In case SFI is configured and not received

        - A repetition is not transmitted if it conflicts with a semi-static flexible symbol.

- For the first Type 2 CG PUSCH (including all the repetitions) activated by an UL grant,

  - Alt 1: same behavior as DG PUSCH

  - Alt 2: same behavior as CG PUSCH without an associated UL grant

  - …

- FFS: in case of a repetition not being transmitted (as in the above bullets), whether a repetition is a nominal repetition or a repetition after segmentation due to semi-static DL symbol(s)/slot boundary

- FFS: whether to postpone or not, and if yes, under what condition(s)

- FFS: whether/how guard period is handled

- Note that segmentation at slot boundary is always performed, even though it is not explicitly mentioned in the bullets above.

- FFS: the handling of conflict with SSB/PRACH symbols, the handling of conflict with semi-statically configured DL reception, etc.

- Other options are not precluded

### RAN1#98bis (Oct. 2019)

Agreements:

- Do not support PUSCH mapping type A for Option 4.

Agreements:

- Rel-16 enhanced PUSCH scheme (including dynamic indication of the number of repetitions) is supported for DCI format 0_1 and new UL DCI format (for DG and type 2 CG).

- Rel-16 enhanced PUSCH scheme is not supported for DCI format 0_0 for DG and type 2 CG

Agreements:

For the dynamic indication of the number of repetitions for dynamic grant:

- Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table

  - The maximum TDRA table size is increased to 64

  - No other spec impact is expected

Agreements:

- Support dynamic indication of the number of repetitions for Rel-15 PUSCH with slot aggregation using DCI formats 0_1 & the new UL DCI format

  - The dynamic indication is done by using the same Rel-16 mechanism (Jointly coding the number of repetitions with SLIV in TDRA table)

Agreements:

For frequency hopping for Rel-16 PUSCH, the number of actual hopping locations in frequency is 2.

Agreements:

In case frequency hopping is enabled for Rel-16 PUSCH, to determine the frequency locations of the two hops, reuse Rel-15 RRC parameters and equations for format 0_1, and introduce new RRC parameters (same as those of Rel-15) for new DCI UL format.

- FFS time domain hopping pattern

Agreements:

In terms of how to interpret L and K for Rel-16 PUSCH transmissions (for both DG & CG), Alt. 1 is adopted.

- That is, for the Rel-16 PUSCH with enhanced repetition transmission, the time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.

Conclusion:

Definitions:

- “Rel-16 PUSCH transmission scheme”: Option 4

- “Rel-15 PUSCH transmission scheme”: the transmission is done according to Rel-15 behavior, either with or without slot aggregation. With slot aggregation, the number of repetitions can be either semi-statically configured (as in Rel-15) or dynamically indicated (as agreed for Rel-16).

Agreements:

For DG and retransmission of CG, introduce one RRC parameter for each of the DCI format 0_1 and the new UL DCI format, to indicate whether UE follows the behavior for “Rel-16 PUSCH transmission scheme” or the behavior for “Rel-15 PUSCH transmission scheme”.

- FFS: whether to restrict that “Rel-16 PUSCH transmission scheme” cannot be enabled for both DCI formats simultaneously

For Type 1 CG, introduce an RRC parameter per CG configuration to indicate whether UE follows the behavior for “Rel-16 PUSCH transmission scheme” or the behavior for “Rel-15 PUSCH transmission scheme”.

Agreements:

For Type 2 CG, UE uses the PUSCH transmission scheme (“Rel-16 PUSCH transmission scheme” or “Rel-15 PUSCH transmission scheme”) associated with the activating DCI format.

Agreements:

For the interaction with DL/UL directions, if dynamic SFI is configured, Option 1-4 is not further considered for both DG and CG

For the interaction with DL/UL directions, if dynamic SFI is configured, Option 1-2 is not further considered for DG.

Agreements:

For the interaction with DL/UL directions, if dynamic SFI is configured, Option 2-2 and 2-3 is not further considered for DG.

Agreements:

- For both DG and CG with “Rel-16 PUSCH transmission scheme”, if dynamic SFI is not configured, semi-static flexible symbols are used for PUSCH. Segmentation occurs at least around semi-static DL symbols.

  - FFS segmentation also around dynamically indicated invalid symbols for UL transmissions in the UL grant (if supported for DG and/or Type 2 CG) and/or semi-statically configured invalid symbols for UL transmissions (if supported)

  - FFS how to handle the conflict with dynamic DL transmission for CG

### RAN1#99 (Nov. 2019)

Agreements:

- For the initial Type 2 CG PUSCH transmission, the TDRA table follows the activating DCI.

- For the initial Type 2 CG PUSCH transmission with PUSCH repetition type A or B, the number of repetitions is provided by the activating DCI via numberofrepetitions if it is present in the corresponding TDRA table; otherwise, the number of repetitions is provided by repK.

Agreements:

- For the initial Type 1 CG PUSCH transmission with PUSCH repetition type B,

  - If one and only one of DCI formats 0_1 and 0_2 is configured with PUSCH repetition type B, the TDRA table corresponding to the DCI format (0_1 or 0_2) configured with PUSCH repetition type B is used.

  - If both 0_1 and 0_2 are configured with PUSCH repetition type B, the TDRA table corresponding to DCI format 0_1 is used.

  - Note: For the initial Type 1 CG PUSCH transmission with PUSCH repetition type B, the case of none of the DCI formats 0_1 and 0_2 is configured with PUSCH repetition type B is an error case

- For the initial Type 1 CG PUSCH transmission, if it is configured with PUSCH repetition type A, use the TDRA table for USS in Rel-15.

- For the initial Type 1 CG PUSCH transmission with PUSCH repetition, the number of repetitions is provided via numberofrepetitions if it is present in the corresponding TDRA table; otherwise, the number of repetitions is provided by repK.

- FFS the value range of repK is extended for R16 repetition type A and/or type B

Agreements:

- For PUSCH repetition type B, L<=14

Agreements:

For PUSCH repetition type B, support the following frequency hopping:

- Inter-PUSCH-repetition FH

  - Details FFS

- Inter-slot FH

- FFS Intra-PUSCH-repetition FH

Agreements:

The column on the number of repetitions numberofrepetitions is always present in PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_1 and PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_2.

- For DG with PUSCH repetition type A, if numberofrepetitions is present in the corresponding TDRA table, the number of repetitions is given by numberofrepetitions. Elseif the UE is configured with pusch-AggregationFactor, the number of repetitions is given by pusch-AggregationFactor. Otherwise the number of repetitions is 1.

- For DG with PUSCH repetition type B, the number of repetitions is given by numberofrepetitions.

  - Note that pusch-TimeDomainAllocationList-ForDCIformat0_1/2 needs to be configured for PUSCH repetition type B.

Agreements:

For PUSCH repetition type A and type B, the number of bits to indicate numberofrepetitions is 3.

- {1, 2, [3], 4, [6], 7, [8], 12, 16} are supported.

- FFS whether to have a limit on the number of nominal repetitions in a slot

Agreements:

For how to indicate S and L in the TDRA table for PUSCH repetition type B, S and L are separately indicated (4-bit for S and 4-bit for L).

- S is from 0 and [13], L is from [1] to 14.

  - Note: The additional restrictions for a particular waveform and/or DMRS mapping type from R15 are still applicable

Agreements:

For both DG and CG with PUSCH repetition type B, the TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism.

Agreements:

For Type 1 CG with PUSCH repetition type B, introduce a new RRC parameter frequencyHopping-PUSCHRepTypeB per CG configuration to indicate the frequency hopping scheme, and reuse Rel-15 parameter frequencyHoppingOffset to determine the frequency locations.

- For Type 1 CG with PUSCH repetition type B, if frequencyHopping-PUSCHRepTypeB is not configured, frequency hopping is not enabled.

Agreements

Introduce a new RRC parameter frequencyHopping-ForDCIFormat0_1.

- This parameter can only be configured when PUSCHRepTypeIndicator-ForDCIFormat0_1 is set to ‘pusch-RepTypeB’.

Agreement (RRC impact)

For DG PUSCH with PUSCH repetition type B, if dynamic SFI is configured, introduce a first RRC parameter that indicates one pattern for invalid symbols for PUSCH transmission repetition type B applicable to both DCI format 0_1 and 0_2, and introduce a second RRC parameter for each of DCI format 0_1 and 0_2 to indicate the presence of an additional bit in the DCI to indicate whether the pattern applies or not.

- If the first RRC parameter is not configured, semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

- If the first RRC parameter is configured and the additional bit exists in a DCI,

  - Value ‘0’ means semi-static flexible symbols are used for PUSCH, and segmentation occurs only around semi-static DL symbols.

  - Value ‘1’ means that segmentation occurs around semi-static DL symbols and invalid symbols in the pattern, and the remaining symbols are used for PUSCH.

- If the first RRC parameter is configured and the additional bit does not exist in a DCI, segmentation occurs around semi-static DL symbols and invalid symbols in the pattern, and the remaining symbols are used for PUSCH.

- The first RRC parameter reuses the pattern definition of rateMatchPattern in time domain for PDSCH.

Note: Qualcomm has concerns over the above feature in terms of UE complexity. Majority of companies do not see this issue.

Agreement

For CG PUSCH with PUSCH repetition type B, if dynamic SFI is configured, segmentation occurs at least around semi-static DL symbols, which results in actual repetitions.

- If dynamic SFI is received for the entire duration of an actual repetition, an actual repetition is not transmitted if it conflicts with a dynamic DL/flexible symbol.

- If dynamic SFI is not received for at least one symbol of an actual repetition, an actual repetition is not transmitted if it conflicts with a semi-static flexible symbol.

- FFS the handling of semi-statically configured invalid symbols for PUSCH repetition type B transmissions (if supported)

Note that the cancellation behavior is the same as Rel-15, including Rel-15 cancellation timeline

Agreement

For DG PUSCH with PUSCH repetition type B, the RV for the first repetition is provided by DCI, and RV cycling is done across the repetitions using the RV sequence of {0, 2, 3, 1}.

- FFS “repetition” means nominal or actual repetition

  - FFS In case “repetition” means nominal repetition, whether the same RV applies to all the actual repetitions corresponding to a nominal repetition.

Agreements:

For CG PUSCH with PUSCH repetition type B, RV cycling is done across repetition following the sequence in repK-RV,

- the first repetition uses the first value in repK-RV

- “repetition” means actual repetition

### RAN1#100-e (Feb. 2020)

[100e-NR-L1enh_URLLC-PUSCH_Enh-01]

Agreements:

For numberofrepetitions for PUSCH repetition type A and type B, {3, 8} are additionally supported. That is, {1, 2, 3, 4, 7, 8, 12, 16} are supported for numberofrepetitions. (RRC impact)

Agreements:

The value range for repK remains the same as in Rel-15.

Agreements:

For PUSCH repetition Type B, S is from 0 to 13, and L is from 1 to 14. (RRC impact)

Agreements: (RRC impact)

Introduce reportSlotOffsetList-r16-ForDCIFormat0_1 and reportSlotOffsetList-r16-ForDCIFormat0_2 and update TS 38.214 accordingly

- FFS whether or not to always assume the number of nominal repetitions is equal to 1 when PUSCH with repetition Type B carries A-CSI/SP-CSI only.

Agreements:

For PUSCH repetition Type B, PUSCH transmit power is determined based on the nominal repetition duration.

Agreements:

Adopt the following TP to TS 38.214:

TP to TS 38.214, Sec. 5.2.1.4 and Sec. 6.1.2.1 5.2.1.4 Reporting configurations <Unchanged text is omitted> For a semi-persistent or aperiodic CSI report on PUSCH, the allowed slot offsets are configured by the following higher layer parameters: - if triggered/activated by DCI format 0_2 and the higher layer parameter reportSlotOffsetListForDCI-Format0-2 is configured, the allowed slot offsets are configured by the higher layer parameter reportSlotOffsetListForDCI-Format0-2 reportSlotOffsetList-r16-ForDCIFormat0_2, and - if triggered/activated by DCI format 0_1 and the higher layer parameter reportSlotOffsetListForDCI-Format0-1 reportSlotOffsetList-r16-ForDCIFormat0_1 is configured, the allowed slot offsets are configured by the higher layer parameter reportSlotOffsetListForDCI-Format0-1 reportSlotOffsetList-r16-ForDCIFormat0_1, and - otherwise, the allowed slot offsets are configured] by the higher layer parameter reportSlotOffsetList. The offset is selected in the activating/triggering DCI. <Unchanged text is omitted> 6.1.2.1 Resource allocation in time domain <Unchanged text is omitted> When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a CSI request field on a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an the allocated table as defined in Clause 6.1.2.1.1 which is defined by the higher layer configured pusch-TimeDomainAllocationList in pusch-Config. The indexed row defines the start and length indicator SLIV, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where are the corresponding list entries of the higher layer parameter - [reportSlotOffsetListForDCI-Format0-2 reportSlotOffsetList-r16-ForDCIFormat0_2, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListForDCI-Format0-2 is configured; - reportSlotOffsetListForDCI-Format0-1 reportSlotOffsetList-r16-ForDCIFormat0_1, if PUSCH is scheduled by DCI format 0_1 and reportSlotOffsetListForDCI-Format0-1 reportSlotOffsetList-r16-ForDCIFormat0_1 is configured]; - reportSlotOffsetList, [otherwise;] in CSI-ReportConfig for the triggered CSI Reporting Settings and is the (m+1)th entry of . <Unchanged text is omitted>

[100e-NR-L1enh_URLLC-PUSCH_Enh-02]

Conclusion on how FH is enabled/disabled for Type 2 CG with DCI format 0_1 in Rel-15:

- For Type 2 CG in Rel-15 activated by DCI format 0_1, if frequencyHopping in configuredGrantConfig is not configured, FH is disabled. If frequencyHopping in configuredGrantConfig is configured, FH for Type 2 CG is enabled if the frequency hopping flag field in the activation DCI is set to 1, and FH is disabled if the frequency hopping flag field in the activation DCI is set to 0.

Agreements:

For Type 2 CG PUSCH activated by a DCI format configured with PUSCH repetition Type B, the frequency hopping enabling/disabling and the frequency offset follows the indication in the activation DCI, and the frequency hopping scheme follows the corresponding RRC parameter for the activation DCI format. (RRC impact)

Agreements:

For PUSCH with repetition Type B, with inter-repetition FH, frequency hopping occurs for each nominal repetition.

Agreements:

For PUSCH repetition Type B, intra-PUSCH-repetition frequency hopping is not supported. (RRC impact)

Agreements:

Adopt the following TP to TS 38.212 (changes in red):

TP to TS 38.212, Sec. 7.3.1.1.2 ##### 7.3.1.1.2 Format 0_1 ## <Unchanged text is omitted> - Frequency hopping flag – 0 or 1 bit: - 0 bit if only resource allocation type 0 is configured, or if both the higher layer parameter frequencyHopping is not configured and the higher layer parameter frequencyHopping-ForDCIFormat0_1 pusch-RepTypeIndicatorForDCI-Format0-1-r16 is are not configured to ‘pusch-RepTypeB’, or if the higher layer parameter frequencyHoppingForDCI-Format0-1-r16 is not configured and pusch-RepTypeIndicatorForDCI-Format0-1-r16 is configured to ‘pusch-RepTypeB’, or if only resource allocation type 2 is configured; - 1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214]. ## <Unchanged text is omitted>

Agreements:

Adopt the following TP to TS 38.214 (changes in red):

TP to TS 38.214, Sec. 6.3.2 6.3.2 Frequency hopping for PUSCH repetition Type B For PUSCH repetition Type B (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHopping-ForDCIFormat0_2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, by frequencyHopping-ForDCIFormat0_1 provided in pusch-Config for PUSCH transmission scheduled by DCI format 0_1, and by frequencyHopping-PUSCHRepTypeB provided in rrc-ConfiguredUplinkGrant configuredGrantConfig for [Type 1] configured PUSCH transmission. [The frequency hopping mode for Type 2 configured PUSCH transmission follows the configuration of the activating DCI format]. One of two frequency hopping modes can be configured: - Inter-repetition frequency hopping - Inter-slot frequency hopping In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHopping- PUSCHRepTypeB is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211]. <Unchanged text is omitted>

Agreements:

Adopt the following TP to TS 38.214 (changes in red):

TP to TS 38.214, Sec. 6.3.2 6.3.2 Frequency hopping for PUSCH repetition Type B <Unchanged text is omitted> In case of inter-repetition frequency hopping, [details to be added when agreements become available]. the starting RB for an actual repetition within the n-th nominal repetition (as defined in Clause 6.1.2.1) is given by: , where is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Subclause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. In case of inter-slot frequency hopping, the starting RB during slot follows that of inter-slot frequency hopping for PUSCH Repetition Type A in Clause 6.3.1.

[100e-NR-L1enh_URLLC-PUSCH_Enh-03]

Agreements:

The semi-static and dynamic indication of invalid symbols (related to InvalidSymbolPattern) for DG PUSCH repetition Type B in case dynamic SFI is not configured follows the same behaviour as for DG PUSCH repetition Type B in case dynamic SFI is configured.

Agreements:

For Type 1 CG PUSCH with repetition Type B, regardless of whether dynamic SFI is configured or not, if InvalidSymbolPattern is configured, the configured pattern is applied (that is, segmentation occurs around semi-static DL symbols and invalid symbols indicated by InvalidSymbolPattern).

Agreements:

For the first Type 2 CG PUSCH with repetition Type B (including all repetitions) after activation, regardless of whether dynamic SFI is configured or not, if InvalidSymbolPattern is configured, whether the configured pattern is applied follows the same procedure as specified for DG PUSCH according to the activation DCI.

Agreements:

For Type 2 CG PUSCH with repetition Type B (excluding the first Type 2 CG PUSCH, with all repetitions, after activation), regardless of whether dynamic SFI is configured or not, if InvalidSymbolPattern is configured, whether the configured pattern is applied follows the activation DCI.

Agreements:

For PUSCH repetition Type B, a UE is not expected to be indicated with an antenna port configuration that is invalid for the duration of any actual repetition.

Agreements:

For PUSCH with repetition Type B, an actual repetition with a single symbol is not transmitted.

Agreements:

Adopt the following TP to TS 38.214:

TP to TS 38.214, Sec. 6.1.2.1 6.1.2 Resource allocation 6.1.2.1 Resource allocation in time domain <unchanged text omitted> For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - [If a UE is configured with higher layer parameter SlotFormatInficator, the] The UE may be configured with the higher layer parameter InvalidSymbolPattern, which provides a symbol level bitmap spanning one or two slots (higher layer parameter symbols given by InvalidSymbolPattern). A bit value equal to 1 in the symbol level bitmap symbols indicates that the corresponding symbol is an invalid symbol for PUSCH repetition Type B transmission. The UE may be additionally configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by InvalidSymbolPattern), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap symbols, and a bit value equal to 1 indicates that the symbol level bitmap symbols is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40ms. The first symbol of periodicityAndPattern every 40ms/P periods is a first symbol in frame 𝑛𝑓 mod 4 = 0, where P is the duration of periodicityAndPattern in units of ms. When periodicityAndPattern is not configured, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. If InvalidSymbolPattern is configured, when the UE applies the invalid symbol pattern is determined as follows: - if InvalidSymbolPatternIndicator-ForDCIFormat0_1 is configured when the PUSCH is scheduled by DCI format 0_1, or if InvalidSymbolPatternIndicator-ForDCIFormat0_2 is configured when the PUSCH is scheduled by DCI format 0_2, - if [invalid symbol pattern indicator] field is set 1, the UE applies the invalid symbol pattern; - otherwise, the UE does not apply the invalid symbol pattern; - If the PUSCH is scheduled by DCI format 0_1, or corresponds to a Type 2 configured grant activated by DCI format 0_1, and if InvalidSymbolPatternIndicator-ForDCIFormat0_1 is configured, - if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern; - otherwise, the UE does not apply the invalid symbol pattern; - If the PUSCH is scheduled by DCI format 0_2, or corresponds to a Type 2 configured grant activated by DCI format 0_2, and if InvalidSymbolPatternIndicator-ForDCIFormat0_2 is configured, - if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern; - otherwise, the UE does not apply the invalid symbol pattern; - otherwise, the UE applies the invalid symbol pattern. For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. [If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot.] An actual repetition is omitted according to the conditions in Clause 11.1 of [6, TS38.213]. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2.

Agreements:

Adopt the following TP to TS 38.214:

TP to TS 38.214, Sec. 6.1.2.1 6.1.2 Resource allocation 6.1.2.1 Resource allocation in time domain <unchanged text omitted> For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. [If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot.] An actual repetition with a single symbol is omitted except for the case of L =1. An actual repetition is omitted according to the conditions in Clause 11.1 of [6, TS38.213]. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2.

Agreements:

Adopt the following TP to TS 38.214:

TP to TS 38.214, Sec. 6.2.2 ## 6.2.2 UE DM-RS transmission procedure <unchanged text omitted> ## For PUSCH repetition Type B, the DM-RS transmission procedure is applied for each actual repetition separately based on the allocation duration of the actual repetition. A UE is not expected to be indicated with an antenna port configuration that is invalid for the allocated duration of any actual repetition.

### RAN1#100bis-e (April. 2020)

[100b-e-NR-L1enh-URLLC-PUSCH-01]

Agreements:

Adopt the following TP to TS 38.213:

TP to TS 38.213, Sec. 7 # 7 Uplink Power control Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions. A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by IE SRS-Positioning-Config as described in Clause 7.3.1. A PUSCH/PUCCH/SRS/PRACH transmission occasion is defined by a slot index within a frame with system frame number , a first symbol within the slot, and a number of consecutive symbols . For PUSCH with repetition Type B, a PUSCH transmission occasion is a nominal repetition, as described in [6, TS 38.214 Clause 6.1.2]. ## 7.1 Physical uplink shared channel ## <omitted text> ### 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP of carrier of serving cell using parameter set configuration with index and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power in PUSCH transmission occasion as [dBm] where, <omitted text> - for and for where is provided by deltaMCS for each UL BWP of each carrier and serving cell . If the PUSCH transmission is over more than one layer [6, TS 38.214], . and , for active UL BWP of each carrier and each serving cell , are computed as below - for PUSCH with UL-SCH data and for CSI transmission in a PUSCH without UL-SCH data, where - is a number of transmitted code blocks, is a size for code block , and is a number of resource elements determined as , where is a number of symbols for PUSCH transmission occasion on active UL BWP of carrier of serving cell , is a number of subcarriers excluding DM-RS subcarriers and phase-tracking RS samples [4, TS 38.211] in PUSCH symbol (assuming no segmentation for a nominal repetition in case of PUSCH repetition type B), , and , are defined in [5, TS 38.212] <omitted text>

[100b-e-NR-L1enh-URLLC-PUSCH-02]

Agreements:

In case of PUCCH overlapping with PUSCH with repetition Type B,

- Option A: Multiplexing timeline conditions in Clause 9.2.5 of TS 38.213 shall be satisfied for all the overlapping actual repetitions. Otherwise it is considered as an error case.

Agreements:

In case PUCCH overlaps with multiple repetitions of PUSCH repetition Type B that satisfy the multiplexing timeline conditions, UCI is multiplexed on only one actual repetition (including the case where a PUCCH overlaps with a PUSCH with repetition Type B in multiple slots). To determine which actual repetition, down-select from the following 3 options:

- Option 1: the first overlapping actual repetition in the first overlapping slot that satisfies the multiplexing timeline

Conclusion:

The number of possible indices for beta offset that dynamic-ForDCIFormat0_2 can indicate is not increased.

[100b-e-NR-L1enh-URLLC-PUSCH-03]

Agreements:

For operation in unpaired spectrum, symbols that are indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered invalid symbols for PUSCH repetition Type B, and segmentation occurs around these invalid symbols.

Agreements:

For operation in unpaired spectrum, symbols indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS are considered as invalid symbols for PUSCH repetition Type B, and segmentation occurs around these symbols.

Agreements:

For operation in unpaired spectrum, introduce a new RRC parameter numberInvallidSymbolsForDL-UL-Switching to indicate the number of symbols after the last semi-static DL symbol that are invalid symbols for PUSCH repetition Type B.

- The candidate values include {1, 2, 3, 4}.

- If not configured, it means no symbols are explicitly defined for DL-to-UL switching.

Agreements:

Adopt the following TP for TS 38.213:

TP for TS 38.213 Section 8.1 8.1 Random access preamble < Unchanged parts are omitted > For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where for or , for or , and is the SCS configuration for the active UL BWP. This applies to each actual repetition for PUSCH repetition Type B (as described in [6, TS 38.214 Clause 6.1.2]) < Unchanged parts are omitted >

[100b-e-NR-L1enh-URLLC-PUSCH-04]

Agreements:

Adopt the following TP for TS 38.214:

TP for TS 38.214 #### 6.1.2.1 Resource allocation in time domain When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, and the number of repetitions (if numberofrepetitions is present in the resource allocation table) to be applied in the PUSCH transmission. When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a CSI request field on a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where are the corresponding list entries of the higher layer parameter - reportSlotOffsetListForDCI-Format0-2, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListForDCI-Format0-2 is configured; - reportSlotOffsetListForDCI-Format0-1, if PUSCH is scheduled by DCI format 0_1 and reportSlotOffsetListForDCI-Format0-1 is configured; - reportSlotOffsetList, otherwise; < Unchanged parts are omitted >

Agreements:

For PUSCH repetition Type B, S is from 0 to 11, and L is from 1 to 12 for extended cyclic prefix. Adopt the following TP for Section 6.1.2.1 in TS 38.214:

TP for TS 38.214 - 6.1.2.1 Resource allocation in time domain < Unchanged parts are omitted > - For PUSCH repetition Type B, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are provided by startSymbol and length of the indexed row of the resource allocation table, respectively. - For PUSCH repetition Type A, the PUSCH mapping type is set to Type A or Type B as defined in Clause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row. - For PUSCH repetition Type B, the PUSCH mapping type is set to Type B. The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations Table 6.1.2.1-1: Valid S and L combinations PUSCH mapping type; Normal cyclic prefix; Extended cyclic prefix. S; L; S+L; S; L; S+L. Type A; 0; {4,…,14}; {4,…,14} (repetition_Type A only); 0; {4,…,12}; {4,…,12}. Type B; {0,…,13}; {1,…,14}; {1,…,14} for repetition Type A, {1,…,27} for repetition Type B; {0,…, 11}; {1,… ,12}; {1,…,12} for repetition Type A, {1,...,23} for repetition Type B.

< Unchanged parts are omitted >

Agreements:

Adopt the following TP for TS 38.214:

TP for TS 38.214 - 6.1.2.1 Resource allocation in time domain < Unchanged parts are omitted > For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 11.1 of [6, TS38.213]. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2. < Unchanged parts are omitted >

Agreements:

Adopt the following TP for TS 38.214:

TP for TS 38.214 #### 6.1.4.2 Transport block size determination < Unchanged parts are omitted > The UE shall first determine the number of REs (NRE) within the slot: - A UE first determines the number of REs allocated for PUSCH within a PRB by - , where is the number of subcarriers in the frequency domain in a physical resource block, Nsymbslotis the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH or Clause 6.1.2.3 for configured PUSCH, is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1 or DCI format 0_2 or as described for DCI format 0_0 in Clause 6.2.2, and is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the is not configured (a value from 6, 12, or 18), the is assumed to be 0. For Msg3 transmission the is always set to 0. In case of PUSCH repetition Type B, is determined assuming a nominal repetition with the duration of L symbols without segmentation. < Unchanged parts are omitted >

# Appendix B: Related Rel-15 RRC parameters

PUSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPUSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

txConfig                                ENUMERATED {codebook, nonCodebook}                                  OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

pusch-PowerControl                      PUSCH-PowerControl                                                  OPTIONAL,   -- Need M

frequencyHopping                        ENUMERATED {intraSlot, interSlot}                                   OPTIONAL,   -- Need S

frequencyHoppingOffsetLists             SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

OPTIONAL,   -- Need M

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pusch-TimeDomainAllocationList          SetupRelease { PUSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pusch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

mcs-TableTransformPrecoder              ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

transformPrecoder                       ENUMERATED {enabled, disabled}                                      OPTIONAL,   -- Need S

codebookSubset                          ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL, -- Cond codebookBased

maxRank                                 INTEGER (1..4)                                            OPTIONAL, -- Cond codebookBased

rbg-Size                                ENUMERATED { config2}                                     OPTIONAL, -- Need S

uci-OnPUSCH                             SetupRelease { UCI-OnPUSCH}                               OPTIONAL, -- Need M

tp-pi2BPSK                              ENUMERATED {enabled}                                      OPTIONAL, -- Need S

...

}

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                    INTEGER (1..64)                                                     OPTIONAL,   -- Need R

rrc-ConfiguredUplinkGrant               SEQUENCE {

timeDomainOffset                        INTEGER (0..5119),

timeDomainAllocation                    INTEGER  (0..15),

frequencyDomainAllocation               BIT STRING (SIZE(18)),

antennaPort                             INTEGER (0..31),

dmrs-SeqInitialization                  INTEGER (0..1)                                                  OPTIONAL,   -- Need R

precodingAndNumberOfLayers              INTEGER (0..63),

srs-ResourceIndicator                   INTEGER (0..15)                                                 OPTIONAL,   -- Need R

mcsAndTBS                               INTEGER (0..31),

frequencyHoppingOffset                  INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                   OPTIONAL,   -- Need R

pathlossReferenceIndex                  INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...

}                                                                                                           OPTIONAL,   -- Need R

...

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic                                 SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic                              BetaOffsets

}

-- ASN1START

-- TAG-DMRS-UPLINKCONFIG-START

DMRS-UplinkConfig ::=               SEQUENCE {

dmrs-Type                           ENUMERATED {type2}                                                  OPTIONAL,   -- Need S

dmrs-AdditionalPosition             ENUMERATED {pos0, pos1, pos3}                                       OPTIONAL,   -- Need S

phaseTrackingRS                     SetupRelease { PTRS-UplinkConfig }                                  OPTIONAL,   -- Need M

maxLength                           ENUMERATED {len2}                                                   OPTIONAL,   -- Need S

transformPrecodingDisabled          SEQUENCE {

scramblingID0                       INTEGER (0..65535)                                              OPTIONAL,   -- Need S

scramblingID1                       INTEGER (0..65535)                                              OPTIONAL,   -- Need S

...

}                                                                                                       OPTIONAL,   -- Need R

transformPrecodingEnabled           SEQUENCE {

nPUSCH-Identity                     INTEGER(0..1007)                                                OPTIONAL,   -- Need S

sequenceGroupHopping                ENUMERATED {disabled}                                           OPTIONAL,   -- Need S

sequenceHopping                     ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

...

}                                                                                                       OPTIONAL,   -- Need R

...

}

-- TAG-DMRS-UPLINKCONFIG-STOP

-- ASN1STOP

# Appendix C: PUSCH prioritization rules for UCI multiplexed on PUSCH

Conclusion (RAN1#97) For the issue raised in the draft CR R1-1906302, the intended UE behavior per specification is commonly understood as follows: - For UCI multiplexing, within a PUCCH group, on PUSCH, the following two steps are performed with step 1 first, then followed by step 2: - Step 1: UCI in overlapped PUCCH transmissions is multiplexed into one PUCCH resource (resource Z). This step is done per PUCCH slot. - Step 2: UCI, that doesn’t include SR, in Z is multiplexed into one PUSCH, if Z overlaps with at least one PUSCH, following the priorities (sequentially from high to low) as listed below. - First priority: PUSCH with A-CSI as long as it overlaps with Z - Second priority: earliest PUSCH slot(s) based on the start of the slot(s) - If there are still multiple PUSCHs overlap with Z in the earliest PUSCH slot(s), follow the following priorities (sequentially from high to low) - Third priority: Dynamic grant PUSCHs > PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH - Fourth priority: PUSCHs on serving cell with smaller serving cell index > PUSCHs on serving cell with larger serving cell index - Fifth priority: Earlier PUSCH transmission > later PUSCH transmission Note: The clarification applies to both cases with the same (except the second priority part) and different numerologies among PUCCH and PUSCHs.