3GPP TSG RAN WG1 Meeting #102-e 	R1-2005252

E-meeting, August 17th – 28th, 2020

Agenda Item:	8.5.2

Source:	Huawei, HiSilicon

Title:	Performance evaluation for Rel-17 positioning

Document for:	Discussion and Decision

Introduction

This contribution presents the simulation results with Rel-16 positioning techniques for the IIoT scenario specified in [2] so as to identify the performance gaps.

Positioning performance accuracy

Scenarios for evaluations

The scenarios to be presented in this contribution include

- Baseline scenarios with InF-SH/InF-DH with fixed UE/gNB height and without UE/gNB calibration error

- InF-DH with variable UE/gNB height

- InF-SH/In-DH with UE/gNB calibration error

Baseline Scenario

Evaluated cases

The following cases in Table 1 are evaluated for FR1 and FR2. We will evaluate DL positioning, UL positioning, and DL+UL positioning solutions in InF-SH and InF-DH. In the following, the results are provided for DL-TDOA, UL-TDOA, UL+TDOA+UL-AOA and Multi-RTT in FR1, DL-TDOA, DL-TDOA+DL-AOD, UL+TDOA+UL-AOA and Multi-RTT in FR2.

Table 1 Evaluated parameter combinations

| Parameter | Case 1 (InF-SH, FR1) | Case 2 (InF-SH, FR1) | Case 3 (InF-SH, FR1) | Case 4 (InF-SH, FR1) | Case 5 (InF-DH, FR1) | Case 6 (InF-DH, FR1) | Case 7 (InF-DH, FR1) | Case 8 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |  |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |  |  |  |  |
| Additional notes, if any | Fixed UE/gNB height. |  |  |  |  |  |  |  |
| Parameter | Case 9 (InF-SH, FR2) | Case 10 (InF-SH, FR2) | Case 11 (InF-SH, FR2) | Case 12 (InF-SH, FR2) | Case 13 (InF-DH, FR2) | Case 14 (InF-DH, FR2) | Case 15 (InF-DH, FR2) | Case 16 (InF-DH, FR2) |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |  |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution | Super resolution | Super resolution | Super resolution | Super resolution | Super resolution | Super resolution | Super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping Rx beam sweeping |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based Rx codebook based |  |  |  |  |  |  |  |
| Additional notes, if any | Fixed UE/gNB height. |  |  |  |  |  |  |  |

Simulation result

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 2. The detailed simulation results can be found in Figure 11~ Figure 14 in the Appendix A.

Table 2  Positioning accuracy (in unit of meter)

|  | Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 1 | All UE | 0.077 | 0.153 | 0.419 | 2.789 | 10.288 |
|  |  | Central UE | 0.053 | 0.090 | 0.224 | 1.964 | 8.759 |
|  | 2 | All UE | 0.0871 | 0.1708 | 0.4013 | 1.8678 | 5.8576 |
|  |  | Central UE | 0.0621 | 0.1096 | 0.2410 | 1.0277 | 3.9456 |
|  | 3 | All UE | 0.0465 | 0.0731 | 0.1271 | 0.299 | 0.7889 |
|  |  | Central UE | 0.0434 | 0.0648 | 0.1116 | 0.2682 | 0.5521 |
|  | 4 | All UE | 0.0721 | 0.2035 | 0.6628 | 1.9481 | 4.4855 |
|  |  | Central UE | 0.0665 | 0.1798 | 0.5628 | 1.6992 | 4.3519 |
|  | 5 | All UE | 0.861 | 3.448 | 8.103 | 15.212 | 23.030 |
|  |  | Central UE | 0.703 | 2.904 | 8.399 | 15.635 | 23.030 |
|  | 6 | All UE | 0.4309 | 1.6317 | 4.9693 | 9.9453 | 16.811 |
|  |  | Central UE | 0.3354 | 1.2393 | 4.6237 | 10.163 | 17.66 |
|  | 7 | All UE | 0.0702 | 0.1673 | 0.3825 | 1.0453 | 2.8012 |
|  |  | Central UE | 0.0643 | 0.1529 | 0.3206 | 0.8016 | 2.1747 |
|  | 8 | All UE | 0.0699 | 0.1260 | 0.3916 | 7.8992 | 15.2156 |
|  |  | Central UE | 0.0667 | 0.1164 | 0.3338 | 7.3110 | 14.4738 |
| FR2 | 9 | All UE | 0.0089 | 0.0329 | 0.1935 | 1.1281 | 4.4335 |
|  |  | Central UE | 0.0077 | 0.0265 | 0.1586 | 0.9633 | 4.1343 |
|  | 10 | All UE | 0.0118 | 0.0203 | 0.0345 | 0.0918 | 0.2518 |
|  |  | Central UE | 0.0143 | 0.0231 | 0.0387 | 0.0654 | 0.1293 |
|  | 11 | All UE | 0.0102 | 0.0191 | 0.0365 | 0.0991 | 0.3826 |
|  |  | Central UE | 0.0084 | 0.0156 | 0.0296 | 0.0694 | 0.2516 |
|  | 12 | All UE | 0.0049 | 0.0143 | 0.0881 | 0.4697 | 1.4975 |
|  |  | Central UE | 0.0048 | 0.0137 | 0.0841 | 0.4496 | 1.3076 |
|  | 13 | All UE | 0.0466 | 0.5795 | 3.6866 | 9.2401 | 16.2349 |
|  |  | Central UE | 0.0321 | 0.4003 | 2.7974 | 9.6798 | 17.9321 |
|  | 14 | All UE | 0.0755 | 0.1861 | 0.4228 | 0.9619 | 2.1831 |
|  |  | Central UE | 0.0587 | 0.1452 | 0.3221 | 0.7197 | 1.7271 |
|  | 15 | All UE | 0.0732 | 0.1728 | 0.3750 | 0.9277 | 2.1001 |
|  |  | Central UE | 0.0603 | 0.1309 | 0.2717 | 0.7086 | 1.4854 |
|  | 16 | All UE | 0.0317 | 0.1905 | 1.0572 | 4.1555 | 10.5087 |
|  |  | Central UE | 0.0259 | 0.1425 | 0.8398 | 4.2895 | 11.5554 |

Observation 1:  The hybrid positioning can help to improve the positioning accuracy, e.g. UL-TDOA+UL-AOA, DL-TDOA+DL-AOD.

Observation 2:  The positioning accuracy of the center area UEs is generally higher than the edge area UEs.

Observation 3:  For InF-SH, the accuracy of less than 0.2m@90% can be achieved with DL-TDOA+DL-AOD and UL-TDOA+UL-AOA in FR2, and the accuracy of less than 0.5m@90% can be achieved with UL-TDOA+UL-AOA in FR1 and Multi-RTT in FR2.

Variable height with modified DH

Evaluated cases

The following cases in Table 3 are evaluated for modified InF-DH with clutter parameters {40%, 3m, 5m}. We will evaluate the 2D localization with fixed UE/gNB height and 3D localization with variable UE/gNB height. In the following, the results are provided for UL+TDOA+UL-AOA and Multi-RTT in FR1 and FR2, respectively.

Table 3  Evaluated parameter combinations

| Parameter | Case 101 (InF-DH, FR1) | Case 102 (InF-DH, FR1) | Case 103 (InF-DH, FR1) | Case 104 (InF-DH, FR1) | Case 105 (InF-DH, FR2) | Case 106 (InF-DH, FR2) | Case 107 (InF-DH, FR2) | Case 108 (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |  |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping |  |  |  | Tx beam sweeping Rx beam sweeping |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  | Tx codebook-based Rx codebook-based |  |  |  |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 4.The detailed simulation results can be found in Figure 15~Figure 18 in the Appendix A.

Table 4 Positioning accuracy (in unit of meter)

|  | Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 101 | All UE | 0.0742 | 0.1792 | 0.4392 | 1.3838 | 3.3913 |
|  |  | Central UE | 0.0628 | 0.1571 | 0.3844 | 1.3012 | 2.8376 |
|  | 102 | All UE | 0.0713 | 0.1249 | 0.4734 | 8.6557 | 9.0699 |
|  |  | Central UE | 0.0663 | 0.1082 | 0.4952 | 9.8411 | 9.0856 |
|  | 103 | All UE | 0.3405(H) | 1.0103(H) | 2.3157(H) | 5.1203(H) | 7.9726(H) |
|  |  |  | 0.0905(V) | 0.2923(V) | 0.7287(V) | 1.3239(V) | 1.8648 (V) |
|  |  | Central UE | 0.2923(H) | 0.8122(H) | 2.0088(H) | 4.3405(H) | 6.6352(H) |
|  |  |  | 00682(V) | 0.2182(V) | 0.5301(V) | 1.1585(V) | 1.7265(V) |
|  | 104 | All UE | 0.1721(H) | 1.3411(H) | 7.4143(H) | 15.4966(H) | 23.8238(H) |
|  |  |  | 0.2176(V) | 0.6510(V) | 1.1742(V) | 1.7622(V) | 2.1000(V) |
|  |  | Central UE | 0.1258(H) | 1.4345(H) | 8.9295(H) | 16.0515(H) | 22.7440(H) |
|  |  |  | 0.1666(V) | 0.5005(V) | 1.0346(V) | 1.6675(V) | 2.0241(V) |
| FR2 | 105 | All UE | 0.0943 | 0.2314 | 0.4973 | 1.3007 | 2.6211 |
|  |  | Central UE | 0.0865 | 0.2101 | 0.4707 | 1.1486 | 2.1881 |
|  | 106 | All UE | 0.3364 | 0.9128 | 2.4777 | 5.7209 | 10.7382 |
|  |  | Central UE | 0.3079 | 0.8594 | 2.2392 | 5.4600 | 10.481 |
|  | 107 | All UE | 0.1297(H) | 0.4173(H) | 1.1042(H) | 2.6878(H) | 5.1519(H) |
|  |  |  | 0.0361(V) | 0.1075(V) | 0.2471(V) | 0.5573(V) | 0.9884(V) |
|  |  | Central UE | 0.1109(H) | 0.3273(H) | 0.8667(H) | 2.4365(H) | 4.4973(H) |
|  |  |  | 0.0288(V) | 0.0796(V) | 0.1879(V) | 0.4593(V) | 0.7747(V) |
|  | 108 | All UE | 0.5012(H) | 2.2343(H) | 6.6096(H) | 15.6304(H) | 24.2192(H) |
|  |  |  | 0.3436(V) | 0.9022(V) | 1.5000(V) | 1.9555(V) | 2.1766(V) |
|  |  | Central UE | 0.4304(H) | 2.4466(H) | 7.9926(H) | 15.5828(H) | 21.7159(H) |
|  |  |  | 0.2871(V) | 0.7927(V) | 1.3316(V) | 1.8800(V) | 2.1606(V) |

Observation 4: For modified InF-DH, the accuracy of less than 0.5m@90% cannot be achieved without NLOS/LOS detection.

UE/gNB calibration error

Evaluated cases

- The following cases in Table 5 are evaluated for InF-SH/In-DH with UE/gNB calibration error. In the following, the results are provided for DL-TDOA, UL-TDOA, UL+TDOA+UL-AOA and Multi-RTT in FR1.

Table 5  Evaluated parameter combinations

| Parameter | Case 201 (InF-SH, FR1) | Case 202 (InF-SH, FR1) | Case 203 (InF-SH, FR1) | Case 204 (InF-SH, FR1) | Case 205 (InF-SH, FR1) | Case 206 (InF-SH, FR1) | Case 207 (InF-SH, FR1) | Case 208 (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | In-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |  |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | gNB Rx/Tx Time error T1=1.4ns UE Rx/Tx time error T1=5.6ns |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |  |  |  |  |
| Additional notes, if any | Fixed UE/gNB height. |  |  |  |  |  |  |  |

Simulation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized inTable 6. The detailed simulation results can be found in Figure 19 and Figure 20 in the Appendix A.

Table 6 Positioning accuracy (in unit of meter)

|  | Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 201 | All UE | 0.587 | 0.874 | 1.462 | 3.540 | 8.841 |
|  |  | Central UE | 0.462 | 0.614 | 0.829 | 1.458 | 4.584 |
|  | 202 | All UE | 0.5937 | 0.8672 | 1.3135 | 2.6395 | 6.4564 |
|  |  | Central UE | 0.4653 | 0.6145 | 0.8120 | 1.2343 | 3.2444 |
|  | 203 | All UE | 0.0699 | 0.1180 | 0.2034 | 0.3878 | 0.7570 |
|  |  | Central UE | 0.0661 | 0.1094 | 0.1750 | 0.3251 | 0.6576 |
|  | 204 | All UE | 1.7167 | 2.3902 | 3.3592 | 4.4076 | 5.7510 |
|  |  | Central UE | 1.7105 | 2.3963 | 3.3574 | 4.2662 | 5.6313 |
|  | 205 | All UE | 1.719 | 4.034 | 8.588 | 15.330 | 24.388 |
|  |  | Central UE | 1.391 | 3.407 | 8.570 | 15.039 | 24.121 |
|  | 206 | All UE | 1.0065 | 2.0631 | 4.7037 | 10.459 | 15.899 |
|  |  | Central UE | 0.8741 | 1.5739 | 4.2751 | 9.4102 | 16.813 |
|  | 207 | All UE | 0.0810 | 0.184 | 0.4251 | 1.1961 | 3.0589 |
|  |  | Central UE | 0.0717 | 0.1577 | 0.3371 | 0.8662 | 2.1206 |
|  | 208 | All UE | 2.2640 | 3.2695 | 4.7279 | 8.9874 | 14.3457 |
|  |  | Central UE | 2.2784 | 3.3381 | 5.0728 | 9.5701 | 13.7623 |

Observation 5: The positioning accuracy of Rel-16 Multi-RTT is deteriorated greatly than other positioning methods with UE/gNB calibration error.

Observation 6: The positioning accuracy of less than 0.5m@90% can be achieved with UL-TDOA+UL-AOA.

UE power consumption

The evaluation results for UE power consumption are based on the evaluated cases discussed in [2], including the following five cases involving PRS measurement and SRS transmission

- PRS with no CDRX

- PRS with CDRX and PRS always in on-duration

- PRS with CDRX and PRS always outside on-duration

- SRS with no CDRX

- SRS with CDRX and SRS always in on-duration

The power model is based on TR 38.840 and presented in [2].

PRS with no CDRX

The simulation report on power consumption is summarized below, with the total power of 208507400 power units in 1000 seconds (2 million slots).

State Time TimeRatio Power PwrRatio PDCCH_PDSCH 5037 0.25185% 1511100 0.72472% PRS_RRM 75000 3.75000% 15000000 7.19399% PDCCH_Only 1919963 95.99815% 191996300 92.08129% Total Number of State: 3 Total Number of Slots: 2000000 Total Power: 208507400

The ratio of time and power for each UE power state is provided in Figure 1.

Figure 1 Ratio of time and power for each UE power state

The power state transit in time is shown in Figure 2 for 2.5 seconds duration (5000 slots).

Figure 2 UE power state transmit in 5000 slots (2.5s)

PRS with CDRX and PRS always in on-duration

The simulation report on power consumption is summarized below, with the total power of 81234601 power units in 1000 seconds (2 million slots).

ĀState Time TimeRatio Power PwrRatio LightSleep 2663 0.13315% 63460 0.07812% MicroSleep 269 0.01345% 12105 0.01490% PDCCH_Only 608347 30.41735% 60834700 74.88767% PDCCH_PDSCH 5035 0.25175% 1510500 1.85943% PRS_RRM 75000 3.75000% 15000000 18.46504% DeepSleep 1308686 65.43430% 3813836 4.69484% Total Number of State: 6 Total Number of Slots: 2000000 Total Power: 81234601

The ratio of time and power for each UE power state is provided in Figure 3.

Figure 3 Ratio of time and power for each UE power state

The power state transit in time is shown in Figure 4 for 2.5 seconds duration (5000 slots).

Figure 4 UE power state transmit in 5000 slots (2.5s)

PRS with CDRX and PRS always outside on-duration

The simulation report on power consumption is summarized below, with the total power of 90052867 power units in 1000 seconds (2 million slots).

State Time TimeRatio Power PwrRatio LightSleep 3472 0.17360% 82940 0.09210% MicroSleep 307 0.01535% 13815 0.01534% PDCCH_Only 680824 34.04120% 68082400 75.60270% PDCCH_PDSCH 5035 0.25175% 1510500 1.67735% PRS_RRM 75000 3.75000% 15000000 16.65688% DeepSleep 1235362 61.76810% 5363212 5.95563% Total Number of State: 6 Total Number of Slots: 2000000 Total Power: 90052867

The ratio of time and power for each UE power state is provided in Figure 5.

Figure 5 Ratio of time and power for each UE power state

The power state transit in time is shown in Figure 6 for 2.5 seconds duration (5000 slots).

Figure 6 UE power state transmit in 5000 slots (2.5s)

SRS with no CDRX

The simulation report on power consumption is summarized below, with the total power of 202319900 power units in 1000 seconds (2 million slots).

State Time TimeRatio Power PwrRatio PDCCH_PDSCH 5023 0.25115% 1506900 0.74481% PDCCH_SRS 6236 0.31180% 1933160 0.95550% PDCCH_PDSCH_SRS 14 0.00070% 7140 0.00353% PDCCH_Only 1988727 99.43635% 198872700 98.29616% Total Number of State: 4 Total Number of Slots: 2000000 Total Power: 202319900

The ratio of time and power for each UE power state is provided in Figure 7.

Figure 7 Ratio of time and power for each UE power state

The power state transit in time is shown in Figure 8 for 2.5 seconds duration (5000 slots).

Figure 8 UE power state transmit in 5000 slots (2.5s)

SRS with CDRX and SRS always in on-duration

The simulation report on power consumption is summarized below, with the total power of 76138397 power units in 1000 seconds (2 million slots).

State Time TimeRatio Power PwrRatio LightSleep 2740 0.13700% 65300 0.08576% MicroSleep 267 0.01335% 12015 0.01578% PDCCH_Only 688274 34.41370% 68827400 90.39775% PDCCH_PDSCH 4987 0.24935% 1496100 1.96497% PDCCH_SRS 6202 0.31010% 1922620 2.52516% PDCCH_PDSCH_SRS 48 0.00240% 24480 0.03215% DeepSleep 1297482 64.87410% 3790482 4.97841% Total Number of State: 7 Total Number of Slots: 2000000 Total Power: 76138397

The ratio of time and power for each UE power state is provided in Figure 9.

Figure 9 Ratio of time and power for each UE power state

The power state transit in time is shown in Figure 10 for 2.5 seconds duration (5000 slots).

Figure 10 UE power state transmit in 5000 slots (2.5s)

Summary of UE power consumption

|  | Total Power | Positioning power | Positioning slot power ratio |
| --- | --- | --- | --- |
| PRS w/o CDRX | 208507400 | 15000000 | 7.19399% |
| PRS w/ CDRX in on-duration | 81234601 | 15000000 | 18.46504% |
| PRS w/ CDRX outside on-duration | 90052867 | 15000000 | 16.65688% |
| SRS w/o CDRX | 202319900 | 1933160+7140 | 0.95550%+0.00353%Note 1 |
| SRS w/ CDRX in ou-duration | 76138397 | 1922620+24480 | 2.52516%+0.03215% Note 1 |
| Note 1: This includes power contribution from PDCCH and PDSCH (if any) in the SRS slot. |  |  |  |

Observation 7: PRS measurement takes 7% power consumption without C-DRX and ~18% power consumption with C-DRX.

Observation 8: SRS transmission takes 1% power consumption without C-DRX and 2.7% power consumption with C-DRX.

Conclusion

Based on the simulation results and the discussion, we have the following observations:

Observation 1:  The hybrid positioning can help to improve the positioning accuracy, e.g. UL-TDOA+UL-AOA, DL-TDOA+DL-AOD.

Observation 2:  The positioning accuracy of the center area UEs is generally higher than the edge area UEs.

Observation 3:  For InF-SH, the accuracy of less than 0.2m@90% can be achieved with DL-TDOA+DL-AOD and UL-TDOA+UL-AOA in FR2, and the accuracy of less than 0.5m@90% can be achieved with UL-TDOA+UL-AOA in FR1 and Multi-RTT in FR2.

Observation 4: For modified InF-DH, the accuracy of less than 0.5m@90% cannot be achieved without NLOS/LOS detection.

Observation 5: The positioning accuracy of Rel-16 Multi-RTT is deteriorated greatly than other positioning methods with UE/gNB calibration error.

Observation 6: The positioning accuracy of less than 0.5m@90% can be achieved with UL-TDOA+UL-AOA.

Observation 7: PRS measurement takes 7% power consumption without C-DRX and ~18% power consumption with C-DRX.

Observation 8: SRS transmission takes 1% power consumption without C-DRX and 2.7% power consumption with C-DRX.

- References

- 3GPP RP-200928, Revised SID: Study on NR Positioning Enhancements, CATT, Intel, RAN#88e, Electronic Meeting, 29th June 29 – 3rd July, 2020.

- 3GPP R1-2005251, Additional consideration on evaluation methodology, Huawei, HiSilicon, RAN1#102-e, 17th August – 28th August, 2020.

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

- Appendix A

The detailed simulation results are as follows:

Figure 11 CDF results for FR1 InF-SH

Figure 12 CDF results for FR1 InF-DH

Figure 13 CDF results for FR2 InF-SH

Figure 14 CDF results for FR2 InF-DH

Figure 15 CDF results for FR1 with modified InF-DH

Figure 16 CDF results for FR1 with modified InF-DH and variable height

Figure 17 CDF results for FR2 with modified InF-DH

Figure 18 CDF results for FR2 with modified InF-DH and variable height

Figure 19 CDF results for FR1 InF-SH with UE/gNB calibration error

Figure 20 CDF results for FR1 InF-DH with UE/gNB calibration error