3GPP TSG RAN WG1 #102		R1-2005393

e-meeting, August 17th – August 28th, 2020

Source:	vivo

Title:	Evaluation on NR coverage performance for FR1

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

- Introduction

In RAN1 101-e meeting, NR coverage evaluation assumptions are discussed, following agreements have been achieved in [1].

The agreements of previous meeting aim to align the evaluation assumptions of specific scenarios, including evaluation metric, evaluation methodology, etc. The main objectives are as follows. Agreements: - The basic evaluation methodology is based on link-level simulation for FR1. - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements. - Step 2: Obtain the baseline performance based on required SINR and link budget template. - Note: aspects related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately - FFS: The evaluation methodology based on system-level simulation is optional for FR1. - Note: The simulation assumptions for SLS are up to companies’ reports. Agreement: - Down selection on the following options for the link budget template for FR1 in next meeting. - Option 1: Adopt single link budget template based on IMT-2020 self-evaluation with necessary revisions, including adding/removing/revising some parameters. - FFS: The template provided by FL in Tdoc R1-2005005. - Option 2: Adopt both templates, i.e. link budget template in IMT-2020 self-evaluation and link budget template in TR 36.824. - Option 3: Adopt single link budget template in TR 36.824 with necessary revisions, including adding/revising some parameters. Agreement: Down selection on the following options for antenna array gain for LLS based methodology for FR1 in next meeting. - Option 1: Antenna array gain is included in the link budget template. - FFS: array gain = 10 * 1og10 (number of antenna elements/number of TxRUs) - FFS: For TDL channel model - FFS: Values reflective of realistic implementation and network operation. - Option 2: Antenna array gain is included in LLS. - FFS: For CDL channel model

To identify the bottlenecks of Rel-15/Rel-16 physical channels, we provide coverage performance of different channels in urban and rural scenario for NR FR1 in this contribution. Link level simulation is adopted and link budget template provided in [2] is reused.

- Evaluation assumptions

In previous meeting, link budget template from IMT-2020 self-evaluation and MCL defined in TR 36.824 are proposed as options for coverage evaluations. Considering evaluation methodologies have been well discussed in ITU self-evaluation, and the max available pathloss (MPL) value is a more accurate metric to reflect the coverage compared with max coupling loss (MCL), MPL defined in ITU link budget template is used as the evaluation metric in this contribution. For link level simulation, the detailed parameters are shown in Appendix Table 1.

Proposal 1: The coverage evaluation methodology based on link budget calculation and link level simulation in ITU self-evaluation can be considered as baseline in NR coverage enhancement SI.

Besides, as discussed in previous meeting, antenna array gain is considered as 10*log10(number of antenna elements/number of antenna chains), while for broadcast channels, the beamforming(BF) gain loss due to fixed and wider beam should be considered. To obtain the BF gain differences, RSRP distribution of unicast and broadcast beamforming is counted through system level simulation, and the CDF curves of the RSRP in each frequency are provided in Figure 1. The result shows that beamforming gain of broadcast beamforming is about 8dB lower than the unicast beamforming gain, thus 8dB beamforming gain loss compared to unicast BF gain is considered in the evaluation for the broadcast channels. Detailed simulation assumptions for counting RSRP using broadcast and unicast beamforming can be found in [3].

Figure 1 CDF of RSRP distribution of broadcast beamforming and unicast beamforming

Proposal 2: The difference between broadcast and unicast beamforming gain should be considered in the evaluation.

- About 8dB broadcast beamforming gain loss is observed compared to unicast beamforming gain.

For 700MHz FDD, the BF gain difference is not considered in evaluation, and sector beamforming is considered for both broadcast channels and unicast channels.

- Evaluation results

In this section, the evaluation results of MPL for different scenarios are given. Channel model A is considered as the pathloss model.

- Urban

As agreed in previous meeting, only NLOS outdoor-to-indoor is considered for urban scenario. And following carrier frequency and frame structures are considered,

- 4GHz, DDDSU

- 4GHz, DDDSUDDSUU

- 2.6GHz, DDDDDDDSUU

- 4GHz,

(a) DDDSU                                    (b) DDDSUDDSUU

Figure 2 MPL in urban scenario for 4GHz, NLOS O-to-I

- 2.6GHz, DDDDDDDSUU

Figure 3 MPL in urban scenario for 2.6GHz, NLOS O-to-I

Observation 1: In urban scenario, PUSCH eMBB is the obvious bottleneck, PRACH format B4 and PUCCH format 3 also have poor coverage among the physical channels.

- Rural

Both NLOS outdoor-to-outdoor and NLOS outdoor-to-indoor are considered for rural scenario. And following carrier frequency and frame structures are considered,

- 4GHz, DDDSU

- 4GHz, DDDSUDDSUU

- 2.6GHz, DDDDDDDSUU

- 2GHz/700MHz FDD

- 4GHz, DDDSU

Figure 4 MPL in rural scenario for 4GHz DDDSU

- 4GHz, DDDSUDDSUU

Figure 5 MPL in rural scenario for 4GHz DDDSUDDSUU

- 2.6GHz, DDDDDDDSUU

Figure 6 MPL in rural scenario for 2.6GHz DDDDDDDSUU

Observation 2: In rural scenario in TDD band, PUSCH eMBB, PRACH and PUCCH format 3 are bottleneck channels.

- 2GHz, FDD

Figure 7 MPL in rural scenario for 2GHz FDD

- 700MHz, FDD

Figure 8 MPL in rural scenario for 700MHz FDD

In FDD spectrum, due to lower carrier frequency and sufficient UL resources, the coverage of UL channels is larger than or close to the MPL corresponding to 1732m ISD. Coverage enhancement is not necessary.

- Rural with long distance

Both LOS outdoor-to-outdoor and LOS outdoor-to-indoor are considered for rural scenario with long distance. And following carrier frequency and frame structures are considered,

- 4GHz, DDDSU

- 4GHz, DDDSUDDSUU

- 700MHz FDD

- 4GHz, DDDSU

Figure 9 MPL in rural with long distance scenario for 4GHz DDDSU

- 4GHz, DDDSUDDSUU

Figure 10 MPL in rural with long distance scenario for 4GHz DDDSUDDSUU

- 700MHz, FDD

Figure 11 MPL in rural with long distance scenario for 700MHz FDD

In rural scenario with long distance coverage, although the uplink channels still have poorer coverage compared to DL channels, the MPL is large enough to meet 5km target ISD due to LOS propagation environment. No further enhancement is needed.

- Bottleneck of the physical channels

Based on the evaluation results provided in section 3, the UL channels have lower MPL compared to DL channels in general. PUSCH eMBB is obviously the bottleneck. For PUSCH voip, since PUSCH repetition and retransmission is assumed, better coverage can be achieved. However, for a high load cell, it may be too ideal to use so many repetitions, as shown in Appendix, for voip transmission. If limited number of PUSCH transmission occasions are allocated for PUSCH voip, coverage enhancement maybe still needed.

Besides, PUCCH and PRACH also have poor coverage among the physical channels. As discussed in [3], although PRACH formats with longer duration are already specified, the formats with long duration may not be deployed in some TDD frame structure. For example, PRACH format 0, which has about 1ms duration, is not feasible for DDDSU with 30kHz SCS, hence coverage enhancements for PRACH cannot be achieved by simply using a PRACH format with long duration.

Proposal 3: PUSCH, PUCCH and PRACH need to be enhanced for coverage enhancements.

- Conclusion

In this contribution, we provide our evaluation results in urban scenario Indoor hotspot scenario with the following observations and proposals:

Observation 1: In urban scenario, PUSCH eMBB is the obvious bottleneck, PRACH format B4 and PUCCH format 3 also have poor coverage among the physical channels.

Observation 2: In rural scenario in TDD band, PUSCH eMBB, PRACH and PUCCH format 3 are bottleneck channels.

Proposal 1: The coverage evaluation methodology based on link budget calculation and link level simulation in ITU self-evaluation can be considered as baseline in NR coverage enhancement SI.

Proposal 2: The difference between broadcast and unicast beamforming gain should be considered in the evaluation.

Proposal 3: PUSCH, PUCCH and PRACH need to be enhanced for coverage enhancements.

- Appendix A: Evaluation assumptions for physical channels

Table 1 evaluation assumption for FR1

| Parameter | Evaluation assumptions |
| --- | --- |
| Number of gNB antenna port in LLS | 4 for 700MHz FDD 2 for others |
| Number of UE antenna port in LLS | 1Tx/2Rx for 700MHz FDD 1Tx/4Rx for others |
| Tx Power at gNB | 53dBm for 100MHz, 46dBm for 20MHz (the same as actual network deployment) |
| Power assignment | Concentrated on occupied BW for UL Uniformed allocated to channel BW for DL |
| PDSCH (eMBB) | Around 1/3 code rate. All DL slots allocated for PDSCH for eMBB |
| PDSCH (voip) | 4 repetitions, 4 HARQ transmission times within 20ms period |
| PDSCH (MSG4) | TBs = 3000bits, around 1/3 code rate. |
| PUSCH (eMBB) | 30 PRB for 1Mbps and 4 PRB for 100Kbps, intra-slot frequency hopping is assumed, one DMRS for each hop |
| PUSCH (voip) | Repetitions and HARQ retransmissions are confined within 20ms period, inter-slot frequency hopping is assumed 2 repetitions, 4 HARQ transmission times for DDDSU and DDDDDDSUU 2 repetitions, 6 HARQ transmission times for DDDSUDDSUU 4 repetitions, 4 HARQ transmission times for FDD |
| PUCCH | Format 1, 2bit, 0.1% Nack to Ack probability Format 3, 22bit, 1% BLER No repetition |

- Appendix B: Performance gap between MPL and target ISD requirements

The target ISD of each scenario is recommended in TR 38.913, which are provided in Table 2. Target MPL can be calculated based on the target ISD and the pathloss model. To intuitively reflect the gap between the MPL of the physical channels and target MPL, the performance gaps are calculated and provided in Table 3.

Table 2 target ISD in each scenario

| scenarios | Urban | Rural | Rural long distance |
| --- | --- | --- | --- |
| target ISD | 500m | 1732m | 5000m |

Table 3 performance gap between MPL and target coverage requirement

| MCL gap(dB) | Freq/DL-UL-Config | PDSCH (eMBB) | PDSCH (voip) | PDCCH (UE) | PDSCH MSG4 | PDCCH (BC) | SSB | PUSCH (eMBB) | PUSCH (voip) | PUSCH MSG3 | PRACH format B4 | PRACH format 0 | PUCCH (PF1) | PUCCH (PF3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(500m) | 4GHz- DDDSU | -12.68 | -20.15 | -12.82 | -4.46 | -4.81 | -7.94 | 14.17 | -0.79 | 0.17 | 5.95 | 1.93 | 2.52 | 4.54 |
| Urban(350m) |  | -18.74 | -26.21 | -18.87 | -10.51 | -10.87 | -14.00 | 8.11 | -6.84 | -5.89 | -0.11 | -4.13 | -3.53 | -1.52 |
| Urban(500m) | 4GHz-DDDSUDDSUU | -12.61 | -20.15 | -12.82 | -4.46 | -4.81 | -7.94 | 12.71 | -2.50 | 0.17 | 5.95 | 1.93 | 2.52 | 4.54 |
| Urban(350m) |  | -18.66 | -26.21 | -18.87 | -10.51 | -10.87 | -14.00 | 6.65 | -8.56 | -5.89 | -0.11 | -4.13 | -3.53 | -1.52 |
| Urban(500m) | 2.6GHz-DDDDDDDSUU | -16.44 | -25.33 | -16.49 | -8.20 | -8.52 | -11.66 | 10.47 | -6.14 | -3.76 | 2.15 | -1.80 | -0.96 | 0.75 |
| Urban(350m) |  | -22.49 | -31.39 | -22.54 | -14.25 | -14.58 | -17.72 | 4.41 | -12.19 | -9.81 | -3.91 | -7.85 | -7.01 | 0.75 |
| Rural O2O | 4GHz DDDSU | -11.89 | -21.83 | -12.56 | -4.76 | -4.60 | -7.57 | 4.55 | -2.76 | -0.53 | 5.60 | 2.82 | 2.23 | 4.81 |
| Rural O2I |  | -10.12 | -18.73 | -11.09 | -2.97 | -3.08 | -6.21 | 6.41 | 0.70 | 1.66 | 7.68 | 3.66 | 4.26 | 6.27 |
| Rural O2O | 4GHz DDDSUDDSUU | -12.37 | -21.82 | -12.56 | -4.76 | -4.60 | -7.53 | 3.35 | -4.31 | -0.54 | 5.60 | 2.82 | 2.23 | 4.81 |
| Rural O2I |  | -10.64 | -18.66 | -11.09 | -2.97 | -3.08 | -6.07 | 4.96 | -1.01 | 1.66 | 7.68 | 3.66 | 4.26 | 6.26 |
| Rural O2O | 2.6GHz DDDSUDDSU | -15.73 | -25.75 | -16.28 | -8.52 | -8.31 | -11.53 | 0.47 | -6.67 | -4.40 | 2.23 | -1.48 | -1.19 | 1.02 |
| Rural O2I |  | -13.92 | -23.84 | -14.76 | -6.69 | -6.79 | -10.30 | 2.56 | -4.65 | -2.27 | 3.88 | -0.07 | 0.77 | 2.48 |
| Rural LOSO2O | 4GHz DDDSU | -29.14 | -38.73 | -32.42 | -21.27 | -24.33 | -27.07 | -15.05 | -25.73 | -19.49 | -15.90 | -18.88 | -18.34 | -15.33 |
| Rural LOS O2I |  | -25.90 | -35.13 | -28.79 | -18.18 | -20.73 | -23.66 | -12.30 | -20.87 | -16.23 | -12.17 | -17.25 | -14.51 | -11.63 |
| Rural LOS O2O | 4GHz DDDSUDDSUU | -24.13 | -34.48 | -29.44 | -24.02 | -29.45 | -32.65 | -19.73 | -26.02 | -20.91 | -11.99 | -14.00 | -21.09 | -17.75 |
| Rural LOS O2I |  | -20.35 | -29.71 | -25.81 | -20.25 | -25.76 | -28.58 | -16.12 | -20.78 | -17.33 | -9.12 | -11.29 | -17.23 | -14.27 |
| Rural O2O | 2GHz FDD | -15.30 | -25.60 | -16.54 | -7.95 | -8.66 | -12.42 | -5.35 | -12.94 | -7.46 | 0.24 | -0.88 | -4.25 | -1.76 |
| Rural O2I |  | -13.25 | -22.35 | -15.05 | -6.06 | -6.99 | -10.42 | -3.73 | -9.74 | -5.80 | 2.15 | 0.32 | -2.57 | -0.43 |
| Rural O2O | 700MHz FDD | -14.14 | -26.16 | -19.45 | -15.21 | -19.26 | -23.87 | -12.31 | -18.88 | -14.20 | -5.21 | -7.33 | -11.61 | -9.37 |
| Rural O2I |  | -11.62 | -21.06 | -17.56 | -13.43 | -17.45 | -21.62 | -10.41 | -14.86 | -12.24 | -3.26 | -5.41 | -10.15 | -7.69 |
| Rural LOS O2O | 700M FDD | -26.64 | -37.49 | -32.45 | -27.03 | -32.46 | -35.66 | -19.23 | -26.02 | -20.91 | -11.99 | -14 | -21.09 | -20.43 |
| Rural LOS O2I |  | -22.86 | -32.72 | -28.82 | -23.26 | -28.77 | -31.59 | -15.62 | -20.78 | -17.33 | -9.12 | -11.29 | -17.23 | -16.85 |

Note: Target ISD of 500m is too strict for uplink channels in urban scenario, therefore the performance gap is calculated assuming the target ISD is 350m.

The negative values in green means the coverage of the physical channel can meet the target requirement in corresponding scenario, and the positive values in red shows performance gap compared with the target available path loss. Meanwhile, the values which are close to the target threshold with limited performance margin are marked with yellow.

It can be observed that PUSCH eMBB channel may need to be enhanced in many scenarios, while PRACH and PUCCH channels also have poor coverage, e.g. in rural TDD scenario. In urban scenario, if 500m is assumed as target ISD, all uplink channels cannot meet the target, and the gap of PUSCH eMBB may be as much as 14dB, which is too large to be compensated. Therefore, target ISD of 350m seems realistic.

- References

- Chairman’s Notes RAN1#101-e final.

- Link budget template for 3GPP 5G NR RIT - Channel Model A.

- R1-2005398, “Considerations on evaluation assumptions for coverage enhancements”, vivo.