3GPP TSG RAN WG1 Meeting #102-e				           R1-2005743

e-Meeting, August 17th – August 28th, 2020

______________________________________________________________________Agenda item: 7.2.4.3

Source: LG Electronics

Title: Discussion on essential corrections in sidelink synchronization mechanism

Document for: Discussion and decision

- Introduction

In RAN WG1 #101-e meeting, the following agreements were made:

Agreements: - For indication of TDD configuration, the pattern(s) indication (X) and periodicity indication (Y) follows the two tables below: Table 1. Periodicity indication Y with single TDD pattern (X=0) Periodicity indication Y; P (ms); Single pattern. 0; 0.5; 0.5. 1; 0.625; 0.625. 2; 1; 1. 3; 1.25; 1.25. 4; 2; 2. 5; 2.5; 2.5. 6; 4; 4. 7; 5; 5. 8; 10; 10. 9-15; Reserved. Table 2. Periodicity indication Y with two TDD patterns (X=1) Periodicity indication Y; P+P2 (ms); Two patterns. P; P2. 0; 1; 0.5; 0.5. 1; 1.25; 0.625; 0.625. 2; 2; 1; 1. 3; 2.5; 0.5; 2. 4; 2.5; 1.25; 1.25. 5; 2.5; 2; 0.5. 6; 4; 1; 3. 7; 4; 2; 2. 8; 4; 3; 1. 9; 5; 1; 4. 10; 5; 2; 3. 11; 5; 2.5; 2.5. 12; 5; 3; 2. 13; 5; 4; 1. 14; 10; 5; 5. 15; 20; 10; 10. Agreements: - Confirm the working assumptions in RAN1#99 for the PSBCH contents for NR SL Rel-16, and reserve bits are 2. Agreements: - When transmitting PSBCH, UE derives the values of X/Y/Z; from RRC configuration of TDD-UL-DL-ConfigCommon and maps them to the “indication of TDD configuration” field of PSBCH. Agreements: - If at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where - X is sl-LengthSymbols - Y is sl-StartSymbol this slot can be indicated by PSBCH. - NOTE: X and Y in this proposal are different from the X/Y/Z in PSBCH. Agreements: - For indication of the UL slots by Z, - If single pattern is configured, Z bits indicate the UL slot number in the pattern is n. - If two patterns are configured, Z bits indicate the state index derived by the UL slots, n=n2w*P*2μ+1w+n1w where - n1 is the number of UL slots in the first pattern, - n2n2 is the number of UL slots in the second pattern, - PP is the periodicity in units of ms of the first pattern, - w is the granularity of resource indication, - μ is (working assumption) - 0/1/2/3 corresponds to the 15/30/60/120 kHz SCS for SL respectively. Agreements: - For indication of the granularity of UL resources, - If single pattern is configured, the granularity of the number of UL resources indicated by SL-TDD-Config is 1 slot. - If two patterns are configured, the granularity of the number of UL resources indicated by SL-TDD-Config follows the table below (as a working assumption) Periodicity indication Y; P+P2 (ms); Two patterns; Granularity in slots with different SCS. P; P2; 15kHz; 30 kHz; 60 kHz; 120 kHz. 0; 1; 0.5; 0.5; 1. 1; 1.25; 0.625; 0.625. 2; 2; 1; 1. 3; 2.5; 0.5; 2. 4; 2.5; 1.25; 1.25. 5; 2.5; 2; 0.5. 6; 4; 1; 3; 1; 2. 7; 4; 2; 2. 8; 4; 3; 1. 9; 5; 1; 4. 10; 5; 2; 3. 11; 5; 2.5; 2.5. 12; 5; 3; 2. 13; 5; 4; 1. 14; 10; 5; 5; 1; 2; 4. 15; 20; 10; 10; 1; 2; 4; 8. The latest TP (38.213) is endorsed, as in R1-2005099. ------------------------------------------------------ Start of Draft TP of 38.213-------------------------------------------- 16.1 Synchronization procedures For transmission of an S-SS/PSBCH block, a UE assumes that PSBCH payload containing a bit sequence a0, a1, a2, a3, …, a11 indicated by sl-TDD-Config provides the slot format over a number of slots: - A number of patterns indicated by TDD-UL-DL-ConfigCommon as described in Subclause 11.1 by a0 - If a pattern1 is provided, a0=0 - If a pattern1 and a pattern2 are provided, a0=1 - A period or two periods indicated by dl-UL-TransmissionPeriodicity by a1, a2, a3,a4 - j=i=14ai∙24-i denotes an index in Table 16.1-1 if a0=0 or Table 16.1-2 if a0=1 - uslotsSL, the indicator of number of slots including uplink symbols by a5, a6, a7, a8, a9, a10, a11 - if a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1, - if a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w where - L is the number of symbols in a slot: L = 12 if cyclicPrefix-SL is enabled to “ECP”, otherwise L = 14; - I1 is 1 if usym*2μ-μref mod L≥ L-Y, otherwise I1 is 0; I2 is 1 if usym,2*2μ-μref mod L≥L-Y, otherwise I2 is 0. Y is the sidelink starting symbol index represented by sl-StartSymbol; - w is the granularity of slots indication as described in Table 16.1-2, - μref, uslots, usym, uslots,2, usym,2, P msec and P2 msec are described in Subclause 11.1, - μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211]. - a5, a6, a7, a8, a9, a10, a11 equal the binary representation of uslotsSL according to i=511ai∙211-i. Table 16.1-1: Slot configuration period when one pattern is indicated Index j; Slot configuration period of pattern1 P (msec). 0; 0.5. 1; 0.625. 2; 1. 3; 1.25. 4; 2. 5; 2.5. 6; 4. 7; 5. 8; 10. 9 – 15; Reserved. Table 16.1-2: Slot configuration period and granularity when two patterns are indicated Index j; Slot configuration period of pattern1 P (msec); Slot configuration period of pattern2 P2 (msec); Granularity w in slots with different SCS. 15kHz; 30 kHz; 60 kHz; 120 kHz. 0; 0.5; 0.5; 1. 1; 0.625; 0.625. 2; 1; 1. 3; 0.5; 2. 4; 1.25; 1.25. 5; 2; 0.5. 6; 1; 3; 1; 2. 7; 2; 2. 8; 3; 1. 9; 1; 4. 10; 2; 3. 11; 2.5; 2.5. 12; 3; 2. 13; 4; 1. 14; 5; 5; 1; 2; 4. 15; 10; 10; 1; 2; 4; 8. ------------------------------------------------ < Unchanged parts are omitted > ------------------------------------ ----------------------------------------------------- End of Draft TP of 38.213 ----------------------------------------. Agreements: - The first OFDM symbol in an S-SS/PSBCH block is mapped to the first OFDM symbol in the slot. ----------------------------------------------------Start of draft TP for 38.211---------------------------------------------------- 8.4.3.1 Time-frequency structure of an S-SS/PSBCH block In the time domain, an S-SS/PSBCH block consists of NsymbS-SSBNsymbS-SSB OFDM symbols, numbered in increasing order from 0 to NsymbS-SSBNsymbS-SSB - 1 within the S-SS/PSBCH block, where S-PSS, S-SSS, and PSBCH with associated DM-RS are mapped to symbols as given by Table 8.4.3.1-1. The number of OFDM symbols in an S-SS/PSBCH block NsymbS-SSBNsymbS-SSB = 13 for normal cyclic prefix and NsymbS-SSBNsymbS-SSB= 11 for extended cyclic prefix. The first OFDM symbol in an S-SS/PSBCH block is the first OFDM symbol in the slot. --------------------------------------------------< Unchanged parts are omitted >------------------------------------------ -----------------------------------------------------End of draft TP for 38.211----------------------------------------------. Agreements: - For sidelink transmission, when gNB/eNB is used as the synchronization reference, the timing determination mechanism in LTE V2X is reused in NR V2X, i.e. DL timing is used. - Send an LS to RAN4 asking for feedback, if any - Final LS in R1-2005098 Agreements: - Adopt the following 38.213 TP 16.2.X Operation for in-device coexistence If a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access occur to a UE, when - the first channel/signal using E-UTRA radio access and the second channel/signal using NR radio access are time-division multiplexed, and - the frame indexes of the first channel/signal using E-UTRA radio access and the frame indexes of the second channel/signal using NR radio access are known to the UE, the UE transmits or receives the channel/signal so that the subframe boundary of the second channel/signal using NR radio access is aligned with the subframe boundary of the first channel/signal using E-UTRA radio access. It is up to UE implementation how to achieve subframe boundary alignment. --------------------------------------------< Unchanged parts are omitted >-------------------------------------. Agreements: - S-SSB transmission/reception slots are in cell-specific UL resources in Uu. Conclusion: - UE assumes that the same SL BWP is (pre-)configured for both RRC idle (or out of coverage) UEs and RRC connected UEs. Agreements: - Adopt the following TP (38.213, Section 16.1) ----------------------------------------------------Start of TP for 38.213------------------------------------------- 16.1 Synchronization procedures For reception of a S-SS/PSBCH block, a UE assumes a frequency location corresponding to the subcarrier with index 66 in the S-SS/PSBCH block [4, TS 38.211], is provided by absoluteFrequencySSB-SL. The UE assumes that a S-PSS symbol, a S-SSS symbol, and a PSBCH symbol have a same transmission power. The UE assumes a same numerology of the S-SS/PSBCH as for a SL BWP of the S-SS/PSBCH block reception, and that a bandwidth of the S-SS/PSBCH is within a bandwidth of the SL BWP. The UE assumes the subcarrier with index 0 in the S-SS/PSBCH block is aligned with a subcarrier with index 0 in a RB of the SL BWP. --------------------------------------------------< Unchanged parts are omitted >-------------------------------- ------------------------------------------------------End of TP for 38.213------------------------------------------. Agreements: - PSBCH-DMRS is used for S-SSB RSRP measurement. - S-SSS may be used for S-SSB RSRP measurement additionally, in which case, it is up to UE implementation to use PSBCH DMRS only or S-SSS + PSBCH DMRS. Agreements: - Adopt the following TP ----------------------------------------------------Start of draft TP for 38.215----------------------------------------- 5.1.22 PSBCH reference signal received power (PSBCH-RSRP) - Definition; - PSBCH Reference Signal Received Power (PSBCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with physical sidelink broadcast channel (PSBCH). - For PSBCH-RSRP sidelink secondary synchronization signals in addition to demodulation reference signals for PSBCH may be used. PSBCH-RSRP using sidelink secondary synchronization signals shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals. - For frequency range 1, the reference point for the PSBCH RSRP shall be the antenna connector of the UE. For frequency range 2, PSBCH-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported PSBCH-RSRP value shall not be lower than the corresponding PSBCH-RSRP of any of the individual receiver branches.. - Applicable for; - RRC_IDLE intra-frequency, - RRC_IDLE inter-frequency, - RRC_CONNECTED inter-frequency. NOTE 1: The number of resource elements within the considered measurement frequency bandwidth and within the measurement period that are used by the UE to determine PSBCH-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled. NOTE 2: The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP. NOTE 3: It is up to UE implementation to use PSBCH DMRS only or both S-SSS and PSBCH DMRS for PSBCH-RSRP. -----------------------------------------------------End of draft TP for 38.215-----------------------------------------.

This contribution addresses the essential corrections on NR sidelink synchronization mechanisms.

- Discussion

Among the agreements made so far, there are a few points that needs further clarification or are not captured properly.  This contributions addresses those remaining issues.

Regarding the SL timing that was agreed to follow the DL timing, we need to clarify the value of the SL timing advance offset for the UE’s RF switching time. The SL transmission timing is determined based on the SL timing advance NTA, SL, and the timing advance offset NTAoffset that is configured by gNB to enable UE to prepare switching from one type of RF transmission to another. The value of NTA, SL is always zero for SL transmission. Similar description is clearly specified in TS 36.211 specification, but is missing in the current TS 38.211 specification.

Proposal 1: According to the agreement made in RAN1#101-e meeting, the following TP is included in TS 38.211.

Agreements:

- For sidelink transmission, when gNB/eNB is used as the synchronization reference, the timing determination mechanism in LTE V2X is reused in NR V2X, i.e. DL timing is used.

TP for TS 38.211

## 8.5 Timing Transmission of a sidelink radio frame number from the UE shall start seconds before the start of the corresponding timing reference frame at the UE. The UE is not required to receive the downlink transmissions in TDD configuration or the sidelink transmission earlier than NTAoffset after the end of a sidelink transmission, where NTA offset is given by [7.1.2, TS 38.133]. If the UE has a serving cell fulfilling the S criterion according to [5.2.3.2, TS 38.304] - the timing of reference radio frame equals that of downlink radio frame in the cell with the same uplink carrier frequency as the sidelink and - is given by [7.1.2, TS 38.133], otherwise - the timing of reference radio frame is implicitly obtained from [TS 38.213] and - . Figure xxx: Sidelink timing relation. The quantity equals 0.

In NR Uu link, the reference SCS of tdd-UL-DL-ConfigurationCommon is configured to be equal to or smaller than any SCS of the configured BWPs for the serving cell. The reference SCS is common across all carriers and independent of the actual subcarrier spacing. The same principle is applied for SL BWP, therefore the reference SCS of tdd-UL-DL-ConfigurationCommon is configured to be equal to or smaller than the SL SCS.

In addition, for SL pre-configuration for out-of-coverage UEs, the same rule is applied. That is, the reference SCS of sl-TDD-Configuration is not greater than the SL SCS.

Proposal 2: Reference SCS for tdd-UL-DL-ConfigurationCommon or sl-TDD-Configuration is equal to or smaller than the sidelink SCS. Following TP is included in TS 38.213

TP for TS 38.213

## 16.1 Synchronization procedures <Unchanged parts omitted> - a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively - for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1 - for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w where - L is the number of symbols in a slot: L=12 if cyclicPrefix-SL = “ECP”; else, L=14 - I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0 - I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0 - Y is the sidelink starting symbol index provided by sl-StartSymbol - w is the granularity of slots indication as described in Table 16.1-2 - μref, uslots, usym, uslots,2, usym,2 are described in Clause 11.1 - μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211] and μ≥μref

In current TS 38.213 specification, UE operation with respect to the TDD configuration is missing as only tdd-UL-DL-ConfigurationCommon is considered for determining sl-TDD-Config in PSBCH content. For OOC UE, there is no connection with gNB so no information about tdd-UL-DL-ConfigurationCommon. The OOC UE should use the pre-configuration for this purpose, which is defined as sl-TDD-Configuration in TS 38.331. This point needs to be captured in TS 38.213.

Proposal 3: For transmission of an S-SS/PSBCH block, sl-TDD-Configuration in SL-PreconfigurationNR can be used for sl-TDD-Config generation. Following TP is included in TS 38.213.

TP for TS 38.213

## 16.1 Synchronization procedures <Unchanged parts omitted> For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots, where - a0=0 if pattern1 is provided by sl-TDD-Configuration or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1 - a1, a2, a3,a4 are determined based on - P in pattern1 as described in Table 16.1-1 for a0=0 - P in pattern1 and P2 in pattern2 as described in Table 16.1-2 for a0=1 where P and P2 are as described in Clause 11.1 - a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively - for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1 - for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w where - L is the number of symbols in a slot: L=12 if cyclicPrefix-SL = “ECP”; else, L=14 - I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0 - I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0 - Y is the sidelink starting symbol index provided by sl-StartSymbol - w is the granularity of slots indication as described in Table 16.1-2 - μref, uslots, usym, uslots,2, usym,2 are the parameters of TDD-UL-ConfigurationCommon as described in Clause 11.1, or the parameters of sl-TDD-Configuration as defined in [9.3, TS 38.331] - μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211] and μ≥μref

- Conclusion

This contribution discussed on NR SL synchronization mechanism. The discussions can be summarized as follows.

Proposal 1: According to the agreement made in RAN1#101-e meeting, the following TP is included in TS 38.211.

Agreements:

- For sidelink transmission, when gNB/eNB is used as the synchronization reference, the timing determination mechanism in LTE V2X is reused in NR V2X, i.e. DL timing is used.

TP for TS 38.211

## 8.5 Timing Transmission of a sidelink radio frame number from the UE shall start seconds before the start of the corresponding timing reference frame at the UE. The UE is not required to receive sidelink or downlink transmissions earlier than NTAoffset after the end of a sidelink transmission, where NTA offset is given by [7.1.2, TS 38.133]. If the UE has a serving cell fulfilling the S criterion according to [5.2.3.2, TS 38.304] - the timing of reference radio frame equals that of downlink radio frame in the cell with the same uplink carrier frequency as the sidelink and - is given by [7.1.2, TS 38.133], otherwise - the timing of reference radio frame is implicitly obtained from [TS 38.213] and - . Figure xxx: Sidelink timing relation. The quantity equals 0.

Proposal 2: Reference SCS for tdd-UL-DL-ConfigurationCommon or sl-TDD-Configuration is equal to or smaller than the sidelink SCS. Following TP is included in TS 38.213

TP for TS 38.213

## 16.1 Synchronization procedures <Unchanged parts omitted> - a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively - for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1 - for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w where - L is the number of symbols in a slot: L=12 if cyclicPrefix-SL = “ECP”; else, L=14 - I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0 - I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0 - Y is the sidelink starting symbol index provided by sl-StartSymbol - w is the granularity of slots indication as described in Table 16.1-2 - μref, uslots, usym, uslots,2, usym,2 are described in Clause 11.1 - μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211] and μ≥μref

Proposal 3: For transmission of an S-SS/PSBCH block, sl-TDD-Configuration in SL-PreconfigurationNR can be used for sl-TDD-Config generation. Following TP is included in TS 38.213.

TP for TS 38.213

## 16.1 Synchronization procedures <Unchanged parts omitted> For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots, where - a0=0 if pattern1 is provided by sl-TDD-Configuration or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1 - a1, a2, a3,a4 are determined based on - P in pattern1 as described in Table 16.1-1 for a0=0 - P in pattern1 and P2 in pattern2 as described in Table 16.1-2 for a0=1 where P and P2 are as described in Clause 11.1 - a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively - for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1 - for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w where - L is the number of symbols in a slot: L=12 if cyclicPrefix-SL = “ECP”; else, L=14 - I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0 - I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0 - Y is the sidelink starting symbol index provided by sl-StartSymbol - w is the granularity of slots indication as described in Table 16.1-2 - μref, uslots, usym, uslots,2, usym,2 are the parameters of TDD-UL-ConfigurationCommon as described in Clause 11.1, or the parameters of sl-TDD-Configuration as defined in [9.3, TS 38.331] - μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211] and μ≥μref