3GPP TSG RAN WG1 #102-e	R1-2005894

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.10.2

Source: 	Intel Corporation

Title: 	Discussion on IAB Backhaul DL Power Control

Document for:	Discussion and decision

# Introduction

Regarding SDM/FDM-based resource management, RAN1#98bis-RAN1#99 have made the following agreements. [1]-[2].

- The donor CU and the parent node can be made aware of the multiplexing capability between MT and DU (TDM required, TDM not required) of an IAB node to for any {MT CC, DU cell} pair.

  - Signaling details up to RAN2/RAN3.

- The indication of the multiplexing capability for the case of no-TDM between IAB MT and IAB DU is additionally provided with respect to each transmission-direction combination (per MT CC/DU cell pair):

  - MT-TX/DU-TX

  - MT-TX/DU-RX

  - MT-RX/DU-TX

  - MT-RX/DU-RX

Note: This agreement does not require any additional specification impact in RAN1 in Rel-16, i.e. in Rel-16 the behaviour of the IAB node is only defined for TDM cases. The behaviour for no-TDM is left to IAB-node/network implementation in Rel-16.

In this contribution, based on those discussion and agreements, we further elaborate on IAB backhaul DL power control mechanisms to support IAB MT/DU simultaneous operations.

# IAB Backhaul DL Power Control to Support MT/DU Simultaneous Operations

In Rel-16, the IAB discussion are focused on supporting TDM based DU function and MT function within an IAB node. In Rel-17 WID [3], there are duplexing enhancements to support simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT TX/DU TX, MT TX/DU RX, MT RX/DU TX, MT RX/DU RX), as shown in Figure 1.

Figure 1: IAB MT/DU simultaneous transmission four cases

If IAB MT/DU simultaneous transmission is allowed, there will be additional power control demands other than UL power control mechanism in current specification. For example, in Figure 1 case(b), an IAB MT is receiving from its parent DU and the co-located DU is receiving from its child. Since DL transmission usually has higher EIRP than UL transmission, there will be high chance that the MT received power will be much higher than the co-located DU received power, which will cause high interference. Since UL transmission power is limited by the UE/MT capability, UL power control in current specification may not fully solve this issue. Hence, DL power control (MT informs its parent DU to adjust its DL transmission power) is needed for IAB backhaul.

Similar to UL power control, there can be open-loop and closed-loop DL power control mechanisms, which can be applied for different DL channels/DL signals (e.g. PDCCH and PDSCH). In open-loop DL power control mechanism (illustrated in Figure 2), the parent DU’s DL transmission power setting algorithm will be based on some configured parameters and UL reference signal measurement for path-loss calculation. The parameter configurations can include UL reference signal power, target DL reception power, fractional path-loss compensation factor, etc.

Figure 2: Open-loop DL power control illustration

For example, a parent DU DL transmission power can be determined on active DL BWP b of carrier f of serving cell c in transmission occasion i as

PDL,b,f,ci=min⁡{PDMAX,f,ci, PDLtarget,b,f,c+αb,f,c·PLb,f,c}                              (1)

where

- PDMAX,f,ci is the parent DU’s configured maximum output power for carrier f of serving cell c in transmission occasion i;

- PDL,target,b,f,c is the DL target reception power according to parameter configurations for the active DL BWP b of carrier f of serving cell c;

- αb,f,c is the open-loop fractional path-loss compensation factor for the active DL BWP b of carrier f of serving cell c;

- PLb,f,c is the pathloss for the active DL BWP b of carrier f of serving cell c based on the associated UL RS measurement. It is calculated by UL reference signal power (according to parameter configurations) and UL RS reception measurements.

In closed-loop DL power control mechanism (illustrated in Figure 3), there will be additional transmit power control (TPC) command transmitted from MT to parent DU according to parent DU’s DL transmission measurements. The parent DU’s DL transmission power setting algorithm will be based on the configured parameters, UL reference signal measurement for path-loss calculation and TPC command.

Figure 3: Closed-loop DL power control illustration

For example, a parent DU DL transmission power can be determined on active DL BWP b of carrier f of serving cell c in transmission occasion i, with open-loop fractional path-loss compensation index j, beam pair index q and close-loop process index l as

PDL,b,f,ci,j,q,l=min⁡{PDMAX,f,ci, PDLtarget,b,f,cj+αb,f,cj·PLb,f,cq+10·log102μ·MRB,b,f,c(i)+ΔTF,b,f,ci+fTPC,b,f,c(i,l)}                                                                                     (2)

where

- PDMAX,f,ci is the parent DU’s configured maximum output power for carrier f of serving cell c in transmission occasion i;

- PDL,target,f,c(j) is the DL target reception power according to parameter configurations for the active DL BWP b, carrier f, serving cell c and open-loop fractional path-loss compensation index j;

- αb,f,cj is the open-loop fractional path-loss compensation factor for the active DL BWP b, carrier f, serving cell c with open-loop fractional path-loss compensation index j;

- PLb,f,cq is the pathloss for the active DL BWP b of carrier f of serving cell c based on the associated UL RS index q measurement;

- μ relates to the subcarrier spacing used for the DL transmission, Δf=2μ·15kHz.

- MRB,b,f,c(i) is the number of resource blocks assigned for the DL transmission for the active DL BWP b, carrier f, serving cell c in transmission occasion i.

- ΔTF,b,f,ci is related to the modulation scheme and channel-coding rate used for the DL transmission for the active DL BWP b, carrier f, serving cell c in transmission occasion i.

- gTPC,b,f,c(i,l) is the power adjustment due to the closed-loop TPC command for the active DL BWP b, carrier f, serving cell c in transmission occasion i, with closed-loop process index l.

Regarding the implementation of IAB backhaul DL power control mechanisms, parameter configurations related to IAB backhaul DL power control can be signaled from MT to parent DU through L1/L2 signaling, or notified by the CU to the parent IAB node through RRC signaling or F1AP signaling; SRS can be applied as UL reference signal for PL calculation; TPC command can be transmitted through UCI or MAC CE.

Compare with open-loop and closed-loop IAB backhaul DL power control mechanisms, open-loop power control is mostly considered to use during initial access procedure, which an IAB node does not expect simultaneously transmission/reception. Closed-loop mechanism is more suitable for IAB backhaul DL power control.

Proposal 1: To fulfil simultaneous operation of IAB-node’s child and parent links, IAB backhaul DL power control mechanisms should be supported.

Proposal 2: At least closed-loop mechanism can be considered for IAB backhaul DL power control.

# Conclusion

In this contribution, we discussed IAB backhaul DL power control mechanisms to support simultaneous operation of IAB-node’s child and parent links.  It is summarized by the following proposal.

Proposal 1: To fulfil simultaneous operation of IAB-node’s child and parent links, IAB backhaul DL power control mechanisms should be supported.

Proposal 2: At least closed-loop mechanism can be considered for IAB backhaul DL power control.

# References

[1] Chairman’s Note RAN1#98bis, Chongqing, China, Oct. 2019.

[2] Chairman’s Note RAN1#99, Reno, USA, Nov. 2019.

[3] RP-201293, “New WID on enhancements to integrated access and backhaul”, RAN#88e, July 2020.