3GPP TSG RAN WG1 #102-e			R1-20xxxxx

e-Meeting, August 17th – 28th, 2020

Source:	NTT DOCOMO, INC.

Title:	Session Notes for NR UE Features

Agenda Item:	7.2.11

Document for: 	Discussion and Decision

    - 7.2.11	NR Rel-16 UE Features (19)

R1-2005944	Summary of UE features for IAB	Moderator (AT&T)

R1-2005946	Summary of UE features for 5G V2X	Moderator (AT&T)

R1-2005947	Summary of UE features for eMIMO	Moderator (AT&T)

R1-2005948	Summary of UE features for UE power savings	Moderator (AT&T)

R1-2005949	Summary of UE features for NR mobility enhancements	Moderator (AT&T)

R1-2006708	Summary on UE features for two-step RACH	Moderator (NTT DOCOMO, INC.)

R1-2006709	Summary on UE features for NR-U	Moderator (NTT DOCOMO, INC.)

R1-2006710	Summary on UE features for URLLC/IIoT	Moderator (NTT DOCOMO, INC.)

R1-2006711	Summary on UE features for NR positioning	Moderator (NTT DOCOMO, INC.)

R1-2006712	Summary on UE features for MR-DC/CA	Moderator (NTT DOCOMO, INC.)

R1-2006713	Summary on UE features for CLI/RIM	Moderator (NTT DOCOMO, INC.)

R1-2006714	Summary on UE features for TEIs	Moderator (NTT DOCOMO, INC.)

R1-2006715	Summary on NR UE features for others	Moderator (NTT DOCOMO, INC.)

[102-e-NR-UEFeature-LS] Email discussion/approval on updated UE features list and LS for NR (8/19-8/21)

    - 2-step (1)

[102-e-NR-UEFeatures-2Step-01] Email discussion/approval on UE features for two-step RACH (17th – 19th August), Hiroki (DCM)

- Whether the note in description field of FG9-1 is removed or not

- Whether FG9-6 is removed or not

Agreements:

- Remove the note in description field of FG9-1.

Agreements:

- Remove FG[9-6].

    - NR-U (2)

[102-e-NR-UEFeatures-NRU-01] Email discussion/approval on UE features for NR-U (17th – 20th August), Hiroki (DCM)

- Whether the FG10-2f is removed for all deployment scenarios or retained for at least specific scenario(s)

- Whether the term “for NR-U” is replaced by “for operation with shared spectrum channel access” or not for FG10-2i/26/26a/27

- Whether each of FGs10-9/9b/9c/9d/15/16/20a is applicable to licensed bands or not (i.e., the note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added)

  - Whether the note is added for 10-1/1a/2/2a/2b/2c/2d/2f/2g/2h/2i, 10-19a/b/c/d/e/f, 10-23, 10-25, 10-27, 10-29, 10-30, 10-26/26a, 10-3, 10-3a, 10-12, 10-13a, 10-18, 10-21a/21b, 10-24, 10-31

Agreements:

- The term “for NR-U” in FG10-2i/26/26a/27 is replaced by “for operation with shared spectrum channel access”.

Agreements:

- The note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added for following FGs

  - 10-1/1a/2/2a/2b/2c/2d/2f/2g/2h/2i, 10-19a/b/c/d/e/f, 10-23, 10-25, 10-27, 10-29, 10-30, 10-26/26a, 10-3, 10-3a, 10-12, 10-13a, 10-18, 10-21a/21b, 10-24.

Agreements:

- The note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added for FG10-2e and 10-7.

[102-e-NR-UEFeatures-NRU-02] Email discussion/approval on basic feature groups for NR-U deployment scenarios (after the completion of the NRU-01 email thread) till 8/26 – Hiroki (DCM)

- How to define basic FG(s) for each of particular NR-U deployment scenarios based on completed FGs

Updated FL proposal 1:

Alt.1:

- The FG10-9/9b/9c/9d/15/16/20a are only applicable to unlicensed bands, and the note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added for the FGs.

Alt.2:

- The FG10-9/9b/9c/9d/20a are only applicable to unlicensed bands, and the note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added for the FGs.

- The FG10-15/16 are also applicable to licensed bands.

Alt.3:

- The FG10-9/9b/9c/9d/15/16/20a are also applicable to licensed bands.

FL proposal 2

- Decide classification of NR-U deployment scenarios for the purpose of defining basic FGs first

- SCell (DL-Only) in band for shared spectrum channel access (maps to Scenario A)

- SCell (DL + UL) in band for shared spectrum channel access (maps to Scenario A)

  - For dynamic channel access mode

  - For semi-static channel access mode

- PCell (DL + UL) in band for shared spectrum channel access (maps to Scenario C)

  - For dynamic channel access mode

  - For semi-static channel access mode

- PCell (DL + UL) in band for shared spectrum channel access + SUL in licensed band (maps to Scenario D)

  - For dynamic channel access mode

  - For semi-static channel access mode

- PSCell (DL + UL) in band for shared spectrum channel access (maps to Scenarios B,E)

  - For dynamic channel access mode

  - For semi-static channel access mode

FL proposal 3

- Decide basic FGs for each of NR-U deployment scenarios

|  | FG | Components | Scenario where the FG is required as part of basic FGs (index is based on FL proposal 2) |
| --- | --- | --- | --- |
| 10-1 | UL channel access for dynamic channel access mode | 1. Type 1 channel access and contention window size adjustment 2. Type 2A channel access 3. Type 2B channel access 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol for PUSCH/PUCCH transmission | 2a, 3a, 4a, 5a |
| 10-1a | UL channel access for semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth 4. CP extension up to 1 symbol for PUSCH/PUCCH transmission | 2b, 3b, 4b,5b |
| 10-2 | SSB-based RRM for dynamic channel access mode | 1. SSB-based RRM with Q for dynamic channel access mode | 1, 2a, 3a, 4a, 5a |
| 10-2a | SSB-based RRM for semi-static channel access mode | 1. SSB-based RRM with Q for semi-static channel access mode, when SMTC window is no longer than the fixed frame period | 1, 2b, 3b, 4b,5b |
| 10-2b | MIB reading on unlicensed cell | 1. MIB reading on unlicensed cell for PCell and PSCell | 3a, 3b, 4a, 4b, 5a, 5b |
| 10-2c | SSB-based RLM for dynamic channel access mode | 1. SSB-based RLM with Q for dynamic channel access mode | 3a, 4a, 5a |
| 10-2d | SSB-based RLM for semi-static channel access mode | 1. SSB-based RLM with Q for semi-static channel access mode, when DRS window is no longer than the fixed frame period | 3b, 4b,5b |
| 10-2e | SIB1 reception on unlicensed cell | 1. SIB1 reception on unlicensed cell for PCell | 3a, 3b, 4a, 4b |
| 10-2f | Support monitoring of extended RAR window | 1. Support of RAR extension from 10ms to 40ms by decoding of the 2-bit SFN indication in DCI 1_0 | 3a, 3b, 4a, 4b, 5a, 5b |
| 10-3 | PRB interlace mapping for PUSCH | 1. PRB interlace frequency domain resource allocation for PUSCH | [3a, 3b] |
| 10-3a | PRB interlace mapping for PUCCH | - PRB interlace frequency domain resource allocation for PUCCH format 0 and format 1 - PRB interlace frequency domain resource allocation for PUCCH format 2 PRB interlace frequency domain resource allocation for PUCCH format 3 | [3a, 3b] |
| 10-27 | Wideband PRACH | - Enhanced PRACH design for NR-U by adopting a single long ZC sequence, with ZC sequence = 1151 for 15kHz and ZC sequence = 571 for 30kHz | [3a, 3b] |
| 10-29 | Support available RB set indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read availableRB-Sets-r16 |  |
| 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read COT duration | [1, 2a, 2b, 3a, 3b, 4a, 4b, 5a, 5b] |
| 10-31 | Support of P/SP-CSI-RS reception with CSI-RS-ValidationWith-DCI-r16 configured | 1. Validate P/SP-CSI-RS reception when receiving a DCI granting a PDSCH over the same set of symbols 2. Validate P/SP-CSI-RS reception when receiving a DCI triggering a A-CSI-RS over the same set of symbols | [1, 2a, 2b, 3a, 3b, 4a, 4b, 5a, 5b] |

IAB (1)

[102-e-NR-UEFeatures-IAB-01] Email discussion/approval for remaining issues on UE features for IAB, till 8/20 – Ralf (AT&T)

- whether FG 20-2 and 20-3 are basic IAB feature groups

V2X (2)

[102-e-NR-UEFeatures-V2X-01] Email discussion/approval for remaining issues on UE features for NR V2X, till 8/20 – Ralf (AT&T)

- Resolve FFS in FG 15-2, components (9), Notes column

- Resolve FFS in FG 15-2 whether to mandate an SCS

- Resolve FFS in FG 15-6, components (2) incl. consequence if the feature is not supported by the UE

- Whether to add “in the same band per band combination” to “UE supports LTE V2X sidelink” as prerequisite for FG 15-6

- Resolve FFS in FG 15-11 for “Need for the eNB to know if the feature is supported”

- Resolve FFS in FG 15-11 for “Applicable to the capability signalling exchange between UEs”

- Resolve FFS in FG 15-18 for “Applicable to the capability signalling exchange between UEs”

[102-e-NR-UEFeatures-V2X-02] Email discussion/approval for basic UE feature groups for NR V2X, till 8/26 – Ralf (AT&T)

- resolve any open issue in the last two columns of a FG pertaining to, if applicable, whether such a FG is a basic NR V2X feature (e.g., “FFS: This is the basic FG for NR sidelink”).

    - URLLC/IIoT (2)

[102-e-NR-UEFeatures-URLLC/IIoT-01] Email discussion/approval on UE features for URLLC/IIoT (17th – 20th August) – Hiroki (DCM)

- Whether/how to define a new FG for “TB CRC for cancelled initial PUSCH with CBG based re-transmission” based on outcome of discussion in maintenance email discussion

- Whether/how to define FG11-3c/d/e/f/g and 11-4c/d/e/f/g/h/i

- Whether the component 3 of FG11-3 is kept, removed or replaced by another component

- Whether the component 4 of FG11-4/4a and the component 1 of FG12-1 are kept, removed or replaced by other component(s)

- Whether the component 6 of FG11-4/4a is kept, removed or modified, and what are candidate values for the component 6

Agreements:

- Confirm working assumption on FG11-3c/d/e/f/g and FG11-4c/d/e/f/g/h/i and keep FGs

  - Add a note: For ECP, “7 symbols” is replaced by “6 symbols” in above FGs

  - Whether “at most” is kept or removed can be discussed in RAN1

Agreements:

- “at most once per subslot for HARQ-ACK” is changed to “once per subslot for HARQ-ACK”

- “at most once per subslot for SR” is changed to “once per subslot for SR”

Proposal:

- Discuss “once per subslot per codebook for SR” part

Agreements:

- For FG11-3c/d/e/f/g and FG11-4c/d/e/f/g/h/i

  - Reporting type is per FS

  - Remove FFS note from FG11-4e

Agreements:

- Component 3 is removed from FG11-3

Agreements:

- Component 6 (Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot) is kept for FG11-4/4a

  - Candidate values for the component 6 of FG11-4 is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration.

  - Candidate values for the component 6 of FG11-4a is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration.

  - Add a note to FG 11-4:

    - Component 6 is applied to the sub-slot HARQ-ACK codebook. It is assumed that only 1 actual PUCCH transmission for HARQ-ACK within a slot for slot-based HARQ-ACK codebook.

    - Component 6 is reported for 2-symbol duration sub-slot configuration. For 6 or 7-symbol duration sub-slot configuration, the value of component 6 is {2} for both NCP and ECP cases.

    - For component 6, maximum of 1 actual PUCCH transmission for HARQ-ACK within a slot for a slot-based HARQ-ACK codebook. Thus, value reported for component 6 does not apply for “slot-based + slot based”.

  - Add a note to FG 11-4a:

    - Component 6 is applied to each of the two sub-slot HARQ-ACK codebooks separately.

    - Component 6 is reported for 2-symbol duration sub-slot configuration. For 6 or 7-symbol duration sub-slot configuration, the value of component 6 is {2} for both NCP and ECP cases.

Agreements:

- Component 4 of FG11-4 is kept.

Agreements:

- Component 4 of FG11-4a and component 1 of FG12-1 are kept.

[102-e-NR-UEFeatures-URLLC/IIoT-02] Email discussion/approval on UE features for URLLC/IIoT (17th – 20th August) – Hiroki (DCM)

- Whether to add licensed/unlicensed differentiation for FG11-6 or not

- Whether to add components for the restriction on the number of monitoring occasions per slot/half-slot to FG11-2 or not

- Whether/how to add new FGs for the reference cell number for DC PDCCH BD/CCE limit

- Whether/how to add new FG for independent cancellation of the overlapping channels in an intra-band UL CA

Agreements:

- The licensed/unlicensed differentiation is introduced for FG11-6.

  - Copy the note in FG19-2 “Note: RAN1 agreed it should be possible to separately indicate support of this FG based on whether the UE is operated with or without shared spectrum access. It is left to RAN2 how to implement this while leaving the type as “per UE”

FL proposal 2:

- The following components for the restriction on the number of monitoring occasions per slot/half-slot are added to FG11-2.

  - The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7.

  - The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell.

Agreements:

- Agree in principle to introduce FG11-2d and 11-2e to URLLC UE features list as below

  - FFS details

| 11. NR_L1enh_URLLC | 11-2d | Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span for MCG and for SCG when configured with NR-NR DC with Rel-16 PDCCH monitoring capability on all the serving cells | - Supported combination of (pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is 1 to pdcch-BlindDetectionCA-r16-1 - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is 1 to pdcch-BlindDetectionCA-r16-1 | 11-2 | Yes | N/A |  | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-2e | Number of carriers for CCE/BD scaling for MCG and for SCG when configured with NR-NR DC with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers | - Supported combination(s) of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) - Candidate values for pdcch-BlindDetectionMCG-UE-r15 is 0 to pdcch-BlindDetectionCA-r15 - Candidate values for pdcch-BlindDetectionSCG-UE-r15 is 0 to pdcch-BlindDetectionCA-r15 - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is 0 to pdcch-BlindDetectionCA-r16 - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is 0 to pdcch-BlindDetectionCA-r16 | 11-2b | Yes | N/A |  | Per BC | N/A | N/A | N/A | One combination of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) corresponds to one combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16) | Optional with capability signalling |

Agreements:

- For FG11-2d and 11-2e,

  - In FG name, “configured with NR-NR DC” is changed to “configured for [synchronous] NR-DC operation”

  - The component description of FG11-2d is updated as below and moved to Note:

    - If the UE reports pdcch-BlindDetectionCA-r16,

      - “pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= pdcch-BlindDetectionCA-r16” is added to component 1 description

      - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is 1 to pdcch-BlindDetectionCA-r16-1

      - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is 1 to pdcch-BlindDetectionCA-r16-1

    - Otherwise, if NNR-DC,  max, r16DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r16monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

      - the value of pdcch-BlindDetectionMCG-UE-r16 or of pdcch-BlindDetectionSCG-UE-r16 is 1,

      - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= NNR-DC,  max, r16DL,cells.

  - The component description of FG11-2e is updated as below and moved to Note:

  - If the UE reports pdcch-BlindDetectionCA-r15,

    - Candidate values for pdcch-BlindDetectionMCG-UE-r15 is 0 to pdcch-BlindDetectionCA-r15

    - Candidate values for pdcch-BlindDetectionSCG-UE-r15 is 0 to pdcch-BlindDetectionCA-r15

    - pdcch-BlindDetectionMCG-UE-r15 + pdcch-BlindDetectionSCG-UE-r15>= pdcch-BlindDetectionCA-r15

  - Otherwise, if NNR-DC,  max, r15DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r15monitoringcapability

    - Candidate values for pdcch-BlindDetectionMCG-UE-r15 is [0, 1, 2]

    - Candidate values for pdcch-BlindDetectionSCG-UE-r15 is [0, 1, 2]

    - pdcch-BlindDetectionMCG-UE-r15 + pdcch-BlindDetectionSCG-UE-r15 >= NNR-DC,  max, r15DL,cells

  - If the UE reports pdcch-BlindDetectionCA-r16,

    - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is 0 to pdcch-BlindDetectionCA-r16

    - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is 0 to pdcch-BlindDetectionCA-r16

    - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16>= pdcch-BlindDetectionCA-r16

  - Otherwise, if NNR-DC,  max, r16DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r16monitoringcapability

    - Candidate values for pdcch-BlindDetectionMCG-UE-r16 is [0, 1]

    - Candidate values for pdcch-BlindDetectionSCG-UE-r16 is [0, 1]

    - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= NNR-DC,  max, r16DL,cells

FL proposal 4:

- A new FG for independent cancellation of the overlapping channels in an intra-band UL CA is introduced as below.

| 12. NR_IIOT | 12-1b | Independent cancellation of the overlapping channels in an intra-band UL CA | - For a UE indicating the capability of pa-PhaseDiscontinuityImpacts, and if the PUCCH or PUSCH on at least one serving cell is cancelled due to the overlapping with a high priority PUCCH or PUSCH transmission, the UE may cancel the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell. | 6-23, one of {11-4, 12-1} | Yes | N/A |  | Per band | N/A | N/A | N/A | If UE indicates 6-23 but does not support this FG, UE is not expected to be scheduled simultaneous PUCCH/PUSCHs on multiple carriers but receiving cancellation only for subset of carriers in intra-band carriers. This FG is for operation with two PUCCH groups in the same band. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MIMO (1)

[102-e-NR-UEFeatures-eMIMO-01] Email discussion/approval for remaining issues on UE features for NR MIMO enhancements, till 8/20 – Ralf (AT&T)

- Finalize open issues of FG 16-1a-1: component descriptions, component candidate values, default values if any, how CSI-RS is counted when it is configured as CMR without dedicated IMR, whether/how capability of SP L1-SINR reporting is supported

- Finalize open issues of FG 16-1g: component descriptions, component candidate values, type

- Finalize open issues of FG 16-2a: component descriptions, component candidate values, type, prerequisites, FFS in component (5)

- Finalize open issues of FG 16-2a-0: type

- Finalize open issues of FG 16-2a-1: type

- Finalize open issues of FG 16-2a-2: type

- Finalize open issues of FG 16-2a-3: type and note per Section  3.1 in R1-2005947

- Finalize open issues of FG 16-2a-10: prerequisites per Section  3.2 in R1-2005947

- Finalize open issues of FG 16-2c: component descriptions

- Finalize open issues of FG 16-2b-1: type

- Finalize open issues of FG 16-2b-1b: prerequisites per Section  3.3 in R1-2005947

- Finalize open issues of FG 16-2b-2: type

- Finalize open issues of FG 16-2b-3: type

- Finalize open issues of FG 16-2b-3a: type

- Finalize open issues of FG 16-2b-4: type

- Finalize open issues of FG 16-2b-5: component descriptions, type

- Finalize open issues of FG 16-3a: component descriptions, component candidate values

- Finalize open issues of FG 16-3b: component descriptions, component candidate values

- Finalize open issues of FG 16-5b: component descriptions

- Finalize open issues of FG 16-5c-2: component descriptions

- Finalize open issues of FG 16-5c-3: component descriptions

- Finalize open issues of FG 16-8: prerequisites and the following FFS

  - whether introduce codebook 3, where codebook 3 is down selected from {Type II, Type II PS, eType II R=1, eType II R=2, eType II PS R=1, eType II PS R=2, NULL}

  - the max number of combinations can be signaled in component 1

  - the minimum requirement for component 2

- Maximum number of unicast PDSCHs for FDMSchemeB / TDMSchemeA per Section  3.5 in R1-2005947

PowerSaving (1)

[102-e-NR-UEFeatures-PowSav-01] Email discussion/approval for remaining issues on UE features for UE Power Savings, till 8/20 – Ralf (AT&T)

- clarify the interpretation of the FR1/FR2 differentiation bit for FG 19-1

- add, if any, a new component "Support of extended value range for aperiodic CSI-RS triggering offset" to FG 19-2

    - Positioning (1)

[102-e-NR-UEFeatures-Positioning-01] Email discussion/approval on UE features for NR positioning (17th – 20th August) – Hiroki (DCM)

- Whether to add a component of additional path report to FG13-6 and 13-11 or not

- Whether to replace FG13-12/12a with four FGs as proposed in R1-2005814 or not

- Whether/how to modify or merge FG13-15 and FG13-15a

- Whether/how to define new FG for simultaneous positioning SRS and MIMO SRS transmission for intra-band CA and inter-band CA

- Whether to add a note “Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination” in FG13-1/2a/2b/3a/3b/4a/4b or not

- Whether to add a note “Note: if the UE does not indicate this capability for a band, the UE does not support any pathloss estimates in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions in that band” in FG13-9e or not

- Whether FG13-11a is changed to per BC or not

- Whether to move component 1 of FG13-2/3/4 inside FG13-2a/3a/4a respectively or not

- Whether to add a note “Note: if the UE does not indicate this capability for a band in a band combination, the UE does not support SRS for Positioning in this band in the band combination” in FG13-8 or not

Agreements:

- Replace the current FG13-12 and FG13-12a with the following new FGs 13-12, 13-12a, 13-12b, and 13-12c

| 13. NR Positioning | 13-12 | SS-RSRP RRM measurements for NR E-CID Positioning | - Support of cell-specific SS-RSRP RRM measurements with LPP report for NR E-CID Positioning - Support of beam-specific SS-RSRP RRM measurements with LPP report for NR E-CID Positioning | 1-1 | No | N/A |  | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR Positioning | 13-12a | SS-RSRQ RRM measurements for NR E-CID Positioning | - Support of cell-specific SS-RSRQ RRM measurements with LPP report for NR E-CID Positioning - Support of beam-specific SS-RSRQ RRM measurements with LPP report for NR E-CID Positioning | 1-1 | No | N/A |  | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-12b | CSI-RSRP RRM measurements for NR E-CID Positioning | - Support of cell-specific CSI-RSRP RRM measurements with LPP report for NR E-CID Positioning - Support of beam-specific CSI-RSRP RRM measurements with LPP report for NR E-CID Positioning | 1-4 | No | N/A |  | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-12c | CSI-RSRQ RRM measurements for NR E-CID Positioning | - Support of cell-specific CSI-RSRQ RRM measurements with LPP report for NR E-CID Positioning - Support of beam-specific CSI-RSRQ RRM measurements with LPP report for NR E-CID Positioning | 1-4 | No | N/A |  | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |

Agreements:

- Add the following Note in FG 13-1, 13-2a, 13-2b, 13-3a, 13-3b, 13-4a, 13-4b:

  - Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.

Agreements:

- Add the following Note in FG 13-9e:

  - Note: if the UE does not indicate this capability for a band, the UE does not support any pathloss estimates in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions in that band.

Agreements:

- Merge the existing FG13-15 and FG13-15a, with the note as below

- A new UE capability of simultaneous SRS transmission across multiple CCs within a band is introduced. With the candidate value {2}.

  - The note on omission of capability reporting is added

| 13. NR Positioning | 13-15 | Simultaneous SRS transmission within a band across multiple CCs | - The number of SRS resources for positioning on a symbol within a band Candidate values {2} Note: if the UE does not indicate this capability for a band, the UE does not support the feature in this band | 13-8 | Yes | N/A |  | Per band | N/A | N/A | N/A | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR Positioning | 13-15a | Simultaneous SRS transmission for a given BC | - The number of SRS resources for positioning on a symbol for a given BC Candidate values {2} Note: For single-band BCs, it defines the capability for intra-band CA, and for BCs with at least two bands, it defines the capability for inter-band CA. Note: if the UE does not indicate this capability for a band combination, the UE does not support the feature in this band combination | 13-8 | Yes | N/A |  | Per BC | N/A | N/A | N/A | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |

Agreements:

- A new UE capability of simultaneous positioning SRS and MIMO SRS transmission across multiple CCs within a band is introduced. With the candidate value {2}.

- A new UE capability of simultaneous positioning SRS and MIMO SRS transmission for a given BC is introduced. With the candidate value {2}.

| 13. NR Positioning | 13-19 | Simultaneous positioning SRS and MIMO SRS transmission within a band across multiple CCs | - The number of SRS resources for positioning and SRS resource for MIMO on a symbol within a band Candidate values {2} Note: SRS resource for MIMO refers to SRS resource configured by SRS-Resource. Note: If UE reports 2 for the candidate value, it means both the number of SRS resource for positioning and SRS resource for MIMO equals to 1. Note: if the UE does not indicate this capability for a band, the UE does not support the feature in this band | 13-8 | Yes | N/A |  | Per band | N/A | N/A | N/A | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR Positioning | 13-19a | Simultaneous positioning SRS and MIMO SRS transmission for a given BC | - The number of SRS resources for positioning and SRS resource for MIMO on a symbol for a given BC Candidate values {2} Note: SRS resource for MIMO refers to SRS resource configured by SRS-Resource. Note: If UE reports 2 for the candidate value, it means both the number of SRS resource for positioning and SRS resource for MIMO equals to 1. Note: For single-band BCs, it defines the capability for intra-band CA, and for BCs with at least two bands, it defines the capability for inter-band CA. Note: if the UE does not indicate this capability for a band combination, the UE does not support the feature in this band combination | 13-8 | Yes | N/A |  | Per BC | N/A | N/A | N/A | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signaling |

Agreements:

- Add the following Note in FG 13-8:

  - Note: if the UE does not indicate this capability for a band in a band combination, the UE does not support SRS for Positioning in this band in the band combination.

    - UE not supporting FG13-8 does not support FG13-8a or FG13-8b in the band in the band combination.

    - The same approach is applicable to FG13-8c, FG13-8d, and FG13-8e.

Mobility Enhancements (2)

[102-e-NR-UEFeatures-MobEnh-01] Email discussion/approval for remaining issues on UE features for NR mobility enhancements, till 8/20 – Ralf (AT&T)

- whether to update the consequence if FG 21-1a and/or 21-1b is not supported by the UE

[102-e-NR-UEFeatures-MobEnh-02] Email discussion/approval for basic UE feature groups for NR mobility enhancements, till 8/26 – Ralf (AT&T)

- FG 21-1a is a basic FG for NR mobility enhancements

- FG 21-1b is optional with capability signaling

    - MR-DC/CA (1)

[102-e-NR-UEFeatures-MRDCCA-01] Email discussion/approval on UE features for MR-DC/CA (17th – 20th August) – Hiroki (DCM)

- Whether and how to define FG 18-4b.

- Whether and how to define FG 18-5c/5d, including values of X, reporting type, applicable PDCCH monitoring capability and corresponding potential further separate capability

- Whether to add “different only” as candidate value for FG18-5a/6a

- Whether/how to add candidate values for FG18-7

- Whether to modify “DL DCI” to “unicast DL DCI” in FG18-9 FG name

- Whether the note “This FG is for synchronous EN-DC” is kept or removed for FG18-2a/2b/3/3a/3b

- Whether/how to modify the prerequisite FG of FG18-3a/3b

Agreements:

- FG[18-4b] is removed

  - Note: with assumption that relaxation of N value for SCell dormancy indication HARQ-ACK minimum processing timeline can be discussed in maintenance email discussion for SCell dormancy.

Agreements:

- For FG18-5c/5d,

  - It is optional with capability signaling

  - At least X=4 for (15,120), (15,60), (30,120) and X=2 for (15,30), (30,60), (60,120 kHz) are supported

    - With smaller value(s) of X as candidate values for reporting

    - FFS: coupling/decoupling FGs for DL/UL scheduling

    - FFS: FDD/TDD differentiation on the value X

  - FFS: Reporting type is per BC

  - The note “This FG is applicable to the basic PDCCH monitoring capability 3-1” is added

Agreements:

- For FG18-5c/5d,

  - X={1,2,4} for (15,120), (15,60), (30,120) and X={2} for (15,30), (30,60), (60,120 kHz)

  - Keep decoupling FGs for DL/UL scheduling

  - The note is updated to “This FG is only applicable to the basic PDCCH monitoring capability 3-1”

  - Reporting type is per FS

  - Prerequisite FG is FG18-5 and FG18-5b for FG18-5c and FG18-5d, respectively

Agreements:

- For FG18-5a/6a,

  - candidate values are {different only, both}.

Agreements:

- Include the following sentence into the LS to be sent to RAN2/4 with UE features list:

RAN1 understands that RAN2/RAN4 may discuss potential issues that may be specific to the following case related to FG18-7:

“PCell/PSCell lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfig > SCell lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfig”

RAN1 therefore asks RAN2/RAN4 to consider whether to add the following candidate values to FG18-7:

  - PCell/PSCell lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfig <= SCell lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfig

  - No restriction for SCS between PCell/PSCell and SCell

Agreements:

- Modify “DL DCI” to “unicast DL DCI” in FG18-9

Updated FL proposal 6:

- The note “[This FG is for synchronous EN-DC]” is removed from FG18-2a/3/3a

Agreements:

- The note “This FG is for synchronous EN-DC” is kept for FG18-2b/3b

Agreements:

- Change the prerequisite FG of FG 18-3a to “one of {18-2a, 18-3}” and change the prerequisite FG of 18-3b to “18-2”.

    - CLI/RIM (1)

[102-e-NR-UEFeatures-CLI/RIM-01] Email discussion/approval on UE features for CLI/RIM (17th – 19th August), Hiroki (DCM)

- Whether the slot in FG17-2 component 3 is a slot using the SCS of UE’s active DL BWP or a slot in SCS = 15kHz.

Agreements:

- Clarify that component 2 is the maximum number of measurement resources across all CCs configured for SRS-RSRP measurement

- Clarify that component 3 is the maximum number of measurement resources across all CCs configured for SRS-RSRP measurement within a slot

  - RAN1 continues discussion on the definition of "within a slot" and counting rule

    - TEIs (2)

[102-e-NR-UEFeatures-TEI-01] Email discussion/approval on UE features for NR TEI (17th – 20th August) – Hiroki (DCM)

- Whether to change type of FG14-2 to “per UE” and to apply the DSS bands the UE support or not

Agreements:

- The reporting type of FG14-2 is kept as “per band”

[102-e-NR-UEFeatures-TEI-02] Email discussion/approval on mandatory/optional for NR TEI (after the completion of TEI-01) by 8/26 – Hiroki (DCM)

- Whether the FG14-8 is mandatory with capability signaling or optional with capability signaling, including possibility of mandatory for UEs supporting with 4 or more carriers

    - Others (1)

[102-e-NR-UEFeatures-Others-01] Email discussion/approval on new FGs that are not dedicated to a specific Rel-16 work item/TEI (17th – 20th August), Hiroki (DCM)

- Whether/how to define new FGs related to PUCCH group based on proposals in R1-2006482 and potentially following points

  - A UE should not be mandated to support the case where cells from different NR PUCCH groups are in the same TAG, but there is no such capability signalling in Rel.15.

  - A UE should not be mandated to support the case where cells from two NR PUCCH groups are in the same band, but there is no such capability signalling in Rel.15

- Whether/how to define new FG for supporting offset between the end of PDCCH triggering A-SRS and the SRS transmission for CB PUSCH and antenna switching for UEs supporting PDCCH capabilities besides FG 3-1

- Whether/how to define new FG for supporting partial cancellation of configured PUCCH/PUSCH/PRACH due to dynamic SFI, dynamically granted PDSCH and CSI-RS, and UE behavior for UE not supporting the FG

FL proposal 1:

- A new FG to indicate whether UE supports 3 different numerologies in the same PUCCH group and the restrictions on PUCCH configuration is introduced.

| 22. NR Others | 22-5a | Support of three different numerologies in the same PUCCH group for EN-DC, NGEN-DC, NE-DC, NR-DC and NR-CA | For EN-DC, NGEN-DC, NE-DC, NR-DC and NR-CA, support three different numerologies in the same PUCCH group - Which FR can be configured to transmit NR PUCCH |  | Yes | N/A |  | Per BC | N/A | N/A | N/A | Candidate value for component 1, {FR1, FR1 and FR2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- A new FG to indicate whether UE supports 4 different numerologies in the same PUCCH group and the restrictions on PUCCH configuration is introduced.

| 22. NR Others | 22-5b | Support of four different numerologies in the same PUCCH group for EN-DC, NGEN-DC, NE-DC, NR-DC and NR-CA | For EN-DC, NGEN-DC, NE-DC, NR-DC and NR-CA, support four different numerologies in the same PUCCH group - Which FR can be configured to transmit NR PUCCH |  | Yes | N/A |  | Per BC | N/A | N/A | N/A | Candidate value for component 1, {FR1, FR1 and FR2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- A new FG to indicate UE’s capability regarding supported PUCCH grouping configuration is introduced.

  - FFS: details discussed in RAN1 or just ask RAN2 to design the signaling

- A new FG to indicate whether UE supports FR1 + (FR1 + FR2) PUCCH group configuration is introduced.

- Introduce capability signalling for PUCCH-grouping for NR-CA with two PUCCH groups

  - For a given band combination, the UE reports how the bands in the band combination can be grouped into the two PUCCH groups subject to the capability of twoPUCCH-Group

    - Cell-grouping capability for NR-DC would be reusable, but details can be up to RAN2

  - If the UE does not report the capability signalling for PUCCH-grouping for NR-CA with two PUCCH groups for a given band combination,

    - If the UE reports the capability of twoPUCCH-Group for the band combination, the capability is same as in Rel.15, i.e., the UE supports NR-CA with two PUCCH groups with arbitral grouping for the two PUCCH groups subject to the capability of twoPUCCH-Group

- A new FG to indicate, for FR1 + (FR1 + FR2) PUCCH group configuration, whether PUCCH can be configured on FR2 on the secondary PUCCH group, or SCG is introduced.

- A new FG to indicate whether UE supports cells from different NR PUCCH groups in the same TAG is introduced.

- A new FG to indicate whether UE supports cells from different NR PUCCH groups in the same band is introduced.

FL proposal 2:

- A new FG for supporting offset between the end of PDCCH triggering A-SRS and the SRS transmission for CB PUSCH and antenna switching for UEs supporting PDCCH capabilities besides FG 3-1 is introduced.

FL proposal 3:

- A new FG for supporting partial cancellation of configured PUCCH/PUSCH/PRACH due to dynamic SFI, dynamically granted PDSCH and CSI-RS is introduced.

R1-2005294	Remaining details of features for Rel-16 V2X	FUTUREWEI

R1-2005361	Remaining issues on Rel-16 UE features	vivo

R1-2005423	Discussion on NR Rel-16 UE Features	ZTE

R1-2005666	Remaining issues on Rel-16 UE features	CATT

R1-2005781	Views on Rel-16 UE features	MediaTek Inc.

R1-2005814	Remaining details of Rel-16 NR UE features	Huawei, HiSilicon

R1-2005857	Rel-16 UE feature	Intel Corporation

R1-2005945	Remaining details of UE features for IAB	AT&T

R1-2005980	Discussion on Rel-16 UE features	OPPO

R1-2006003	Discussion on remaining issues of NR feature list for 5G_V2X_NRSL	OPPO

R1-2006124	Remaining issues on NR Rel-16 UE features	Samsung

R1-2006195	LS on UE behavior for P/SP-CSI-RS reception in NR-U	MediaTek Inc.

R1-2006462	Updated RAN1 UE features list for Rel-16 NR	Moderators (AT&T, NTT DOCOMO, INC.)

R1-2006482	Discussions on NR Rel-16 UE features	Apple

R1-2006646	Discussion on NR Rel-16 UE features	LG Electronics

R1-2006677	Remaining aspects of Rel-16 UE features	Nokia, Nokia Shanghai Bell

R1-2006703	Discussion on NR Rel-16 UE features	NTT DOCOMO, INC.

R1-2006704	Summary on [101-e-Post-NR-UE-Features-11]	Moderator (NTT DOCOMO, INC.)

R1-2006705	Summary on [101-e-Post-NR-UE-Features-12]	Moderator (NTT DOCOMO, INC.)

R1-2006706	Summary on [101-e-Post-NR-UE-Features-13]	Moderator (NTT DOCOMO, INC.)

R1-2006707	Summary on [101-e-Post-NR-UE-Features-14]	Moderator (NTT DOCOMO, INC.)

R1-2006788	Discussion on NR Rel-16 UE features	Qualcomm Incorporated

R1-2006874	Remaining details of Rel-16 NR UE features	Ericsson

R1-2006954	Summary on [101-e-Post-NR-UE-Features-08]	Moderator (AT&T)

R1-2006955	Summary on [101-e-Post-NR-UE-Features-09]	Moderator (AT&T)

R1-2006956	Summary on [101-e-Post-NR-UE-Features-10]	Moderator (AT&T)