3GPP TSG RAN WG1 #103e		R1-2008090

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.8.1.2

Source: 	Xiaomi

Title:	Evaluation on NR coverage performance for FR2

Document for:	Discussion

- Introduction

In RAN1#102e meeting, some agreement about estimation of coverage for FR2 had been achieved as follows: - For link level simulations in FR2, only PUCCH format 1 and format 3 are considered for baseline performance evaluation. - For link level simulations in FR2, only PUCCH duration of 14 OFDM symbols is considered for baseline performance evaluation. - For link level simulations in FR2, consider 4 DMRS symbol for PUCCH Format 3. - Consider only one panel at the UE in link budget in FR2. - For link budget calculation in FR2, downlink transmit power is scaled by the occupied bandwidth. The following downlink transmit power vs occupied bandwidth values are considered as baseline for the calculations: - 40 dBm for 100 MHz Urban scenario. - 23 dBm for 100 MHz Indoor scenario. - For link budget calculation in FR2, an uplink transmit power of 23dBm is considered for baseline performance evaluations. Other values can be reported by companies - Confirm the target throughput values of the REL-17 SID for the suburban scenario: - DL: 1 Mbps, UL: 50 kbps - Study performance of PUSCH in FR2 only for DFT-s-OFDM. - For link level simulations, only 1% BLER should be considered for baseline performance evaluation of PDDCH in FR2. - Suburban scenario is deprioritized for NR coverage enhancement SI. - Only 1% BLER target should be considered for baseline performance evaluation of PUCCH in FR2, regardless of whether UCI includes CSI feedback or not. - Simulation assumptions for SLS in FR2 are up to companies’ reports, i.e., no more clarification is needed, as per agreement during RAN1#101-e.

In this document, the required SNR is obtained for difference channels and scenarios by the link level simulation. And the coverage performance based on the simulation results for FR2 will be discussed.

- Link level simulation prefomence

Link level simulation about PDCCH, PUCCH, PUSCH are listed in this document. The TDL-A channel model is considered in the simulation, and one DMRS with type 1 for PUSCH, two DMRS for PUCCH. Above parameters has been discussed in last meeting and aligned with other companies.

2.1  Analysis of PDCCH performance

Table 1. simulation assumption for PDCCH

| Parameters | value |
| --- | --- |
| Scenario and frequency | - 28 GHz |
| SCS | - 120 kHz |
| BWP BW | - 100MHz (66 RBs) |
| Channel model | - TDL-A, NLoS |
| Scenario and Delay spread | - Indoor: 30ns - urban: 100ns |
| Antenna correlation | - Low |
| UE velocity | - 3 km/h |
| Number of antenna | - 2T2R |
| AL | - 16 |
| Payload | - 40bits |
| CORESET | - 2 symbols, 48RBs |
| BLER | - 1% |
| Tx diversity | - Precoder cycling with bundle size of 6 |

indoor and urban are considered in this document. As the agreements from RAN1#102e meeting, only 1% BLER is considered as baseline performance evaluation for PDCCH in FR2. It can be see from Table 2., the required SNR is -9.6dB and -7.9dB in case of indoor and urban respectively with AL=16 base on the simulation assumption in Table 1.

Table 2. Required SNR (dB) when the BLER is 0.01

|  | indoor | urban |
| --- | --- | --- |
| PDCCH (40bit) | -9.6 | -7.9 |

2.2  Analysis performance for PUCCH

Last meeting agree that only PUCCH format 1 and format 3 are considered for baseline performance evaluation. In this section, the simulation results for format 3 with 11 bit and 22 bit with difference number of antenna are shown. The simulation assumption for PUCCH is given in Table 3.

Table 3. simulation assumption for PUCCH

| Parameters | value |
| --- | --- |
| Scenario and frequency | - 28 GHz |
| SCS | - 120 kHz |
| BWP BW | - 200MHz (12 RBs) |
| Channel model | - TDL-A, NLoS |
| Scenario and Delay spread | - 100ns |
| Antenna correlation | - Low |
| UE velocity | - 3 km/h |
| Format and payload | - Format 3: 22 bits UCI - Format 3: 11bits UCI |
| BLER | - Format 3: 1% BLER for A/N/SR, 10% for CSI |
| FDRA | - 1 RB |
| TDRA | - 14 OFDM symbols |
| Number of antenna | - 1T2R / 1T4R |
| Tx diversity | - intra-slot frequency hopping |

Table 4. Required SNR (dB) when the BLER is 0.01

|  |  | 11bit | 22bit |
| --- | --- | --- | --- |
| PUCCH format 3 | 1T2R | -1.8 | 1 |
|  | 1T4R | -7.7 | -5.6 |

As shown in Table 4., the PUCCH format 3 with 11 bit has about 3dB gain in comparison with 22 bit. And the increasing the number of receive antenna from 2 to 4 also can bring about 6dB gain.

2.3  Analysis performance for PUSCH

To evaluate the coverage performance for PUSCH, the required SNR in three scenarios for PUSCH are discussed in this section. The simulation result in Table 6. base on the detail assumption in Table 5. And the different number of antenna is also considered.

Table 5. simulation assumption for PUSCH

| Parameters | value |
| --- | --- |
| frequency | - 28 GHz |
| Frame structure | - DDDSU |
| SCS | - 120 kHz |
| BWP BW | - 100M(66RB) |
| RBs | - Indoor: 30RB - Urban: 30RB - Suburban: 4RB |
| TBS | - Indoor: 2792 - Urban: 2792 - Suburban: 144 |
| Modulation | - QPSK |
| Channel model | - TDL-A, NLoS |
| Scenario and Delay spread | - Indoor: 30ns - Urban: 100ns - Suburban: 100ns |
| Scenario and UE velocity | - Indoor: 3 km/h - Urban: 3km/h, 30km/h - Suburban: 3km/h, 30km/h |
| MCS | - Indoor: MCS4 - Urban: MCS4 - Suburban: MCS0 |
| BLER | - 10% |
| Symbol number of per slot | - 13 OFDM symbols |
| Number of antenna | - 1T2R / 1T4R |
| DMRS | - 1 OFDM symbol |

Table 6. Required SNR (dB) when the BLER is 0.1

|  |  | Indoor | Urban | suburban |
| --- | --- | --- | --- | --- |
| PUSCH | 1T2R | -1 | -1.34 | -2.5 |
|  | 1T4R | -4.95 | -4.85 | -5.8 |

- Conclusion

In this contribution, with the discussion result of RAN1#102e meeting, we have provided our link level simulation results for FR2. The required SNR of PUCCH, PDCCH, PUSCH in difference scenario have been discussed.

- References

[1] RAN1 chairman note, Aug, 2020