3GPP TSG RAN WG1 #103-e			R1-2008544

e-Meeting, October 26th – November 13th, 2020

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary on UE features for NR TEIs

Agenda Item:	7.2.11

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the discussions and proposals in AI 7.2.11 regarding UE features for NR TEIs.

Based on the discussions summarized in Section 2, followings are parts of the suggested email discussions/approvals for AI 7.2.11.

FL proposal of email discussion/approval:

[103-e-NR-UEFeature-TEI-01] Email discussion/approval on UE features for NR TEIs (26th Oct – 29th Oct)

- Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG14-7

- How to reply to RAN2 LS on beamSwitchTiming

Companies are encouraged to check above FL proposal and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Moderator | Discussion point #2 is added according to draft summary on AI 5 from RAN1 chairman. |
| Apple | - Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG14-7 - We are fine to clarfy as the scheduled/triggered/indicated cell - How to reply to RAN2 LS on beamSwitchTiming - RAN2 intepretation is current - Answer to the questions Question A: For CSI-RS configured with repetition and set to “ON”, whether the UE indicates switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signaling or the default value of 48? Answer: the signalled value from the set {224, 336} Question B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling? Answer: 48 Question C: For CSI-RS configured without repetition, whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling? Answer: 48 |
| Intel | We are fine with FL proposal. |
| Moderator | Thank you very much for the feedbacks. I see that the proposed email discussion scope seems fine for all. |

- Discussion on NR Rel-16 UE features

  - Interpretation of UE capabilities in case of cross-carrier operation

| 14. NR TEI | 14-7 | New capability for beamSwitchTiming values of 224 and 336 | - Indicates the minimum number of required OFDM symbols {224, 336} between the DCI triggering aperiodic CSI-RS and the corresponding aperiodic CSI-RS transmission in a CSI-RS resource set configured with repetition ‘ON’ - Candidate values: {224, 336} | 2-28 | Yes | N/A |  | Per band | N/A | N/A (FR2 only) | N/A | Agreements: ・48 is used as the beam switching threshold for Ues reporting 224 or 336 ØWhen using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is made in a contribution.

| [2] | As shown in the introduction section, RAN1 agreed that Interpretation#1 is adopted for beamSwitchTiming in Rel-15. The same interpretation (Interpretation#1) for 14-7 can be adopted. Note that FG14-7 is only for FR2, Interpretation#3 would preclude the case of DCI in FR1 triggering CSI-RS in FR2. Proposal 3 (TEI): - Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on the support of this capability for the band of the scheduled/triggered/indicated cell only. - FG14-7 14. NR TEI; 14-7; New capability for beamSwitchTiming values of 224 and 336; - Indicates the minimum number of required OFDM symbols {224, 336} between the DCI triggering aperiodic CSI-RS and the corresponding aperiodic CSI-RS transmission in a CSI-RS resource set configured with repetition ‘ON’ - Candidate values: {224, 336}. |
| --- | --- |

Discussion point #1

- Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG14-7

  - LS reply on beamSwitchTiming [3]

| 14. NR TEI | 14-7 | New capability for beamSwitchTiming values of 224 and 336 | - Indicates the minimum number of required OFDM symbols {224, 336} between the DCI triggering aperiodic CSI-RS and the corresponding aperiodic CSI-RS transmission in a CSI-RS resource set configured with repetition ‘ON’ - Candidate values: {224, 336} | 2-28 | Yes | N/A |  | Per band | N/A | N/A (FR2 only) | N/A | Agreements: ・48 is used as the beam switching threshold for Ues reporting 224 or 336 ØWhen using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In [3], following requests from RAN2 are provided.

- For Rel-15 UEs, - if the UE indicates a value from the set {14, 28, 48} in Rel-15 capability signaling, the reported value is used for all cases, including CSI-RS with repetition is configured and set to “ON” or “OFF”. - if the UE indicates a value from the set {224, 336} in Rel-15 capability signaling, there is no specified beahviour for value of 224 and 336. - If the Rel-15 capability signaling (beamSwitchTiming) is not included, the beam switch timing is up to 48 OFDM symbols for each supported sub-carrier spacing. - For Rel-16 UEs, the UE signals beamSwitchTiming and beamSwitchTiming-r16. The first one, i.e., the Rel-15 capability signaling from the set {14, 28, 48}, is always signaled (if this capability needs to be reported). The second one, i.e., the newly introduced Rel-16 capability signaling from the set {224, 336}, is optional. - If the UE doesn’t include the Rel-16 capability, it behaves in all respects as the Rel-15 UEs above. - If the UE does include the Rel-16 capability, then, the UE may or may not receive configured parameter enableBeamSwitchTiming-r16 in the RRC configuration: - If the UE doesn’t receive parameter enableBeamSwitchTiming-r16, it behaves in all respect as the Rel-15 UE above. - If the UE receives parameter enableBeamSwitchTiming-r16, then, the UE does the following: - Question A: For CSI-RS configured with repetition and set to “ON”, whether the UE indicates switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signaling or the default value of 48? - Question B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling? - Question C: For CSI-RS configured without repetition, whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling? # To RAN WG1 # RAN2 kindly requests RAN1 to: - Confirm whether the above RAN2 understanding is correct or not. - Provide response to the above Questions.

Following proposals are made in contributions (from AI 5).

| [4] | Firstly, from RAN1 perspective, the following RAN2 understanding about beamSwitchingTiming is correct. [RAN1]: From RAN1 perspective, if the UE signals beamSwitchTiming-r16 and receives parameter enableBeamSwitchTiming-r16 - Regarding Question A: For CSI-RS configured with repetition and set to “ON”, the UE applies switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signalling. - Regarding Question B: For CSI-RS configured with the repetition and set to “OFF”, the UE applies a value of 48 as beam switch time regardless of signalled value in the Rel-16 capability signalling and signalled value in Rel-15 capability signal (i.e., even if it signalled < 48 in the Rel-15 capability). - Regarding Question C: For CSI-RS configured without repetition and without trs-info (e.g., CSI-RS for CSI), the UE applies a value of 48 as beam switch time regardless of signalled value in the Rel-16 capability signalling and signalled value in Rel-15 capability signal (i.e., even if it signalled < 48 in the Rel-15 capability). For CSI-RS configured without repetition and with trs-info (i.e., CSI-RS for tracking), the UE applies switch time of the signalled value in Rel-15 capability signalling, when the signalled value is one of the set {14, 28, 48}. |
| --- | --- |
| [5] | For a UE reporting both the Rel.15 capability (beamSwitchTiming) and Rel.16 capability (beamSwitchTiming-r16), if the UE receives parameter enableBeamSwitchTiming-r16, - Answer A: For CSI-RS configuration with repetition and set to “ON”, the UE assumes switch time is equal to 48 symbols, per RAN1 agreement. - Answer B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), no agreement has been reached in RAN1 on new beam switch behaviour in Rel.16, and in our understanding the new UE capability does not apply to this case. UE behaviour remains the same as Rel.15, e.g. UE assumes switch time of the reported capability beamSwitchTiming when the reported value is {14, 28, 48}, and otherwise undefined. - Answer C: For CSI-RS without repetition, no agreement has been reached in RAN1 on new beam switch behaviour in Rel.16, and in our understanding the new UE capability does not apply to this case. UE behaviour remains the same as Rel.15, e.g. UE assumes switch time of the reported capability beamSwitchTiming when the reported value is {14, 28, 48}, and otherwise undefined. |
| [6] | Answer to Question A: RAN1 understands that the switch time is indicated from a gNB, not a UE. gNB configures offset between triggering DCI and triggered AP-CSI-RS considering the UE capability parameters beamSwitchTiming and beamSwitchTiming-r16. The intention of the parameter beamSwitchTiming-r16 is to inform gNB that a UE needs large switch time, {224, 336}, on CSI-RS for Rx beam sweeping (CSI-RS configured with repetition=ON), while the intention of beamSwitchTiming is to inform gNB on the switch time on CSI-RS for other purposes. gNB can schedule AP-CSI-RS for any purpose (except TRS) with either i) larger/equal or ii) smaller triggering offset than the UE capability explained above, and the corresponding UE behaviour on each case is specified in RAN1 as follows. For the case when RRC parameter enableBeamSwitchTiming-r16 is configured, - If the triggering offset is equal to or greater than 48, UE receives the AP-CSI-RS with the TCI state indicated by DCI. - If the triggering offset is smaller than 48, UE receives the AP-CSI-RS with a default TCI state. Answer to Question B: Please refer to the answer for Question A. Answer to Question C: Please refer to the answer for Question A. |
| [7] | Proposed reply: The understanding from RAN2 is correct. For Question A/B/C, when the UE receives parameter enableBeamSwitchTiming-r16, RAN1 has the following understanding: Question A: For CSI-RS configured with repetition and set to “ON”, whether the UE indicates switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signaling or the default value of 48? - Answer: For CSI-RS configured with repetition and set to “ON”, UE can report a value from the set {224, 336} in beamSwitchTiming-r16, but 48 is used as the beam switching threshold for UEs reporting 224 or 336 in beamSwitchTiming-r16. Question B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling? - Answer: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), the UE applies the signalled value in Rel-15 capability signaling as the beam switching threshold. Question C: For CSI-RS configured without repetition, whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling? - Answer: For CSI-RS configured without repetition and without trs-info, the UE applies the signalled value in Rel-15 capability signaling as the beam switching threshold. |
| [8] | A1: RAN1 confirms the above understanding. A2: RAN1 has the following understanding - Both beamSwitchTiming and beamSwitchTiming-r16 are optional to report - If the UE doesn’t include the beamSwitchTiming-r16, its behaviour is same as Rel-15 - If the UE does include the Rel-16 capability, UE may or may not receive enableBeamSwitchTiming-r16 - If the UE does include the Rel-16 capability but doesn’t receive enableBeamSwitchTiming-r16, its behaviour is same as Rel-15 - If the UE receives enableBeamSwitchTiming-r16 - For CSI-RS configured with repetition “ON”, the UE indicates switch time that is the same as the signalled value from the set {224, 336} - For CSI-RS configured with repetition “OFF”, the UE indicates switch time of 48 if beamSwitchTiming-r16 is reported - For CSI-RS configured without repetition, the UE indicates switch time of 48 if beamSwitchTiming-r16 is reported |
| [9] | from RAN1’s understanding, descriptions of the first, second and third bullets are correct. It is correct understanding that Rel-16 capability signaling from the set {224, 336} is optional, and Rel-15 capability signaling from the set {14, 28, 48} is always signaled (if this capability needs to be reported). It is also correct that the UE behaves as Rel-15 UE in all respects if the UE doesn’t include Rel-16 capability or doesn’t receive parameter enableBeamSwitchTiming-r16. Below please find RAN1’s Answers to Questions A, B and C. Answer A: for CSI-RS configured with repetition and set to “ON”, when the Rel-16 UE capability signaling indicates one of the values {224, 336}, the beam switch time is 48. Answer B: for CSI-RS configured with repetition and set to “OFF”, the beam switch time is equal to the signaled value in Rel-15 capability signaling, if reported. Answer C: for CSI-RS configured without repetition, the beam switch time is equal to the signaled value in Rel-15 capability signaling, if reported. |
| [10] | Understanding 1: For Rel-15 UEs, if the UE indicates a value from the set {14, 28, 48} in Rel-15 capability signaling, the reported value is used for all cases, including CSI-RS with repetition is configured and set to “ON” or “OFF”. Response 1: This is not correct. The UE behavior for CSI-RS configured with repetition (set as ‘ON’ or ‘OFF’) was unspecified in RAN1 in R15. This is one of the reasons that the Rel-16 TEI was agreed, i.e., to include CSI-RS with repetition, by changing ‘without higher layer parameter trs-Info and without the higher layer parameter repetition’ as ‘without higher layer parameter trs-Info’. Understanding 2: For Rel-15 UEs, if the UE indicates a value from the set {224, 336} in Rel-15 capability signaling, there is no specified behaviour for value of 224 and 336. Response 2: This is not correct. In R15, when UE reports {224, 336}, it will be used to determine UE expectation/behavior for aperiodic CSI-RS for tracking and latency requirements for L1-RSRP reporting, according the specs highlighted below. Understanding 3: If the Rel-15 capability signaling (beamSwitchTiming) is not included, the beam switch timing is up to 48 OFDM symbols for each supported sub-carrier spacing. Response 3: In R15, RAN1 did not agree on using 48 as default value for beamSwitchTiming. Instead, in R15, RAN1 listed ‘No recommendation on the desired beam switching timing’ as ‘Consequences if the feature is not supported by the UE’ in UE feature list R1-1907862. It would be clearer if the UE reports this UE capability. Answers to questions The following assume enableBeamSwitchTiming-r16 is provided to the UE. Question A: For CSI-RS configured with repetition and set to “ON”, whether the UE indicates switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signaling or the default value of 48? Answer A: RAN1 did not define default value for beamSwitchTiming-r16, as listed in the LS sent from RAN1 to RAN2 in R1-2007326. The LS sent from RAN1 to RAN2 in R1-1909895 indicated the required beam switching time is the signalled value from the set of {224, 336} for CSI-RS configured with repetition set to ‘ON’. Question B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling? Answer B: For CSI-RS configured with repetition set to ‘OFF’, the required beam switching time is 48 symbols. Question C: For CSI-RS configured without repetition, whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling? Answer C: RAN1 assume this question is for Rel-16 UE. CSI-RS without repetition can be CSI-RS for CSI (without repetition and without trs-Info) and CSI-RS for tracking (without repetition and with trs-Info). For CSI-RS for tracking, the indicated value for beamSwitchTiming applies (if belongs to the set of {14, 28, 48} according to the Rel-16 spec quoted below). For CSI-RS for CSI, it is handled the same way as CSI-RS for BM (with repetition, set to ‘OFF’), where the required beam switching time is 48 symbols. |

Discussion point #2

- How to reply to RAN2 LS on beamSwitchTiming

- Conclusion

Based on the email discussion in preparation phase, it was agreed to have following email discussion in RAN1#103-e.

[103-e-NR-UEFeature-TEI-01] Email discussion/approval on UE features for NR TEIs (26th Oct – 29th Oct) – Hiroki (DCM)

- Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG14-7

- How to reply to RAN2 LS on beamSwitchTiming

# Reference

[1]	R1-2007326	Updated RAN1 UE features list for Rel-16 NR including remaining RAN1 issues				Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2007739	Discussion on NR Rel-16 UE Features	ZTE

[3]	R1-2007519	LS to RAN1 on beamSwitchTiming	RAN2, vivo

[4]	R1-2007757	Draft reply LS on beamSwitchTiming	ZTE

[5]	R1-2007801	Draft reply LS on  beamSwitchTiming	CATT

[6]	R1-2008116	Draft reply LS on beamSwitchTiming	Samsung

[7]	R1-2008228	Discussion on LS on beamSwitchTiming for AP CSI-RS	OPPO

[8]	R1-2008591	Draft reply LS to RAN 2 LS on beamSwitchTiming	Qualcomm Incorporated

[9]	R1-2008651	Draft Reply LS on beamSwitchTiming	vivo

[10]	R1-2008791	[Draft] Reply LS on beamSwitchTiming	Huawei, HiSilicon

# Appendix: UE features list for NR TEIs in [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14. NR TEI | 14-1 | Multiple LTE-CRS rate matching patterns | - Maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS - Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS | 5-28 (Rate-matching around LTE CRS) | Yes | N/A |  | Per band | N/A | N/A (FR1 only) | N/A | For DSS The number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot UE reporting component 1 for 14-1 also reports component 2. Reporting of values of Component 1 larger than two is only applicable when reporting values of Component 2 larger than one. | Optional with capability signalling Component 1:{2, 3, 4, 5, 6} Component 2: {1, 2, 3} |
| 14. NR TEI | 14-1a | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier | - Support of two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz SCS overlapping with a LTE carrier | 14-1 | Yes | N/A |  | Per band | N/A | N/A (FR1 only) | N/A | For DSS The number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot | Optional with capability signaling |
| 14. NR TEI | 14-2 | PDSCH Type B mapping of length 9 and 10 OFDM symbols | - support of PDSCH Type B scheduling of length 9 and 10 OFDM symbols - support of DMRS shift for length-10 symbols | 5-6a (PDSCH mapping type B) | Yes | N/A |  | Per band | N/A | N/A (FR1 only) | N/A | For DSS FG10-8 covers PDSCH type B mapping without DMRS shift due to CRS collision. | Optional with capability signaling |
| 14. NR TEI | 14-3 | One slot periodic TRS configuration for FR1 | - UE can be configured with one-slot periodic TRS configuration only when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated | 2-51 (CSI-RS for tracking) | Yes | N/A |  | Per band | N/A (TDD only) | N/A (FR1 only) | N/A | UE can be configured with one-slot periodic TRS configuration only when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated. This FG is not also applicable for the case that all slots are indicated as flexible | Optional with capability signalling |
| 14. NR TEI | 14-4 | SRS Tx switch with allowing downgrading configuration | 1) Support SRS Tx port switch | 2-55 | Yes | N/A |  | Per BC (same reporting type as srs-TxSwitch in Rel-15) | N/A | N/A | N/A | Agreement: •Rel-16 UE capability design for SRS antenna switching in conjunction with the existing Rel-15 UE capability should allow UE to indicate support of one of the following combinations o{t1r1, t1r2} o{t1r1, t1r2, t1r4} o{t1r1, t1r2, t2r2, t2r4} o{t1r1, t2r2} o{t1r1, t2r2, t4r4} o{t1r1, t1r2, t2r2, t1r4, t2r4} oNote: Detailed signaling design is up to RAN2 | Optional with capability signalling Component 1: Candidate value set: { o{t1r1, t1r2} o{t1r1, t1r2, t1r4} o{t1r1, t1r2, t2r2, t2r4} o{t1r1, t2r2} o{t1r1, t2r2, t4r4} o{t1r1, t1r2, t2r2, t1r4, t2r4} } Component2: Candidate value set: {yes, no} Component 3: Candidate value set: {yes, no} |
| 14. NR TEI | 14-5 | Half-duplex UE behaviour in TDD CA for same SCS | - Support for directional collision handling between reference and other cell(s) for half-duplex operation in CA with same SCS | 6-5 and simultaneousRxTxInterBandCA not supported | Yes | N/A |  | Per BC | N/A (TDD only) | N/A | N/A | Half duplex UEs that do not indicate this capability should still be able to operate half-duplex TDD CA (i.e. simultaneousRxTxInterBandCA not supported) per Rel15 specifications if network ensures same transmission direction across all the serving cells | Optional with capability signaling |
| 14. NR TEI | 14-6 | New RACH configuration for FR1 TDD | - new RACH configuration entries with subframe number 2 and/or 7 for RACH periodicity longer than 10 ms |  | No | N/A |  | N/A | N/A (TDD only) | N/A (FR1 only) | N/A | Agreement: •A new UE capability is not introduced for this TEI, i.e., it is a mandatory UE feature for Rel-16. | Mandatory without capability signalling |
| 14. NR TEI | 14-7 | New capability for beamSwitchTiming values of 224 and 336 | - Indicates the minimum number of required OFDM symbols {224, 336} between the DCI triggering aperiodic CSI-RS and the corresponding aperiodic CSI-RS transmission in a CSI-RS resource set configured with repetition ‘ON’ - Candidate values: {224, 336} | 2-28 | Yes | N/A |  | Per band | N/A | N/A (FR2 only) | N/A | Agreements: ・48 is used as the beam switching threshold for Ues reporting 224 or 336 ØWhen using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. | Optional with capability signaling |
| 14. NR TEI | 14-8 | CSI trigger states containing non-active BWP | - CSI trigger states containing non-active BWP |  | Yes | N/A |  | Per UE | No | No | N/A | Agreements: TEI – “CSI trigger states containing non-active BWP” ・When a UE is triggered with a CSI report for a DL BWP that is non-active, the UE is not expected to report the CSI for the non-active BWP and the CSI report associated with the BWP is omitted. ・When a UE is triggered with aperiodic CSI-RS in a DL BWP that is non-active, the UE is not expected to measure the aperiodic CSI-RS. ・The above non-active BWP is the non-active BWP when receiving the associated CSI-RS with the following relaxation for UE processing. In the CC of the associated CSI-RS, if the active BWP when receiving the CSI-RS is different from the active BWP when receiving the triggering DCI The last symbol of the PDCCH span of the DCI carrying the BWP switching shall be no later than the last symbol of the PDCCH span of the CSI trigger DCI, irrespective of whether they are in the same CC or not and irrespective of whether they are in the same SCS or not. The UE is not expected to have any other BWP switching in that CC after the last symbol of the PDCCH span covering CSI trigger DCI and before the first symbol of the triggered CSI-RS resource. ・Note: the UE is not required to measure P/SP-CSI-RS in the non-active BWP per current specification | Optional with capability signaling |