3GPP TSG-RAN WG1 Meeting #103-e	R1-2009093

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.4.3

Source:	Ericsson

Title:	On HARQ enhancements for NTN

Document for:	Discussion

# 1	Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the work item description (WID) is updated in [1]. One objective is to specify HARQ enhancements for NTN. In this contribution, we discuss HARQ enhancements for NTN per the WID objective:

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

# 2	On the support of 32 HARQ processes

At RAN1#102e, the following agreement was made to support up to 32 HARQ processes for NTN.

RAN1#102-e:

Agreement:

The extension of maximal HARQ process number can be considered with following assumptions:

- The maximal supported HARQ process number is up to 32.

- FFS: Support on the maximal HARQ process number is up to UE capability

- Minimizing the impacts on specification and scheduling

The use of 32 HARQ processes should be under network control and can be configured by RRC, preferably for downlink and uplink separately.

Whether 32 HARQ processes are used or not in the uplink can be configured by RRC.

Whether 32 HARQ processes are used or not in the downlink can be configured by RRC.

If the network configures 32 HARQ processes in the downlink and/or uplink, the HARQ process ID field in some DCI formats would need to be extended by a configurable bit to accommodate 32 HARQ processes.

- In fallback DCI 0_0/1_0, the HARQ process ID field has a fixed number of 4 bits. Note that fallback DCI is generally not configurable. For backward compatibility, the HARQ process ID field in fallback DCI 0_0/1_0 should not be extended to accommodate 32 HARQ processes. This is acceptable as fallback DCI 0_0/1_0 is used for basic scheduling, while the use of 32 HARQ processes is intended for high throughput use cases, the scheduling of which can be carried out with non-fallback DCI 0_1/1_1 or 0_2/2_2.

- In non-fallback DCI 0_1/1_1, the HARQ process ID field also has a fixed number of 4 bits. Non-fallback DCI is configurable. In this case, if 32 HARQ processes are configured, an additional bit is present. This additional bit, together with the existing 4 bits, can be used to indicate 32 HARQ processes.

- In non-fallback DCI 0_2/2_2, the size of the HARQ process ID field is 0, 1, 2, 3 or 4 bits determined by the higher layer configured number of HARQ processes. So, the HARQ process ID field is already configurable. It is straightforward to support 32 HARQ processes using DCI 0_2/2_2: the size of the HARQ process ID field is 5 bits if 32 HARQ processes are configured.

There are proposals about using time window-based method (and its variations) to support 32 HARQ processes. It should be noted that the fundamental NR HARQ design is asynchronous and thus flexible, relaxed from early LTE HARQ design (which is synchronous to some extent). Time window-based method (and its variations) is not aligned with this principle and re-introduces unnecessary scheduling restriction from LTE. This is against the agreement that the solution should be “minimizing the impacts on specification and scheduling.”

In contrast, adding a configurable bit to DCI 0_1/1_1 or 0_2/2_2 is a much clean way to support 32 HARQ processes. It minimizes the impacts on specification and scheduling.

It is not necessary to schedule 32 HARQ processes using fallback DCI 0_0/1_0.

Adding an additional configurable bit in DCI 0_1/1_1 or 0_2/2_2 to support 32 HARQ processes minimizes the impacts on specification and scheduling.

Time window-based method (and its variations) for supporting 32 HARQ processes is not aligned with asynchronous NR HARQ design principle, introduces unnecessary scheduling restriction, and thus is against the RAN1 agreement of “minimizing the impacts on specification and scheduling.”

If 32 HARQ processes are configured, an additional bit is present in DCI 0_1/1_1. This additional bit, together with the existing 4 bits in the HARQ process ID field, can be used to indicate 32 HARQ processes.

If 32 HARQ processes are configured, the size of the HARQ process ID field in DCI 0_2/2_2 is 5 bits.

# 3	Configuration for enabled / disabled HARQ

At RAN1#102e, the following agreement was made for enabling / disabling HARQ feedback for downlink transmission.

RAN1#102-e:

Agreement:

Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling

HARQ is a basic functionality in NR. Many physical layer functionalities depend on the use of HARQ. If the HARQ would be completely turned off, there would be significant RAN1 specification impact. For example, the MAC CE activation/deactivation of a certain feature becomes effective 3 msec after the UE transmits HARQ-ACK corresponding to the PDSCH carrying the MAC CE command. The MAC CE activation/deactivation timing relationships (in many places of RAN1 specifications) would become undefined if HARQ would be completely turned off.

To avoid the unnecessarily large RAN1 specification impact, at least one HARQ process should be configured with UL HARQ feedback when HARQ is disabled. Alternatively, UE does not expect a MAC CE activation/deactivation command, which would become effective 3 msec after the UE would transmit the corresponding HARQ-ACK, to be scheduled on a downlink HARQ process with disabled feedback. This would solve the unnecessary complications resulted from completely turning off HARQ.

RAN1 to down select one option: Option 1 – UE expects that at least one HARQ process is configured with UL HARQ feedback; Option 2 – UE does not expect a MAC CE activation/deactivation command, which would become effective 3 msec after the UE would transmit the corresponding HARQ-ACK, to be scheduled on a downlink HARQ process with disabled feedback.

Given that enabling / disabling of HARQ feedback should be configurable per HARQ process, it is pointed out in Section 7.2.1.4 in TR 38.821 [2] that “If the feedback is disabled for a selective number (i.e. not all) of HARQ processes, the configuration parameters for different HARQ processes may need to be different.”

Disabling HARQ feedback may increase the residual Block Error Rate (BLER) since retransmissions based on HARQ feedback are not possible anymore. For robustness and to minimize the need of RLC AM retransmission, the BLER target of the first transmission should be reduced. If the feedback is disabled for some HARQ processes but not for the others, it is not desirable to apply the same configuration. For example, for a HARQ process with feedback enabled, an MCS table with higher BLER target can be used, while for a HARQ process with feedback disabled, another MCS table with lower BLER target can be used.

Similarly, for a HARQ process with feedback enabled, aggregation factor may not be needed, while for a HARQ process with feedback disabled, aggregation factor may be needed.

RAN1 to discuss what parameters need to be configured differently for HARQ processes with feedback and HARQ processes without feedback. Example parameters include MCS table and aggregation factor.

# 4	HARQ codebooks for enabled/disabled HARQ

Note that RAN1 agreement clearly states that “Enabling/disabling on HARQ feedback for downlink transmission…”. If the network schedules a PDSCH on a HARQ process with feedback disabled, it is clear that the network is not interested in receiving the feedback. In this case, the UE should not send the feedback. This is the meaning of “disabling HARQ feedback”.

As an example, in transparent GEO, the round-trip time is ~500 ms. Using a HARQ process with feedback disabled, the network schedules PDSCH #1 in subframe #1, PDSCH #2 in subframe #2, …. The network is not interested in receiving a feedback for PDSCH #1 after ~500 ms. In this case, sending feedback from UE leads to waste of radio resource and UE power consumption, as well as increased interference.

The intention of disabling HARQ feedback for a downlink transmission is not to send HARQ feedback for the downlink transmission.

If the network schedules a PDSCH on a HARQ process with feedback disabled, it is clear that the network is not interested in receiving the feedback.

Sending feedback from UE for a HARQ process with disabled feedback leads to waste of radio resource and UE power consumption, as well as increased interference.

With the UE being configured with both feedback disabled HARQ processes and feedback enabled HARQ processes, UE procedures on how to determine the entries in the NR Type-1 and Type-2 HARQ codebooks need to be defined.  For instance, a UE may receive PDSCHs associated with either a feedback disabled HARQ process or a feedback enabled HARQ process.

Additionally, Type-3 HARQ codebook is introduced in Rel-16, motivated by NR operation in unlicensed spectrum that often requires listen-before-talk (LBT) to assess the availability of the carrier since the carrier may be shared with other networks and may not be available at any given time. While NTN uses licensed spectrum, the Type-3 HARQ codebook can be used to increase robustness of HARQ operation in any network and may be useful for NTN as well in some circumstances.

In the case of the NR Type-1 HARQ codebook, as the size of the codebook is fixed based on configuration and should not change dynamically, one simple approach is to insert NACKs in the positions corresponding to PDSCHs associated with feedback disabled HARQ processes.  For PDSCHs associated with feedback enabled HARQ processes, the UE inserts ACK or NACK depending on the decoding outcome of the PDSCHs.  An example of a NR Type 1 HARQ codebook with feedback enabled/disabled HARQ processes is shown in Figure 1.  In the DL association set for HARQ codebook in slot n, the UE receives a PDSCH associated with a feedback disabled HARQ process in slot n-3 (corresponding to K1=3) and a PDSCH associated with a feedback enabled HARQ process in slot n-1 (corresponding to K1=1).  As there is no ACK-NACK feedback for the PDSCH associated with feedback disabled HARQ process, the UE will insert a NACK (N) in position K1=3 corresponding to the PDSCH received in slot n-3.  For PDSCH received in slot n-1 which is associated with a feedback enabled HARQ process, the UE inserts ACK-NACK bit X which depends on the outcome of the PDSCH associated with the feedback enabled HARQ process.

Figure 1:  An example of a NR Type 1 HARQ codebook with feedback enabled/disabled HARQ processes

In the case of the NR Type-2 HARQ codebook, a UE can simply ignore counter DAI from a PDCCH that is associated with a feedback disabled HARQ process.  Furthermore, the gNB does not need to increment the counter DAI for a PDCCH that is associated with a feedback disabled HARQ process.  If total DAI is present, it indicates the sum of all the scheduled PDCCHs associated with feedback enabled HARQ process.

Two examples are shown in Figure 2. The numbers (cDAI,tDAI) in the boxes indicate counter DAI and total DAI, respectively. The upper example shows the legacy case where all PDSCHs are associated with feedback enabled HARQ processes and the DAIs are incremented for each PDCCH. In the lower example, the green boxes indicate scheduling of PDSCH associated with feedback disabled HARQ processes. The counter DAI is not incremented in the PDCCHs scheduling the HARQ disabled processes and the total DAI does not take these PDCCHs into account. But the true DAI values are still signaled to increase the robustness of the dynamic codebook construction of HARQ processes with feedback enabled.

Figure 2:  An example of a NR Type 2 HARQ codebook with feedback enabled/disabled HARQ processes

In the case of the NR Type-3 HARQ codebook, feedback is provided on all HARQ processes in one shot. In order to reduce unnecessary overhead one simple approach is to only provide ACK/NACK information in the Type-3 codebook for those HARQ processes that are enabled. This allows for an optimized codebook size that does not vary dynamically. The Type 3 codebook can thus be used to increase reliability in some situations.

When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-1 HARQ codebook, the UE inserts NACKs in positions corresponding to PDSCHs associated with feedback disabled HARQ processes.

When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-2 HARQ codebook, the UE ignores counter DAI from a PDCCH that is associated with a feedback disabled HARQ process and counter DAI is not incremented for such a PDCCH.

When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-2 HARQ codebook, the total DAI (if present) indicates the sum of all the scheduled PDCCHs associated with feedback enabled HARQ process.

When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-3 HARQ codebook, the codebook size is dimensioned to include ACK/NACK information only for HARQ processes that are enabled.

# 5	PDSCH/PUSCH scheduling on a disabled HARQ process

The motivation of disabled HARQ is to avoid throughput reduction due to the stop-and-wait protocol in HARQ. Specifically, the enhancement is to enable the gNB and UE to reuse a HARQ process ID without waiting for HARQ feedback.

To complete the functionality of disabled HARQ, it is necessary to revisit the restrictions in RAN1 specification on reusing the same HARQ process ID.

According to section 6.1 in TS 38.214, the UE is not expected to be scheduled to transmit another PUSCH for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process.

Section 6.1, TS 38.214:

The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process.

It is noted that some company has proposed to clarify the interpretation of the specification text “scheduled to transmit” at RAN1#103e as part of the Rel-15 maintenance [3]:

- Interpretation 1: “scheduled to transmit” is the point in time where the actual PUSCH transmission takes place.

  - This case is illustrated in Figure 3. In this case, for a given HARQ process, network can send 2 back-to-back DCIs to schedule 2 back-to-back PUSCHs.

- Interpretation 2: “scheduled to transmit” is the point in time where the PDCCH with the UL grant is received.

  - This case is illustrated in Figure 4. In this case, for a given HARQ process, network cannot send the second DCI to schedule a second PUSCH until after the end of the expected transmission of the first PUSCH. So, there is a small gap between the 2 PUSCHs scheduled on the same HARQ process. But the gap can be filled by scheduling PUSCHs on a different HARQ process. As a result, continuous PUSCH transmission is still possible.

Figure 3:  Reusing HARQ process ID in the UL: “scheduled to transmit” is interpreted as the point in time where the actual PUSCH transmission takes place.

Figure 4:  Reusing HARQ process ID in the UL: “scheduled to transmit” is interpreted as the point in time where the actual PUSCH transmission takes place.

Regardless of which interpretation of the text “scheduled to transmit” is taken, it does not impose restriction that would lead to throughput reduction in the presence of large RTT, and it is not impacted by unaligned DL and UL frame timings at the gNB/UE.

The situation is different in the DL. According to section 5.1 in TS 38.214, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process.

Section 5.1, TS 38.214:

The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6].

Clearly, with disabled HARQ feedback, the gNB does not need to receive the HARQ feedback and the UE does not need to prepare the HARQ feedback. So, the aforementioned restriction based on HARQ feedback is not applicable. Instead, similar to the uplink, we can allow reuse of a HARQ process ID for another PDSCH after the end of the previous PDSCH. This new rule of reusing HARQ process in the DL is illustrated in Figure 5.

Figure 5:  Reusing HARQ process ID in the DL for a HARQ process with disabled feedback

The uplink HARQ process reuse rule that “the UE is not expected to be scheduled to transmit another PUSCH for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process” does not impose restriction that would lead to throughput reduction in the presence of large RTT, regardless of whether the DL and UL frame timings are aligned or not at the gNB/UE.

The downlink HARQ process reuse rule that “the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process” is not applicable to a HARQ process with feedback disabled.

Keep the existing uplink HARQ process reuse rule.

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH for the given HARQ process until after the end of the expected reception of the last PDSCH for that HARQ process.

# Conclusion

In the previous sections, we discuss HARQ enhancements for NTN. We made the following observations:

Observation 1	It is not necessary to schedule 32 HARQ processes using fallback DCI 0_0/1_0.

Observation 2	Adding an additional configurable bit in DCI 0_1/1_1 or 0_2/2_2 to support 32 HARQ processes minimizes the impacts on specification and scheduling.

Observation 3	Time window-based method (and its variations) for supporting 32 HARQ processes is not aligned with asynchronous NR HARQ design principle, introduces unnecessary scheduling restriction, and thus is against the RAN1 agreement of “minimizing the impacts on specification and scheduling.”

Observation 4	The intention of disabling HARQ feedback for a downlink transmission is not to send HARQ feedback for the downlink transmission.

Observation 5	If the network schedules a PDSCH on a HARQ process with feedback disabled, it is clear that the network is not interested in receiving the feedback.

Observation 6	Sending feedback from UE for a HARQ process with disabled feedback leads to waste of radio resource and UE power consumption, as well as increased interference.

Observation 7	The uplink HARQ process reuse rule that “the UE is not expected to be scheduled to transmit another PUSCH for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process” does not impose restriction that would lead to throughput reduction in the presence of large RTT, regardless of whether the DL and UL frame timings are aligned or not at the gNB/UE.

Observation 8	The downlink HARQ process reuse rule that “the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process” is not applicable to a HARQ process with feedback disabled.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Whether 32 HARQ processes are used or not in the uplink can be configured by RRC.

Proposal 2	Whether 32 HARQ processes are used or not in the downlink can be configured by RRC.

Proposal 3	If 32 HARQ processes are configured, an additional bit is present in DCI 0_1/1_1. This additional bit, together with the existing 4 bits in the HARQ process ID field, can be used to indicate 32 HARQ processes.

Proposal 4	If 32 HARQ processes are configured, the size of the HARQ process ID field in DCI 0_2/2_2 is 5 bits.

Proposal 5	RAN1 to down select one option: Option 1 – UE expects that at least one HARQ process is configured with UL HARQ feedback; Option 2 – UE does not expect a MAC CE activation/deactivation command, which would become effective 3 msec after the UE would transmit the corresponding HARQ-ACK, to be scheduled on a downlink HARQ process with disabled feedback.

Proposal 6	RAN1 to discuss what parameters need to be configured differently for HARQ processes with feedback and HARQ processes without feedback. Example parameters include MCS table and aggregation factor.

Proposal 7	When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-1 HARQ codebook, the UE inserts NACKs in positions corresponding to PDSCHs associated with feedback disabled HARQ processes.

Proposal 8	When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-2 HARQ codebook, the UE ignores counter DAI from a PDCCH that is associated with a feedback disabled HARQ process and counter DAI is not incremented for such a PDCCH.

Proposal 9	When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-2 HARQ codebook, the total DAI (if present) indicates the sum of all the scheduled PDCCHs associated with feedback enabled HARQ process.

Proposal 10	When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-3 HARQ codebook, the codebook size is dimensioned to include ACK/NACK information only for HARQ processes that are enabled.

Proposal 11	Keep the existing uplink HARQ process reuse rule.

Proposal 12	For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH for the given HARQ process until after the end of the expected reception of the last PDSCH for that HARQ process.

# References

RP-201256, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #88e, June 2020.

TR 38.821, Solutions for NR to support non-terrestrial networks.

R1-2008767, “Clarification on the PUSCH scheduling constraint in Rel-15,” 3GPP TSG RAN1 #103e, Apple Inc., October/November 2020.