3GPP TSG RAN WG1 #103-e			R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

Title:	[draft] Reply LS to RAN1 on beamSwitchTiming

Response to:	R2-2008318

Release:	Rel-16

Work Item:	TEI16

Source:	vivo [RAN1]

To:	RAN2

Cc:

- Contact Person:

- Name:	                Rakesh Tamrakar

E-mail Address:	 rakesh@vivo.com

- 1. Overall Description:

RAN1 thanks RAN2 for the LS on beamSwicthTiming.

On Rel-15 UE capability of beamSwitchTiming,

- For Rel-15 UEs,

- if the UE indicates a value from the set {14, 28, 48} in Rel-15 capability signaling, the reported value is used for all cases, including CSI-RS with repetition is configured and set to “ON” or “OFF”.

- if the UE indicates a value from the set {224, 336} in Rel-15 capability signaling, there is no specified beahviour for value of 224 and 336.

- If the Rel-15 capability signaling (beamSwitchTiming) is not included, the beam switch timing is up to 48 OFDM symbols for each supported sub-carrier spacing.

from RAN1’s understanding, descriptions in the first, second and third bullets are correct.

Observation from first round of discussion, majority of companies agree with RAN2 understanding.

| company | comments |
| --- | --- |

For Rel-16 UEs, it is correct understanding that Rel-16 capability signaling BeamSwitchTiming-r16 from the set {224, 336} is optional. It is also correct that the UE behaves according to Rel-15 specification in all respects if the UE doesn’t include Rel-16 capability or doesn’t receive parameter enableBeamSwitchTiming-r16. In addition, the following answers are irrelevant to aperiodic TRS. Below please find RAN1’s Answers to Questions A, B and C.

- For Rel-16 UEs, the UE signals beamSwitchTiming and beamSwitchTiming-r16. The first one, i.e., the Rel-15 capability signaling from the set {14, 28, 48}, is always signaled (if this capability needs to be reported). The second one, i.e., the newly introduced Rel-16 capability signaling from the set {224, 336}, is optional.

- If the UE doesn’t include the Rel-16 capability, it behaves in all respects as the Rel-15 UEs above.

- If the UE does include the Rel-16 capability, then, the UE may or may not receive configured parameter enableBeamSwitchTiming-r16 in the RRC configuration:

  - If the UE doesn’t receive parameter enableBeamSwitchTiming-r16, it behaves in all respect as the Rel-15 UE above.

  - If the UE receives parameter enableBeamSwitchTiming-r16, then, the UE does the following:

    - Question A: For CSI-RS configured with repetition and set to “ON”, whether the UE indicates switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signaling or the default value of 48?

    - Question B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling?

    - Question C: For CSI-RS configured without repetition, whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling?

Answer A: for CSI-RS configured with repetition and set to “ON”, when the Rel-16 UE capability signaling indicates one of the values {224, 336}, the beam switch time is equal to [48 or the same as signaled value from the set {224, 336}].

Answer B: for CSI-RS configured with repetition and set to “OFF”, the beam switch time is equal to [48 or the signaled value in Rel-15 capability signaling], if reported.

Answer C: for CSI-RS configured without repetition, the beam switch time is equal to [48 or the signaled value in Rel-15 capability signaling,] if reported.

Observation from first round of discussion,

6 companies are in favor of (Option 1)

Answer A: the same as signaled value from the set {224, 336}

Answer B: 48

Answer C: 48

3 companies are in favor of (Option 2)

Answer A: 48

Answer B: the signaled value in Rel-15 capability signaling

Answer C: the signaled value in Rel-15 capability signaling

| company | comments |
| --- | --- |

- 2. Actions:

- To RAN WG2

- RAN1 respectfully requests RAN2 to take above answers into account.

- 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #104-e		25 Jan – 5 Feb 2021				e-Meeting

TSG RAN WG1 Meeting #104-bis-e		12 – 20 Apr 2021					e-Meeting