| 3GPP TR 36.763 V0.0.0 (2020-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Non-Terrestrial Networks overview and scenarios	10

4.1	Non-Terrestrial Networks overview	10

4.2	Non-Terrestrial Networks reference scenarios	10

5	Radio Layer 1 issues and related solutions	10

5.1	IoT NTN Reference Parameters	10

5.2	Link Budget Analysis	10

5.3	Timing Relationships	10

5.4	UL Synchronization	10

6	User Plane Aspects	10

6.1	Random Access Procedure	10

6.2	MAC Timers	10

6.3	HARQ	11

7	Control Plane Aspects	11

7.1	System Information Enhancements	11

7.2	Idle Mobility Enhancements	11

7.3	Radio Link Failure Enhancements (NB-IoT Only)	11

7.4	Connected Mobility Enhancements (eMTC only)	11

7.5	Tracking Area Update Enhancements	11

8	Recommendations on the way forward	11

8.1	Recommendations from RAN1	11

8.2	Recommendations from RAN2	11

Annex A:	Satellite ephemeris	12

Annex B:	KPI and evaluation assumptions	12

Annex C (Informative): Change history	13

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

TBA

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)"

[3]	3GPP TS 38.821 v16.0.0: " Solutions for NR to support non-terrestrial networks (NTN) (Release 16)"

[4]	TBA

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Availability: % of time during which the RAN is available for the targeted communication. Unavailable communication for shorter period than [Y] ms shall not be counted. The RAN may contain several access network components among which an NTN to achieve multi-connectivity or link aggregation.

Feeder link: Wireless link between NTN Gateway and satellite

Geostationary Earth orbit: Circular orbit at 35,786 km above the Earth's equator and following the direction of the Earth's rotation. An object in such an orbit has an orbital period equal to the Earth's rotational period and thus appears motionless, at a fixed position in the sky, to ground observers.

Low Earth Orbit: Orbit around the Earth with an altitude between 300 km, and 1500 km.

Medium Earth Orbit: region of space around the Earth above low Earth orbit and below geostationary Earth Orbit.

Minimum Elevation angle: minimum angle under which the satellite or UAS platform can be seen by a terminal.

Mobile Services: a radio-communication service between mobile and land stations, or between mobile stations

Mobile Satellite Services: A radio-communication service between mobile earth stations and one or more space stations, or between space stations used by this service; or between mobile earth stations by means of one or more space stations

Non-Geostationary Satellites: Satellites (LEO and MEO) orbiting around the Earth with a period that varies approximately between 1.5 hour and 10 hours. It is necessary to have a constellation of several Non-Geostationary satellites associated with handover mechanisms to ensure a service continuity.

Non-terrestrial networks: Networks, or segments of networks, using an airborne or space-borne vehicle to embark a transmission equipment relay node or base station.

NTN-gateway: an earth station or gateway is located at the surface of Earth, and providing sufficient RF power and RF sensitivity for accessing to the satellite. NTN Gateway is a transport network layer (TNL) node.

On Board processing: digital processing carried out on uplink RF signals aboard a satellite or an aerial.

On board NTN eNB: eNB implemented in the regenerative payload on board a satellite.

On ground NTN eNB: eNB of a transparent satellite payload implemented on ground.

One-way latency: time required to propagate through a telecommunication system from a terminal to the public data network or from the public data network to the terminal. This is especially used for voice and video conference applications.

Round Trip Delay: time required for a signal to travel from a terminal to the sat-gateway or from the sat-gateway to the terminal and back. This is especially used for web-based applications.

Satellite: a space-borne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO), Medium-Earth Orbit (MEO), or Geostationary Earth Orbit (GEO).

Satellite beam: A beam generated by an antenna on-board a satellite

Service link: Radio link between satellite and UE

Transparent payload: payload that changes the frequency carrier of the uplink RF signal, filters and amplifies it before transmitting it on the downlink

User Connectivity: capability to establish and maintain data / voice / video transfer between networks and Terminals

User Throughput: data rate provided to a terminal

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ECEF	Earth-Centered, Earth-Fixed

EIRP	Equivalent Isotropic Radiated Power

GEO	Geostationary Earth Orbiting

eNB	4G Node B

GW	Gateway

LEO	Low Earth Orbiting

Mbps	Mega bit per second

MS	Mobile Services

MSS	Mobile Satellite Services

NGEO	Non-Geostationary Earth Orbiting

NTN	Non-Terrestrial Network

RAN	Radio Access Network

RTD	Round Trip Delay

SNR	Signal-to-Noise Ratio

TLE	Two-Line Element

Rx	Receiver

UE	User Equipment

# 4	IoT Non-Terrestrial Networks overview and scenarios

## 4.1	IoT Non-Terrestrial Networks overview

Editor’s notes: This section can include IoT NTN specific overview within scope of IoT NTN study Item description.

## 4.2	IoT Non-Terrestrial Networks reference scenarios

Editor’s notes: This section can include IoT NTN specific scenarios in addition to scenarios common to NR NTN in TR 38.821

# 5	Radio Layer 1 issues and related solutions

## 5.1	IoT NTN Reference Parameters

Editor’s notes: This section can include IoT NTN specific parameters in addition to parameters common to NR NTN in TR 38.821

## 5.2	Link Budget Analysis

Editor’s notes: This section can include link budget analysis with parameter sets specific to IoT NTN scenarios,

## 5.3	Timing Relationships

Editor’s notes: This section should aim to maximize synergies with timing relationships enhancements in TR 38.821 with adaptations for IoT NTN as needed.

## 5.4	UL Synchronization

Editor’s notes: This section should aim to maximize synergies with UL synchronization enhancements in TR 38.821 with adaptations for IoT NTN as needed.

# 6	User Plane Aspects

## 6.1	MAC

### 6.1.1	Random Access Procedure

Editor’s notes: This section should aim to maximize synergies with Random Access procedure enhancements in TR 38.821 with adaptations for IoT NTN as needed.

### 6.1.2	Discontinuous Reception (DRX)

Editor’s notes: This section should aim to maximize synergies with DRX enhancements in TR 38.821 with adaptations for IoT NTN as needed.

### 6.1.3	Scheduling Request

Editor’s notes: This section should aim to maximize synergies with Scheduling Request enhancements in TR 38.821 with adaptations for IoT NTN as needed.

### 6.1.4	HARQ

Editor’s notes: This section should aim to maximize synergies with HARQ enhancements in TR 38.821 with adaptations for IoT NTN as needed.

## 6.2	RLC

### 6.2.1	Status Reporting

Editor’s notes: This section should aim to maximize synergies with Status Reporting enhancements in TR 38.821 with adaptations for IoT NTN as needed.

### 6.2.2	RLC Sequence Numbers

Editor’s notes: This section should aim to maximize synergies with HARQ enhancements in TR 38.821 with adaptations for IoT NTN as needed.

## 6.3	PDCP

### 6.3.1	SDU Discard

Editor’s notes: This section should aim to maximize synergies with discardTimer in SDU discard enhancements in TR 38.821 with adaptations for IoT NTN as needed.

### 6.3.2	Reordering and In-order Delivery

Editor’s notes: This section should aim to maximize synergies with timer t-Reordering in reordering and in-order delivery enhancements in TR 38.821 with adaptations for IoT NTN as needed.

### 6.3.3	PDCP Sequence Number and Window Size

Editor’s notes: This section should aim to maximize synergies with PDCP Sequence Number and Window Size enhancements in TR 38.821 with adaptations for IoT NTN as needed.

# 7	Control Plane Aspects

## 7.1	System Information

Editor’s notes: This section should aim to maximize synergies with System Information enhancements in TR 38.821 with adaptations for IoT NTN as needed. In particular ephemeris data and satellite specific system information.

## 7.2	Idle Mobility

Editor’s notes: This section should aim to maximize synergies with Idle Mobility enhancements in TR 38.821 with adaptations for IoT NTN as needed.

## 7.3	Tracking Area Update

Editor’s notes: This section should aim to maximize synergies with Tracking Area Update enhancements in TR 38.821 with adaptations for IoT NTN as needed. In particular potential paging issues should be considered for IoT NTN.

## 7.4	Radio Link Failure

Editor’s notes: This section should aim to maximize synergies with Radio Link Failure for connected NB-IoT device with adaptations for IoT NTN as needed.

## 7.5	Connected Mobility

Editor’s notes: This section should aim to maximize synergies with mobility enhancements in TR 38.821 with adaptations for connected eMTC device IoT NTN as needed.

# 8	Recommendations on the way forward

## 8.1	Recommendations from RAN1

TBA

## 8.2	Recommendations from RAN2

TBA

# Annex A:	Satellite ephemeris

Editor’s notes: This section should aim to maximize synergies with Satellite ephemeris format in TR 38.821 with adaptations for IoT NTN as needed.

# Annex B:	KPI and evaluation assumptions

Editor’s notes: This section should aim to maximize synergies with KPI and evaluation for IoT NTN  (i.e. UE density per km2) in TR 38.821.

# Annex C (Informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-11 | RAN#90e | R3-185304 |  |  |  | Skeleton TR | 0.0.0 |