3GPP TSG RAN WG1 meeting #104-e                                                                             R1-2100315

e-Meeting, January 25th – February 5th, 2021

Title:	Draft reply LS on per table MCS range for mode-2

Response to:	R2-2010933

Release: 	Rel-16

Work Item: 	5G_V2X_NRSL-Core

Source:	CATT, GOHIGH [RAN1]

To: 	RAN2

Cc:

Contact person:	Rui Zhao

zhaorui@gohigh.com.cn

Attachments:	None

Overall Description

RAN1 would like to thank RAN2 for the LS R1-2009644[1] on per table MCS range for mode-2.

Q1: Should the MCS ranges for mode-2 operation (i.e., in the PSSCH transmission parameter table based on UE speed and synchronization source, and the PSSCH transmission parameter table based on CBR and priority) be defined per MCS table?

Answer: Yes, the MCS ranges for mode-2 operation (i.e., in the PSSCH transmission parameter table based on UE speed and synchronization source, and the PSSCH transmission parameter table based on CBR and priority) should be defined per MCS table.

In RAN1#98bis meeting, the following agreements were archived:

Agreements: - Congestion control can restrict the values of at least the following PSSCH/PSCCH TX parameters per resource pool: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions – already agreed in mode 2 AI - Upper bound of TX power (including zero TX power) - Congestion control can set an upper bound on channel occupancy ratio (CR), CRlimit. - Ranges/bounds of the transmission parameters and CRlimit are functions of QoS and CBR.

In RAN1#100b-e meeting, the following agreements were archived:

Agreement: - In addition to congestion control (in use or not in use), the following PSSCH/PSCCH TX parameters per resource pool can be restricted by reusing the same mechanism as in LTE: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions

According the above agreements, the MCS ranges based on congestion control, UE speed and synchronization source should be defined per MCS table.

Actions

To RAN2

ACTION: RAN1 respectfully asks RAN2 to take the above information into account.

Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #104b-e                                       12 Apr. – 20 Apr. 2021                       e-Meeting

TSG-RAN WG1 Meeting #105-e                                         19 May – 27 May 2021                       e-Meeting

Reference

  - R1-2009644, “LS on per-table MCS range for mode-2”, RAN2.