3GPP TSG RAN WG1 #104-e Meeting	R1-2101007

e-Meeting, 25th January – 5th February 2021

Agenda item:		8.1.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancements to enable inter-cell multi-TRP operations

Document for:		Discussion and Decision

# Introduction

In the Rel-17 work item for enhancements on MIMO for NR includes an objective to extend specification support for inter-cell multi-TRP operation was agreed together with few other objectives as follows [1]:

Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2:

  - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline

  - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception

  - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

  - Enhancement to support HST-SFN deployment scenario:

    - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission

    - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In RAN1#102-e, following agreements were made [2]:

Agreement

Study the following aspects of QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation.

- Details on configuration of non-serving cell RS;

- Allowed source and target RS types for RS transmitted from the non-serving cell TRP ;

- Allowed QCL types for RS transmitted from the non-serving cell TRP ;

- Measurement and reporting related to QCL /TCI enhancement except for that in 8.1.1, if any;

- Clarification on potential UE behavior for associating/multiplexing non-serving cell RS with other RS/channels;

- Other details not precluded.

In RAN1#103-e, following agreements were made [3]:

Agreement

For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information

- Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”

  - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement

  - FFS : Whether the association is explicit or implicit

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

In this contribution, we discuss further on the enhancement to support the inter-cell multi-TRP operation.

# Discussion

## 2.1 Details on the configuration of non-serving cell RS;

To enable configuration of non-serving cell RS within the current beam management framework, the TCI framework should be enhanced and extended. The SSB is used as main/initial QCL source in the beam management RS configuration, but the index value space is currently defined to be unique within a serving cell. Thus, to be able to use the non-serving cell SSB as QCL source, an extension or additional indication would be needed that associates a specific SSB index to a specific cell (serving and non-serving). As an example, a new field could be added to the referenceSignal (Figure 1. TCI State IE with an example enhancement with non-serving cell SSB information) parameter in the TCI State QCL Info.

For providing non-serving cell information, several options were also discussed in the last RAN1 meeting. One example was to use the SSB-InfoNcell-r16 field which was defined in the Rel-16 for positioning purposes as a starting point for configuring non-serving cell RS in inter-cell mTRP. SSB-InfoNcell-r16 includes values for ssb-index (ssb-IndexNcell-r16) and non-serving cell identifier (physicalCellId). In addition the SSB-Configuration-r16 field in SSB-InfoNcell-r16 allows full configuration of SSB information.

Observation 1: SSB is the main QCL source for beam management reference signals.

Observation 2: Associating SSB with a cell-specific identifier enables configuration of non-serving cell RS within the beam management framework.

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                                                   OPTIONAL,   -- Need R

...

}

QCL-Info ::=                        SEQUENCE {

cell                                ServCellIndex           OPTIONAL,  -- Need R

bwp-Id                              BWP-Id                  OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal                     CHOICE {

csi-rs                              NZP-CSI-RS-ResourceId,

ssb                                 SSB-Index

ssb-non-serving-cell::=           SEQUENCE {

physicalCellId                 PhysCellId,

ssb                            SSB-Index

}

},

qcl-Type                            ENUMERATED {typeA, typeB, typeC, typeD},

...

}

Figure 1. TCI State IE with an example enhancement with non-serving cell SSB information

Enhancing TCI State with a parameter that associates SSB index to an index space of a specific cell enables further the configuration of NZP-CSI-RS as non-serving cell RS. Configuring the TCI State with a non-serving cell SSB as a QCL source (TCIState ID, Figure 2. NZP-CSI-RS IE) for NZP-CSI-RS can be used to associate the NZP-CSI-RS to a non-serving cell without defining additional fields for the NZP-CSI-RS-Resource or Resource set. In case the QCL source for NZP-CSI-RS is another NZP-CSI-RS the QCL chain up to the SSB can be used to determine the associated cell for the RS. In addition to referenceSignal parameter in TCI State IE, similar updates would be needed for other IEs such as SRS-SpatialRelationInfo, PUSCH-PathlossReferenceRS-r16, PUSCH-PathlossReferenceRS, PUCCH-SpatialRelationInfo and PUCCH-PathlossReferenceRS-r16.

Observation 3: To associate NZP-CSI-RS with a non-serving cell, a QCL source (e.g. SSB) associated with non-serving cell identifier can be used.

Observation 4: The referenceSignal parameter is used for SRS-SpatialRelationInfo, PUSCH-PathlossReferenceRS-r16, PUSCH-PathlossReferenceRS, PUCCH-SpatialRelationInfo and PUCCH-PathlossReferenceRS-r16.

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}  OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                          OPTIONAL,   -- Cond Periodic

...

}

Figure 2. NZP-CSI-RS IE.

Proposal 1: To configure SSB as non-serving cell RS, indicate the associated cell (PCI) and SSB-index for the SSB in the referenceSignal parameter.

Proposal 2: To configure NZP-CSI-RS resource as non-serving cell RS, configure the RS with a QCL source RS that is associated with a non-serving cell.

## 2.2 Measurement and reporting related to QCL /TCI enhancement

Following high-level agreement was made in multibeam (Issue 1a) regarding the measurements and reporting of non-serving cells. In our view, both L1/L2 centric mobility and inter-cell mTRP should share the same methods for QCL/TCI enhancement for measurement and reporting. The following proposals for measurement and reporting related enchancent for non-serving-cell signals are also discussed in our contribution in AI 8.1.1.

  - In RAN1#103-e, depending on the outcome of 2a), further identify additional components –along with the associated alternatives –required for supporting inter-cell mobility based on the same unified TCI framework as that for intra-cell mobility (including dynamic TCI state update signaling), including

    - Method(s) for incorporating non-serving cell information associated with TCI

    - Method(s) for DL measurements and UE reporting (e.g. L1-RSRP) associated with non-serving cell(s)

    - UE behavior for reception of signals and non-UE-specific control and data channels associated with non-serving cell(s)

    - UL-related enhancements, e.g. related to RA procedure including TA

    - Beam-level event-driven mechanism for L1/L2-centric inter-cell mobility

To enable beam management measurements and reporting on the non-serving cell (NSC), the non-serving cell SSB should be able to be indicated in the CSI-SSB-ResourceSet. The CSI-SSB-ResourceSet list contains SSB-index which implicitly refers to the serving cell SSB-index space and thus to support SSB-based beam management measurements on non-serving cells it should be possible to indicate the associated cell for the SSB-index space in the information element. To indicate the associated cell for SSB or set of SSBs in CSI-SSB-ResourceSet the PCI can be used.

Observation 5: SSB based measurements can be supported by BM framework by associating the SSBs with a cell-specific identifier.

From 38.331 (CSI-SSB-ResourceSet):

The IE CSI-SSB-ResourceSet is used to configure one SS/PBCH block resource set which refers to SS/PBCH as indicated in ServingCellConfigCommon.

CSI-SSB-ResourceSet information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESET-START

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...

}

-- TAG-CSI-SSB-RESOURCESET-STOP

-- ASN1STOP

To support inter-cell NZP-CSI-RS measurements within a beam management framework requires the association of NZP-CSI-RS with the non-serving cell. As discussed in Section 2.1, the NZP-CSI-RS may require a TCI state as QCL source where the TCI state can include the association of SSB with a non-serving cell identifier. Determining the associated cell for NZP-CSI-RS resource itself can be based on the QCL source (TCI State ID) indicated by the qcl-InfoPeriodicCSI-RS. Thus, a separate/new parameters may not be needed for NZP-Resource or resource set to associate the reference signals with non-serving cell.

Similarly for the measurement purposes, as the TCI state is used a QCL source for NZP-CSI-RS, the TCI State list should be allowed include RS that are associated with non-serving cell.

Since the RS configuration would implicitly indicate the cell for which the measurements are reported, it is FFS whether there is a need to enhance the reporting configuration itself.

Observation 6: NZP-CSI-RS measurements can be supported by BM framework by configuring the SSB with a cell-specific identifier as a QCL source in the TCI State.

Proposal 3: For L1 SSB based beam measurements and reporting, enhance the CSI-SSB-ResourceSet IE to associate set of SSBs with a cell-specific identifier (PCI).

Proposal 4: For non-serving cell CSI-RS measurements, configure the NZP-CSI-RS with a QCL source RS that is associated with a non-serving cell identifier.

## 2.3 Inter Cell mTRP Configuration and PDCCH Beam Indication

To follow release 16 mDCI framework, the (CORESETs) non-serving cell TRP can be considered as a part of the active BWP configuration.

For UE to understand that a CORESET or set of CORESETs are transmitted from a TRP associated with non-serving cell, in one option, UE could be explicitly indicated that a CORESET(s) under the same CORESETpoolIndex are associated with a non-serving cell. Alternatively, UE can determine the inter-cell mTRP configuration/PDCCH reception from a non-serving cell implicitly based on the RS or the QCL source RS indicated by the active TCI state for PDCCH (in multi-DCI MTRP).

Figure 3. CORESET(s) of the serving cell and CORESETs of the non-serving cell (inter-cell MTRP cell) under the same PDCCH-config

For PDCCH reception, when a UE receives activation for a TCI state for PDCCH, it can determine based on the QCL chain (QCL source info) that an RS indicated by the active TCI state is associated with non-serving cell. The TCI State framework would need to be updated so that e.g. SSB can be associated with a specific PCI.

Observation 7: UE can determine the inter-cell mTRP configuration/PDCCH reception through the QCL source for the RS indicated by active TCI state for a CORESET.

Thus, in view of the observation, beam indication enhancement is not necessary for configuring the reception of PDCCH from a non-serving cell. The CORESET configuration can follow the rel16 mTRP principles, e.g. non-serving cell CORESET can be pooled using the CORESETpoolIndex and the reception assumption for the CORESET is based on the active TCI state; the RS (or the QCL source RS through he QCL chain) indicated by the active TCI State for a CORESET determines the reception assumption for PDCCH i.e. whether the PDCCH is to be received from a serving cell or non-serving cell.

Proposal 5: For inter-cell multi-DCI based multi-TRP support, the CORESETs of non-serving cell are pooled under the same CORESETpoolIndex.

# Conclusion

In this contribution, we discuss the support of inter-cell multi-TRP/panel transmission. The following observations and proposals are made.

Observation 1: SSB is the main QCL source for beam management reference signals.

Observation 2: Associating SSB with a cell-specific identifier enables configuration of non-serving cell RS within the beam management framework.

Observation 3: To associate NZP-CSI-RS with a non-serving cell, a QCL source (e.g. SSB) associated with non-serving cell identifier can be used.

Observation 4: The referenceSignal parameter is used for SRS-SpatialRelationInfo, PUSCH-PathlossReferenceRS-r16, PUSCH-PathlossReferenceRS, PUCCH-SpatialRelationInfo and PUCCH-PathlossReferenceRS-r16.

Observation 5: SSB based measurements can be supported by BM framework by associating the SSBs with a cell-specific identifier.

Observation 6: NZP-CSI-RS measurements can be supported by BM framework by configuring the SSB with a cell-specific identifier as a QCL source in the TCI State.

Observation 7: UE can determine the inter-cell mTRP configuration/PDCCH reception through the QCL source for the RS indicated by active TCI state for a CORESET.

Proposal 1: To configure SSB as non-serving cell RS, indicate the associated cell (PCI) and SSB-index for the SSB in the referenceSignal parameter.

Proposal 2: To configure NZP-CSI-RS resource as non-serving cell RS, configure the RS with a QCL source RS that is associated with a non-serving cell.

Proposal 3: For L1 SSB based beam measurements and reporting, enhance the CSI-SSB-ResourceSet IE to associate set of SSBs with a cell-specific identifier (PCI).

Proposal 4: For non-serving cell CSI-RS measurements, configure the NZP-CSI-RS with a QCL source RS that is associated with a non-serving cell identifier.

Proposal 5: For inter-cell multi-DCI based multi-TRP support, the CORESETs of non-serving cell are pooled under the same CORESETpoolIndex.

# References

- RP-193133, “Further enhancements on MIMO for NR,” 3GPP WID RAN1 #86.

- RAN1 Chairman’s Notes 102-e

- RAN1 Chairman’s Notes 103-e