3GPP TSG-RAN WG1 Meeting #103-e	R1-2101758

e-Meeting, January 25th– February 5th, 2021

Agenda Item:	7.2.8

Source:	Ericsson

Title:	Maintenance of NR positioning support

Document for:	Discussion, Decision

# Introduction

This contribution covers the maintenance issues remaining for NR positioning support.

## Maintenance of NR positioning support

## SRS transmit power

In 38.213 v16.4.0, the sounding reference signals description for the UE behaviour is split between SRS for MIMO and SRS for positioning. However, in the header section, the applicability for the power split is unclear as the specification does not distinguish between SRS for Mimo and SRS for postioning for this part. Clearly the parameter  is refering to the Mimo SRS, we propose therefore to clarify with the following TP:

TP 1 ---------- unchanged text omitted--------------- ## 7.3 Sounding reference signals For SRS configured with SRS-ResourceSet, a UE splits a linear value of the transmit power on active UL BWP of carrier of serving cell equally across the configured antenna ports for SRS. ---------- unchanged text omitted---------------

Endorse the proposed TP 1

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Endorse the proposed TP 1