3GPP TSG RAN WG1 Meeting #104-e                                     R1-2102672

e-Meeting, April 12th – 20th, 2021

Source:               TCL Communication

Title:                    Reduced maximum UE bandwidth for RedCap

Agenda item:      8.6.1.1

Document for:    Discussion and Decision

- Introduction

In 3GPP TSG RAN#90-e meeting, Rel.17 supporting of reduced capability NR devices [1] was agreed. The working scope includes the following objectives.

This WI has the following objectives: Additional objectives may be added in RAN#91e based on the finalization of the RAN2 part of the RedCap SI. - Specify support for the following UE complexity reduction features [RAN1, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access of 20 MHz is supported. The possibility of, and any associated conditions for, optional support of a wider bandwidth up to 40MHz after initial access for this case will be further discussed at RAN#91e. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz - No other relaxations of maximum modulation order are specified for a RedCap UE.

In 3GPP TSG RAN WG1#104-e meeting, the following agreements about RACH occasions are achieved:

Agreements: - Study further how to enable/support that a RACH occasion associated with the best SSB falls within the RedCap UE bandwidth, with the following options: - Option 1: Proper RF-retuning for RedCap - Option 2: Separate initial UL BWP(s) for RedCap UEs - Option 3: gNB configuration (e.g., restrictions on existing PRACH configurations, or FDM-ed ROs, or always restricting the initial UL BWP to within RedCap UE bandwidth) - Option 4: Dedicated PRACH configurations (e.g., ROs) for RedCap UEs - Other options are not precluded

This contribution discusses the potential solutions to enable a RACH occasion associated with the best SSB falling within the RedCap UE bandwidth.

- Discussion

If the Redcap UEs are configured with 8-FDMed Rach occasions, the required BW for 8 FDMed RACH occasions for 30 or 120 kHz SCS is greater than the UE bandwidth (20 MHz in FR1 and 100 MHz in FR2). A RACH occasion associated with the best SSB can fall outside the UE bandwidth.

During the initial access to a cell, it is beneficial if a gNB knows which beam is the strongest for UE reconceiving.  One SSB corresponds to one beam, therefore the SSB RSRP is used to determine which beam is strongest. Each SSB is associated with one or more Rach Occasions, and once a SSB is selected, the ROs that can be selected is determined. In NR specifications, a UE is provided by higher layer with a set of SSB RSRP thresholds used for selecting a proper SSB, and the information about the number of SSBs per RACH occasion.

Figure 1 shows how to select SSB and RO. It can be seen that the SSB RSRP threshold is the only factor in selecting the SSB. If more than one SSB’s RSRP are greater than the RSRP threshold, the SSB may be selected randomly among them. If more than one ROs associated with the selected SSB, select one associated RO randomly with equal probability; otherwise select the associated RO.

Figure 1 Steps of the existing random access procedure

Option#1 and option#2, which agreed in RAN1#104e, imply that step 1 and step 2 are unchanged, but the frequency retuning is required between step 2 and step 3.  A back-retuning might be performed after transmitting preamble. As we know, the frequency retuning requires transition time. This complicates the initial access procedure and spends more time, thus may increase access latency. The configurations in option#3 and option#4 can avoid the selected RO outside the UE bandwidth, but cause inflexibility or fragmentation of PUSCH resources.

By the current mechanism, the RSRP threshold is the only consideration for selecting SSB, and UE may select an improper SSB (e.g. SSB3 in Figure 2a/2b) and the associated RO.  If the frequency position of the associated RO is also taken as a factor, SSB3 can be excluded and SSB2 will be selected. Therefore, optimizing the selection of SSB (step 1) may be a feasible option.

Observation 1：By the current specification, UE may select an improper SSB-RO even with a proper SSB-RO.

Proposal 1: Whether the associated RO is within the UE bandwidth is a consideration for SSB selection.

For one SSB is associated more than one RO, under the current specification for selecting RO, UE also may select an improper RO(RO#8 in Figure 3) even with a proper RO.

Figure 3 SSB associated with multiple ROs

Observation 2：By a mechanism with random equal probability, UE may select an improper RO even with a proper SSB-RO.

Proposal 2: Whether the associated RO is within the UE bandwidth is a consideration for RO selection.

- Conclusion

In this contribution the following proposals and observations have been made:

Observation 1：By the current specification, UE may select an improper SSB-RO even with a proper SSB-RO.

Proposal 1: Whether the associated RO is within the UE bandwidth is a consideration for SSB selection.

Observation 2：By a mechanism with random equal probability, UE may select an improper RO even with a proper RO.

Proposal 2: Whether the associated RO is within the UE bandwidth is a consideration for RO selection.

References

[1] RP-202701, “New WID on support of reduced capability NR devices”

[2] Meeting report, “Chairman’s Notes RAN1#104-e”

[3] TS 38.321, “Medium Access Control (MAC) protocol specification”