3GPP TSG RAN WG1 #104b-e                                                             R1-2103032

e-Meeting, 12th – 20th April, 2021

Source: 	Intel Corporation

Title:	On timing relationship enhancements for NTN

Agenda item:	8.4.1

Document for:	Discussion and Decision

# Introduction

At the 3GPP TSG RAN meeting #86 the new work item (WI) on solutions for NR to support non-terrestrial networks (NTN) was approved in [1]. The NTN WI aims to specify enhancements identified for NR NTN (especially LEO and GEO) with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles:

- FDD is assumed for core specification work for NR-NTN.

  - NOTE: This does not imply that TDD cannot be used for relevant scenarios e.g. HAPS, ATG

- Earth fixed Tracking area is assumed with Earth fixed and moving cells

- UEs with GNSS capabilities are assumed.

According to 3GPP TR 38.821 [2] changes for NR specification are required to support NTN due to long propagation delays, Doppler effects and moving cells. Multiple features of NR specification should be enhanced to support NTN including timing relationship. In this contribution timing relationships enhancements for NR to support NTN are discussed.

# Discussion

Timing relationships enhancements were discussed in the study item on solutions for NR to support NTN [2]. For NTN it is assumed that large timing advance (TA) is applied by the UE in order to compensate large propagation delay. In order to avoid the case where UE should transmit PUSCH before reception of UL grant for that PUSCH, slot offset from the DCI to the target slot of PUSCH reception should be larger than TA applied by the UE. In order to solve this issue and several similar issues it was agreed to support common slot offset K_offset [3].

Agreement: - Introduce K_offset to enhance the following timing relationships: - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH). - The transmission timing of RAR grant scheduled PUSCH. - The transmission timing of HARQ-ACK on PUCCH. - The CSI reference resource timing. - The transmission timing of aperiodic SRS. - Note: Additional timing relationships that require K_offset of the same or different values can be further identified.

Details of configuration for K_offset value are still under discussion in RAN1. At the RAN1#103-e meeting [4] it was agreed that at least a cell specific K_offset configuration is supported for initial access. Beam specific K_offset configured for initial access is FFS. Based on values specified in table 4.2-2 from [2], the maximum differential delay within a cell is 10.3 ms for GEO and 3.18 ms for LEO at 1200 km. The number of slots within the maximum differential delay is represented in the below table for different subcarrier spacing (SCS).

Table 1. Number of slots to cover the maximum differential delay within a cell

| Orbit\SCS | 15 kHz | 30 kHz | 60 kHz | 120 kHz |
| --- | --- | --- | --- | --- |
| GEO | 11 slots | 21 slots | 42 slots | 83 slots |
| LEO @1200km | 4 slots | 7 slots | 13 slots | 26 slots |

As it can be seen from the above table, maximal number of slots to cover the differential delay within a cell is 83. If difference of beam-specific Koffset relative to cell-specific Koffset is indicated, 7 bits per beam is required. In order to decrease the overhead, reporting of slot offset difference between adjacent beams can be considered. Furthermore, subsampling of value range can be considered in order to further decrease the overhead. Thus, 2 bits per beam can be enough in order to support beam-specific Koffset for initial access.

Proposal 1:

- Support beam specific K_offset configured in system information for initial access

  - Support indication of K_offset difference between adjacent beams with up to X bits (e.g. X = 2)

At the RAN1#103-e meeting [4] the following agreement was made in agenda item for enhancements on UL time and frequency synchronization.

Agreement: - In NTN, the network may broadcast - A common timing offset value - FFS details of the common timing offset - FFS: A common timing drift rate - Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows: Where: is derived from the User specific TA self-estimation is derived at least from the common timing offset value if broadcasted by the network. The granularity of and whether is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed. - depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2. - is specified in TS 38.211 section 4.1. - Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

In the above agreement it is stated that indication of common time offset (common TA) is supported. Since the common TA and common slot offset (K_offset) are related to common propagation delay, the value of K_offset can be fully or partially derived from common TA value. For example, for deployment with transparent payload common TA can be used in order to compensate feeder link delay, K_offset for this case is used to compensate maximum delay for the deployment which is sum of feeder link delay and maximum service link delay. Thus, K_offset can be represented as sum of common TA (in slots) and common slot offset for service link.

Since the common slot offset K_offset is a function of common propagation delay which is determined by the deployment it should be the same for all physical channels and procedure which utilise K_offset.

Proposal 2:

- Common timing advance (TA) value should be used to determine common slot offset (K_offset)

- K_offset value should be common for all applicable physical layer procedures

Since the common propagation delay for the same cell and same beam can be changing in time (e.g. for non-GEO satellite), the value of common slot offset K_offset needs to be updated after initial access. At the last RAN1 meeting [5] it was agreed to support update of K_offset after initial access, however, details on K_offset update are not agreed. In order to improve the delay characteristics of UL transmission and CSI reference resource K_offset may be updated from cell-specific to beam-specific or from beam-specific to UE-specific. However, currently it is not clear if UE-specific K_offset is necessary considering that PUSCH and PUCCH supports flexible configuration of slot offset (K1 and K2 values) in UE-specific manner.

Proposal 3:

- Update by the gNB after initial access via higher layer signaling (RRC or MAC CE) is supported for cell-specific and beam-specific K_offset

# Conclusion

In this contribution aspects related to timing relationships for NTN were discussed. The following proposals are made.

Proposal 1:

- Support beam specific K_offset configured in system information for initial access

  - Support indication of K_offset difference between adjacent beams with up to X bits (e.g. X = 2)

Proposal 2:

- Common timing advance (TA) value should be used to determine common slot offset (K_offset)

- K_offset value should be common for all applicable physical layer procedures

Proposal 3:

- Update by the gNB after initial access via higher layer signaling (RRC or MAC CE) is supported for cell-specific and beam-specific K_offset

# References

- RP-193234 Solutions for NR to support non-terrestrial networks (NTN)

- 3GPP TR 38.821 V16.0.0 (2019-12); 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16)

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #102-e, e-Meeting

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #103-e, e-Meeting

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting