3GPP TSG RAN WG1 Meeting #104bis-e    	          	     	  R1-2103375

e-Meeting, April 12th – 20th, 2021

Agenda Item:	7.2.4

Source: 	LG Electronics

Title: 	Discussion on essential corrections in physical layer structure

Document for:	Discussion and decision

Introduction

In this contribution, we provide text proposals for NR specification to capture the agreements on physical layer structure for NR sidelink made in Rel-16 NR V2X WI. Furthermore, we continue to discuss remaining issues on physical layer structure as follows:

- Issue 1: For RS sequence generation, whether the slot number is logical slot index or physical slot index considering in-coverage scenario with different serving cells.

- Issue 2: How to generate OFDM baseband signal for NR sidelink considering the center frequency/RB boundary of UL BWP.

Discussion

  - Reference Signal design

For PSCCH DMRS, PSSCH DMRS, sidelink CSI-RS, and cyclic shift hopping sequence generation, pseudo-random sequence generation is used with random seed of which value is determined based on higher layer signaled offset, PSCCH CRC, symbol index within a slot, and/or slot index within a frame. Meanwhile, it is necessary to clarify whether the slot index used for the sequence generation is logical slot index within a resource pool or physical slot index. In LTE V2X, PSSCH DMRS/scrambling sequence generator uses logical slot index. Considering that the sidelink communication between UEs camping on different serving cells, logical slot index would need to be used instead of physical slot index. To be specific, for paired-spectrum operation (FDD cells), time location of SFN0 for each cell could be different even though slot boundaries are aligned each other. In this case, for the same slot location, physical slot index would be different, so TX UE and RX UE would be have different understanding on the RS sequence. On the other hand, by using suitable resource pool configuration with respect to their own SFN0, it would be possible that the logical slot index is the same between UEs with different serving cells. Moreover, it could be beneficial to achieve sequence randomization between different resource pools.

In those points of views, it can be considered that the logical slot index within a resource pool is used for sequence generation for PSCCH DMRS, PSSCH DMRS, sidelink CSI-RS

Proposal 1: Capture that the logical slot index within a resource pool is used for generations of the following sequences in the specification:

- PSCCH DMRS

- PSSCH DMRS

- Sidelink CSI-RS

- Cyclic shift hopping for PSFCH

Proposal 2: Adopt following TP for TS38.211:

#### 8.3.4.2 PSFCH format 0 ##### 8.3.4.2.1 Sequence generation The sequence xn shall be generated according to xn=ru,vα,δn n=0,1,…,NscRB-1 where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions: - mcs is given by clause 16.3 of [5, TS 38.213]; - m0 is given by clause 16.3 of [5, TS 38.213]; - l is the OFDM symbol number in the PSFCH transmission where l=0 corresponds to the first OFDM symbol of the PSFCH transmission; - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213] - u=nID mod 30 and v=0 with nID given by the higher-layer parameter sl-PSFCH-HopID if configured; otherwise, u=0. - cinit=nID with nID given by the higher-layer parameter sl-PSFCH-HopID if configured; otherwise, cinit=0. - ns,fμ is the slot number in logical slots of a resource pool in the radio frame ##### <Unchanged part is omitted> ##### 8.4.1.1.1 Sequence generation The sequence rlm shall be generated according to rlm=121-2c2m+j121-2c2m+1 where the pseudo-random sequence cm is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with cinit=217Nsymbslotns,fμ+l+12NID+1+2NID mod 231 where l is the OFDM symbol number within the slot, ns,fμ is the slot number in logical slots of a resource pool within a frame, and NID=NIDX mod 216 where the quantity NIDX equals the decimal representation of CRC on the PSCCH associated with the PSSCH according to NIDX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in [4, TS 38.212]. <Unchanged part is omitted> ##### 8.4.1.3.1 Sequence generation The sequence rlm shall be generated according to rlm=121-2c2m+j121-2c2m+1 where the pseudo-random sequence cm is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with cinit=217Nsymbslotns,fμ+l+12NID+1+2NID mod 231 where - l is the OFDM symbol number within the slot, - ns,fμ is the slot number in logical slots of a resource pool within a frame, and - NID∈{0,1,…,65535} is given by the higher-layer parameter sl-DMRS-ScrambleID. <Unchanged part is omitted> ##### 8.4.1.5.2 Sequence generation The sequence rm shall be generated according to rm=121-2c2m+j121-2c2m+1 where the pseudo-random sequence ci is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialised with cinit=210Nsymbslotns,fμ+l+12nID+1+nID mod 231 at the start of each OFDM symbol where ns,fμ is the slot number in logical slots of a resource pool within a radio frame, l is the OFDM symbol number within a slot, and NID=NIDX mod 210 where the quantity NIDX equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the CSI-RS according to NIDX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in [4, TS 38.212].

  - OFDM baseband signal generation

In NR Uu link, more than one BWP can be supported, and each BWP can have different numerology. When the OFDM baseband signal is generated, a certain offset is applied to ensure the center of all the resource grids with different sub-carrier spacing are aligned as in Figure 1.

Figure 1: Example of alignment of center frequency of resource grids with different SCS.

To be specific, resource gird with SCS μ can be shifted such that its center is the same as that of resource grid with the largest configured subcarrier spacing. According to TS38.211, this alignment is done by offset k0μ.

### 5.3.1 OFDM baseband signal generation for all channels except PRACH and RIM-RS The time-continuous signal on antenna port p and subcarrier spacing configuration μ for OFDM symbol in a subframe for any physical channel or signal except PRACH is defined by sl(p,μ)t=sl(p,μ)ttstart,lμ≤t<tstart,lμ+Tsymb,lμ0otherwise sl(p,μ)t=k=0Ngrid,xsize,μNscRB-1ak,l(p,μ)ej2πk+k0μ-Ngrid,xsize,μNscRB2Δft-NCP,lμTc-tstart,lμ k0μ=Ngrid,xstart,μ+Ngrid,xsize,μ2NscRB-Ngrid,xstart,μ0+Ngrid,xsize,μ02NscRB2μ0-μ Tsymb,lμ=Nuμ+NCP,lμTc where t=0 at the start of the subframe, and - is given by clause 4.2; - is the subcarrier spacing configuration; - μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList.

In case of NR sidelink, only one SL BWP is configured, therefore, only one SCS value will be (pre)configured. In this case, the offset k0μ will be always 0 according to the current specification. Considering that the sidelink resource is parts of cell-specific UL resources and switching delay between active UL BWP and SL BWP, it would be preferable that the resource grid of UL and resource grid of SL are aligned in boundary. However, depending on the value of k0μ for UL, it is not always guaranteed that this alignment of resource grid between UL and SL for the same SCS.

Observation 1: When resource grids of UL and SL are not aligned in PRB boundary, it will cause switching delay between active UL BWP and SL BWP even though these BWP have the same numerology.

In the current stage, it is not preferable to introduce new higher layer signaling to adjust frequency location of SL resource grid for alignment with UL resource grid. Next, a UE can consider UL resource grid with the largest configured SCS value for OFDM baseband signal generation for the alignment, but it would not be possible for out-of-coverage UE. In other words, SL resource grid could be misaligned between in-coverage UE and out-of-coverage UE. In those points of view, for best effort, it can be considered that the network ensures the UL resource grid and SL resource gird are aligned in boundary. If there is no way to make the alignment between UL resource grid and SL resource grid for a given SCS, SL BWP will be deactivated. If any specification change including this restriction on the network configuration is not introduced, it can force that all the UEs are implemented to have no switching delay even for the case when resource grids of UL and SL are not aligned in PRB boundary. This situation is not desirable.

Proposal 3: A UE deactivates SL BWP when resource grid of sidelink and resource grid of uplink for a given SCS are not aligned in boundary.

Proposal 4: Adopt following TP for TS38.213:

# 16 UE procedures for sidelink A UE is provided by locationAndBandwidth-SL a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS38.211]. For a resource pool within the SL BWP, the UE is provided by numSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by subchannelsize. The first RB of the first sub-channel in the SL BWP is indicated by startRB-Subchannel. Available slots for a resource pool are provided by timeresourcepool and occur with a periodicity provided by 'periodResourcePool'. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by startSLsymbols and be within a number of consecutive symbols indicated by lengthSLsymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined. The UE expects to use a same numerology in the SL BWP and in an active UL BWP in a same carrier of a same cell. If the active UL BWP numerology is different than the SL BWP numerology, the SL BWP is deactivated. The UE expects that the difference of k0μ as described in [4, TS 38.211] between the SL BWP and an active UL BWP in a same carrier of a same cell is a multiple of NscRB. If the difference of k0μ between the SL BWP and the active UL BWP is not a multiple of NscRB, the SL BWP is deactivated. A UE transmitting using a Mode-1 grant uses the corresponding fields in SCI to reserve the next resource(s) allocated by the same grant. <Unchanged part is omitted>

Conclusions

In this contribution, some aspects of physical layer structure for NR V2X sidelink was discussed. Based on the above discussion, our proposals and observations are summarized as follows:

Observation 1: When resource grids of UL and SL are not aligned in PRB boundary, it will cause switching delay between active UL BWP and SL BWP even though these BWP have the same numerology.

Proposal 1: Capture that the logical slot index within a resource pool is used for generations of the following sequences in the specification:

- PSCCH DMRS

- PSSCH DMRS

- Sidelink CSI-RS

- Cyclic shift hopping for PSFCH

Proposal 2: Adopt following TP for TS38.211:

#### 8.3.4.2 PSFCH format 0 ##### 8.3.4.2.1 Sequence generation The sequence xn shall be generated according to xn=ru,vα,δn n=0,1,…,NscRB-1 where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions: - mcs is given by clause 16.3 of [5, TS 38.213]; - m0 is given by clause 16.3 of [5, TS 38.213]; - l is the OFDM symbol number in the PSFCH transmission where l=0 corresponds to the first OFDM symbol of the PSFCH transmission; - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213] - u=nID mod 30 and v=0 with nID given by the higher-layer parameter sl-PSFCH-HopID if configured; otherwise, u=0. - cinit=nID with nID given by the higher-layer parameter sl-PSFCH-HopID if configured; otherwise, cinit=0. - ns,fμ is the slot number in logical slots of a resource pool in the radio frame ##### <Unchanged part is omitted> ##### 8.4.1.1.1 Sequence generation The sequence rlm shall be generated according to rlm=121-2c2m+j121-2c2m+1 where the pseudo-random sequence cm is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with cinit=217Nsymbslotns,fμ+l+12NID+1+2NID mod 231 where l is the OFDM symbol number within the slot, ns,fμ is the slot number in logical slots of a resource pool within a frame, and NID=NIDX mod 216 where the quantity NIDX equals the decimal representation of CRC on the PSCCH associated with the PSSCH according to NIDX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in [4, TS 38.212]. <Unchanged part is omitted> ##### 8.4.1.3.1 Sequence generation The sequence rlm shall be generated according to rlm=121-2c2m+j121-2c2m+1 where the pseudo-random sequence cm is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with cinit=217Nsymbslotns,fμ+l+12NID+1+2NID mod 231 where - l is the OFDM symbol number within the slot, - ns,fμ is the slot number in logical slots of a resource pool within a frame, and - NID∈{0,1,…,65535} is given by the higher-layer parameter sl-DMRS-ScrambleID. <Unchanged part is omitted> ##### 8.4.1.5.2 Sequence generation The sequence rm shall be generated according to rm=121-2c2m+j121-2c2m+1 where the pseudo-random sequence ci is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialised with cinit=210Nsymbslotns,fμ+l+12nID+1+nID mod 231 at the start of each OFDM symbol where ns,fμ is the slot number in logical slots of a resource pool within a radio frame, l is the OFDM symbol number within a slot, and NID=NIDX mod 210 where the quantity NIDX equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the CSI-RS according to NIDX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in [4, TS 38.212].

Proposal 3: A UE deactivates SL BWP when resource grid of sidelink and resource grid of uplink for a given SCS are not aligned in boundary.

Proposal 4: Adopt following TP for TS38.213:

# 16 UE procedures for sidelink A UE is provided by locationAndBandwidth-SL a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS38.211]. For a resource pool within the SL BWP, the UE is provided by numSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by subchannelsize. The first RB of the first sub-channel in the SL BWP is indicated by startRB-Subchannel. Available slots for a resource pool are provided by timeresourcepool and occur with a periodicity provided by 'periodResourcePool'. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by startSLsymbols and be within a number of consecutive symbols indicated by lengthSLsymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined. The UE expects to use a same numerology in the SL BWP and in an active UL BWP in a same carrier of a same cell. If the active UL BWP numerology is different than the SL BWP numerology, the SL BWP is deactivated. The UE expects that the difference of k0μ as described in [4, TS 38.211] between the SL BWP and an active UL BWP in a same carrier of a same cell is a multiple of NscRB. If the difference of k0μ between the SL BWP and the active UL BWP is not a multiple of NscRB, the SL BWP is deactivated. A UE transmitting using a Mode-1 grant uses the corresponding fields in SCI to reserve the next resource(s) allocated by the same grant. <Unchanged part is omitted>

Reference