3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103872

E-meeting, April 12th – 20th, 2021

Agenda Item:	8.1.4

Source:	(Moderator) Huawei, HiSilicon

Title:	Summary of CSI enhancements for MTRP and FDD (Round 2)

Document for:	Discussion and Decision

# Agreements

Agreement

For rank=1, polarization-common based free-selection should be supported for W1.

- FFS: Whether there is a need to restrict the number of CSI-RS ports for which this is supported

Agreement

Support the indication of following RI combinations by a joint RI field for a NCJT measurement hypothesis in CSI part 1, when the maximal transmission layers is less than or equal to 4:

- {1, 1}, {1, 2}, {2,1}, {2,2}

- FFS: CBSR and/or RI restrictions per TRP or across TRPs

Agreement

With regarding to the maximal values of Nmax for N,  Ks,max  for Ks:

- Support of Nmax=2 is a UE optional feature

- Support of Ks,max=X is a UE optional feature

  - X can be up to 8 and other candidate values can be discussed as part of UE features

- FFS: Default value of Nmax, Ks,max

- FFS: Which combinations of N<=Nmax, Ks<=Ks,max are supported

# Proposals

FDD CSI

Proposal 2: Combinatorial coefficient is used for port selection for W1.

Yes：ZTE, Lenovo/Motorola, vivo, Intel, Ericsson, Nokia/NSB, Apple, CATT, OPPO, MTK, LG, DOCOMO

No: Samsung

Proposal 4-0: Confirm following WA of Wf for R17 PS CB

- Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters.

- FFS: whether the optionality is expressed in combination with values of K1

Yes：ZTE, Apple, Samsung, OPPO, Spreadtrum, LG, QC, vivo, MTK, Intel, [Nokia/NSB] with FFS

No: Lenovo/Motorola

Proposal 4-1: For Wf  in CN3*Mv, Mv = 2 is supported for R17 PS CB

- FFS: whether there is a need of restriction for certain number of CSI-RS ports

- FFS: whether Mv>2 is needed

Yes: Nokia/NSB, Lenovo/Motorola, DOCOMO, MTK, Intel, Ericsson

No: Samsung

Revised Proposal 8: A bitmap for indication non-zero coefficients should be supported for W2 with a compression coefficient beta<=1 whereas

- FFS values of beta < 1

- FFS: whether/how such a bitmap can be absent for specific codebook configuration parameters

- FFS: whether a bitmap is polarization-common or polarization-specific

- FFS: whether to report the total number of non-zero coefficients

- FFS: possible parameter combinations/dependence for beta with other PS CB parameters.

Yes: ZTE, Apple, vivo, MTK, Intel, Ericsson, Nokia/NSB

MTRP CSI

Revised Proposal 11: With regarding to for possible restriction between K1 and K2, down select from following Alternatives for K1 and K2

- Alt 1: K1=K2=Ks/2

- Alt 2: No restriction as long as K1+K2=Ks

[Alt 1:7 companies]: QC, DCM, VIVO, Nokia, Spreadtrum, OPPO, Apple

[Alt 2: 14 companies] Lenovo/MotM, Intel, MTK, Ericsson, LG, Futurewei, NEC, Samsung, CMCC, ZTE, CATT, HW, Hisi

Proposal 14-2: The UE may assume that QCL-Type D of CMRs associated with a NCJT measurement hypothesis are applied to the corresponding CSI-IM resource.

Support: VIVO, CATT, CMCC, Futurewei, LG, MTK, DCM, Ericsson

Not support: Apple, MotM

Proposal 15-2:  For the UE be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses (i.e. Option 2),

- Alt 1: Single CRI is reported whereas CRI bit size depends on total number of valid CMR pairs for NCJT measurement hypothesis and valid CMRs for single-TRP measurement hypotheses.

  - FFS further mapping mechanism between each CRI codepoint and Single-TRP/NCJT measurement hypothesis.

- Alt 2: additional UCI field is needed on top of CRI reporting. The gNB is informed that CRI reporting may correspond to either Single-TRP or NCJT measurement hypothesis.

- Alt 3: reuse RI reporting implicitly, e.g. depending on the number of reported RIs. The gNB is informed that CRI reporting may correspond to either Single-TRP or NCJT measurement hypothesis.

Alt 1 (17): QC, ZTE, Intel, Apple, Ericsson, Docomo, LG, Fraunhofer IIS, Fraunhofer HHI, Nokia/NSB, Futurewei, Samsung, InterDigital, MediaTek, Huawei, HiSilicon

Alt 2 (1): Vivo

Alt 3 (3): Lenovo/MotM, Oppo

Revised Proposal 17:  For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, an NCJT CSI hypothesis based on a pair of CMRs is assuming to occupy two CPUs, two active NZP CSI-RS resources, and a number of active ports corresponding to both CMRs.

- If a NZP CSI-RS resource is referred X times by CMR pairs for NCJT measurement hypothesis and CMR for Single-TRP measurement hypothesis, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times for active resources and active ports.

Revised Proposal 18:  A 2-part CSI report is supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis with following clarifications:

- Within CSI part 1

  - CRI, RI, WB CQI and SB CQI for the first CW are reported with consistent payload and zero padding (if needed). FFS further details

  - FFS whether RI can be shared between NCJT CSI and single-TRP CSIs to reduce CSI feedback overhead

  - FFS whether additional field is needed, at least for Option 2

- Within CSI part 2:

  - FFS further compression/omission/Sharing for PMI reporting among Single-TRP and NCJT hypotheses

[Mod] Minor change is suggested by Lenovo/MotM and Ericsson