3GPP TSG-RAN WG1 Meeting #104-bis-e	R1-2104117

# e-Meeting, April 12th – April 20th, 2021

# Title:	LS response on New Standardized 5QIs for 5G-AIS (Advanced Interactive Services)

# Response to:	S4-210283 (R1-2102308), LS on SA4 Reply LS on New Standardized 5QIs for 5G-AIS

# Release:	Rel-17

# Work Item:	FS_5GXR, FS_XRTraffic, 5G_AIS

# Source:	Qualcomm Incorporated

# To:	SA2, SA4

# Cc:	2

Contact Person:

Name: Eddy Kwon

E-mail Address: eddykwon@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

SA2 is discussing new standardized 5QIs for 5G AIS and sent an LS to RAN1 and SA4 [1].

SA4’s reply to the above LS includes standardised 5QIs newly proposed by SA4 and is asking RAN1 which of the proposed new standardised 5QIs can be supported by NG-RAN [2].

Standardised 5QIs newly proposed by SA4

[…]

Finally, it is relevant to understand that due to the periodic nature of the video traffic based on video frames with frame rates of for example 60 fps or 90 fps (i.e. every 17ms or 11ms) and certain video rate control mechanisms, the size of one frame is restricted, typically 30 kByte, but also up to possibly 100 kByte. This implies that the peak throughput requirement can be higher than the average throughput requirement for this traffic. This information may be useful for appropriate QoS settings, for example in order to define peak throughputs, and decisions on admitting this traffic.

Based on these observations, SA4 kindly invites SA2 to consider the following new parameters to the table, reflecting the SA4 vision on how to guarantee the media quality experience:

- Relax the latency targets (potentially increasing the radio capacity) in the downlink but support lower loss rates from 5ms and 10ms to 10ms and 20ms, respectively.

- Invert the loss rate requirements for uplink and downlink based on the considerations above to provide lower loss rates in downlink and accept higher ones in the uplink.

- Consider adding delay Critical GBR type to downlink with associated default MDBV values in the order of 50-100kB to support sufficiently high peak-to-average throughputs.

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New Value#1 | GBR (NOTE 1) | 25 | 105ms | 10-4 | N/A | 2000 ms | Interactive Service - visual content for cloud/edge/split rendering, (see TS 22.261 [2]) |
| New Value#2 |  | 25 | 2010ms | 10-4 | N/A | 2000 ms | Interactive Service - visual content for cloud/edge/split rendering, (see TS 22.261 [2]) |

Question: SA4 kindly asks RAN1 which of these proposed new standardised 5QIs can be supported by NG-RAN and provides feedback to SA2 and SA4.

Answer: In addition to the response LS from RAN1#104-e in February 2021 to SA2, SA4 (cc: RAN2) in R1-2101976 that already addresses new SA4 5QIs values #3 and #4, RAN1 would like to provide the following additional information in response to the LS from SA4: RAN1 has  finalized in RAN1#104bis-e traffic models and evaluation methodology for AR/VR/CG applications over NG-RAN for baseline scenarios and configurations, and is expecting to have additional evaluation results available in May RAN1#105-e that may be related to new SA4 5QIs values #1 and #2.

2. Actions:

To SA2 and SA4:

RAN1 respectfully asks SA2 and SA4 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #105-e 	10 – 27 May 2021		Electronic Meeting

TSG-RAN WG1 Meeting #106-e 	16 – 27 Aug 2021		Electronic Meeting

4. References

- R1-2100015	LS on New Standardized 5QIs for 5G-AIS (Advanced Interactive Services)	SA2, Tencent

- R1-2102308	Reply LS on New Standardized 5QIs for 5G-AIS (Advanced Interactive Services)	SA4, Qualcomm Incorporated.